Microsoft®
Memphis Developer's Release Device Driver Kit

HIDMINI
HID Class Sample Minidriver Design Notes

This document is for informational purposes only. MICROSOFT MAKES NO WARRANTIES, EXPRESS OR IMPLIED, IN THIS DOCUMENT.

Microsoft Corporation may have patents or pending patent applications, trademarks, copyrights, or other intellectual property rights covering subject matter in this document. The furnishing of this document does not give you any license to the patents, trademarks, copyrights, or other intellectual property rights except as expressly provided in any written license agreement from Microsoft Corporation.

Microsoft does not make any representation or warranty regarding specifications in this document or any product or item developed based on these specifications. Microsoft disclaims all express and implied warranties, including but not limited to the implied warranties or merchantability, fitness for a particular purpose and freedom from infringement. Without limiting the generality of the foregoing, Microsoft does not make any warranty of any kind that any item developed based on these specifications, or any portion of a specification, will not infringe any copyright, patent, trade secret or other intellectual property right of any person or entity in any country. It is your responsibility to seek licenses for such intellectual property rights where appropriate. Microsoft shall not be liable for any damages arising out of or in connection with the use of these specifications, including liability for lost profit, business interruption, or any other damages whatsoever. Some states do not allow the exclusion or limitation of liability or consequential or incidental damages; the above limitation may not apply to you.

ActiveMovie, ActiveVRML, ActiveX, BackOffice, Developer Studio, Direct3D, DirectDraw, DirectInput, DirectMusic, DirectPlay, DirectSound, DirectX, Microsoft, NetMeeting, OpenType, Visual Basic, Visual C++, Win32, Windows, and Windows NT are trademarks or registered trademarks of Microsoft Corporation in the United States and/or other countries. Other product and company names mentioned herein may be the trademarks of their respective owners.

© 1997 Microsoft Corporation. All rights reserved.

Revision History

| Version | Description |
| --- | --- |
| 3/21/97 | March 97 Developer's Release draft |

Contents

Overview	3

Background Notes	3

Notes on Writing HID Class Minidrivers	4

Anatomy of the Sample Minidriver	4

Event Sequence for the Sample Minidriver	7

Reference to Other Microsoft-provided Samples	8

# Overview

This Design Note documents HIDMINI, a sample Human Interface Device (HID) class minidriver provided by Microsoft in source-code form. The sample is intended to assist the user in writing their own HID class minidriver.

HIDMINI demonstrates how to use the lower edge interface of the HIDCLASS bus driver. A developer uses this interface to implement a minidriver for a non-Universal Serial Bus (USB) device that must appear in the system as a HID device. USB HID devices are supported by default in the Microsoft Windows “Memphis” operating system.

HIDMINI emulates a keyboard typing a fixed string. This reduces the amount of code in the sample by not talking to a physical device. The focus of this Design Note is on the HID minidriver interface. No attempt is made here to explain either the HID class driver above the minidriver, or how it connects to an underlying piece of hardware. The latter case in particular requires code specific to the actual physical device.

The source code in this sample is synthesized, allowing you to see how the minidriver works without the complications of servicing an actual device. After installing the WDM DDK, take the following steps to enable the sample:

- In the Windows folder, open the hidden sub-folder \Inf.

- Open the Hiddev.inf file in Notepad or a similar text editor.

- Locate the [USBHIDDevice.AddReg] section.

- One line in this section reads:

- HKR,,NTMPDriver,,”hidusb.sys”

- Change this line to read:

- HRK,,NTMPDriver,,”hidmini.sys”

- Close the file, saving changes, and then reboot the system.

- Plug in a real HID device.

(Note:  doing this disables the use of the real HID device, because it loads the hidmini.sys minidriver instead of hidusb.sys, which would normally create the linkage from the USB stack to hidclass.sys, the HID class driver.)

You can now make hidmini.sys “type” it’s string, character by character, each time you hit any of the keys CAPS LOCK, NUM LOCK, or SCROLL LOCK on your PS/2 keyboard.

# Background Notes

A minidriver is a hardware-specific DLL that uses a Microsoft-provided class driver to accomplish most actions through IOCTL IRPs and provides only device-specific controls. Under WDM, the minidriver registers each adapter with the class driver and the class driver creates a device object to represent each adapter that registers. The minidriver uses the class driver’s device object to make system calls.

Some design tradeoffs in the definition of the HID class driver and minidriver are outlined below, and apply to this sample.

- The minidriver does not create a device object, but “shares” the class driver’s device object as necessary. This choice was made to save system resources.

- The HID class driver creates one or more device objects for each physical device presented by the minidriver. Each of these objects represents a top level collection in the Report Descriptor. Operations on these top-level collection objects are converted into requests on the underlying device object. These collection objects are not visible to the minidriver, and the HID class driver converts all requests on the collections into operations on the underlying device.

- The HID class driver is responsible for understanding HID operations. The minidriver is responsible for conveying the semantics of these operations to the underlying implementation if it is not a native HID device.

- The HID class driver’s internal interface to the minidriver is primarily a set of internal device controls passed in Interrupt Request Packets (IRPs) between the class driver and the minidriver. System power and PNP IRPs are also passed to the minidriver.

- The Microsoft-provided HID class driver controls the request flow, passing an IRP to the appropriate minidriver when access to the hardware is required. Once the minidriver registers with the HID class driver it is passive and is called only by the HID class driver. Communication from the minidriver to the HID class driver is in the form of completed IRPs.

## Notes on Writing HID Class Minidrivers

- The following points apply to any HID class minidriver.

- Minidrivers must run on other processors in addition to Intel x86 chipsets, and therefore must be written in C or another high-level language. Minidrivers must not contain assembly-language source code.

- Some hardware requires that firmware be loaded before it will function. Minidrivers must embed this firmware in the minidriver binary rather than trying to load it from disk.

- Hardware resources (such as port or memory addresses) must be accessed through the macros found in the wdm.h file. For example, use the macro WRITE_PORT_USHORT to write to a word port, use the macro WRITE_PORT_BUFFER_USHORT to write a buffer to a port, and so forth.

- Minidrivers must not have unprotected spin loops. If the minidriver needs to “spin” waiting for the value of a port to change, the loop must be protected by a loop counter.

- Minidrivers that need to synchronously wait for a small period of time must use the KeStallExecutionProcessor routine. This routine shall be used sparingly.

- Minidrivers that need to wait for a long period of time (more than a few microseconds) for some operation to complete must either be interrupt-driven or use the StreamClassScheduleTimer function to schedule a timed callback. Minidrivers must not “spin” for a long time waiting for a status change, as this is deleterious to overall system performance.

- A minidriver that passes Write IRPs to a parent WDM driver must check the IRP for Pending Returned after completion. If the IRP is marked Pending Returned, the minidriver must call IoMarkIrpPending on the IRP before returning. This requirement is not illustrated in the sample.

# Anatomy of the Sample Minidriver

The following source files comprise the HIDMINI sample.

| File | Description |
| --- | --- |
| hid.c | Contains code for handling operations to the underlying device, such as Read, Write, Set and Get Feature. |
| hidmini.c | Contains code for basic installation and removal of the minidriver: DeviceEntry, AddDevice and driver unload functions. |
| ioctl.c | Contains code to recognize all legal IOCTLs from the HID class driver and dispatches them to the lower level routines in hid.c. |
| pnp.c | Contains code for handling PNP IRPs such as Start and Stop Device. |
| hidmini.h | Contains all of the minidriver global data structures and macros, and declarations of global data. |

These sample files are heavily commented, to aid the user in understanding their purpose and their relationship to the HID class driver.

The following table lists the interface functions in the various source code modules.

| Module | Minidriver DDI | Description |
| --- | --- | --- |
| hid.c | NTSTATUS HidMiniGetHIDDescriptor( IN PDEVICE_OBJECT DeviceObject, IN PIRP Irp ); | Obtains the HID descriptor structure from the device. |
| hid.c | NTSTATUS HidMiniGetReportDescriptor( IN PDEVICE_OBJECT DeviceObject, IN PIRP Irp ); | Obtains the Report descriptor structure from the device. |
| hid.c | VOID HidMiniIncrementPendingRequestCount( IN PDEVICE_EXTENSION DeviceExtension ); | Should be called whenever an IO operation is queued up. This keeps count of the number of outstanding IRPs, in case they might need to be aborted later. |
| hid.c | VOID HidMiniDecrementPendingRequestCount( IN PDEVICE_EXTENSION DeviceExtension ); | Should be called whenever an IO operation completes. When the count reaches zero, if the device is no longer in running state, it signals the all clear event. |
| hid.c | NTSTATUS HidMiniReadReport( IN PDEVICE_OBJECT DeviceObject, IN PIRP Irp ); | Accepts read HID report requests. |
| hid.c | NTSTATUS HidMiniReadCompletion( IN PDEVICE_OBJECT DeviceObject, IN PIRP Irp, IN PVOID Context ); | Completion routine for reading HID reports. |
| hid.c | NTSTATUS HidMiniWriteReport( IN PDEVICE_OBJECT DeviceObject, IN PIRP Irp ); | Accepts write HID report requests. |
| hid.c | NTSTATUS HidMiniWriteCompletion( IN PDEVICE_OBJECT DeviceObject, IN PIRP Irp, IN PVOID Context ); | Completion routine for writing HID reports. |
| hid.c | NTSTATUS HidMiniGetStringDescriptor( IN PDEVICE_OBJECT DeviceObject, IN PIRP Irp ); | Obtains the list of string descriptors, if any, from the device. |
| hid.c | NTSTATUS HidMiniOpenCollection( IN PDEVICE_OBJECT DeviceObject, IN PIRP Irp ); | Called in response to the opening of a top level collection device by a client of HID class. |
| hid.c | NTSTATUS HidMiniCloseCollection( IN PDEVICE_OBJECT DeviceObject, IN PIRP Irp ); | Called in response to the closing of a top level collection device by a client of HID class. |
| hidmini.c | NTSTATUS DriverEntry( IN PDRIVER_OBJECT DriverObject IN PUNICODE_STRING registryPath ); | Installable driver initialization entry point. This entry point is called directly by the I/O system. It must call the HID class registration entry point. |
| hidmini.c | NTSTATUS HidMiniAddDevice( IN PDRIVER_OBJECT DriverObject, IN PDEVICE_OBJECT FunctionalDeviceObject ); | Called by the HID class driver in response to an AddDevice IRP. The minidriver may use this opportunity to initialize data structures or the underlying device. |
| hidmini.c | VOID HidMiniUnload( IN PDRIVER_OBJECT DriverObject ); | Frees all the allocated resources and handle other issues in anticipation of the minidriver being unloaded. |
| ioctl.c | NTSTATUS HidMiniIoctl( IN PDEVICE_OBJECT DeviceObject, IN PIRP Irp ); | Processes the Control IRPs sent to this device. Serves as a dispatch routine for all of the functions in hid.c. |
| pnp.c | NTSTATUS HidMiniPnP( IN PDEVICE_OBJECT DeviceObject, IN PIRP Irp ); | Processes the PNP IRPs sent to this device. |
| pnp.c | NTSTATUS HidMiniStartDevice( IN PDEVICE_OBJECT DeviceObject ); | Provides the ability to do initialization before parent drivers. |
| pnp.c | NTSTATUS HidMiniStartCompletion( IN PDEVICE_OBJECT DeviceObject, IN PIRP Irp, IN PVOID Context ); | Provides the ability to do initialization after parent drivers. |
| pnp.c | NTSTATUS HidMiniInitDevice( IN PDEVICE_OBJECT DeviceObject ); | Called from HidMiniStartCompletion. Gets the device information and attempts to initialize a configuration for a device. If we cannot identify this as a valid HID device or configure the device, the Start Device function is failed. |
| pnp.c | NTSTATUS HidMiniStopDevice( IN PDEVICE_OBJECT DeviceObject ); | Provides the ability to perform tasks related to stopping a device before parent drivers act. |
| pnp.c | NTSTATUS HidMiniAbortPendingRequests( IN PDEVICE_OBJECT DeviceObject ); | Takes whatever actions are required to stop all outstanding IOs. |
| pnp.c | NTSTATUS HidMiniStopCompletion( IN PDEVICE_OBJECT DeviceObject, IN PIRP Irp, IN PVOID Context ); | Provides the ability to perform tasks related to stopping a device after parent drivers act. |
| pnp.c | NTSTATUS HidMiniRemoveDevice( IN PDEVICE_OBJECT DeviceObject ); | Allows the minidriver to free resources associated with its device extension. |
| pnp.c, | NTSTATUS HidMiniQueryIDCompletion( IN PDEVICE_OBJECT DeviceObject, IN PIRP Irp, IN PVOID Context ); | The QueryID IOCTL asks for the device’s PNP name. This may be used in constructing a corresponding HID device name. The name must be returned in an allocated buffer, as this buffer will be freed by hidclass.sys. |
| hid.c | NTSTATUS HidMiniGetDeviceAttributes( IN PDEVICE_OBJECT DeviceObject, IN PIRP Irp ); | Returns device attributes to hidclass.sys, so it can respond to clients. The Product ID (PID) and Vendor ID (VID) are required. |

# Event Sequence for the Sample Minidriver

The following steps briefly describe initialization, use, stopping, and unloading of the sample HID class minidriver. The sample minidriver routines mentioned here are described in greater detail in the comments of the sample source code.

- The adapter supported by the minidriver is detected by the PCI (or some other) enumerator. In the case of the sample minidriver, you can build HIDMINI.SYS and change the HIDDEV.INF file to load HIDMINI.SYS instead of HIDUSB.SYS. This results in a generic keyboard that is always typing something.

- Plug and Play loads the minidriver and calls its DriverEntry routine (in hidmini.c). Loading HIDMINI.SYS automatically loads HIDCLASS.SYS and HIDPARSE.SYS, because they are statically linked together. The minidriver initializes the dispatch routines of the driver object.

- DriverEntry calls the HID class driver’s HidRegisterMinidriver function. HidRegisterMinidriver takes the HID_MINIDRIVER_REGISTRATION structure as a parameter. This moves control of the driver object (and therefore the device object) from the minidriver to the HID class driver, and allows the class driver to verify that the minidriver will be using an interface compatible with its current version. The HID class driver changes the driver object dispatch routines to its own, but saves the minidriver’s routine addresses so that they can be called.

- From this point on, the HID class driver receives all requests for the device object. Eventually, the HID class driver’s AddDevice function will be called, and in turn the minidriver’s AddDevice (HidMiniAddDevice() in hidmini.c).

- PNP and Power IRPs are passed along to the minidriver. Note that support for Power IRP functionality is not included in the sample. The minidriver is responsible for performing any necessary operations and passing the IRPs to any parent drivers (see HidMiniPnP() in pnp.c). After the AddDevice, an IRP_MJ_PNP / IRP_MN_START_DEVICE request will come along. The following messages are associated with this activity:

- IRP_MN_START_DEVICE—Dispatched by HidMiniPnP() to HidMiniStartDevice() in pnp.c to verify and configure the device. The minidriver may configure the hardware and initialize structures (using HidMiniStartDevice() and HidMiniStartCompletion() in pnp.c).

- IRP_MN_QUERY_ID—Retrieves the underlying device’s name.  HIDCLASS.SYS may use this name to construct the device names for its corresponding top level collection devices.  The name must be returned in an allocated buffer that HIDCLASS.SYS can subsequently pass to ExFreePool().

- IRP_MN_STOP_DEVICE—Causes all operations on the device to be canceled (by calling HidMiniAbortPendingRequests() in pnp.c).

- IRP_MN_REMOVE_DEVICE—Verifies that all operations on the device are completed and then frees any data structures associated with the device.

- After the device is started, the HID class driver requests the HID and Report descriptors so that the collection structure of the device can be determined and top level collection devices can be created. Like all other communication from the class driver to the minidriver, these requests occur via IRP_MJ_INTERNAL_DEVICE_CONTROL IRPs. The following messages are associated with this activity:

- IOCTL_HID_GET_DEVICE_DESCRIPTOR—Returns the HID descriptor in the buffer pointed to by the IRP.

- IOCTL_HID_GET_REPORT_DESCRIPTOR—Returns the Report descriptor in the buffer point to the by the IRP.

- IOCTL_HID_GET_DEVICE_ATTRIBUTES—Returns the Vendor ID (VID) and Product ID (PID).  A client of HIDCLASS.SYS can request these items, perhaps to associate a HID collection with another device.

- Normal operations on the device, such as Read, Write, Get Feature and Set Feature, also go through IRP_MJ_INTERNAL_DEVICE_CONTROL IRPs. The following messages are associated with this activity:

- IOCTL_HID_ACTIVATE_DEVICE—Sent the first time any of the top level collections for a device is opened.

- IOCTL_HID_DEACTIVATE_DEVICE—Sent when the last open top level collection is closed on a device.

- IOCTL_HID_OPEN_COLLECTION—Sent when a top level collection is opened on a device.

- IOCTL_HID_CLOSE_COLLECTION—Sent when a top level collection is closed on a device.

- IOCTL_HID_READ_REPORT—Sent to read the next available HID report packet from the device.

- IOCTL_HID_WRITE_REPORT—Sent to write a HID report packet to the device.

- IOCTL_HID_GET_FEATURE—Sent to get a HID feature packet from the device.  Not supported in this sample, but might be useful in some situations.

- IOCTL_HID_SET_FEATURE—Sent to write a HID feature packet to the device. Not supported in this sample, but might be useful in some situations.

- The HID class driver supports some IOCTLs of its own. These are converted into IOCTLs that the minidriver can process and transmitted via IRP_MJ_INTERNAL_DEVICE_CONTROL IRPs. The following messages are associated with this activity:

- IOCTL_HID_GET_STRING—Sent to obtain the string descriptors, if any, from the device.

- IOCTL_HID_GET_PHYSICAL_DESCRIPTOR—Sent to obtain the designator descriptors, if any, from the device.

# Reference to Other Microsoft-provided Samples

The following Microsoft-supplied minidrivers are available as part of this Memphis DDK.

SCSAMPLE: Stream Class Sample Minidriver Design Notes documents the SCSAMPLE sample minidriver, which is a skeleton minidriver in the sense that it simply demonstrates the relationship between the Stream class driver and a generic Stream class minidriver. No device-specific issues are covered.

TESTCAP: Stream Class Video Capture Sample Minidriver Design Notes documents the TESTCAP sample minidriver. It requires no hardware to operate; it simply synthesizes and streams images. It implements a number of property sets. TESTCAP is designed to act as a testbed for further development, and attempts to illustrate the basic technique of writing a WDM Stream class video capture driver.

SONYDCR: IEEE 1394 Bus Class Sample Minidriver Design Notes documents the SONYDCR sample minidriver. SONYDCR is a skeleton driver for the Sony VX-1000 Camcorder. It demonstrates how a minidriver gets loaded, and how it submits requests to the Microsoft-provided IEEE 1394 bus driver.

I82930: USB Device Sample Minidriver Design Notes documents the I82930 sample minidriver. I82930 is a driver for the Intel 82930 USB test board and demonstrates how a minidriver gets loaded, and how it submits requests to the USB Driver Interface (USBDI).

WDMSB16: Stream Class Audio Sample Minidriver Design Notes documents the WDMSB16 sample minidriver. WDMSB16 demonstrates the basic path from Stream class to SB16 PCM audio device output.