Universal Serial Bus HID Usages Tables

- Version 0.9

June 11, 1997

For Review and discussion only
Document subject to revision and rejection

- Warning!!

This document is a draft.  It’s contents will change.  You may use these values as place holders if designing product but do not interpret these as final.  They are not!

- Contributors

- Brian M. Bates – ELO Touchsystems

- Robert Dezmelyk – LCS/Telegraphics

Robert Ingman – Microsoft Corporation

Steve McGowan – Intel (Editor)

Kenneth Ray – Microsoft Corporation

Steve Schumacher – LCS/Telegraphics

Mike Van Flandern – Microsoft Corporation

Remy Zimmerman – Logitech International

And many others.

- Revision History

| Revision | Issue Date | Comments |
| --- | --- | --- |
| 0.7a | April 22, 1996 | Initial draft. |
| 0.7c | August 14, 1996 | Third draft. |
| 0.7d | October 13, 1996 | 4th draft. |
| 0.7e | November 26, 1996 | 5th draft. Added VESA Monitor Pages. |
| 0.7g | February 7, 1997 | Updated per DWG meeting discussions |
| 0.7h | February 28, 1997 | Update – Deleted VESA Monitor Pages. |
| 0.7i | March 24, 1997 | Update |
| 0.8rc | April 1, 1997 | 0.8 Release Candidate |
| 0.9rc | May 8, 1997 | 0.9 Release Candidate |
| 0.9 | June 11, 1997 | 0.9 – Incorporate DWG changes. Posted for public comment |

USB Usage Tables
Copyright © 1996-1997, USB Implementers Forum

- All rights reserved.

- INTELLECTUAL PROPERTY DISCLAIMER

THIS SPECIFICATION IS PROVIDED “AS IS” WITH NO WARRANTIES WHATSOEVER INCLUDING ANY WARRANTY OF MERCHANTABILITY, FITNESS FOR ANY PARTICULAR PURPOSE, OR ANY WARRANTY OTHERWISE ARISING OUT OF ANY PROPOSAL, SPECIFICATION, OR SAMPLE.

A LICENSE IS HEREBY GRANTED TO REPRODUCE AND DISTRIBUTE THIS SPECIFICATION FOR INTERNAL USE ONLY.  NO OTHER LICENSE, EXPRESS OR IMPLIED, BY ESTOPPEL OR OTHERWISE, TO ANY OTHER INTELLECTUAL PROPERTY RIGHTS IS GRANTED OR INTENDED HEREBY.

AUTHORS OF THIS SPECIFICATION DISCLAIM ALL LIABILITY, INCLUDING LIABILITY FOR INFRINGEMENT OF PROPRIETARY RIGHTS, RELATING TO IMPLEMENTATION OF INFORMATION IN THIS SPECIFICATION.  AUTHORS OF THIS SPECIFICATION ALSO DO NOT WARRANT OR REPRESENT THAT SUCH IMPLEMENTATION(S) WILL NOT INFRINGE SUCH RIGHTS.

All product names are trademarks, registered trademarks, or service marks of their respective owners.

- Please send comments via electronic mail to usbdevice@fsp008.fm.intel.com

- Table of Contents

1.	Introduction	1

1.1	Scope	1

1.2	Purpose	1

1.3	Related Documents	1

1.4	Terms and Abbreviations	1

2.	Management Overview	3

3.	Usage Pages	4

3.1	Conventions	4

3.2	Unknown Usage Handling	5

3.2.1	Usages	5

3.2.2	Usages attached to Collections	5

3.3	Usages and Units	5

3.4	Usage Types	5

3.4.1	Alternate Types	10

3.5	System Controls	11

3.5.1	Keyboard	11

3.5.2	Mice	11

3.5.3	Joysticks	11

4.	Generic Desktop Page (0x01)	12

4.1	Application Usages	12

4.2	Axis Usages	13

4.3	Miscellaneous Controls	13

4.4	Vector Usages	14

4.5	System Controls	15

4.5.1	Power Controls	15

4.6	Buffered Bytes	16

5.	Simulation Controls Page (0x02)	17

5.1	Flight Simulation Devices	18

5.2	Automobile Simulation Devices	21

5.3	Other Simulation Devices	21

6.	VR Controls Page (0x03)	24

7.	Sport Controls Page (0x04)	26

7.1	Stick Devices	26

7.1.1	Golf Clubs	27

7.2	Exercise Machines	27

8.	Game Controls Page (0x05)	28

8.1	3D Game Controller	28

8.2	Pinball Device	29

8.3	Gun Device	30

9.	Key Code Page (0x07)	31

10.	LED Page (0x08)	32

11.	Button Page (0x09)	35

12.	Ordinal Page (0x0A)	37

13.	Telephony Device Page (0x0B)	38

13.1	Key Pad	39

13.2	Call Control	40

13.3	Speed Dial Controls	41

13.4	Voice Mail Controls	41

13.5	Locally Generated Tones	41

14.	Consumer Page (0x0C)	44

14.1	Numeric Key Pad	45

14.2	General Controls	46

14.3	Menu Controls	46

14.4	Display Controls	46

14.5	Selection Controls	47

14.6	Transport Controls	48

14.7	Search Controls	49

14.8	Audio Controls	50

14.9	Speed Controls	50

14.10	Home and Security Controls	50

15.	Digitizers (0x0D)	52

15.1	Digitizer Devices	52

15.2	Digitizer Transducer Collection Usages	53

15.3	Digitizer Report Field Usages	54

15.3.1	Digitizer-Specific Fields	54

15.3.2	Tilt Orientation	55

15.3.3	Azimuth-Altitude Orientation	55

15.4	Digitizer Switch Usages	55

16.	Unicode Page (0x10)	56

17.	Alphanumeric Display Page (0x14)	57

17.1	Flags	57

17.2	Display Control	58

17.3	Scrolling	58

17.4	Character Transfers	59

17.5	Display Status	60

17.6	Cursor Control	60

17.7	Font Loading	61

17.8	Display ASCII Character Set	62

18.	Monitor Pages	64

19.	Power Pages	64

20.	Bar Code Scanner Pages	64

21.	Point of Sale Pages	64

22.	Appendix A: Usage Examples	65

22.1	Volume Control	65

22.1.1	Up/Down Buttons	65

22.1.2	Knob	65

22.2	Tape Jog Wheel	65

22.3	Radio Buttons	66

22.3.1	Mechanically Linked	66

22.3.2	No Mechanical Linkage	66

22.4	Named Array Field	66

22.5	Multiple Instances of a Control	67

22.6	Multiple Instances of a Multi Mode LED	67

22.7	Desktop Tablet Example	69

22.8	A device with a Display	72

22.9	Remote Control	75

22.10	Telephone	77

22.11	Joystick	80

22.12	Game Pad	82

23.	Appendix B: Delimiter Example	83

24.	Appendix C: Physical Descriptor Example	85

- Table of Tables

Table 1: Usage Page Summary	4

Table 2: Usage Types (Controls)	6

Table 3: Usage Types (Data)	7

Table 4: Usage Types (Collection)	9

Table 5: Generic Desktop Page	12

Table 6: Simulation Controls Page	17

Table 7: VR Controls Page	24

Table 8: Sport Controls Page	26

Table 9: Game Controls Page	28

Table 10: LED Usage Page	32

Table 11: Button Usage Page	35

Table 12: Ordinal Usage Page	37

Table 13: Telephony Usage Page	38

Table 14: Consumer Usage Page	44

Table 15: Digitizer Page	52

Table 16: Alphanumeric Display Usage Page	57

Table 17: ASCII Display Character Set	63

- Table of Figures

Figure 1: LED Report fields	68

Figure 2: Example Digitizer 2-Button Stylus Input Report	71

Figure 3: Example Digitizer 16-Button Puck Input Report	71

Figure 4: Example Digitizer Pressure Stylus Input Report	71

Figure 5: Example Display Attributes Feature Report	74

Figure 6: Example Display Device Input Report	74

Figure 7: Example Display Device Display Position Feature Report	74

Figure 8: Example Display Device Display Data Feature Report	74

Figure 9: Example Display Device Font Load Output Report	75

Figure 10: Example Remote Control Input Report	76

Figure 11: Example Telephony Device Input Report	79

Figure 12: Example Telephony Device Output Report	79

Figure 13: Example Joystick Input Device Report	81

Figure 14: Example Game Pad Input Device Report	82

Figure 15: Joystick button layout	85

# Introduction

Usages are part of the HID Report descriptor and supply an application developer with information about what a control is actually measuring or reporting. In addition, a Usage tag can be used to indicate the vendor’s suggested use for a specific control or group of controls. While most of the items within a Report descriptors describe the format of the data—for example, three 8-bit fields—the Usage tags define what should be done with the data—for example, x, y, and z input. This feature allows a vendor to ensure that the user sees consistent function assignments to controls across applications. It is also the key feature within HID Report descriptors that allows systems or applications software to know the meaning of data items, or collections of data items so the data items can be correctly interpreted or routed to the systems or applications software that consumes them

## Scope

Many usages are defined within the USB HID Class Specification. This document is a  the most current and complete list of currently defined Usages, and is a superset of the list of usages in the USB HID Class Specification.

## Purpose

USB Usage tables define constants which can be interpreted by an application to ultimately identify the purpose and meaning of a data field in a HID report.

Usages are also used to define the meaning of groups of related data items. This is accomplished by the hierarchical assignment of Usage information to collections. Usages identify the purpose of a Collection, and the items within it.   The Input, Output, Feature, and/or Collection data items within the Collection item, each can be assigned a purpose with its own Usage item. Usages assigned to a collection apply to the items within the collection.

In some cases a usage applied to a Collection can redefine the meaning of the Usages it contains. An example of this is the Usage Selected Indicator on the LED page.

Usages are also used to specify the meaning of each element within an Array data item.

## Related Documents

Universal Serial Bus Specification, 1.0Version (also referred to as the USB Specification)

USB PC Legacy Compatibility Specification

Universal Serial Bus Device Class Definition for Human Interface Devices

## Terms and Abbreviations

Control	A control is used to operate, or regulate a particular aspect of a device. In this document a Control refers broadly to the physical entity on the device that the Usage identifies.

Field	The Input, Output, and Feature main items create a bit field in a report. The Report Size determines the field’s width and the associated usage determines field’s purpose. The offset of a field in a report is determined by the fields that are declared before it.

Array Field	The bit field created by an Input, Output, or Feature main item which is declared as an Array. An Array Field contains the index of a usage not the usage value itself.

Usage	Defines the purpose or meaning of an item.

Pad	If a field is marked as a Constant and there is no Usage associated with it will be treated as pad bits and ignored by host software.

Note: Fields created by main items that do not have usages attached to them will not necessarily be accessible by applications.

Applications is used in this document to refer to software programs that consume the data generated by the HID device, or control the HID device through Feature reports, or output data through Output reports. Applications may be games or other programs used by end users, or systems software components.

# Management Overview

This document provides lists of usages and their descriptions that significantly extend the list of usages provided in the HID specification. HID Usages communicates the intended function or meaning of a particular control. Usages provide a description of the data items in a HID device’s Input, Output, and Feature reports. The existence of a defined Usage does not guarantee that systems or applications software will recognize or utilize the data item. While Usages can be very powerful, there is a potential for misuse. The detail provided in this document will help minimize the misuse or misinterpretation of Usages when they are applied by a device developer.

Usages have been organized into Pages of related controls. Each Usage has an ID, Name and description. The Names were chosen as a convenient short form references. To avoid misleading interpretations that could be created by a quick look at the Name, it is very important that a developer review a Usage’s description in detail to ensure that it properly identifies the purpose of the control or device that the Usage is attached to.

In theory, a usage may be attached to any type of HID control, variable, array, collection, etc. In reality, usages only make sense when they are attached to particular controls and are used in certain ways. There is a relatively small set of Usage Types that have been defined to help the applications software developer better understand what to expect when they find a particular usage. In this document each Usage has a Usage Type associated with it. They identify the item types, flag settings and bit fields organizations that the developer should expect when they find the usage.

Usages are also used to identify functional devices as a whole, thus providing an easy method for an application to identify devices which provide functions of interest. These usages are found attached to Application Collections that are wrapped around all the items that describe a particular functional device, or a function in a complex device. Generally an application will query the HID driver for all application collection usages that it knows pertain to it. For instance, a gaming device API will look for Joysticks and Game Pads. While a system mouse driver may look for Mouse, Digitizer Tablet and Touch screen usages.

As a general rule the usages selected by a device developer should be specific enough to dissuade inappropriate use by applications while remaining general enough to allow applications to take advantage of their features if they can. If uncertain, favoring the more general usage will encourage broader application support for you device. An alternative is to use Delimited Sets to define multiple usages associated with a single control or a device.

Some usage pages that are in the HID Specification are also found in this document. They are included here because additional text has been provided to clarify how the Usages are to be used , new usages have been added to the page, or both. No changes have been made to the Usage values assigned in the HID specification.

# Usage Pages

The following is a list of currently defined Usage Pages.

- Table 1: Usage Page Summary

| Page ID | Page Name | Page ID | Page Name |
| --- | --- | --- | --- |
| 00 | Undefined | 0C | Consumer |
| 01 | Generic Desktop Controls | 0D | Digitizer |
| 02 | Simulation Controls | 0E-0F | Reserved |
| 03 | VR Controls | 10 | Unicode |
| 04 | Sport Controls | 11-13 | Reserved |
| 05 | Game Controls | 14 | Alphanumeric Display |
| 06 | Reserved | 15-7F | Reserved |
| 07 | Keyboard/Keypad Keys | 80-83 | Monitor Pages |
| 08 | LEDs | 84-87 | Power Pages |
| 09 | Button | 88-8B | Bar Code Scanner Pages |
| 0A | Ordinal | 8C-8F | Point of Sale Pages |
| 0B | Telephony | 90-FEFF | Reserved |
|  |  | FF00-FFFF | Vendor Defined |

The bold Usage definitions below identify a collection. The non-bold definitions are specific features related to a device that would be applied to individual controls that generate data. In many cases specific usages can be used by a number of device types.

Note: In many cases the coordinate system assumes that the values can vary positive and negative around zero (0).

## Conventions

Usage ID 0 should always be reserved.

Usage IDs 1 through 0x1F are reserved for “top level” collections. These Usage IDs are not necessarily application level but are used to identify general device types.

Usage Page values are limited to 16 bits.

Usage ID values are limited to 16 bits.

Usages are 32 bit identifiers, where: the high order 16 bits represents the Usage Page and the low order 16 bits represents the Usage ID. To allow more compact Report Descriptors Usage Page items can be declared to specify the high order bits of the Usage item and the Usage items can declare only the ID portion of the usage. If the bSize field of the Usage item equals 1 or 2 then the (1 or 2 byte) data portion of the item is interpreted as a Usage ID. If the bSize field equals3 then the (4 byte) data portion of the item is interpreted as a Usage Page (bits 16-31) and Usage ID (bits 0-15). This interpretation of Usages applies to Usage, Usage Minimum, and Usage Maximum items. The notation for a 32-bit Usage  (sometimes called an Extended Usage) in the examples is Usage(Usage Page: Usage ID).

## Unknown Usage Handling

### Usages

If a usage is unknown to an application then the application should ignore it.

### Usages attached to Collections

If the usage attached to a collection is unknown to an application then the application should ignore the collection and all usages contained in the collection. Collections can be used to modify the meaning of the usages that it contains, therefore “known” usages within an unknown collection may not represent their original meaning. An example of this is the Usage Selected Indicator on the LED page.

Commonly system software interprets the Usages associated with top level Application Collections as a key to invoke the drivers associated with the collections or if a Usage is unknown the operating system may ask the user to load the appropriate driver.

## Usages and Units

For usages that declare data items as a measurement of time, distance, force, etc. an application must look at the units to properly interpret the value defined by a Usage. The exceptions are if 1) a Usage specifically declares Units as optional or 2) the Usage description defines the Units that the value will be presented in. If Units are Optional or set to None (have not been declared) then an application can assume the Usage represents a dimensionless value. Any application that ignores Units will do so at it's own risk.

A usage that declares itself to be a measurement of time would specify whether it was seconds or milliseconds by declaring Units and Unit Exponent prior to the respective Main Item declaration. An example is LEDs : Flash On Time – “Duration that the indicator is illuminated in flash mode”. The duration would be qualified by the values of Units and Unit Exponent.

## Usage Types

Usages define a wide variety of device features, However, the way the data that they generate is treated by an application falls into a relatively small set of  categories. This section provides descriptions of the often used types of usages, primarily to save redundant text throughout this document. This list is an not exhaustive list of the possible Usage Types. Individual Usage pages can declare their own Usage Types.

Each Usage Type describes how an application should treat the bit field generated by the Main Item that the usage is attached to.

Usage Type names are followed by an abbreviation that is used in the detailed Usage description to identify the default type of a usage. In some cases Usage Types do not apply and the detailed description will identify how the Usage is to be interpreted.

There are three basic types of information that are described by Usages: Controls, Collections and Data. In this context, Controls are identified with the state of a device (on/off, enable/disable, etc.), Collections group related Controls and Data together, and Data comprises the remaining information that is passed between a device and the host.

The following table summarizes the control related Usage Types.

- Table 2: Usage Types (Controls)

| Type | Log Min | Log Max | Flags | Signal | Operation |
| --- | --- | --- | --- | --- | --- |
| Linear Control (LC) | -1 | 1 | Relative, | Edge | A 1 will increment the value. A –1 will decrement the value. |
|  | 0 | n | Absolute | N/A | The value reported by the control is used directly by the host. |
| On/Off (OOC) | -1 | 1 | Relative, No Preferred | Edge | A 1 will assert an On condition. A –1 will assert an Off condition. |
|  | 0 | 1 | Relative, Preferred | Edge | A 0 to 1 transition will toggle the current On/Off state. |
|  | 0 | 1 | Absolute, No Preferred | Level | A value of 1 asserts an On condition and a value of 0 asserts an Off condition. |
| Momen-tary (MC) | 0 | 1 | Absolute, Preferred | Level | A value of 1 asserts a condition and a value of 0 deasserts the condition. |
| One shot (OSC) | 0 | 1 | Relative, Preferred | Edge | A 0 to 1 transition triggers an event. A 1 to 0 transition must occur before another event will occur. |
| Re-trigger (RTC) | 0 | 1 | Absolute, Preferred | Level | A value of 1 triggers an event. When an event completes, if value is 1 then the event will start again. |

Linear Control (LC)
There are many cases where a control of a linear value is implemented as a pair of increment / decrement buttons or a linear control like a knob or a slide.

When implemented as an increment / decrement control (2 buttons) the device must be translated into a single value of two or more bits. these two buttons into a signed value and declare the control as a Relative main item. Where: -1 decrements the value and +1 increments. No change occurs when 0 is asserted. The control can be implemented with more a +/- 1 resolution. In these cases a Report Size larger than 2 will be declared.

When implemented as a linear control the device must declare the control as a Absolute main item. See the Volume Control example.

On/Off Control (OOC)
An On/Off control can be implemented as a 2 buttons: On and Off, a single button where each time the button is pressed the On/Off state toggles, or as a toggle switch where the On/Off state is maintained mechanically by the switch. To encode these 3 implementations so that they are recognizable for what the are the following rules will be applied.

1) 2 buttons - The two buttons are encoded into a 2-bit signed value and declare the control as a Relative, No Preferred main item with Logical Minimum and Maximum of –1 and 1, respectively. Where: the transition from0 to -1 generates an Off condition and the transition from 0 to +1 generates an On condition. No change occurs when 0 is asserted.

2) Single throw momentary switch (a button) - The single button is encoded into a 1-bit unsigned value and declare the control as an Relative, Preferred main item with a Logical Minimum and Maximum of 0 and 1, respectively. Where: a transition from 0 to 1 toggles a current On/Off state. No change occurs on the 1 to 0 transition.

3) A single throw switch (Toggle switch) - This control is encoded into a 1-bit unsigned value and declare the control as an Absolute, No Preferred main item with a Logical Minimum and Maximum of 0 and 1, respectively. Where: the assertion of a 1 generates an On condition and 0 generates an Off condition.

Momentary Control (MC)
A Momentary Control is a basic push button. Momentary buttons are encoded into a 1-bit value and declare the control as a Absolute, Preferred main item with a Logical Minimum and Maximum of 0 and 1, respectively. Where: 1 is an asserted condition and 0 generates an non-asserted condition. An example is a mouse button.

One Shot Control (OSC)
A One Shot Control is a push button that triggers a single event or action. One Shot Control are encoded into a 1-bit value and declare the control as a Relative, Preferred main item with a Logical Minimum and Maximum of 0 and 1, respectively. Where: a 0 to 1 transition initiates an event. Nothing occurs on a 1 to 0 transition but they are required before another event can occur. An example is degauss.

Re-Trigger Control (RTC)
A Re-Trigger Control is a push button that triggers repeating event events as long as it is asserted. Re-Trigger Control are encoded into a 1-bit value and declare the control as a Absolute, Preferred main item with a Logical Minimum and Maximum of 0 and 1, respectively. Where: a 0 to 1 transition initiates the first event. When each event terminates if the control is still asserted (1) then another event will occur. An example is an auto-repeat fire button.

The following table summarizes the data related Usage Types.

- Table 3: Usage Types (Data)

| Type | Flags | Description |
| --- | --- | --- |
| Selector (Sel) | Array | Contained in a Named Array |
| Static Value (SV) | Constant, Variable, Absolute | A read only, multi-bit value. |
| Static Flag (SF) | Constant, Variable, Absolute | A read only single bit value. |
| Dynamic Value (DV) | Data, Variable, Absolute | A read/write, multi-bit value. |
| Dynamic Flag (DF) | Data, Variable, Absolute | A read/write only single bit value. |

Selector (Sel)
Selectors come in three forms: “one selection of a set”, “n selections of a set” or “any selection of a set”.

Radio buttons are a mechanically linked set of buttons where one selection is always valid. This is a perfect example of the “one selection of a set” form. A Radio button set is defined by a Main Item with the Array flag set and the Report Count set to 1. The index returned in the Array Field corresponds to the pressed button (or selection). A usage must be declared for each selection. The Array Field never returns an index of NULL value because one usage is always valid.

In the “n selections of a set” form, more than one selection (button) can be valid at a time. Multiple selections can be returned to the system they  at one time in a multi-byte array. The “n selections of a set” form is defined by a Main Item with the Array flag set and the Report Count set to n, where n is the number of selections that can be reported in a single report.


In the case of “any selection of a set” the control is implemented as a set of bit fields where each bit represents a single selection. This control is defined by a Main Item with the Variable flag set and the Report Size equals 1. The Report Count will be equal to the number of selections in the set.

Selectors therefore can be implemented in a number of ways. Array[1] (one selection of a set), Array[n] (n selections of a set), or bitmap (any selection of a set).
Optionally, the Array Field or set can be named by wrapping a set of Selectors in a Logical Collection with a Usage attached to it. See Named Array (NAry) discussion below.

Static Value (SV)
Static values are used to declare a fixed features in a device. They are defined as constants and treated as read-only information. Therefore, asserting this field in a SetReportDescriptor command has no effect on a static value.

Static Flag (SF)
Static flags are used to declare the existence of a fixed feature in a device. If a Static Flag usage is found in a Report Descriptor then the field must be read to determine whether the feature identified by the flag exists. A value of 1 indicates existence, 0 indicates non-existence. The absence of a Static Flag usage implies that the flag is false or the feature defined by the flag is not supported by the device. A Static Flag must be declared as a Constant. To be accessible by applications A Static Flag must have a Usage assigned to it.

Static Flags are typically declared in a Feature report as a single bit field, where the value is always read as ‘1’.  Attempting to modify a this field in a SetReportDescriptor command has no effect on a Static Flag.

Dynamic Flag (DF)
Dynamic Flags are used to declare the existence of a host controllable feature in a device. The absence of a Dynamic Flag usage implies that the flag is false or the feature defined by the flag is not supported by the device.

Dynamic Flags are typically declared in a report as a single bit field, where a value of ‘1’ returned by the device indicates that the feature is enabled. The assertion of a ‘1’ by the host will cause the feature to be evoked and the assertion of a ‘0’ indicates that the feature is to be disabled or ignored if the feature is a one time event (like Degauss or Clear Display). A Dynamic Flag main item must be declared as Data.

Dynamic Value (DV)
An n-bit field that contains a value associated with a control. The associated main item will have the Data and Variable flags set. A Dynamic Value main item must be declared as Data.

Note: More advanced devices may allow a Usage declared as a Static type to be Dynamic. Always check the Constant/Data flag in the Input, Output or Feature main item.

The following table summarizes the data related Usage Types.

- Table 4: Usage Types (Collection)

| Type | Collection Type | Definition |
| --- | --- | --- |
| Named Array (NAry) | Logical | A collection that encompasses an array definition, naming the array set or the field created by the array. |
| Application Collection (CA) | Application | Applies a name to a top level collection which the operating system uses to identify a device and spawn a driver. |
| Logical Collection (CL) | Logical | A logical collection of items. |
| Physical Collection (CP) | Physical | A physical collection of items. |
| Usage Switch (US) | Logical | A Usage Switch modifies the purpose or function of the usages (controls) that it contains. |

Named Array (NAry)
To simplify application finding a set of “selectors”, whether defined as an Array Field or a bitmap, the set of selectors can be named by wrapping them in a Logical Collection and applying a Usage to the collection. Usages used in this way are referred to as Named Array usages. See the Named Array Field example in the Appendix.

Collection Application (CA)
Usages only used for Application level Collections. An Application Collection identifies a HID device, or a functional subset of a complex device. An operating system uses the Usage associated with this collection to link the device to its controlling application or driver. Common examples are a keyboard or mouse. A keyboard with an integrated pointing device could contain two different application collections. Note: Data reports cannot span application collections.

Collection Logical (CL)
A usage applied to a Logical Collection. Logical collections can be used to further define the purpose of the items or controls that they contain.

Collection Physical (CP)
A usage applied to a Physical Collection, usually a collection of axes. A physical collection is used for a set of data items that represent data points collected at one geometric point.  This is useful for sensing devices which may need to associate sets of measured or sensed data with a single point.  It does not indicate that a set of data values comes from one device, such as a keyboard.  In the case of device which reports the position of multiple sensors, physical collections are used to show which data comes from each separate sensor.

Usage Switch (US)
A usage applied to a Logical Collection that modifies the purpose of the Usages that are contained in that collection. An example is indicators. To avoid having to define an Indicator Usage for every control that could possibly use an indicator (i.e. Play/Play Indicator, etc.) a Usage Switch collection can be wrapped around a usage (Play) to create a indicator for the same function. Usage Switches often modify the type of the contained usage as well.

### Alternate Types

Usage Types are a guide not the rule. The flags, Logical Min and Max values, and other main item attributes must be evaluated by applications and system software to determine the true purpose, meaning or interpretation of a control.

There are many cases where a Usage may take on the attributes of a Usage Type other than it’s default type. The alternate type may be declared by a collection that the usage is found in or implied by the way it is declared in a Report Descriptor. For instance, the Usage In Use Indicator usage from the LED page is an example of an alternate Usage Type being applied to a Usage. When a usage is wrapped in a Usage In Use Indicator collection it becomes an On/Off Control.

There are other cases where a usage can be declared as either a Static Value or a Dynamic Value. i.e., a screen saver,  where on one device the Screen Saver Delay is fixed and on another it is variable. The same thing can happen with Static Flags and Dynamic Flags.

Another example is where a Usage is declared as either a On/Off Control or a or a Selector. A device that can support a variety of operational modes will declare individual bits as On/Off Controls to identify which modes are enabled. However when the device is running only one mode will be in effect at a time. The device would then declare the same usage as a Selector and report this in a Named Array field to identify the mode associated with the current data. i.e. A tape transport could have 3 states: Stopped, Paused and Playing. This could be implemented as 3 individual bits where only one bit is true at a time or as a 2 bit field where 0 = Stopped, 1 = Paused and 3 = Playing.

## System Controls

Applications look at the usage applied to top level application collections to identify devices. Systems software which support keyboards, mice and joysticks follow the same conventions. If a device vendor wants their device to be recognized by the system software as one of these devices then they must follow the following conventions:

### Keyboard

Typical system software will search for (top-level) Application Collections tagged with either a Keyboard or a Keypad Usage. When found the usages generated by these collections will be treated as standard system keyboard input. All devices that use these declarations will have their output routed to the same destination. i.e. typing on any device will effect the current application with focus.

### Mice

Typical system software will search for (top-level) Application Collections tagged with either a Mouse or a Pointer Usage. When found the usages generated by these collections will be treated as standard system pointer input. All devices that use these declarations will have their output routed to the same destination. i.e. moving any mouse will effect the system pointer.

### Joysticks

Typical system software will search for (top-level) Application Collections tagged with either a Joystick or a Game Pad Usage. When found the usages generated by these collections will be treated as standard system joystick (gaming device) input. Devices that use these declarations will have their output routed to separate destinations allowing multi-player applications.

# Generic Desktop Page (0x01)

- Table 5: Generic Desktop Page

| Usage ID | Usage Name | Usage ID | Usage Name |
| --- | --- | --- | --- |
| 00 | Undefined | 40 | Vx |
| 01 | Pointer | 41 | Vy |
| 02 | Mouse | 42 | Vz |
| 03 | Reserved | 43 | Vbrx |
| 04 | Joystick | 44 | Vbry |
| 05 | Game Pad | 45 | Vbrx |
| 06 | Keyboard | 46 | Vno |
| 07 | Keypad | 47-7F | Reserved |
| 08-2F | Reserved | 80 | System Control |
| 30 | X | 81 | System Power Down |
| 31 | Y | 82 | System Sleep |
| 32 | Z | 83 | System Wake Up |
| 33 | Rx | 84 | System Context Menu |
| 34 | Ry | 85 | System Main Menu |
| 35 | Rz | 86 | System App Menu |
| 36 | Slider | 87 | System Help Menu |
| 37 | Dial | 88 | System Menu Exit |
| 38 | Wheel | 89 | System Menu Select |
| 39 | Hat switch | 8A | System Menu Right |
| 3A | Counted Buffer | 8B | System Menu Left |
| 3B | Byte Count | 8C | System Menu Up |
| 3C | Motion Wakeup | 8D | System Menu Down |
| 3D-3F | Reserved | 8E-FFFF | Reserved |

## Application Usages

Pointer	CL – A collection of axes that generates a value to direct, indicate, or point user intentions to an application.

Mouse	CA – A hand-held, button-activated input device that when rolled along a flat surface directs an indicator to move correspondingly about a computer screen, allowing the operator to move the indicator freely, as to select operations or manipulate text or graphics. A mouse typically consists of 2 axes (X and Y) and 1, 2 or 3 buttons.

Joystick	CA – A manual control or cursor device. A joystick minimally consists of 2 variable axes (X and Y) and 2 buttons. A joystick is typically a rotational motion sensor, however for legacy reasons it is defined using linear axes.

Classically, joysticks drivers apply their own scaling to values returned from a joystick. i.e. the driver simply linearizes and translates the range of values generated by the stick into normalized values between 0 and 64K, where 32K is centered. The application (game) then interprets the normalized values as necessary. Because of this, joysticks normally do not declare Units or Physical Minimum and Physical Maximum values for their axes. Depending on the driver they may be ignored if they are declared.

Game Pad	CA – A manual control or cursor device. A game pad minimally consists of a thumb activated 2 axes (X and Y)  rocker switch and 4 buttons. The rocker switch consists of 4 contact closures for up, down, right and left.

Keyboard	CA – Primary computer input device. A Keyboard minimally consists of 103 buttons as defined by the Boot Keyboard definition

Keypad	CA – Any Keyboard configuration that does not meet the minimum requirements of the Boot Keyboard. Often refers to a supplementary calculator style keyboard.

## Axis Usages

For X, Y, Z, Rx, Ry and Rz the declaration of Units are optional. If Units are None or not declared then these values should be considered as dimensionless.

X	DV – This usage represents a linear translation in the X direction. Report values should increase as the control’s position is moved from left to right.

Y	DV – This usage represents a linear translation in the Y direction. Report values should increase as the control’s position is moved from far to near.

Z	DV – This usage represents a linear translation in the Z direction. Report values should increase as the control’s position is moved from high to low (Z).

Rx	DV – This usage represents a rotation about the X axis. Angular position report values follow the right hand rule.

Ry	DV – This usage represents a rotation about the Y axis. Angular position report values follow the right hand rule.

Rz	DV – This usage represents a rotation about the Z axis. Angular position report values follow the right hand rule.

## Miscellaneous Controls

Slider	DV – A linear control for generating a variable value, normally in the form of a thumb slide in a slot. Report values should increase as controls are moved from near to far.

Dial	DV – A rotary control for generating a variable value, normally in the form of a knob spun by the index finger and thumb. Report values should increase as controls are spun clockwise.

Wheel	DV – A rotary control for generating a variable value, normally rolled, unlike a dial. Report values should increase as controls are rolled forward, away from the user.

Hat Switch	DV – A Hat Switch is a specialized mechanical configuration of switches generating a variable value with a null state. The switches are arranged around a spring loaded knob. When the knob is tilted in the direction of a switch it’s contacts are  closed. A typical example is 4 switches that are capable of generating information about 4 possible directions that the knob can be tilted. Intermediate positions can also be decoded if the hardware allows 2 switches to be reported simultaneously.

Motion Wakeup
OSC – This usage enables the generation of a USB Remote Wakeup when motion is detected by the device. i.e. a mouse may generate a Remote Wakeup when a button is pressed or when it is moved. For some implementations a laptop user may want to disable the wakeup on motion because it draws more power.

## Vector Usages

For the usages Vx, Vy, Vz, Vbrx, Vbry, Vbrx and Vno Units are always required to determine the meaning of the vector. Rotational vectors are also identified by the Units. These usages are used when declaring velocity, acceleration, force, electric field, etc. vectors in the respective direction and frame of reference.

Vx	DV – This usage represents a vector in the X direction. Report values should increase as the vector increases in the positive X direction (from left to right). Negative values represent vectors in the negative X direction.

Vy	DV – This usage represents a vector in the Y direction. Report values should increase as the vector increases in the positive Y direction (from far to near). Negative values represent vectors in the negative Y direction.

Vz	DV – This usage represents a vector in the Z direction. Report values should increase as the vector increases in the positive Z direction (from high to low). Negative values represent vectors in the negative Z direction.

Vbrx	DV - This usage represents a vector in the X direction relative to the body of an object. Report values should increase as the vector increases in the positive X direction (forward). Negative values represent vectors in the negative X direction. X is the “forward” axis for an object.

Vbry	DV - This usage represents a vector in the Y direction relative to the body of an object. Report values should increase as the vector increases in the positive Y direction (to the right from an observer facing forward on the object). Negative values represent vectors in the negative Y direction.

Vbrz	DV - This usage represents a vector in the Z direction relative to the body of an object. Report values should increase as the vector increases in the positive Z direction (down from an observer facing forward on the object). Negative values represent vectors in the negative Z direction.

Vno	DV- This usage represents a non oriented vector or value. The units define a physical measurement not related to a specific axis or orientation. An example would be pressure or temperature.

## System Controls

System controls are a special category of usages that effect the system as a whole. They are pulled together in a System Control collection to make them easy to identify for system software.

Keyboard page usages found in System Control collection will be interpreted in a operating system specific manner.

System Control
CA – A application level collection that contains System software specific usages. An operating system will search specifically for this collection for those controls that effect the system globally.

System Context Menu 
OSC - Evoke a context sensitive menu. (right mouse click).

System Main Menu
OSC - Evokes OS main level selection (Start) menu.

System App Menu 
OSC - Brings up an application specific menu.

System Menu Help
OSC - Bring up the help menu.

System Menu Exit	
OSC - Exits a menu.

System Menu Select 
OSC - Selects a menu item.

System Menu Right 
RTC - Menu select right.

System Menu Left 
RTC - Menu select left.

System Menu Up 
RTC - Menu select up.

System Menu Down 
RTC - Menu select down.

### Power Controls

The Power Controls can step the system through states: Full Power, Low Power, and Power Down. The state diagram is shown below.

Low

Power

Full

Power

Power

Down

System Power Down

System Wake Up

System Sleep

System Power Down
OSC - This usage is asserted when the intended action is to  initiate system wide power down now from Full Power or Sleep states.

System Sleep
OSC - This usage is asserted when the intended action is to  initiate system wide low power mode now. If the system is already in the Low Power state then there is no effect.

System Wake Up
OSC - This usage is asserted when the intended action is to initiate system wide Full Power state now. If the system is already in the Full Power then there is no effect.

## Buffered Bytes

The following usages provide a standard way of defining the operation of a buffered byte field where the number of valid bytes in the field is less than the total number of bytes in the field and the vendor does not define a NoOp value to mark unused bytes.

When declaring a buffered byte field the global item Report Size should always be set to 8 (for byte cells), and the Report Count should be equal to the maximum size of the buffer to be transferred.

Counted Buffer
CL – This usage is used with buffered byte data to indicate the number of valid bytes in the buffered byte field. This collection always contains two field declarations: Byte Count and a usage that names the “purpose” of the buffered byte field. The main item associated with the “purpose” usage will always have the "Buffered Bytes" attribute set.

Byte Count	DV - Defines a report field that indicates the number of meaningful data bytes in an associated buffered byte field.

# Simulation Controls Page (0x02)

This section provides detailed descriptions of the usages employed by Simulation Devices.

- Table 6: Simulation Controls Page

| Usage ID | Usage Name | Usage ID | Usage Name |
| --- | --- | --- | --- |
| 00 | Undefined | B8 | Elevator |
| 01 | Flight Simulation Device | B9 | Elevator Trim |
| 02 | Automobile Simulation Device | BA | Rudder |
| 03 | Tank Simulation Device | BB | Throttle |
| 04 | Spaceship Simulation Device | BC | Flight Communication |
| 05 | Submarine Simulation Device | BD | Flare Release |
| 06 | Sailing Simulation Device | BE | Landing Gear |
| 07 | Motorcycle Simulation Device | BF | Toe Brake |
| 08 | Sports Simulation Device | C0 | Trigger |
| 09 | Airplane Simulation Device | C1 | Weapons Arm |
| 0A | Helicopter Simulation Device | C2 | Weapons Select |
| 0B | Magic Carpet Simulation Device | C3 | Wing Flaps |
| 0C | Bicycle | C4 | Accelerator |
| 0D - 1F | reserved | C5 | Brake |
| 20 | Flight Control Stick | C6 | Clutch |
| 21 | Flight Stick | C7 | Shifter |
| 22 | Cyclic Control | C8 | Steering |
| 23 | Cyclic Trim | C9 | Turret Direction |
| 24 | Flight Yoke | CA | Barrel Elevation |
| 25 | Track Control | CB | Dive Plane |
| 26 | Driving Control | CC | Ballast |
| 27 - CF | reserved | CD | Bicycle Crank |
| B0 | Aileron | CE | Handle Bars |
| B1 | Aileron Trim | CF | Front Brake |
| B2 | Anti-Torque Control | D0 | Rear Brake |
| B3 | Auto-pilot enable | D1-FFFF | reserved |
| B4 | Chaff Release |  |  |
| B5 | Collective Control |  |  |
| B6 | Dive Brake |  |  |
| B7 | Electronic Counter Measures |  |  |

## Flight Simulation Devices

Flight Simulation Device
CA - This usage definition allows a device to be generally classified as one that uses standard controls found on an airplane.

Aileron	DV - An aileron is one of two movable flaps on the wings of an airplane that can be used to control the plane's rolling and banking movements.  In the zero position the ailerons are centered, positive values will move the right aileron up and the left aileron down, negative values will have the opposite effect on the ailerons.

Aileron Trim
DV - Aileron Trim allows fine adjustment of the Aileron position. The zero position is the nominal position, positive values will move the right aileron up and the left aileron down, negative values will have the opposite effect on the ailerons.

Anti-Torque Control
DV - This control mechanically behaves the same as rudder pedals, as one is pushed forward the other pushes back. In a helicopter this controls the pitch of the tail blade to spin the helicopter in place. The zero position is centered, positive values rotate right, and negative values rotate left.

Autopilot Enable
OOC - A control for enabling or disabling an airplanes autopilot. This should be a toggle switch but it is typically implemented as push-button.

Chaff Release
OSC - Chaff is strips of metal, foil, or glass fiber with a metal content, cut into various lengths and having varying frequency responses, that are used to reflect electromagnetic energy as a radar countermeasure. These materials, usually dropped from aircraft, also can be deployed from shells or rockets. Typically this a Push Button that initiates a release of a fixed amount of material.

Collective Control
DV - This control is specifically for a helicopter. It controls the vertical acceleration or lift of the helicopter. The zero position is centered (level flight), positive values accelerate up, and negative values accelerate down.

Cyclic Control
CP - This control is specifically for a helicopter. A cyclic control is a stick between the pilots legs that moves in two axes. It controls the swash plate which in turn controls horizontal acceleration of the helicopter. The zero position is centered, positive Y values accelerate forward, and negative Y values backwards. Positive X values accelerate right, and negative X values accelerate left. This collection will contain X and Y axes.

Cyclic Trim
CP - This control is specifically for a helicopter. Cyclic Trim allows fine adjustment of the Cyclic position in 2 dimensions. The zero position is the nominal position, positive values adjust the baseline acceleration right or forward, and negative values adjust the baseline acceleration left or backwards, respectively. This collection will contain X and Y axes.

Dive Brake	DV - A flap that can be extended on an aircraft to increase drag and reduce the speed of descent. A lever that generates a dimensionless value between no braking (0) and full braking.

Electronic Counter Measures
OOC - A push-button that allows enables Electronic Counter Measures. This is typically active radar jamming, however Chaff (radar) or Flare (infrared) can be invoked.

Elevator	DV - A movable control surface, usually attached to the horizontal stabilizer of an aircraft, that is used to produce motion up or down. The zero position is centered, positive values are elevator up, and negative values are elevator down.

Elevator Trim
DV - Elevator Trim allows fine adjustment of the Elevator position. The zero position is the nominal position, positive values are elevator offset up, and negative values are elevator offset down.

Flight Communications
OOC - In combat aircraft a communication (comm) button is usually positioned under the index finger. Typically this a two position push-button, where the first position enables communications with the crew and the second position enables the transmitter for communication external to the plane.

Flare Release
OSC - A flare is a device that produces a bright light for signaling, illumination, identification, or heat for infrared missile counter measures. Typically this a Push Button that release a fixed number of flares.

Flight Control Stick
CA - A Flight Control Stick controls the Pitch and Roll of an airplane. It looks like a joystick. The stick may be pushed forward or pulled back to move the tail elevator, down and up, respectively;  pushing forward causes the plane to nose down. Tilting the stick right and left alters the position of the ailerons. In the zero position the ailerons are centered, tilting the stick to the right will move the right aileron up and the left aileron down, tilting the stick to the left direction will have the opposite effect on the ailerons. 

Mechanically a stick presents two degrees of rotational freedom with approximately a +/-45° range however, these axes are represented as Generic Desktop Page translational axes X (Roll) and Y (Pitch).

Flight Stick
CA - A Flight Stick defines a class of device commonly used for flight simulator games. For a device to qualify as a “Flight Stick” the device must support at least 2 axes (Pitch and Roll), a trigger button, three additional buttons, and a hat switch. A Flight Stick is also a Flight Control Stick.

Landing Gear
OOC - A control for raising or lowering an airplanes landing gear. This should be a toggle switch but it is typically implemented as push-button.

Rudder	DV - Nautical: A vertically hinged plate of metal, fiberglass, or wood mounted at the stern of a vessel for directing its course. The movements of the rudder are controlled by a wheel in front of the ships pilot.

Aeronautic: A similar structure at the tail of an aircraft, used for effecting horizontal changes in course. The movements of the rudder are normally controlled from foot pedals in front of the pilot's seat;  the pedals also activate the wheel brakes when the plane is on the ground. The rudder is controlled by two rudder pedals spaced 12 to 18 inches apart, as one pedal is pressed forward the other moves backward. When the right pedal is pressed the aircraft will turn to the right and when the left pedal is pressed the aircraft will turn to the left. Each pedal is also hinged so that pointing the toe initiates braking on that side (see Toe Brake).

The zero position is centered, positive values turn right, and negative values turn left.

Toe Brake	DV - A device for slowing or stopping the motion of an airplane when it is on the ground. Typically Toe Brakes consist of two foot pedals that effect the left and right brakes, respectively. Control of the Toe Brakes can allow steering of the plane as well as braking when it is on the ground. An analog Toe Brake generates a dimensionless value between 0 and full scale. In some implementation the Toe Brake can be simply a push-button (full on or off).

Throttle	DV - A valve that regulates the flow of a fluid, such as the valve in an internal-combustion engine that controls the amount of vaporized fuel entering the cylinders. 2. A lever or pedal controlling such a valve generates a dimensionless value between 0 and full scale.

Trigger	MC - A lever pressed by the finger to release or activate a mechanism. Typically used to discharge a firearm however it can be used for many devices. In combat airplanes the Trigger is usually positioned under the thumb, for a gun it would be positioned under the index finger. Typically this a Push Button.

Weapons Arm
OOC - This device is normally a covered toggle switch which must be selected to enable the weapons system.

Weapons Select
OSC - This device can either be a push-button that steps through the available weapons or a Radio Button that selects them individually.

Wing Flaps
DV - Wing flap controls are usually powered, either hydraulically or by electric motors and are used for low speed control of an airplane. A flap generates a value between 0 and full extension.

Flight Yoke
CA - A Flight Yoke or Control Wheel controls the Pitch and Roll of an airplane. It looks like a bow tie grasped by both hands. The yoke at which the pilot sits may be pushed forward or pulled back to move the tail elevator, down and up, respectively.  In the zero position the elevator is centered for level flight. Pushing forward on the yoke causes the plane to nose down and generates negative values. Pulling back on the yoke causes the plane to nose up and generates positive values. Rotating the yoke alters the position of the ailerons. In the zero position the ailerons are centered, rotating the yoke in a clockwise direction will move the right aileron up and the left aileron down and generate incrementing values, rotating the yoke in the counter-clockwise direction will have the opposite effect on the ailerons and generate decrementing values.

## Automobile Simulation Devices

Automobile Simulation Device
This usage definition allows a device to be generally classified as one that uses standard controls found in an automobile or truck.

Accelerator
DV - A device, especially the gas pedal of a motor vehicle, for increasing speed. An Accelerator is a dimensionless single Degree-of-Freedom Analog Entity. Where the range of values is from zero to maximum acceleration.

Brake	DV - A device for slowing or stopping motion, as of a vehicle, especially by contact friction. A Brake can be a Generic Button or a dimensionless single Degree-of-Freedom Analog Entity. Where the range of values is from zero to maximum braking.

Clutch	DV - A Clutch can be a Generic Button or a dimensionless single Degree-of-Freedom Analog Entity. Where the range of values is from zero to maximum clutch actuation.

Shifter	DV - A Shifter is a specialized mechanical configuration of a Radio Button. A zero value is returned when the shifter is in the “neutral” position. Positive values indicate the forward gear and negative values indicate the reverse gear that the device is in.

Steering	DV - A steering wheel is a single Degree-of-freedom device that rotates about an axis. The zero position is always the neutral or “straight ahead” position, with positive values turning clockwise and negative values counter-clockwise. If the Coordinate Values Wrap attribute is set then the steering wheel can be turned through more that 360 degrees.

## Other Simulation Devices

Tank Simulation Device
CA - This usage definition allows a device to be generally classified as one that uses standard controls found in a tank or a treaded vehicle.

Track Control
CP - A device for controlling the direction and velocity of a vehicle that is driven by tracks. There can be either two sticks with one degree of freedom or a single stick with 2 degrees of freedom. In the two stick case: the neutral position is when the stick is centered (zero), pushing the stick forward causes forward acceleration (positive values), pulling it back causes reverse acceleration (negative values). The right and left hand controls will effect the respective side of the vehicle. In the one stick case: forward/backward acceleration works the same however right and left movement of the stick determines the amount of power applied to the respective track. When the stick is centered horizontally, equal amounts of power are applied to both tracks generating a zero output value. Moving the stick to the right will generate positive values, left - negative. A Tank Track Control is a dimensionless Analog Entity.

Turret Direction
DV - This control determines the right to left positioning of the a tank turret. A value of zero hold maintains the current orientation of the turret. A positive value turns the turret to the right and a negative value turns the turret to the left.

Barrel Elevation
DV - This control elevation of the gun barrel in a turret. A value of zero hold maintains the current orientation of the barrel. A positive value raises the barrel and a negative value lowers the barrel.

Submarine Simulation Device
CA - Classify a device to use the standard controls of a submarine.

Dive Plane	DV - Dive Planes control the vertical ascent of descent of the submarine under power. A zero value indicates level travel. Positive values indicate ascent, negative values indicate descent. A Dive Plane is a dimensionless Analog Entity.

Ballast	DV - Ballast controls the vertical ascent of descent of the submarine. A zero value indicates level travel. Positive values indicate ascent, negative values indicate descent. A Ballast is a dimensionless Analog Entity.

Sailing Simulation Device
CA - Classify a device to use the standard controls of a sail boat.

Motorcycle Simulation Device
CA - Classify a device to use the standard controls of a motorcycle.

Bicycle Crank
DV - A foot-operated assembly of pedals attached to a crank that are used for powering a bicycle. The reported value is the rate that the crank turns.

Handle Bars
DV - Steering control for a motorcycle or bicycle that is held in both hands. A zero output value indicates that the travel is in a direction straight ahead. Pulling back on the right side turns the vehicle to the right and generates a positive output. Pulling back on the left side turns the vehicle to the left and generates a negative output.

Front Brake
DV - Engages the front brake of the motorcycle to slow the vehicle. A Front Brake can be a Generic Button or a dimensionless single Degree-of-Freedom Analog Entity. Where the range of values is from zero to maximum braking.

Rear Brake
DV- Engages the rear brake of the motorcycle to slow the vehicle. A Rear Brake can be a Generic Button or a dimensionless single Degree-of-Freedom Analog Entity. Where the range of values is from zero to maximum braking.

Magic Carpet Simulation Device
CA - Classify a device to use the standard controls of a magic carpet. This control is a bar that is grasped by both hands which controls the Yaw, Pitch and Roll of the carpet. The bar, at which the pilot sits, may be pushed forward or pulled back to cause the carpet to dive and rise, respectively.  In the zero position the carpet is in level flight. Pushing forward on the bar causes the carpet to nose down and generates negative values. Pulling back on the bar causes the carpet to nose up and generates positive values. Turning the bar turns the carpet. In the zero position the carpet travels straight ahead, pulling back on the right side will turn the carpet to the right and generates positive values, pulling back on the left side will turn the carpet to the left and generates negative values. Rotating the bar rolls the carpet. In the zero position the carpet travels level, rotating the bar in a clockwise direction will roll the carpet to the right and generates positive values, rotating the carpet in the counter-clockwise direction will roll the carpet to the left and generates negative values.

# VR Controls Page (0x03)

Virtual Reality controls depend on designators to identify the individual controls. Most of the following are usages are applied to the collections of entities that comprise the actual device.

- Table 7: VR Controls Page

| Usage ID | Usage Name | Usage ID | Usage Name |
| --- | --- | --- | --- |
| 00 | Unidentified | 08 | Oculometer |
| 01 | Belt | 09 | Vest |
| 02 | Body Suit | 0A | Animatronic Device |
| 03 | Flexor | 0B-1F | Reserved |
| 04 | Glove | 20 | Stereo Enable |
| 05 | Head Tracker | 21 | Display Enable |
| 06 | Head Mounted Display | 22-FFFF | Reserved |
| 07 | Hand Tracker |  |  |

Belt	CA - A Belt wraps around the users waist. A tracker would be centered in the small of the users back identifying the orientation of the wearer’s hips.

Body Suit	CA - This type can be used to generally classify a device as one that uses standard controls generated by a Body Suit. A Body Suit typically has a large number of position sensors, typically fixed to the major joints of the body, like the ankles, knees, hips, shoulders, elbows, wrists and head, for measuring the angle and movement of the wearer's joints and limbs.

Flexor	CP - A Flexor describes the angle of bend of a joint or limb in the body. The designator is used to determine which joint a specific Flexor entity represents.

Glove	CA - A Glove reports the positions of the fingers. Up to 20 angular values can be reported. Designators are used to determine the Degrees-of-Freedom that the Glove device is capable of reporting.

Head Tracker	CP - A head tracker represents the position and/or orientation of the head in space. The axes are oriented so that in the zero position the user is looking from the positive Z axis to the negative Z axis and positive Y axis is sticking out the top of their head.

Head Mounted Display
CA - A Head Mounted Display presents the following parameters to the user.

Hand Tracker	CA - A Hand Tracker represents the position of the hand in space. A Hand Tracker attaches to the back of the hand. Holding the hand flat, it is assumed that in the zero position the hand is held upright where the extended thumb is parallel to the Y axis and the fingers are pointing in the negative Z direction.

Oculometer	CA - An oculometer identifies the direction that the eye is looking in rotation about the X and Y axes. The designator for a Oculometer is always Eye.

Vest	CA - A Vest wraps around the chest and abdomen. A tracker would be placed on the users back between the shoulder blades.

Animatronic Device
CA - An input device for the animation of mechanical or Computer Graphic Image “electronic” puppets. An animatronic device is engineered to fit a puppeteer's or performer's body (and/or head and/or face) and comfortably allow a wide range of physical freedom. They measure the angle and movement of the wearer's joints and limbs which are then translated into the motion of the puppet, allowing a puppet mimic the wearer’s movements. There is not necessarily a 1:1 mapping of human to puppet controls. A puppeteer’s arm may be used to control the movement of an elephants trunk or hand controls may be used to control facial expressions. Designators are used to determine which puppeteer joint is being tracked. The controlling application will map these inputs to the electro/mechanical device.

Stereo Enable
OOC - Selects the display mode of the HMD. A value of 0 selects monoscopic mode and 1 selects stereoscopic mode.

Display Enable
OOC - Enables the HMD video output. A value of 0 turns off the display and 1 turns it on.

# Sport Controls Page (0x04)

- Table 8: Sport Controls Page

| Usage ID | Usage Name | Usage ID | Usage Name |
| --- | --- | --- | --- |
| 00 | Unidentified | 50 | Putter |
| 01 | Baseball Bat | 51 | 1 Iron |
| 02 | Golf Club | 52 | 2 Iron |
| 03 | Rowing Machine | 53 | 3 Iron |
| 04 | Treadmill | 54 | 4 Iron |
| 05-2F | Reserved | 55 | 5 Iron |
| 30 | Oar | 56 | 6 Iron |
| 31 | Slope | 57 | 7 Iron |
| 32 | Rate | 58 | 8 Iron |
| 33 | Stick Speed | 59 | 9 Iron |
| 34 | Stick Face Angle | 59 | 10 Iron |
| 35 | Stick Heel/Toe | 5A | 11 Iron |
| 36 | Stick Follow Through | 5B | Sand Wedge |
| 37 | Stick Tempo | 5C | Loft Wedge |
| 38 | Stick Type | 5D | Power Wedge |
| 39 | Stick Height | 5E | 1 Wood |
| 3A-4F | Reserved | 5F | 3 Wood |
|  |  | 60 | 5 Wood |
|  |  | 61 | 7 Wood |
|  |  | 62 | 9 Wood |
|  |  | 63-FFFF | reserved |

## Stick Devices

Stick devices are used in applications where the user swings an object to make contact with another. Typical examples are a baseball bat and a Golf Club. These devices sense various quantities at the point of impact to determine the direction that the target (struck) object will go. The target object is a sphere. The Stick usages identify the measurement quantities.

Baseball Bat	CA - Primary input device for baseball simulation applications. Normally consists of a collection of Stick Usages.

Golf Club 	CA - Primary input device for golf simulation applications. Normally consists of a collection of Stick Usages.

Stick Speed	DV - The velocity with which the Stick strikes the target object. This can be expressed as collection of velocity values to provide a direction or as an absolute magnitude where the Stick Face Angle provides the direction.

Stick Face Angle
DV - The direction with which the Stick strikes the target object. In a golf simulation this will be the horizontal angle. The vertical angle will be determined by the Stick Type.

Stick Heel/Toe
DV - Identifies the contact point relative to the striking surface. This helps to identify the “sweet spot”. This reported as a relative value where; 0 = the sweet spot, positive values are away from the user, and negative values are towards the user.

Stick Follow Through
DV - In a golf simulation the user strikes the ball, swings the club forward over their shoulders, then brings the club back to the tee. Stick Follow Through is a measure of the time that this process takes.

Stick Tempo
DV - In a golf simulation the user starts the swing at the ball, swings the club back over their shoulders, then strikes the ball. Stick Tempo is a measure of the time that this process takes.

Stick Type	NAry  - Identifies the type of club used.

Stick Height	DV - Height of contact point above the ground.

### Golf Clubs

Sel - Irons 1 through 11 and Woods 1, 3, 5, 7 and 9, are enumerated as well as the Sand Wedge, Loft Wedge, Power Wedge and Putter.

## Exercise Machines

Bicycles can be found in Simulation Devices.

Rowing Machine
CA - An exercise device that simulates rowing a boat. Usages typically found in this collection are Oars and Rate. The Rate is typically expressed in strokes per minute.

Oar	DV - Rowing repetition rate. Left, right and two handed oars are distinguished with designators.

Treadmill	CA - An exercise device consisting of an endless moving belt on which a person can walk or jog while remaining in one place. Usages typically found in this collection are Slope and Rate. The Rate is typically expressed in strokes per minute.

Rate	DV - Rate in miles per hour.

Slope	DV - Slope is measured in degrees. Positive angles are uphill, negative angles are downhill and 0° is level.

# Game Controls Page (0x05)

- Table 9: Game Controls Page

| Usage ID | Usage Name | Usage ID | Usage Name |
| --- | --- | --- | --- |
| 00 | Undefined | 2A | Flipper |
| 01 | 3D Game Controller | 2B | Secondary Flipper |
| 02 | Pinball Device | 2C | Bump |
| 03 | Gun Device | 2D | New Game |
| 04-1F | Reserved | 2E | Shoot Ball |
| 20 | Point of View | 2F | Player |
| 21 | Turn Right/Left | 30 | Gun Bolt |
| 22 | Pitch Right/Left | 31 | Gun Clip |
| 23 | Roll Forward/Backward | 32 | Gun Selector |
| 24 | Move Right/Left | 33 | Gun Single Shot |
| 25 | Move Forward/Backward | 34 | Gun Burst |
| 26 | Move Up/Down | 35 | Gun Automatic |
| 27 | Lean Right/Left | 36 | Gun Safety |
| 28 | Lean Forward/Backward | 37-FFFF | reserved |
| 29 | Height of POV |  |  |

## 3D Game Controller

The following controls support the first-person games or those that are played through the eyes of the character or player that represents you in the 3D world.

3D Game Controller	CA - A collection of 3D movement usages.

Point of View
CP - A collection of rotational axes (Rx, Ry, Rz) that represent the orientation of the users head in 3D space. If applied to a Hat Switch only the Rz (Yaw) axis will be controlled.

Turn Right/Left
DV - Identifies the horizontal facing direction of the players hips (Rz = Yaw). A relative value where; 0° is straight ahead, positive values turn right, and negative values turn left. The rate of rotation is determined by the application.

Pitch Right/Left
DV - Identifies the vertical facing direction of the players hip (Rx = Pitch). A relative value where; 0° is straight up, positive values bend back, and negative values bend forward. The rate of rotation is determined by the application.

Roll Forward/Backward
DV - Identifies the vertical facing direction of the players hip (Ry = Roll). A relative value where; 0° is straight up, positive values lean left, and negative values lean right. The rate of rotation is determined by the application.

Move Right/Left
DV - This control allows the player to side slip or move right and left without changing the orientation of the hips. A relative value where; 0 is no lateral motion, positive values move right, and negative move left. The rate of movement is determined by the application.

Move Forward/Backward 
DV - This control allows the player to move forward and backwards in the direction (yaw) the hips are facing. A relative value where; 0 is no motion, positive values move backward, and negative move forward. The rate of movement is determined by the application.

Move Up/Down
DV - This control allows the player to move up and down. It is assumed that the player can fly, is swimming, at a ladder or a climbable wall. A relative value where; 0 is no motion, positive values move down, and negative move up. The rate of movement is determined by the application.1

Lean Right/Left
DV - This control allows the player to lean right and left from the hips, without changing the orientation of the hips. i.e. to look around a corner. A relative value where; 0 is no lateral motion, positive values move right, and negative move left. The rate of movement is determined by the application.

Lean Forward/Backward 
DV - This control allows the player to lean forward and backwards from the hips, in the direction (yaw) the hips are facing. A relative value where; 0 is no motion, positive values move backward, and negative move forward. The rate of movement is determined by the application. 1

Height of POV
DV - This control allows the player to stand, squat and crawl. A absolute value where; 0 is standing and positive values lower the players Point Of View. 1

## Pinball Device

Pinball is a game played on a device in which the player operates a plunger to shoot a ball down or along a slanted surface having obstacles and targets.

Pinball Device
CA - A collection of usages representing the controls of a pinball game.

Flipper	MC - A button that actuates a bar that swings around a pivot to change the direction of the pinball. A right and left pair of these normally reside at the near end of the table.

Secondary Flipper
MC - A button that actuates a bar that swings around a pivot to change the direction of the pinball. Secondary flippers normally reside on the sides halfway up the table.

Bump	MC - A control that indicates that the flipper assembly has been shoved by the player to change the direction of the pinball. This can be as simple as a switch or as complex as a 3D force vector.

New Game	OSC - A button that terminates any current game and reinitializes for a new game.

Shoot Ball	OSC - A control that indicates that the player has placed the pinball into play. This can be as a switch or a force vector.

Player	OSC - A button that cycles through the number of players supported by the game.

## Gun Device

A Gun Device is used in battle and war game simulations.

Gun Device	CA - A collection of Gun usages that describes a hand-held projectile weapon.

Gun Bolt	OOC - A control that indicates the state of the Gun Bolt. A returned value of 1 indicates that the bolt is locked and ready to fire, 0 indicates that the bolt is open, ready for the next shell.

Gun Clip	OOC - A control that indicates whether the Gun Clip is inserted. A returned value of 1 indicates that the clip is inserted, 0 indicates that the clip is missing.

Gun Selector
NAry - This control identifies the firing mode of the weapon. Typically it is a mutually exclusive set of usages for single shot, burst, and fully automatic that are presented as an array input.

Gun Single Shot
Sel - Selects a single shot each time the trigger is pulled.

Gun Burst 	Sel - Selects a short burst (3 shots) each time the trigger is pulled.

Gun Automatic
Sel - Places the gun in fully automatic mode where it will fire continuously while the trigger is pulled.

Gun Safety	OOC - A control that indicates whether safety is enable. A returned value of 1 indicates that the gun is enabled to fire, 0 indicates that gun will not fire when the trigger is pulled.

# Key Code Page (0x07)

The Usage Type of all key codes are Selectors. Refer to the HID Class Specification for more information.

# LED Page (0x08)

An LED or indicator is implemented as an On/Off Control using the “Single button toggle” mode. Where a ‘1’ value will turn on the indicator, and ‘0’ value will turn it off. The only exceptions the Usage  Selected Indicator and Usage In Use Indicator, see their description below.

- Table 10: LED Usage Page

| Usage ID | Usage Name | Usage ID | Usage Name |
| --- | --- | --- | --- |
| 00 | Undefined | 27 | Stand-by |
| 01 | Num Lock | 28 | Camera On |
| 02 | Caps Lock | 29 | Camera Off |
| 03 | Scroll Lock | 2A | On-Line |
| 04 | Compose | 2B | Off-Line |
| 05 | Kana | 2C | Busy |
| 06 | Power | 2D | Ready |
| 07 | Shift | 2E | Paper-Out |
| 08 | Do Not Disturb | 2F | Paper-Jam |
| 09 | Mute | 30 | Remote |
| 0A | Tone Enable | 31 | Forward |
| 0B | High Cut Filter | 32 | Reverse |
| 0C | Low Cut Filter | 33 | Stop |
| 0D | Equalizer Enable | 34 | Rewind |
| 0E | Sound Field On | 35 | Fast Forward |
| 0F | Surround field On | 36 | Play |
| 10 | Repeat | 37 | Pause |
| 11 | Stereo | 38 | Record |
| 12 | Sampling Rate Detect | 39 | Error |
| 13 | Spinning | 3A | Usage Selected Indicator |
| 14 | CAV | 3B | Usage In Use Indicator |
| 15 | CLV | 3C | Usage Multi Mode Indicator |
| 16 | Recording Format Detect | 3D | Indicator On |
| 17 | Off-Hook | 3E | Indicator Flash |
| 18 | Ring | 3F | Indicator Slow Blink |
| 19 | Message Waiting | 40 | Indicator Fast Blink |
| 1A | Data Mode | 41 | Indicator Off |
| 1B | Battery Operation | 42 | Flash On Time |
| 1C | Battery OK | 43 | Slow Blink On Time |
| 1D | Battery Low | 44 | Slow Blink Off Time |
| 1E | Speaker | 45 | Fast Blink On Time |
| 1F | Head Set | 46 | Fast Blink Off Time |
| 20 | Hold | 47 | Usage Indicator Color |
| 21 | Microphone | 48 | Red |
| 22 | Coverage | 49 | Green |
| 23 | Night Mode | 4A | Amber |
| 24 | Send Calls | 4B | Generic Indicator |
| 25 | Call Pickup | 4C-FFFF | Reserved |
| 26 | Conference |  |  |

Note: The Usage Selected Indicator, Usage In Use Indicator, Usage Multi Mode Indicator usages can change the Usage Type of the usage(s) that they contain.

Usage Selected Indicator
CL - This collection allows the usages that it contains to be associated with a visual output (an LED) that indicates that a control identified by the usage is Selected. A one bit field where 1 = selected and 0 = not selected. All usages found in this collection will be treated as a On/Off Controls.

Usage In Use Indicator
CL - This collection allows the usages that it contains to be associated with a visual output (an LED) that indicates that a control identified by the usage is in use. A one bit field where 1 = in use and 0 = not in use. All usages found in this collection will be treated as a On/Off Controls.

An indicator may be support Multi Mode features and multiple colors simultaneously. To tie these functions together they can be wrapped in a logical collection where the usage that is attached to the collection defines the purpose of the control.

Usage Multi Mode Indicator
NAry – This collection allows the usages that it is contained in to be identified as an indicator that supports multiple illumination modes. In this usage one or more of the following Indicator selectors will be found: On, Flash, Slow Blink, Fast Blink and Off.

Indicator On
Sel - Light indicator continuously.

Indicator Flash 
Sel - Single, Momentary illumination of indicator.

Indicator Slow Blink
Sel - Continuous flashing of the indicator at a slow rate.

Indicator Fast Blink
Sel - Continuous flashing of the indicator at a fast rate.

Indicator Off
Sel - Turn Indicator illumination off.

Flash On Time
DV – Duration that the indicator is illuminated in flash mode.

Slow Blink On Time
DV – Duration that the indicator is illuminated in slow blink mode.

Slow Blink Off Time
DV – Duration that the indicator is off in slow blink mode.

Fast Blink On Time
DV – Duration that the indicator is illuminated in fast blink mode.

Fast Blink Off Time
DV – Duration that the indicator is off in fast blink mode.

Usage Indicator Color
NAry – This collection allows the usage that it is contained in to be an indicator that supports multiple colors. All usages found in this collection will be treated as a Selectors where one or more of the following Indicator selectors will be found: Red, Green and Amber.

Indicator Red
Sel - Indicator color set to Red.

Indicator Green
Sel - Indicator color set to Green.

Indicator Amber
Sel - Indicator color set to Amber. This is typically implemented by asserting Red and Green simultaneously.

Generic Indicator
OOC – This usage identifies an indicator that has no permanently assigned function.

# Button Page (0x09)

The Button page is the first place an application should look for user selection controls. OS Graphical User Interfaces typically employ a pointer and a set of hierarchical selectors to select, move and otherwise manipulate their environment. For these purposes the following assignment of significance can be applied to the Button usages.

Button 1 - Primary Button - Button used for object selecting, dragging, and double click activation.  On MacOS, this is the only button. Microsoft operating systems call this a logical left button, although it is not necessarily located on the left of the pointing device.

Button 2 - Secondary Button - Used by newer shells to browse object properties. Exposed by OS to applications which typically assign application-specific functionality.

Button 3 - Tertiary Button - Optional control.  Exposed to application programs, but seldom assigned functionality due to prevalence of two- and one- button devices.

Buttons 4 - 255 - As the button number increases the significance to as a selector decreases.

In many ways the assignment of button numbers is similar to the assignment of Effort in Physical Descriptors. Button 1 would be used to define the button a finger rests on when the hand is in the “at rest” position, that is, virtually no effort is required by the user to activate the button. Button values increment as the finger has to stretch to reach a control. See the Physical Descriptors section in the “USB Device Class Definition for Human Interface Devices (HID)” specification for methods of further qualifying buttons.

- Table 11: Button Usage Page

| Usage ID | Usage Name |
| --- | --- |
| 00 | No button pressed |
| 01 | Button 1 (primary/trigger) |
| 02 | Button 2 (secondary) |
| 03 | Button 3 (tertiary) |
| 04 | Button 4 |
| ... | ... |
| FFFF | Button 65535 |

When defining buttons Usage ID 0 is defined to indicate that no buttons are pressed. When declaring an array of buttons one can:

- Declare all buttons of interest, include the usage No Button Pressed, set the No Null Position flag and declare a Logical Minimum of 0.

- Or, only declare the buttons of interest, set the Null State flag and declare a Logical Minimum of 1. In this case the 0 value is out of range or Null and interpreted as No Buttons Pressed.

In either case, by convention a device that returns a value of 0 for an Array should be indicating that no button is pressed. Radio Buttons are an exception to this rule because one button is always valid. See “Radio Buttons, Mechanically Linked” in the Examples section.

# Ordinal Page (0x0A)

The Ordinal Page allows multiple instances of a control or sets of controls to be declared without requiring the individual enumeration of them in the native Usage Page. i.e. declaring Usages of Pointer 1, Pointer 2, etc. on the Generic Desktop page. When parsed the ordinal instance number is, in essence, concatenated to the usages attached to the encompassing Collection to create Pointer 1, etc.

For an example see “Multiple Instances of a Control” in the Examples section.

By convention Ordinal collections are placed inside the collection that they are declaring multiple instances of.

Instances do not have to be identical.

- Table 12: Ordinal Usage Page

| Usage ID | Usage Name |
| --- | --- |
| 00 | Reserved |
| 01 | Instance 1 |
| 02 | Instance 2 |
| 03 | Instance 3 |
| 04 | Instance 4 |
| ... | ... |
| FFFF | Instance 65535 |

# Telephony Device Page (0x0B)

This usage page defines the  keytop and control usages for telephony devices.

Indicators on a phone are handled by wrapping LED: Usage In Use Indicator and LED: Usage Selected Indicator usages. i.e. A Message indicator LED would by a identified by a Telephony: Message usage declared as a Feature or Output found in a LED: Usage In Use Indicator collection.

See the Consumer Page for audio volume and tone controls.

- Table 13: Telephony Usage Page

| Usage ID | Usage Name | Usage ID | Usage Name |
| --- | --- | --- | --- |
| 00 | Unassigned | 73 | Message |
| 01 | Phone | 74 | Answer On/Off |
| 02 | Answering Machine | 75-8F | Reserved |
| 03 | Message Controls | 90 | Inside Dial Tone |
| 04 | Handset | 91 | Outside Dial Tone |
| 05 | Headset | 92 | Inside Ring Tone |
| 06 | Telephony Key Pad | 93 | Outside Ring Tone |
| 07 | Programmable Button | 94 | Priority Ring Tone |
| 08-1F | Reserved | 95 | Inside Ringback |
| 20 | Hook Switch | 96 | Priority Ringback |
| 21 | Flash | 97 | Line Busy Tone |
| 22 | Feature | 98 | Reorder Tone |
| 23 | Hold | 99 | Call Waiting Tone |
| 24 | Redial | 9A | Confirmation Tone 1 |
| 25 | Transfer | 9B | Confirmation Tone 2 |
| 26 | Drop | 9C | Tones Off |
| 27 | Park | 9D-AF | Reserved |
| 28 | Forward Calls | B0 | Phone Key 0 |
| 29 | Alternate Function | B1 | Phone Key 1 |
| 2A | Line | B2 | Phone Key 2 |
| 2B | Speaker Phone | B3 | Phone Key 3 |
| 2C | Conference | B4 | Phone Key 4 |
| 2D | Ring Enable | B5 | Phone Key 5 |
| 2E | Ring Select | B6 | Phone Key 6 |
| 2F | Phone Mute | B7 | Phone Key 7 |
| 30 | Caller ID | B8 | Phone Key 8 |
| 31-4F | Reserved | B9 | Phone Key 9 |
| 50 | Speed Dial | BA | Phone Key Star |
| 51 | Store Number | BB | Phone Key Pound |
| 52 | Recall Number | BC | Phone Key A |
| 53 | Phone Directory | BD | Phone Key B |
| 54-6F | Reserved | BE | Phone Key C |
| 70 | Voice Mail | BF | Phone Key D |
| 71 | Screen Calls | C0-FFFF | Reserved |
| 72 | Do Not Disturb |  |  |

Phone	CA – An application level collection which identifies a device that contains telephone controls.

Answering Machine
CA - An application level collection which identifies a device that contains primarily voice mail or answering machine controls.

Message Controls
CL – Usages related to voice mail controls.

Handset	CL – Usages related to the handle-shaped part of a telephone, containing the audio receiver and transmitter.

Headset	CL – Usages related to the telephone headset (headphones and microphone), containing the audio receiver and transmitter.

## Key Pad

Telephony Key Pad
NAry - Collection Usage for a standard telephony key pad (Dial buttons 1-9, *, 0, and #). A Telephony Key Pad implies that the keytops are marked with a digit and associated alpha characters. This collection can also be used as a general purpose 1-9 and 0 keypad.

There are two options for describing the buttons in a Telephony Key Pad.

1) Use the Telephony Keypad usages defined in section 13.6 below.

2)Usages for Dial digits can be defined from the Button page where Dial buttons are assigned to Button page Buttons with the following rules:

Button	Dial	Key top label
Usage	Button
Button 1	0	‘0’ & ‘Oper’
Button 2	1	‘1’
Button 3	2	‘2’ & ‘ABC’
Button 4	3	‘3’ & ‘DEF’
Button 5	4	‘4’ & ‘GHI’
Button 6	5	‘5’ & ‘JKL’
Button 7	6	‘6’ & ‘MNO’
Button 8	7	‘7’ & ‘PQRS’
Button 9	8	‘8’ & ‘TUV’
Button 10	9	‘9’ & ‘WXYZ’
Button 11	Star	‘*’ 
Button 12	Pound	‘#’ 
Button 13	A
Button 14	B
Button 15	C
Button 16	D

Programmable Button
NAry - Programmable telephone button. This collection is enumerated using usages from the Button page.

## Call Control

Hook Switch
OOC - Indicates that the handset is Off Hook. Single bit; 1 = Off Hook.

Flash	MC - Generates a momentary On Hook condition to signal the application. Often used for alternate line selection.

Feature	OSC - Select operating feature.

Hold	OOC - Places current call on hold.

Redial	OSC - Redials last number dialed.

Transfer	OSC - Transfers call to another extension.

Drop	OSC - Disconnect the active call.

Park	OOC - Wait for free line.

Forward Calls
OOC - Forward calls to another number.

Alternate Function
MC - Modifier key, similar to a shift key, that provides an alternate function to be selected on specific buttons. Pressing this button enables the alternate function mapping. Pressing an alternate function key terminates alternate function mode.

Line	OSC - Line selection.

Speaker Phone
OOC - Enable speaker phone mode.

Conference	OOC - Initiate conference call.

Ring Enable
OOC - Enable ringer.

Ring Select
OSC - Select ring tone. Typically, press Ring Select then a dial digit to select the tone.

Phone Mute
OOC – Disable audio to called person. The caller can still hear the incoming audio.

Caller ID	MC - Displays ID of caller.

## Speed Dial Controls

Speed Dial	OSC - Initiate Speed Dial operation.

Store Number
OSC - Save Speed Dial number.

Recall Number
OSC - Recall Speed Dial number on display.

Phone Directory
OOC - Display phone directory.

## Voice Mail Controls

Voice Mail	OOC - Enter Voice Mail application.

Screen Calls
OOC – Disable audio to called person and forward calls to voice mail application. The caller can still hear the incoming audio

Do Not Disturb
OOC – Disable ring and speaker phone operation and forward calls to voice mail application.

Message	OSC - Listen to Voice Message.

Answer On/Off
OOC - Toggle Answering machine operation.

## Locally Generated Tones

Some telephony devices generate tones locally vs. being delivered over the audio input. These tones are played to the user via either the handset speaker, or the speaker in a speakerphone telephone.

Inside Dial Tone
MC - Telephone is ready to place an inside call.

Outside Dial Tone
MC - Telephone is ready to place an outside call.

Inside Ring Tone
MC - A destination telephone in-house is ringing.

Outside Ring Tone
MC - A destination telephone outside is ringing.

Priority Ring Tone
MC - A destination telephone with some “Feature” is ringing.

Inside Ringback
MC - A ringback feature has been activated to an inside line.

Outside Ringback
MC - A ringback feature has been activated to an outside line.

Priority Ringback
MC - A ringback feature has been activated to a “Feature” line.

Line Busy Tone
MC - The destination line is currently busy.

Reorder Tone
MC - There are no lines available for the user to place a call.

Call Waiting Tone
MC - The user is currently on a line, and another phone call is coming in.

Confirmation Tone 1
MC - A feature the user has requested has been enabled.

Confirmation Tone 2
MC - A feature the user has requested has been enabled.

Tones Off	OOC - Turn all tones off.

## Phone Keypad

- The Phone keypad is defined here because of its unique keytop markings.

Phone Key 0
Phone keypad key digit 0 and Oper.

Phone Key 1
Phone keypad key digit 1.

Phone Key 2
Phone keypad key digit 2 and A, B, C.

Phone Key 3
Phone keypad key digit 3 and D, E, F.

Phone Key 4
Phone keypad key digit 4 and G, H, I.

Phone Key 5
Phone keypad key digit 5 and J, K, L.

Phone Key 6
Phone keypad key digit 6 and M, N, O.

Phone Key 7
Phone keypad key digit 7 and P, Q, R, S. Q is optional.

Phone Key 8
Phone keypad key digit 8 and T, U, V.

Phone Key 9
Phone keypad key digit 9 and W, X,  Y, Z. Z is optional.

Phone Key Star
Phone keypad key Star (*).

Phone Key Pound
Phone keypad key Pound (#).

Phone Key A
Phone keypad key A

Phone Key B
Phone keypad key B

Phone Key C
Phone keypad key C

Phone Key D
Phone keypad key D

# Consumer Page (0x0C)

All controls on the Consumer Page are application specific. i.e. they effect a specific device not the system as a whole.

- Table 14: Consumer Usage Page

| Usage ID | Usage Name | Usage ID | Usage Name |
| --- | --- | --- | --- |
| 00 | Unassigned | 9B | Media Select Call |
| 01 | Consumer Control | 9C-9F | Reserved |
| 02 | Numeric Key Pad | A0 | VCR Plus |
| 04-1F | Reserved | A1 | Once |
| 20 | +10 | A2 | Daily |
| 21 | +100 | A3 | Weekly |
| 22 | AM/PM | A4 | Monthly |
| 23-3F | Reserved | A5-AF | Reserved |
| 30 | Power | B0 | Play |
| 31 | Reset | B1 | Pause |
| 32 | Sleep | B2 | Record |
| 33 | Sleep After | B3 | Fast Forward |
| 34 | Sleep Mode | B4 | Rewind |
| 35 | Illumination | B5 | Scan Next Track |
| 36 | Function Buttons | B6 | Scan Previous Track |
| 37-3F | Reserved | B7 | Stop |
| 40 | Menu | B8 | Eject |
| 41 | Menu Pick | B9 | Random Play |
| 42 | Menu Up | BA | Select Disc |
| 43 | Menu Down | BB | Enter Disc |
| 44 | Menu Left | BC | Repeat |
| 45 | Menu Right | BD | Tracking |
| 46 | Menu Escape | BE | Track Normal |
| 47 | Menu Value Increase | BF | Slow Tracking |
| 48 | Menu Value Decrease | C0 | Frame Forward |
| 49-5F | Reserved | C1 | Frame Back |
| 60 | Data On Screen | C2 | Mark |
| 61 | Closed Caption | C3 | Clear Mark |
| 62 | Closed Caption Select | C4 | Repeat From Mark |
| 63 | VCR/TV | C5 | Return To Mark |
| 64 | Broadcast Mode | C6 | Search Mark Forward |
| 65 | Snapshot | C7 | Search Mark Backwards |
| 66 | Still | C8 | Counter Reset |
| 67-7F | Reserved | C9 | Show Counter |
| 80 | Selection | C2-DF | Reserved |
| 81 | Assign Selection | E0 | Volume |
| 82 | Mode Step | E1 | Balance |
| 83 | Recall Last | E2 | Mute |
| 84 | Enter Channel | E3 | Bass |
| 85 | Order Movie | E4 | Treble |
| 86 | Channel | E5 | Bass Boost |
| 87 | Media Selection | E6 | Surround Mode |
| 88 | Media Select Computer | E7 | Loudness |
| 89 | Media Select TV | E8 | MPX |
| 8A | Media Select WWW | E9-EF | Reserved |
| 8B | Media Select DVD | F0 | Speed Select |
| 8C | Media Select Telephone | F1 | Playback Speed |
| 8D | Media Select Program Guide | F2 | Standard Play |
| 8E | Media Select Video Phone | F3 | Long Play |
| 8F | Media Select Games | F4 | Extended Play |
| 90 | Media Select Messages | F5 | Slow |
| 91 | Media Select CD | F6-FF | Reserved |
| 92 | Media Select VCR | 100 | Fan Enable |
| 93 | Media Select Tuner | 101 | Fan Speed |
| 94 | Quit | 102 | Light |
| 95 | Help | 103 | Light Illumination Level |
| 96 | Media Select Tape | 104 | Climate Control Enable |
| 97 | Media Select Cable | 105 | Room Temperature |
| 98 | Media Select Satellite | 106 | Security Enable |
| 99 | Media Select Security | 107 | Fire Alarm |
| 9A | Media Select Home | 108 | Police Alarm |
|  |  | 109-FFFF | Reserved |

Consumer Control
CA - General consumer control device.

## Numeric Key Pad

Numeric Key Pad
NAry - Collection Usage for a generic numeric keypad. On a consumer device these are commonly used for channel selection. Usages for digits can be found on the Button page where numeric values starting with 0 are assigned to Button 1, 1 to Button 2, and so on.

+10	OSC - Increment channel by 10.

+100	OSC - Increment channel by 100.

AM/PM	OSC – Toggles between AM and PM for time entry.

## General Controls

Power	OOC - Controls application specific power state. For global power control see System Power on the Generic Desktop Usage page.

Reset	OSC - Reset Device.  All volatile settings revert to default.

Sleep	OSC - Initiate low power state on application specific device now.

Sleep After	OSC - Set  inactivity time-out to value. The Sleep After button will be followed with the time-out value in minutes entered on a Numeric Key Pad.

Sleep Mode	RTC - Cycle through available sleep delays. No sleeping, after 5 min., after 10 min., after 30 min., etc. The last selected mode will be enabled.

Illumination
OOC - Toggles illumination of remote's controls on/off.

Function Buttons
NAry - Collection Usage for a generic function buttons. On a consumer device these are commonly used for user assigned functions. Usages for function buttons can be found on the Button page where Function Button 1 is assigned to Button 1, Function Button 2 to Button 2, and so on.

## Menu Controls

Menu	OOC - Initiates on-device-display main menu.  Sets a mode where the other menu controls are active.  In this mode, a subsequent menu press will cancel the mode.

Menu Pick	OSC - Pick item from on screen menu.

Menu Up	OSC - Move selection up in device-displayed menu.

Menu Down	OSC - Move selection down in device-displayed menu.

Menu Left	OSC - Move selection left in device-displayed menu.

Menu Right	OSC - Move selection right in device-displayed menu.

Menu Escape
OSC - Back up a level in on screen menu system.

Menu Value Increase
OSC - Increment value of currently selected menu item. i.e. After using a menu to select a volume control the user can modify the volume level using this control.

Menu Value Decrease
OSC - Decrement value of currently selected menu item.

## Display Controls

Data On Screen
OOC - Superimpose state data on monitor video.  Typically, channel information is displayed.

Closed Caption
OOC - Closed caption display enabled.

Closed Caption Select
OSC - Cycle through closed caption viewing options.

VCR/TV	OOC - Select record source for VCR.

Broadcast Mode
OSC - Cycles between available broadcast modes. Broadcast, CATV, etc. The last selected mode is enabled.

Snapshot	OSC - Capture screen or image of currently selected window.

Still	OSC – Pause playback screen or image of currently selected window.

## Selection Controls

Selection	NAry - Collection Usage for a number of discreet selections. On a consumer device these are commonly used for “favorite” selections. Usages for the selections can be found on the Button page where the choices are assigned to Button 1 and so on.

Assign Selection
OSC - This button works in conjunction with the Selection usage. To assign the current channel or mode to a selection button the user presses the Assign Selection button followed by a Selection button.

Mode Step	OSC - Steps through devices (TV, VCR, cable) in a multi-mode remote.

Recall Last	OSC - Return to last selected channel or mode.

Enter Channel
OSC - Interpret previous number entry as channel information.

Order Movie	OSC - Request pay-per-view entertainment.

Channel	LC - Channel selection control where the range of possible values is equal to the number of channels supported by the device.

VCR Plus	OSC – Initiates (and optionally terminates) VCR Plus code entry mode.

The following controls Once, Daily, Weekly and Monthly are typically used for programming record operations.

Once	OSC - Perform operation once.

Daily	OSC - Perform operation once a day.

Weekly	OSC - Perform operation once a week.

Monthly	OSC - Perform operation once a month.

Media Selection
NAry - Identifies the media source to be manipulated or displayed. This collection will contain a ‘Media Select’ Consumer usage

Media Select Computer
Sel - Select Computer Display.

Media Select TV
Sel - Select television display

Media Select WWW
Sel - Select World Wide Web access.

Media Select DVD
Sel - Select DVD drive.

Media Select Telephone
Sel - Select telephone mode.

Media Select Program Guide
Sel - Select viewing guide.

Media Select Video Phone
Sel - Select videophone mode.

Media Select Games
Sel - Select Gaming mode.

Media Select Messages
Sel - Select Message mode.

Media Select CD
Sel - Select CD drive.

Media Select VCR
Sel - Select VCR.

Media Select Tuner
Sel - Select Tuner.

Media Select Tape
Sel - Select audio Tape.

Media Select Cable
Sel - Select Cable receiver.

Media Select Satellite
Sel - Select Satellite receiver.

Media Select Security
Sel - Select Security Status display.

Media Select Home
Sel - Select Home system status display.

Media Select Call
Sel - Select telephone Call status display.

Quit	OSC - Exit current mode.

Help	OOC - Bring up help screen.

## Transport Controls

Play	OOC - Begin streaming linear media.

Pause	OOC - Stop streaming linear media.

Record	OOC - Initiate transferring input data to media.

Fast Forward
OOC - Initiate fast forward scan of linear media.

Rewind	OOC - Initiate fast reverse scan of linear media.

Scan Next Track
OSC - Move to next chapter or track boundary.

Scan Previous Track
OSC - Move to previous chapter or track boundary.

Stop	OSC - Halt scanning, streaming, or recording linear media.

Eject	OSC - Remove media from player.

Random Play
OOC - Random selection of tracks.

Repeat	OSC - Repeat selection of tracks.

Select Disc	NAry - This usage is attached to a collection that defines the selection of one of many disks. The allowed disk numbers are enumerated with the declaration of ordinals in the Select Disc Logical collection.

Enter Disc	MC -  This button works in conjunction with the Numeric Key Pad usage. To select a disk the user presses the Enter Disc button followed by the entry of the desired disc number on the Numeric Key Pad.

Tracking	LC - Adjusts media tracking.

Track Normal
OSC - Sets media tracking to default or automatic value.

Slow Tracking
LC - Adjusts media slow tracking.

Frame Forward
RTC - Move forward one video frame.

Frame Back
RTC - Move back one video frame.

## Search Controls

Search controls either place a physical flag, Index or Mark on the magnetic media, or use the position or frame counter to flag points of interest on the media. The search controls allow identifying and moving between these points of interest.

Mark	OSC - Mark reference point in media. Synonymous with "Counter Memory" on some devices.

Clear Mark	OSC - Removes mark reference point in media.

Repeat From Mark
OOC - Mark current position as end of block and repeat-play block starting from marked beginning of block.

Return To Mark
OSC - Position at last detected mark and play.

Search Mark Forward
OSC - Search forward for mark.

Search Mark Backwards
OSC - Search backwards for mark.

Counter Reset
OSC - Reset time, position or frame counter.

Show Counter
OSC - Toggles between position counter and time display.

## Audio Controls

Volume	LC - Audio Volume control.

Balance	LC - Audio Balance control.

Mute	OOC - Audio Mute control.

Bass	LC - Audio Bass control.

Treble	LC - Audio Treble control.

Bass Boost	OOC - Audio Bass Boost enable control.

Surround Mode
OSC - Steps through surround mode options.

Loudness	OOC - Applies boost to audio bass and treble.

MPX	OOC - Enable stereo multiplexer.

## Speed Controls

Speed Select
OSC - Cycles through media speed options.

Playback Speed
NAry - A collection of controls that allow adjustment of playback speed (units relative to normal playback speed). Contains selectors Standard, Long, and Extended Play.

Standard Play
Sel - Use VCR "SP" recording speed.

Long Play	Sel - Use VCR "LP" recording speed.

Extended Play
Sel - Use VCR "EP" recording speed.

Slow	OSC - Enable slow speed transport motion.

## Home and Security Controls

Fan Enable	OOC - Controls the state of a overhead, furnace or ventilation fan.

Fan Speed	LC - Adjusts the speed of a overhead, furnace or ventilation fan.

Light Enable
OOC - Controls the state of a light or lamp.

Light Illumination Level
LC - Adjusts the illumination level of a light or lamp.

Climate Control Enable
OOC – Enables or disables a climate control system.

Room Temperature
LC - Adjusts room temperature level.

Security Enable
OOC - Enables or disables a security system.

Fire Alarm	OSC - Initiates a fire alarm.

Police Alarm 
OSC - Initiates a police alarm.

# Digitizers (0x0D)

This section provides detailed descriptions of the usages employed by Digitizer Devices.

- Table 15: Digitizer Page

| Usage ID | Usage Name | Usage ID | Usage Name |
| --- | --- | --- | --- |
| 00 | Undefined | 32 | In Range |
| 01 | Digitizer | 33 | Touch |
| 02 | Pen | 34 | Untouch |
| 03 | Light Pen | 35 | Tap |
| 04 | Touch Screen | 36 | Quality |
| 05 | Touch Pad | 37 | Data Valid |
| 06 | White Board | 38 | Transducer Index |
| 07 | Coordinate Measuring Machine | 39 | Tablet Function Keys |
| 08 | 3-D Digitizer | 3A | Program Change Keys |
| 09 | Stereo Plotter | 3B | Battery Strength |
| 0A | Articulated Arm | 3C | Invert |
| 0B | Armature | 3D | X Tilt |
| 0C | Multiple Point Digitizer | 3E | Y Tilt |
| 0D | Free Space Wand | 3F | Azimuth |
| 0E-1F | Reserved | 40 | Altitude |
| 20 | Stylus | 41 | Twist |
| 21 | Puck | 42 | Tip Switch |
| 22 | Finger | 43 | Secondary Tip Switch |
| 23-2F | Reserved | 44 | Barrel Switch |
| 30 | Tip Pressure | 45 | Eraser |
| 31 | Barrel Pressure | 46 | Tablet Pick |
|  |  | 47-FFFF | Reserved |

## Digitizer Devices

Digitizer	CA - A device that measures absolute spatial position, typically of two or more dimensions.  This is a generic usage: several specialized types of digitizer exist that are distinguished by their attributes.

Pen	CA - A digitizer with an integrated display that allows use of a stylus.  The OS must ensure that the sensed stylus position and the display representations of that position are the same. A pen digitizer has enough time and space resolution for handwriting input.  A digitizer that may or may not be in an integrated display application should use the more generic Digitizer collection usage.

Light Pen	CA - A display-integrated  digitizer that relies on the underlying video raster to determine position .  The interpretation of light pen coordinates is sensitive to changes of display mode.

Touch Screen	CA - A digitizer with an integrated display that allows the use of a finger or stylus for pointing. Some touch screen technologies can differentiate between the touch of a finger and the touch of a stylus.

Touch Pad	CA - A digitizer that is not integrated with a display, but allows the use of a finger for pointing.

White Board	CA - A digitizer that is mounted vertically, and can optionally be synchronized with a projected video display.

Coordinate Measuring Machine
CA - A specialized digitizing instrument that is used to make spatial measurements of maps or photographic images.  It is not suitable for screen pointing

3D Digitizer	CA - General usage for a digitizer that measures position(s) in 3-dimensional space.

Stereo Plotter	CA - A 3D digitizer that relies on the operator’s binocular vision to determine the position of points on a stereoscopically rendered image.

Articulated Arm
CA - A 3D digitizer that uses a series of instrumented mechanical linkages to determine the position of its tip in space.

Armature	CA - A 3D digitizer that determines the position of several mechanical linkages in space.  An armature typically represents the position of a human body for animation or modeling.

Multiple Point Digitizer
CA - A 3D digitizer that detects the position of multiple points in space, typically through some non-mechanical means such as electromagnetic sensors.

Free Space Wand
CA - A 3D digitizer that detects the position of a point at the end of a hand-held wand.

## Digitizer Transducer Collection Usages

Stylus	CL - A stylus is a hand-held transducer that looks and is used like a pen. A digitizer typically reports the coordinates of the tip of a stylus. The stylus collection is a physical collection containing all the controls physically located on the stylus.

Puck	CL - A puck is a mouse-like transducer that rests on a low friction surface, sometimes called a cursor.  A digitizer typically reports the coordinates of crosshairs marked on the puck. The puck collection is a physical collection containing all the controls physically located on the puck.

Finger	CL - Any human appendage used as a transducer. i.e. To touch a touch screen setting the location of the screen cursor. A digitizer typically reports the coordinates of center of the finger. The Finger collection is minimally a collection of X and Y axes.

## Digitizer Report Field Usages

Not all digitizer field usages are from the digitizer usage page.  In particular, the usages for X and Y displacement come from the Generic Desktop page.

### Digitizer-Specific Fields

Tip Pressure
DV - Force exerted against the tablet surface by the transducer, typically a stylus transducer.

Barrel Pressure
DV - Force exerted directly on a transducer sensor by the user, e.g. a pressure-sensitive button on the barrel of a stylus.

In Range	MC - Indicates that a transducer is located within the region where digitizing is possible.  A bit quantity.

Touch	MC - A bit quantity for touch pads analogous to In Range that indicates that a finger is touching the pad. An OS will typically map a Touch to a primary button.

Untouch	OSC - Release of a finger from the surface of the touch screen. The OS typically maps an untouch to the release of a primary button.

Tap	OSC - On a touch pad, this indicates that the finger has been quickly lifted and replaced on the tablet surface.  This is typically mapped to a button event, but is distinct as no physical button is involved.

Quality	DV - If set, indicates that the transducer is sensed to be in a relatively noise-free region of digitizing.

Data Valid	MC - Indicates that the current data set is valid

Transducer Index
DV - Indicates which transducer generated the current report.  Useful if multiple transducers generate identical reports, otherwise Report IDs should be used to distinguish different transducer types.

Tablet Function Keys
CL - These controls are located on the surface of a digitizing tablet, and may be implemented as actual switches, or as soft keys actuated by the digitizing transducer.  These often used to trigger location-independent macros or other events.

Program Change Keys
CL - These are function key targets specialized to changing some internal aspect of the digitizer’s behavior.

Battery Strength
DV - This indicates the power remaining in a digitizer component that is outside the scope of device power management.  Typically this is the battery for a cordless transducer.

Invert	MC - A bit that indicates that the currently sensed position originates from the end of a stylus opposite the tip.

### Tilt Orientation

X Tilt and Y Tilt are used in conjunction to specify the tilt away from normal of a digitizer transducer.  In its normal position, the values of X Tilt and Y Tilt for a transducer are both zero. The X Tilt / Y Tilt orientation system does not specify the rotation of the transducer about its own normal axis.

X Tilt	DV - This quantity is used in conjunction with Y Tilt to represent the tilt away from normal of a transducer, such as a stylus.  The X Tilt value represents the plane angle between the Y-Z plane and the plane containing the transducer axis and the Y axis. A positive X Tilt is to the right.

Y Tilt	DV - This value represents  the angle between the X-Z and transducer-X planes.  A positive Y Tilt is toward the user.

### Azimuth-Altitude Orientation

Azimuth-altitude is an alternative to the tilt system of specifying a digitizer transducer’s orientation.  This system includes rotation of the transducer about its own axis.

Azimuth	DV - Specifies the counter-clockwise rotation of the cursor about the Z axis through a full circular range.

Altitude	DV - Specifies the angle with the X-Y plane through a signed, semicircular range.  Positive values specify an angle downward and toward the positive Z axis.

Twist	DV - Specifies the clockwise rotation of the cursor about its own major axis.

## Digitizer Switch Usages

Tip Switch	MC - Switch located at the tip of a stylus, indicating contact of the stylus with a surface.  A pen-based OS or OS extension would use this switch to gate the input of handwriting or gesture data.  The OS typically maps tip switch to a primary button in a non-pen context.

Secondary Tip Switch
MC - A secondary switch used in conjunction with the Tip Switch to indicate pressure above a certain threshold applied with the stylus.  The threshold switch is not closed unless the tip switch already is.

Barrel Switch
MC - A non-tip button located on the barrel of a stylus.  Its function is typically mapped to an OS Secondary Button, or to a shift-key modifier that changes the Tip Switch function from Primary Button to Secondary Button.

Eraser	MC - This control is used for erasing objects.  Following the metaphor of a pencil, it is typically located  opposite the writing end of a stylus.  It may be a bit switch or a pressure quantity.

Tablet Pick	MC - The primary button used by CAD systems, typically used to select an object.  Sometimes called Button Zero.

# Unicode Page (0x10)

The Unicode Page directly maps to the two octet form defined in the Unicode Standard.

The Unicode Standard, Version 1.1, is the newest version of the Unicode™ Standard. Unicode 1.1 includes the changes and additions that were made to Unicode 1.0 in the process of alignment with the international character encoding standard, ISO/IEC 10646-1, which was approved by ISO/IEC as an International Standard in June 1992, and published in May 1993. The character content and encoding of Unicode 1.1 is thus identical to that of the ISO/IEC 10646-1 UCS-2 (the two octet form).

See Alphanumeric Display for an example.

# Alphanumeric Display Page (0x14)

This page is targeted at simple alphanumeric displays that are used on consumer devices.

The size the display is identified by the Logical Minimum and Logical Maximum items associated with the Row and Column report fields.

- Table 16: Alphanumeric Display Usage Page

| Usage ID | Usage Name | Usage ID | Usage Name |
| --- | --- | --- | --- |
| 00 | Undefined | 30 | Err Not a loadable character |
| 01 | Alphanumeric Display | 31 | Err Font data cannot be read |
| 02-1F | Reserved | 32 | Cursor Position Report |
| 20 | Display Attributes Report | 33 | Row |
| 21 | ASCII Character Set | 34 | Column |
| 22 | Data Read Back | 35 | Rows |
| 23 | Font Read Back | 36 | Columns |
| 24 | Display Control Report | 37 | Cursor Pixel Positioning |
| 25 | Clear Display | 38 | Cursor Mode |
| 26 | Display Enable | 39 | Cursor Enable |
| 27 | Screen Saver Delay | 3A | Cursor Blink |
| 28 | Screen Saver Enable | 3B | Font Report |
| 29 | Vertical Scroll | 3C | Font Data |
| 2A | Horizontal Scroll | 3D | Character Width |
| 2B | Character Report | 3E | Character Height |
| 2C | Display Data | 3F | Character Spacing Horizontal |
| 2D | Display Status | 40 | Character Spacing Vertical |
| 2E | Stat Not Ready | 41 | Unidcode Character Set |
| 2F | Stat Ready | 42-FFFF | Reserved |

Alpha-Numeric Display
A collection of alpha-numeric related display usages.

## Flags

If a flag is defined as a single bit constant Input item then it is simply a read only bit for the Host. If a flag is defined as an Output item then it can be used to enable or disable the flags’ feature.

Display Attributes Report
CL - This usage applies the logical collection that identifies the report associated with features of the display device.

ASCII Character Set
SF - Finding this usage in a display application descriptor indicates that the device “ASCII character set” is compatible. See Display ASCII Character Set below.

Unicode Character Set
SF - Finding this usage in a display application descriptor indicates that the device displays Unicode character set. If defined, it is implied that 16 bit characters will be transferred in the Display Data field and the Buffered Bytes flag is set.

Data Read Back
SF - Finding this usage in a display application descriptor indicates that the Character Report can be read back. Otherwise the display data is a write-only.

Font Read Back
SF - Finding this usage in a display application descriptor indicates that the Font Report can be read back. Otherwise the display font is a write-only.

## Display Control

Display Control Report
CL - This usage applies the logical collection that identifies the report associated with controlling the features of the display device.

Clear Display
DF - Clears the display to blanks (spaces) and returns the cursor to the home position. This is a write-only control. Returns 0 when read. A single bit data field where: 0 = No operation, 1 = The display is cleared.

Display Enable
DF - Turns the display on or off. A single bit data field where: 0 = display off, 1 = display on. If absent from the Report Descriptor assume that the display is always enabled.

Screen Saver Delay
SV or DV – The delay in ms. between setting the Screen Saver Enable bit and the screen save operation actually taking place.

Screen Saver Enable
DF – When enabled, the display will either put up a vendor defined screen saver or turn the display off after the Screen Saver Delay. If absent from the Report Descriptor assume that the display does not support this feature.

## Scrolling

If Horizontal and Vertical Scrolling are disabled, then characters received after the cursor reaches the right-most column (Column = Columns) will overwrite each other.

If Vertical Scrolling is enabled, then if the cursor is on the last character of a row (Column = Columns), other than the last row (Row != Rows), then the next character received will cause the vertical cursor position to be incremented  (Row++) and the horizontal cursor position set to 0 (Column = 0). If the cursor is on the last character (Column = Columns) of the last row (Row = Rows), then the next character will cause all rows are scrolled up, the last row is cleared, and the horizontal cursor position set to 0 (Column = 0).

If Horizontal Scrolling is enabled and the cursor is on the last character of a row (Column = Columns), then when the next character is received the row will scroll horizontally one character position and the character will be placed on the last column of the row.

NOTE: There are three scrolling modes: none, horizontal and vertical. Only one can be operative at a time. i.e. Horizontal Scrolling and Vertical Scrolling are mutually exclusive.

NOTE: Vertical Scroll or Horizontal Scroll are consider as Static Flags, i.e. if they are not declared then it can be assumed that the mode is not supported. However, if defined as a variable the mode can be enabled or disabled.

NOTE: Any data that scrolls off the display is lost.

Vertical Scroll
DF - Identifies the whether the display will scroll vertically, where: 0 = the display will not scroll vertically, 1 = the display will scroll vertically.

Horizontal Scroll
DF - Identifies the whether the row will scroll horizontally, where: 0 = the display will not scroll horizontally, 1 = the display will scroll horizontally.

## Character Transfers

An Alphanumeric display can be configured to read or write multiple characters in a single message.

Character Report
CL - This usage applies the logical collection that identifies the report associated with character data movement. Flow control is handled by the display NAKing Character reports until it is ready for more characters.

Display Data
The report field that is written to pass characters to the display. When read the character currently indicated by the cursor is returned.

If this field is declared with a Report Count greater than 1 any non NoOp characters will be written to the display. When the same field is read all characters from the current Cursor position forward will be returned. If the range goes beyond the end of the display memory NoOp’s will be returned.

Buffered Byte – When the Display Data field as Buffered Bytes the data in the array is used as an index in to the character ROM of the device. The Report Size will reflect the size of the character set supported by the device.

NAry - If a vendor wishes to identify specific characters other than those found in the ASCII Character Set and does not want to send 16 bit Unicode characters to the display, they can describe the Display Data field as an Named Array where the selector usages are pulled from the Unicode page.

## Display Status

A display will initially generate a Stat Not Ready status until the display is fully initialized. No commands should be issued to the display until the Stat Ready condition is detected. Any error will held in Display Status field until it is read, at which point the Display Status field will return to the Stat Ready condition or be set to the next error code.

Display Status
CL - This is a collection of the status codes that the display supports. The status codes are reported in a single location array.

Stat Not Ready
Sel - The display is not ready for use. These displays are typically slow to initialize.

Stat Ready	Sel - The display is ready for use. No commands can be issued until the Display Status indicates Stat Ready.

Err Not a loadable character
Sel - This error will occur after an attempt is made to load  non-loadable character location.

Err Font data cannot be read
Sel - This error will occur after an attempt is made to read the font bit-map of a character location and the display does not support it.

## Cursor Control

There are two ways of handling cursor positioning. 1) The Row and Column fields may be declared with a report ID that is different from that used by the Data field. This will allow cursor positioning to be independent of writing characters to the display. 2) Or the Row, Column, and data can all be in the same report. If either the Row or Column fields contain an out of range value then the cursor position will not be updated.

In a Display device the cursor position is applied first then any characters are written to the display buffer. Inputting the Row and Column will provide the current cursor position.

If Cursor Mode fields equals increment (1) then nothing will happen if a character is entered when the cursor is on the last column of the last row. If a character is entered when the cursor is on the last column of any other row the cursor will move the first column of the next row.

If Cursor Mode fields equals decrement (0) then nothing will happen if a character is entered when the cursor is on the first column of the first row. If a character is entered when the cursor is on the first column of any other row the cursor will move the last column of the next row.

Cursor Position Report
CL - This usage applies the logical collection that identifies the report associated with cursor positioning.

Row 	DV - Identifies or sets the vertical character position that the cursor is on. 0 is the top most row.

Column	DV - Identifies or sets the horizontal character position that the cursor is on. 0 is the leftmost column.

Rows 	SV - Identifies the number of Rows supported by the display.

Columns	SV - Identifies the number of Columns supported by the display.

Cursor Pixel Positioning
SF - Finding this usage in a Report Descriptor indicates that the display supports pixel level cursor positioning. A single bit data field where: 1 = Cursor Pixel Positioning enabled, 0 = Character cursor positioning enabled. Not finding this usage implies that the display only supports character level positioning.

If pixel positioning is supported then the Character Width plus the Character Spacing Horizontal indicate the number of pixels the cursor will move horizontally if Character positioning is enabled. And the Character Height plus the Character Spacing Vertical indicate the number of pixels the cursor will move vertically if Character positioning is enabled.

Note: If pixel positioning is supported then the Row and Column fields must be large enough to contain a character or pixel address.

Cursor Mode
DF - Sets the cursor movement direction. After each character code is sent to the display the cursor can automatically move either right or left. A single bit data field where: 0 = decrement cursor position (move left), 1 = increment cursor position (move right). If absent from the Report Descriptor assume that increment cursor position is always enabled.

Cursor Enable
DF - Turns the cursor on or off. A single bit data field where: 0 = cursor off, 1 = cursor on. If absent from the Report Descriptor assume that the cursor is always enabled.

Cursor Blink
DF - Turns the cursor blinking on or off. A single bit data field where: 0 = blink off, 1 = blink on. If absent from the Report Descriptor assume that Blink is off.

## Font Loading

Alphanumeric displays that support loadable fonts will contain the following usages.

To download a font the report must contain the row and column of the destination and a buffered bytes data field that contains Character Width times Character Height bits of data.

Font Data is organized as sequential rows of pixels where the least significant bit contains the pixel in the upper right-and corner of the character.

Not all displays support downloading of all character locations so the Display Status field should be checked after each download to ensure that the operation completed successfully. A Err Not a loadable character value will be returned in the Display Status field if an error occurred.

Not all displays support uploading of all character locations so the Display Status field should be checked after each upload to ensure that the operation completed successfully. A Err Font data cannot be read value will be returned in the Display Status field if an error occurred.

Font Report
CL - Finding this usage in a display application descriptor indicates that the display supports downloadable fonts. This usage is applied to a logical collection that defines the font download report.

Font Data	A buffered bytes data field that contains the font data.

The following Usages define display parameters. These are normally static values defined in a feature report.

The Character Spacing Horizontal and Character Spacing Vertical values indicate whether the inter-character spacing must be included in the downloaded font character or it is forced by the pixel layout of the display. A Character Spacing of 0 implies that any inter-character spacing must be included in the downloaded font. If a Character Spacing usage is not declared then it can be assumed that the respective inter-character spacing is forced by the pixel layout of the display and can be assumed to be 1.

Character Width
SV - Identifies the width of a character in pixels.

Character Height
SV -Identifies the height of a character in pixels.

Character Spacing Horizontal
SV -Identifies the horizontal distance between characters in pixels.

Character Spacing Vertical
SV -Identifies the vertical distance between characters in pixels.

## Display ASCII Character Set

The Display ASCII Character Set defines a minimum character set that will be supported by a display. The blank character locations in the table may be optionally defined by a vendor. All characters will be passed to the display, so to take advantage of the other characters the controlling application must know vendor specific information. The total number of character codes supported is vendor specific.

In the following table the high nibble of the character code is labeled across the top and the low nibble is labeled down the left side.

- Table 17: ASCII Display Character Set

| Low | High Nibble |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Nibble | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 0 | NoOp |  | Space | 0 | @ | P | ` | p |
| 1 |  |  | ! | 1 | A | Q | a | q |
| 2 |  |  | “ | 2 | B | R | b | r |
| 3 |  |  | # | 3 | C | S | c | s |
| 4 |  |  | $ | 4 | D | T | d | t |
| 5 |  |  | % | 5 | E | U | e | u |
| 6 |  |  | & | 6 | F | V | f | v |
| 7 |  |  | ‘ | 7 | G | W | g | w |
| 8 |  |  | ( | 8 | H | X | h | x |
| 9 |  |  | ) | 9 | I | Y | i | y |
| A |  |  | * | : | J | Z | j | z |
| B |  |  | + | ; | K | [ | k | { |
| C |  |  | , | < | L | \ | l | ¦ |
| D |  |  | - | = | M | ] | m | } |
| E |  |  | . | > | N | ^ | n |  |
| F |  |  | / | ? | O | _ | o |  |

Where:

NoOp	On operation is performed if received.

Space	Clears the character position.

# Monitor Pages

Refer to the Monitor Class Specification.

# Power Pages

Refer to the Power Class Specification.

# Bar Code Scanner Pages

Refer to the Bar Code Scanner Usage Supplement.

# Point of Sale Pages

Refer to the Point of Sale Class Usage Supplement.

# Appendix A: Usage Examples

This section provides examples of common implementations of controls. A pseudo code is used to describe the Main, Global and Local items.

There are a number cases where Usage Minimum or Usage Maximum could have been used in these examples but were not for clarity purposes.

## Volume Control

Here are two examples of volume controls. The first on defines a pair of buttons that are used to ramp the volume up and down and the second example is a normal volume knob.

### Up/Down Buttons

To define a pair of buttons that ramp a variable i.e. Volume Up and Volume Down buttons. The Input device must be defined as Relative. A value of -1 will reduce and +1 will increase the volume at a rate determined by the vendor. A value of 0 will have no effect on the volume.

UsagePage(Consumer)

Usage(Volume)

Logical Minimum(-1), Logical Maximum(1),

ReportSize(2), ReportCount(1),

Input(Data, Variable, Relative)

### Knob

How to define a Volume knob that turns 270°:

UsagePage(Consumer)

Usage(Volume)

Logical Minimum(0), Logical Maximum(100),

ReportSize(7), ReportCount(1),

Input(Data, Variable, Absolute, No Wrap, Linear, No Preferred)

The Logical Minimum and Logical Maximum values depend on the resolution provided by the vendor. Because the knob only turns 270 degrees the No Wrap flag is set. A volume control usually generates an analog output using an audio taper, however in this example the volume control simply generates a Linear output as a function of its physical position from 0 to 100 percent. The controlling application would apply the audio taper to the output. The No Preferred flag is true because the control will remain in the last position that the user left it in.

## Tape Jog Wheel

A tape jog wheel is a spring loaded knob that rotates ±90°, with a small indent for the users index finger. As the user twists the knob right or left the tape is advanced or backed up at a rate that is proportion to the rotation from the spring loaded center position.

UsagePage(Consumer)

Usage(Tape Jog)

Logical Minimum(-127), Logical Maximum(127),

ReportSize(8), ReportCount(2),

Input(Data ,Variable, Relative, No Wrap, Linear, Preferred)

The Preferred flag is true because the control will return to the center position when the user releases it.

## Radio Buttons

Radio buttons are a group of mutually exclusive buttons. In this example an audio receiver uses 3 radio buttons to select between an Computer, DVD or WWW as a display source.

### Mechanically Linked

Classically, radio buttons implementations have had a mechanical system releases any buttons not pressed and holds the last pressed button in an active state until another button is pressed. In the example below one of three codes will be returned; MS Computer, MS DVD or MS WWW.

Logical Minimum(1), Logical Maximum(3),

UsagePage(Consumer),

Usage(MS Computer),

Usage(MS DVD),

Usage(MS WWW),

ReportSize(2), ReportCount(1),

Input(Data, Array, Absolute, No Wrap, Linear, No Preferred, No Null Position)

The No Preferred flag is set because the report will always present the value of the last button pressed. The No Null Position flag indicates that there is never a state in which it is not sending meaningful data. The returned values are 1 =  Computer, 2 = DVD or 3 = WEB.

### No Mechanical Linkage

Many systems today use a separate display to indicate the current selection and there is no mechanical connection between the buttons. In this example the control will return one of four codes; No Event, Aux, CD or Tuner.

Logical Minimum(1), Logical Maximum(3),

UsagePage(Consumer),

Usage(Computer),

Usage(DVD),

Usage(WWW),

ReportSize(2), ReportCount(1),

Input(Data, Array, Absolute, No Wrap, Linear, Preferred, Null Position)

The No Preferred flag is set because the report will present a Null value (0) when the user is not touching the a button. The Null Position flag indicates that there is a state in which the control is not sending meaningful data. The returned values are Null (a value outside of the Logical Min/Max range) = No Event, 1 =  Computer, 2 = DVD or 3 = WWW.

## Named Array Field

To simplify application finding a “one of many” set of controls the Array Field associated with it, can be named by wrapping the Array declaration in a Logical Collection.

In the following example the device returns one of three status codes: Not Ready, Ready, and Error Not a loadable character. An application can simply query for the Display Status usage to find the Array Field that will contain the status codes.

ReportSize(2), ReportCount(1),

Logical Maximum(2),

Usage(Display Status),

Collection(Logical),

Usage(Stat Not Ready),

Usage(Stat Ready),

Usage(Err Not a loadable character),

Input(Data, Array, Absolute, No Null Position), 	; 3-bit status field

End Collection(),

The No Null Position flag indicates that there is never a state in which it is not sending meaningful data. The returned values are 1 =  Stat Not Ready, 2 = Stat Ready or 3 = Err Not a loadable character.

## Multiple Instances of a Control

This example shows how to implement multiple instances of a set of controls by defining a device with two pointers each with X and Y axes. An application looking for Pointers would find two of each type enumerated.

UsagePage(Generic Desktop),

Usage(Pointer),		;Pointer

Collection(Logical),

UsagePage(Ordinal),

Usage(Instance 1),	; pointer 1

Collection(Physical),

UsagePage(Generic Desktop),

Usage(X-axis),

Usage(Y-axis),

Collection End,

UsagePage(Ordinal),

Usage(Instance 2),

Collection(Physical),

UsagePage(Generic Desktop),

Usage(X-axis),

Usage(Y-axis),

Collection End,

Collection End,

## Multiple Instances of a Multi Mode LED

This example shows how to implement an Indicator that supports blinking as well as multiple colors. In this example there are 2 LEDs (Play and Stop) that can be On, Blinking or Off and when they are illuminated they can be either Red, Green or Orange. The LED page provides Slow and Fast blinking usages, either one could have been chosen here to enable the single blinking mode that this device supports.

Declare the globals that are used by all the main items.

Report Size(2),

Report Count(1),

Logical Minimum(1),

Logical Maximum(3),

Declare the Play LED.

Usage Page(Consumer),

Usage Minimum(Play),

Collection(Logical),

Usage Page(LED),

Usage(Usage Multi Mode Indicator),		; Declare Mode field

Collection(Logical),

Usage(Indicator On),

Usage(Indicator Slow Blink),

Usage(Indicator Off),

Feature(data, Array, Null),	; 3 Modes supported

End Collection(),

Usage(Usage Indicator Color),			; Declare Color field

Collection(Logical),

Usage(Red),		;   of the LED.

Usage(Green),

Usage(Orange),

Feature(data, Array, Null), 	; 3 Colors supported

End Collection(),

;Add the actual control here.

End Collection(),

Declare the controls for the Stop LED.

Usage Page(Consumer),

Usage Minimum(Stop),

Collection(Logical),

Usage Page(LED),

Usage(Usage Multi Mode Indicator),

Collection(Logical),

Usage(Indicator On),

Usage(Indicator Slow Blink),

Usage(Indicator Off),

Feature(data, Array, Null),

End Collection(),

Usage(Usage Indicator Color),

Collection(Logical),

Usage(Red),

Usage(Green),

Usage(Orange),

Feature(data, Array, Null),

End Collection(),

End Collection(),

End Collection(),

- Figure 1: LED Report fields

| Bit |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| Generic LED 2 Color |  | Generic LED 2 Mode |  | Generic LED 1 Color |  | Generic LED 1 Mode |  |

## Desktop Tablet Example

This is the Report Descriptor for a typical desktop digitizing tablet. The  tablet's digitizing region is 12" square, and it reports data in units of .001 inches.  It is optionally equipped with any or all of three cordless transducers: a 16-button cursor, a stylus with a tip and a barrel switch, and a stylus with a pressure transducer.

The example digitizer can distinguish between the different cursors, and sends a report based on the cursor that last changed state.  The Report ID report data entity indicates which cursor is generating the current report.  The X and Y position data and the In Range bit are in the same field for each report type, but the pressure and button data are different for each transducer, with padding in the report where necessary.  The vanilla stylus and puck transducers generate 6-byte reports, whereas the pressure stylus generates a 7-byte report.

The report descriptor below is structured as an application collection containing three physical collections - one for each supported cursor.  The Report ID items precede each cursor collection, which causes a separate, tagged report to be defined for each cursor.  The Push and Pop items are used to save and restore the item state that defines the X and Y fields.  The report descriptor takes advantage of the fact that the tablet is square - that is, the physical and logical ranges of X and Y position are identical.

- ;; Example Digitizer Report Descriptor

Usage Page(Digitizers),				; application collection

Usage(Digitizer),

Collection(Application),

ReportID(1), 					; 2-Button Stylus

Usage(Puck),

Collection(Physical),

Usage Page(Generic Desktop),		; X and Y Position

Usage(X), Usage(Y),

ReportSize(16), ReportCount(2),

Logical Minimum(0), Logical Maximum(12000),

Physical Minimum(0), Physical Maximum(12),

Units(English Linear: Distance),	; inches

Exponent(0),

Push,					; (save position item state)

Input(Data, Variable, Absolute),

Usage Page(Digitizers),

Usage(In Range),			; In Range bit, switches

Usage(Barrel Switch),

Usage(Tip Switch),

Logical Minimum(0), Logical Maximum(1),

Physical Minimum(0), Physical Maximum(1),

Units(None),

Report Size(1), Report Count(3),

Input(Variable, Absolute),

Report Count(1), Report Size(5), 	; padding (5 bits)

Input(Constant),

End Collection,

Report ID(2),					; 16-Button Cursor Tag

Usage(Stylus),

Collection(Physical),

Pop,		; Refer to Global items saved during last Push

Push,

; Report Count (2)

Usage(X), Usage(Y), 			; X and Y position usages

Input(Data, Variable, Absolute),

Usage Page(Digitizer),

Usage(In Range),			; In Range bit

Logical Minimum(0), Logical Maximum(1),

Physical Minimum(0), Physical Maximum(1),

Units(None),

Report Size(1), Report Count(1),

Input(Data, Variable, Absolute),

Usage Page(Buttons),			; button index

Usage Minimum(0), Usage Maximum(16),

Logical Maximum(16),

Report Size(5), Report Count(1),

Input(Data, Array, No Null Position),

Report Count(1), Report Size(2),	; padding (2 bits)

Input(Constant),

End Collection,

Report ID(3),					; Pressure Stylus Tag

Usage Page(Digitizer),

Usage(Stylus),

Collection(Physical),

Pop, 		; Refer to Global items saved during initial Push

Usage(X), Usage(Y),			; X and Y position usages

Input(Date, Variable, Absolute),

Logical Minimum(0), Logical Maximum(1),

Physical Minimum(0), Physical Maximum(1),

Units(None),

Report Size(1), Report Count(6),	; padding (6 bits)

Input(Constant),

Usage Page(Digitizer),

Usage(In Range),			; In Range bit, barrel switch

Usage(Barrel Switch),

Report Count(2),

Input(Variable, Absolute),

Usage(Tip Pressure),	 		; tip pressure

Logical Minimum(0), Logical Maximum(127),

Physical Minimum(0), Physical Maximum(45),

Units(SI Linear: Force), Exponent(4),

Report Size(8), Report Count(1),

Input(Variable, Absolute, Non Linear),

End Collection,

End Collection

- Figure 2: Example Digitizer 2-Button Stylus Input Report

|  | Bit |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| 0 | ReportID = 1 (indicating 2-button stylus report) |  |  |  |  |  |  |  |
| 1 | X Coordinate Bits 0-7 |  |  |  |  |  |  |  |
| 2 | X Coordinate Bits 8-15 |  |  |  |  |  |  |  |
| 3 | Y Coordinate Bits 0-7 |  |  |  |  |  |  |  |
| 4 | Y Coordinate Bits 8-15 |  |  |  |  |  |  |  |
| 5 | Pad |  |  |  |  | In Range | Barrel Switch | Tip Switch |

- Figure 3: Example Digitizer 16-Button Puck Input Report

|  | Bit |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| 0 | ReportID = 2 (indicating 16-button puck transducer report) |  |  |  |  |  |  |  |
| 1 | X Coordinate Bits 0-7 |  |  |  |  |  |  |  |
| 2 | X Coordinate Bits 8-15 |  |  |  |  |  |  |  |
| 3 | Y Coordinate Bits 0-7 |  |  |  |  |  |  |  |
| 4 | Y Coordinate Bits 8-15 |  |  |  |  |  |  |  |
| 5 | Pad |  | Button Index |  |  |  |  | In Range |

- Figure 4: Example Digitizer Pressure Stylus Input Report

|  | Bit |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| 0 | ReportID = 3 (indicating pressure stylus report) |  |  |  |  |  |  |  |
| 1 | X Coordinate Bits 0-7 |  |  |  |  |  |  |  |
| 2 | X Coordinate Bits 8-15 |  |  |  |  |  |  |  |
| 3 | Y Coordinate Bits 0-7 |  |  |  |  |  |  |  |
| 4 | Y Coordinate Bits 8-15 |  |  |  |  |  |  |  |
| 5 | In Range | Barrel Switch | Unused |  |  |  |  |  |
| 6 | Pressure Bits 0-7 |  |  |  |  |  |  |  |

## A device with a Display

The following example is of a 2x16 character display device. The device uses 1 Feature, 1 Input, and 2 Output reports.

A Feature Report is declared for identifying fixed features of the display and display status. All the feature report’s fields are constants.

The Character Spacing usage is not declared so it can be assumed that the respective inter-character spacing is forced by the pixel layout of the display and any downloaded font characters do not have to include it.

In this example the Character Height and Width are fixed. The fields are declared in the report descriptor and the actual values are reported when the Feature report is read. i.e. the Character Height and Width fields will always return 7 and 5, respectively.

Uploading of the font is not supported by this display so there is no Font Report Input report.

UsagePage(Alphanumeric Display),

Usage(Alphanumeric Display),

Logical Minimum(0),

Collection(Application),

The first report defined is a Feature report with 7 fields. The Rows, Columns, Character Height and Width fields are Static Values and their report size is set to 5 to demonstrate how the bit packing takes place in a report. Standard Character Set, Data Read Back and Vertical Scroll are Static Flags.

Usage(Display Attributes Report),

Collection(Logical),

Usage(Rows),						; Constant = 2

Usage(Columns),					; Constant = 16

Usage(Character Width),				; Constant = 5

Usage(Character Height),				; Constant = 7

ReportID(1),

Logical Maximum(31),

ReportSize(5), ReportCount(4),

Feature(Constant, Variable, Absolute),		; 4, 5-bit fields

ReportSize(1), ReportCount(3),

Logical Maximum(1),

Usage(ASCII Character Set),				; Constant = 1

Usage(Data Read Back),					; Constant = 1

Usage(Vertical Scroll),				; Constant = 1

Feature(Constant, Variable, Absolute),		; 3, 1-bit fields

ReportCount(1),

Feature(Constant, Variable, Absolute),		; 1-bit pad

End Collection(),

The second report defined is an Input report that is generated on the interrupt endpoint each time the status of the display changes. Each of the possible states that can be identified by the display are identified in the Display Status collection. This report can also be read over the control pipe to determine the current status.

ReportSize(8), ReportCount(1),

Logical Maximum(2),

Usage(Display Status),

Collection(Logical),

Usage(Stat Not Ready),

Usage(Stat Ready),

Usage(Err Not a loadable character),

Input(Data, Array, Absolute, No Null), 	; 8-bit status field

End Collection(),

A second Feature report is defined for getting or setting the current cursor position.

Usage(Cursor Position Report),

Collection(Logical),

ReportID(2),

ReportSize(4), ReportCount(1),

Logical Maximum(15),

Usage(Column),

Feature(Data, Variable, Absolute, No Preferred State), ;Column

Logical Maximum(1),

Usage(Row),

Feature(Data, Variable, Absolute, No Preferred State), ;Row

End Collection(),

There are a number of ways that data can be transferred between the host and the display: a byte a time, multiple bytes, or the whole screen using a 32-byte Buffered Byte transfer. The choice may depend on whether the device is implemented as a low-speed or a high-speed device. In this example a third Feature report is defined for writing up to 4 sequential characters from the display in a single report. Note that the Data Read Back usage is not declared in the Report Descriptor which implies that the display character data is write only.

Usage(Character Report),

Collection(Logical),

ReportID(3),

ReportSize(8), ReportCount(4),

Logical Maximum(126),

Usage(Display Data),

Feature(Data, Variable, Absolute, Buffered Bytes),	;4-byte data buffer

End Collection(),

A fourth Feature report is defined for updating the font. The Display Data field identifies the character to be modified. And because the Character Height = 7 and the Character Width = 5, 35 bits will be required for a font character. A 40 bit buffered byte field (5x8) is declared to contain the font data.  Note that the Data Read Back usage is not declared in the Report Descriptor which implies that he display font data is write only.

ReportID(4),

Usage(Font Report),

Collection(Logical),

Logical Minimum(0), Logical Maximum(126),

ReportSize(8), ReportCount(1),

Usage(Display Data),

Output(Data, Variable, Absolute),	;Character to write

ReportCount(5),			;assumes a 5x7 font, 35 bits

Usage(Font Data),

Output(Data, Variable, Absolute, Buffered Bytes), 	; Font Data

End Collection(),

End Collection()

- Figure 5: Example Display Attributes Feature Report

|  | Bit |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| 0 | Report ID = 1 |  |  |  |  |  |  |  |
| 1 | Columns (bits 2-0) = 16 |  |  | Rows = 2 |  |  |  |  |
| 2 | Character Height (bits 1-0) | Character Width = 5 |  |  |  |  | Columns (bits 4-3) |  |
| 3 | Pad | Vertical Scroll = 1 | Data Read Back = 1 | ASCII Character Set = 1 | Character Height (bits 5-2) = 7 |  |  |  |

- Figure 6: Example Display Device Input Report

|  | Bit |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| 0 | Report ID = 1 |  |  |  |  |  |  |  |
| 1 | Display Status |  |  |  |  |  |  |  |

- Figure 7: Example Display Device Display Position Feature Report

|  | Bit |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| 0 | Report ID = 2 |  |  |  |  |  |  |  |
| 1 | Row |  |  |  | Column |  |  |  |

- Figure 8: Example Display Device Display Data Feature Report

|  | Bit |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| 0 | Report ID = 3 |  |  |  |  |  |  |  |
| 1 | Display Data 0 |  |  |  |  |  |  |  |
| 2 | Display Data 1 |  |  |  |  |  |  |  |
| 3 | Display Data 2 |  |  |  |  |  |  |  |
| 4 | Display Data 3 |  |  |  |  |  |  |  |

- Figure 9: Example Display Device Font Load Output Report

|  | Bit |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| 0 | Report ID = 4 |  |  |  |  |  |  |  |
| 1 | Display Data = Character to update |  |  |  |  |  |  |  |
| 2 | Font Data 0 |  |  |  |  |  |  |  |
| 3 | Font Data 1 |  |  |  |  |  |  |  |
| 4 | Font Data 2 |  |  |  |  |  |  |  |
| 5 | Font Data 3 |  |  |  |  |  |  |  |
| 6 | Font Data 4 |  |  |  |  |  |  |  |

## Remote Control

The Remote control in this example has 24 buttons with the following labels:

A number pad with 10 digits 1-9 and 0
Channel Up, Channel Down,
Volume Up and Volume Down,
Mute,
Power,
Sleep Timer,
On Screen,
Enter Choice, Choice 1, Choice 2, Choice 3,
Broadcast,
Return.

UsagePage(Consumer),

Usage(Consumer Control),

Collection(Linked),

Usage(Numeric Key Pad),

Collection(Logical),

UsagePage(Button);

Usage(Button 1);			; ‘0’

Usage(Button 2);			; ‘1’

Usage(Button 3);			; ‘2’

Usage(Button 4);			; ‘3’

Usage(Button 5);			; ‘4’

Usage(Button 6);			; ‘5’

Usage(Button 7);			; ‘6’

Usage(Button 8);			; ‘7’

Usage(Button 9);			; ‘8’

Usage(Button 10);			; ‘9’

Logical Minimum(1), Logical Maximum(10),

ReportSize(4), ReportCount(1)

Input(Data, Array, Absolute, Null State)

End Collection(),

UsagePage(Consumer Devices),

Usage(Channel),				; Channel buttons

Usage(Volume),					; Volume buttons

Logical Minimum(-1), Logical Maximum(1),

ReportSize(2), ReportCount(2)

Input(Data, Variable, Relative, Preferred),

Usage(Mute),					; Mute

Usage(Power),					; Power

Usage(Sleep Mode),				; Sleep

Usage(Data On Screen),				; On Screen

Usage(Broadcast Mode),				; Broadcast

Usage(Selection Back),				; Return

Usage(Assign Selection),			; Enter Choice

Logical Minimum(1), Logical Maximum(7),

ReportSize(4), ReportCount(1)

Input(Data, Array, Absolute, Null State),

Usage(Selection),

Collection(Logical),				; 3 Choice buttons

UsagePage(Button),

Usage(Button 1),			; Choice 1

Usage(Button 2),			; Choice 2

Usage(Button 3),			; Choice 3

Logical Minimum(1), Logical Maximum(3),

ReportSize(2), ReportCount(1)

Input(Data, Array, Absolute, Null State),

End Collection(),

Logical Minimum(1), Logical Maximum(2),

ReportSize(2), ReportCount(1)

Input(Constant, Variable, Absolute),		; 2 bit pad

End Collection(),

The resulting report will look like:

- Figure 10: Example Remote Control Input Report

|  | Bit |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| 0 | Volume |  | Channel |  | Numeric Keypad Values |  |  |  |
| 1 | Pad |  | Choice Buttons |  | Remaining Buttons |  |  |  |

## Telephone

This is an example of a speaker phone with the following features:

- 6 programmable buttons each with an In Use indicator LED. The first two Programmable Buttons also have LEDs that can be used to indicate that the function (line) is Selected but not necessarily in use.

- A Message Waiting indicator that can blink when the voice mail box is full.

- A standard telephone keypad.

- 6 permanently marked buttons: Alternate Function, Conference, Transfer, Drop, Hold, Speaker Phone, Volume Up and Volume Down.

- An In Use indicators for the Alternate Function button.

- An Off-Hook indicator used by the handset.

;Declare all the inputs

ReportCount(1),

UsagePage(Telephony Devices),

Usage(Phone),

Collection(Application),

Usage(Programmable Button)

Collection(Logical)

UsagePage(Button),

Usage Minimum(Button 1), Usage Maximum(Button 6),

ReportSize(3),

Logical Minimum(1), Logical Maximum(6),

Input(Data, Array, Absolute, Null State),; 3-bit buffer for prog buttons

End Collection(),

UsagePage(Telephony Devices),

Usage(Telephony Key Pad),

Collection(Logical),

Usage Page(Button),

Usage Minimum(Button 1), Usage Maximum(Button 12),

Logical Maximum(12),			; 12 buttons

ReportSize(4),

Input(Data, Array, Absolute, Null State), ;4-bit field, key pad buttons

End Collection(),

UsagePage(Telephony Devices),

Usage(Hook Switch),

Usage(Alternate Function),

Usage(Conference),

Usage(Transfer),

Usage(Drop),

Usage(Hold),

Usage(Speaker Phone),

Logical Maximum(7),				; 7 buttons

ReportSize(3),

Input(Data, Array, Absolute, Null State),	; 3-bit field for misc. buttons

UsagePage(Consumer Devices),

Usage(Volume),

Logical Minimum(-1),

Logical Maximum(1),

ReportSize(2),

Input(Data, Variable, Absolute),		; 2-bit field for volume

;Pad to byte boundary

ReportSize(4), ReportCount(1),

Input(Constant),				; 4-bit pad

;Declare all the indicator outputs (LEDs)

;Enabled LEDs associated with Programmable buttons 1 and 2

Logical Minimum(0), Logical Maximum(1),

UsagePage(LEDs),

Usage(Usage Selected Indicator),

Collection(Logical),

UsagePage(Telephony Devices),

Usage(Programmable Buttons),

Collection(Logical),

UsagePage(Button),

Usage Minimum(Button 1), Usage Maximum(Button 2),

ReportCount(2),

Output(Data, Variable, Absolute),

End Collection(),

End Collection(),

;In Use LEDs associated with Programmable buttons 1 through 6

; Message Waiting, and Alternate Function.

UsagePage(LEDs)

Usage(Usage In Use Indicator)

Collection(Logical)

UsagePage(Telephony Devices)

Usage(Programmable Key)

Collection(Logical)

UsagePage(Button),

Usage Minimum(Button 1), Usage Maximum(Button 6),

ReportCount(6),

Output(Data, Variable, Absolute),

End Collection(),

UsagePage(Telephony Devices),

Usage(Alternate Function),

ReportCount(1),

Output(Data, Variable, Absolute),

End Collection(),

UsagePage(LEDs)

Usage(Usage Multi Mode Indicator)

Collection(Logical)

UsagePage(Telephony Devices)

Usage(Message),

Collection(Logical),

Usage(Indicator On),

Usage(Indicator Fast Blink),

Usage(Indicator Off),

ReportSize(2),

Output(Data, Array),

End Collection(),

End Collection(),

;Volume Control

UsagePage(Consumer),

Usage(Volume),					; Volume buttons

Logical Minimum(-1), Logical Maximum(1),

ReportSize(2), ReportCount(1)

Output(Data, Variable, Relative, Preferred),	; 2-bit field for volume

;Pad to byte boundary

ReportSize(3), ReportCount(1),

Output(Constant),				; 3-bit pad

End Collection()

In the following Telephony Device Report the Key Usage Value is an input to the system returning any pressed keys. All the Indicator bits are outputs (LEDs).

- Figure 11: Example Telephony Device Input Report

|  | Bit |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| 0 | Misc. Buttons Bit 0 | Telephony Key Pad |  |  |  | Programmable Keys |  |  |
| 1 | Pad |  |  |  | Volume |  | Misc. Buttons Bits 2-1 |  |

- Figure 12: Example Telephony Device Output Report

|  | Bit |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| 0 | In Use Indicator Prog. Button 6 | In Use Indicator Prog. Button 5 | In Use Indicator Prog. Button 4 | In Use Indicator Prog. Button 3 | In Use Indicator Prog. Button 2 | In Use Indicator Prog. Button 1 | Selected Indicator Prog. Button 2 | Selected Indicator Prog. Button 1 |
| 1 | Constant pad = 0 |  |  | Volume |  | In Use Indicator Alternate Function | Multi Mode Indicator Message Waiting |  |

## Joystick

This is an example of a joystick with the following features:

- A 2 axis stick that tilts forwards/backwards and right/left

- A Throttle control on the base

- A 4 position Hat switch on the stick

- 2 buttons on the stick

- 2 buttons on the base

UsagePage(Generic Desktop)

LogicalMin(0)

Usage(Joystick)

Collection(Application)

UsagePage(Simulation Controls),

Usage (Throttle),

Logical Minimum (-127),

Logical Maximum (127),

Report Size (8),

Report Count (1),

Input (Data, Variable, Absolute),

UsagePage(Generic Desktop)

Usage (Pointer),

Collection (Physical),

Usage (X),

Usage (Y),

Report Count (2),

Input (Data, Variable, Absolute),

End Collection(),

Usage (Hat switch),

Logical Minimum (0), Logical Maximum (3),

Physical Minimum (0), Physical Maximum (270),

Unit (English Rotation: Angular Position), 	; degrees

Unit Exponent (0),

Report Size (4), Report Count (1),

Input (Data, Variable, Absolute, Null State),

Usage Page (Buttons), 	; buttons on the stick

Usage Minimum (Button 1),

Usage Maximum (Button 4),

Logical Minimum (0), Logical Maximum (1),

Physical Minimum (0), Physical Maximum (1),

Report Count (4),

Report Size (1),

Unit (None),

Input (Data, Variable, Absolute)

- End Collection()

- Figure 13: Example Joystick Input Device Report

|  | Bit |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| 0 | Report ID = 1 |  |  |  |  |  |  |  |
| 1 | Throttle |  |  |  |  |  |  |  |
| 2 | X-axis |  |  |  |  |  |  |  |
| 3 | Y-axis |  |  |  |  |  |  |  |
| 4 | Button 4 | Button 3 | Button 2 | Button 1 | Hat Switch |  |  |  |

## Game Pad

This is an example of a game pad with the following features:

- A 2 axis rocker that tilts forwards/backwards and right/left

- 6 buttons

UsagePage(Generic Desktop)

Usage(Game Pad)

Collection(Application)

Usage (Pointer),

Collection (Physical),

Usage (X),

Usage (Y),

Logical Minimum (-1), Logical Maximum (-2),

Report Count (2), Report Size (2),

Input (Data, Variable, Absolute, No Null),

End Collection(),

Report Count (4),

Report Size (1),

Input (Constant, Variable, Absolute)		// 4 bit Pad

Usage Page (Buttons), 	; buttons on the stick

Usage Minimum (Button 1),

Usage Maximum (Button 6),

Logical Minimum (0), Logical Maximum (1),

Report Count (6),

Report Size (1),

Input (Data, Variable, Absolute)

Report Count (2),

Input (Constant, Variable, Absolute)		// 2 bit Pad

- End Collection()

- Figure 14: Example Game Pad Input Device Report

|  | Bit |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| 0 | Pad |  |  |  | Y-axis |  | X-axis |  |
| 1 | Pad |  | Button 6 | Button 5 | Button 4 | Button 3 | Button 2 | Button 1 |

# Appendix B: Delimiter Example

Delimiters allow a device to declare multiple uses for a control.

Ideally, an application like a flight simulator would look for the controls that are specific to it. It would search for Flight Simulation Devices attached to the system, once found it would look for controls (usages) like Flight Control Stick, Trigger, Rudder, Throttle, Landing Gear, Toe Brake, etc. The problem with a flight stick declaring these usages is that the same device can also be used as a generic joystick. A device declaring itself as a Flight Simulation Device would mean that it would not be found by an application that searched for a Joystick usage.

A problem that occurs with generic buttons on an application specific device like a Flight Stick is that different applications use the same buttons for different purposes, forcing the user to relearn for each app. Using Delimiters a hardware vendor can suggest usages for the buttons that will allow a consistent user interface across applications.

Delimiters solve this problem by allowing multiple usages to be associated with a device or an individual control.  Below is an example of a single device that can also be identified as a Flight Simulation Device, a Flight Stick or a generic joystick. Most of the controls also have alternate mappings:

The axes of the stick are either a Flight Stick Control or a Pointer. The hat switch can also be used a Point of View control. And each of the 4 buttons have alternate mappings: Button 1 or Trigger; Button 2 or Weapons Select; Button 3 , Electronic Counter Measures or Flare Release; and Button 4, Landing Gear or Chaff Release. Buttons 3 and 4 have two alternate usages.

Note that the report generated by this Report Descriptor is identical to that generated by the Joystick example above.

- No Delimiters on top level collections or arrays.!!!!

UsagePage(Generic Desktop),

LogicalMin(0),

Usage (Joystick),

Collection(Application)

UsagePage(Simulation Controls),

Usage (Throttle),

Logical Minimum (-127),

Logical Maximum (127),

Report Size (8),

Report Count (1),

Input (Data, Variable, Absolute),

Delimiter(Open),

Usage (Flight Control Stick),

Usage (Generic Desktop: Pointer),

Delimiter(Close),

Collection (Physical),

Usage (Generic Desktop: X),

Usage (Generic Desktop: Y),

Report Count (2),

Input (Data, Variable, Absolute),

End Collection(),

Delimiter(Open),

Usage (Game Controls: Point of View),

Usage (Generic Desktop: Hat switch),

Delimiter(Close),

Logical Minimum (0),

Logical Maximum (3),

Physical Minimum (0), Physical Maximum (270),

Unit (English Rotation: Length(1)),	; degrees

Unit Exponent (0)

Report Size (4),

Report Count (1),

Input (Data, Variable, Absolute, Null State),

; Declare the buttons on the stick

Delimiter(Open),

Usage (Buttons: Button 1),

Usage (Trigger),

Delimiter(Close),

Delimiter(Open),

Delimiter(Open),

Usage (Buttons: Button 2),

Usage (Weapons Select),

Delimiter(Close),

Delimiter(Open),

Delimiter(Open),

Usage (Buttons: Button 3),

Usage (Electronic Counter Measures),

Usage (Flare Release),

Delimiter(Close),

Delimiter(Open),

Delimiter(Open),

Usage (Buttons: Button 4),

Usage (Landing Gear),

Usage (Chaff Release),

Delimiter(Close),

Logical Minimum (0), Logical Maximum (1),

Physical Minimum (0), Physical Maximum (1),

Report Count (4),

Report Size (1),

Unit (None),

Input (Data, Variable, Absolute)

- End Collection()

# Appendix C: Physical Descriptor Example

Physical Descriptors allow a device to identify how the user physically interfaces with the device. These are particularly useful for devices like ergonomically designed flight simulator throttle controls.

Attaching a Designator to a control is as simple as adding a Usage. The Designator Index is used to access a Physical Descriptor in a Physical Descriptor Set. In this example the stick is designed to be held in either hand however the way that the user access the buttons will change depending on which hand the user uses.

Consider the joystick below. When the joystick is held by a right handed person their thumb falls on the left button (2). It would make sense to assign this button a function that requires quick access or a fast repeat rate while the button on the right (4) would be assigned a function that does not. This is because a right-handed user must stretch their thumb from the rest position to touch button 4. If a left-handed person held the device the reverse would be true because their thumb would naturally rest on the button on the right (4).

These considerations result in the Effort values that are declared for the right-hand bias Physical Descriptor Set (1) below. Buttons 2, 3, 4 and the hat switch is accessed by the users thumb. The Effort assignments are Button 2 = 0, the Hat switch = 1, Button 3 = 2 and Button 4 = 3. In the case of the Hat switch and Button 3 the thumb has to stretch the same amount. The user must in essence, must “heel and toe” the two controls with their thumb. The Hat switch receives the lower Effort value because tip of the thumb (toe) is considered a more effective manipulator than the first joint of the thumb (heel).

The left hand of a right-handed user normally manipulates the Throttle, while a left-handed user must let go of the stick and use their index finger to manipulate it. This why the Physical Descriptor for both right and left handed users indicates the left index finger however, for the left handed user the Effort is higher.

Hatswitch

Button 4

Button 3

Button 2

Button 1 (Trigger)

Throttle

Note: The Trigger is dotted because it is on the opposite side of the stick.

Figure 15: Joystick button layout

In the following Report Descriptor example Physical Descriptor 1 is attached to the throttle, Physical Descriptor 2 to the stick, and so on. Two Physical Designator Sets are provided: right and left hand. The Physical Descriptor set that is actually referenced depends on whether the user is right or left-handed. It is assumed that the orientation of the user is stored in the user’s profile on the system.

Usage Page (Generic Desktop)

Logical Minimum (0)

Usage (Joystick)

Collection (Application)

Usage Page (Simulation Controls),

Usage (Throttle),

Designator Index (1),

Logical Minimum (-127),

Logical Maximum (127),

Report Size (8),

Report Count (1),

Input (Data, Variable, Absolute),

Usage Page (Generic Desktop)

Designator Index(2),

Usage (Pointer),

Collection (Physical),

Usage (X),

Usage (Y),

Report Count (2),

Input (Data, Variable, Absolute),

End Collection(),

Usage (Hat switch),

Designator Index (3),

Logical Minimum (0), Logical Maximum (3),

Physical Minimum (0), Physical Maximum (270),

Unit (English Rotation: Angular Position), 	; degrees

Unit Exponent (0),

Report Size (4), Report Count (1),

Input (Data, Variable, Absolute, Null State),

Usage Page (Buttons), 	; buttons on the stick

Usage Minimum (Button 1),

Usage Maximum (Button 4),

Physical Minimum (4), Physical Maximum (7),

Logical Minimum (0), Logical Maximum (1),

Physical Minimum (0), Physical Maximum (1),

Report Count (4),

Report Size (1),

Unit (None),

Input (Data, Variable, Absolute)

- End Collection()

The following notation is used to describe two physical descriptor sets for right and left handed use.

; There are 2 Sets defined

Physical Descriptor Set [1]

Bias(Right Hand)

Preference (0)

Physical Descriptor [1]		; Throttle

Designator (Index Finger)

Qualifier (Left)

Effort (0)

Physical Descriptor [2]		; Stick

Designator (Hand)

Qualifier (Right)

Effort (0)

Physical Descriptor [3]		; Hat switch

Designator (Thumb)

Qualifier (Right)

Effort (1)

Physical Descriptor [4]		; Button 1 - Trigger

Designator (Index Finger)

Qualifier (Right)

Effort (0)

Physical Descriptor [5]		; Button 2

Designator (Thumb)

Qualifier (Right)

Effort (0)

Physical Descriptor [6]		; Button 3

Designator (Thumb)

Qualifier (Right)

Effort (2)

Physical Descriptor [7] 		; Button 4

Designator (Thumb)

Qualifier (Right)

Effort (3)

Physical Descriptor Set Header [2]

Bias (Left Hand)

Preference (0)

Physical Descriptor [1]		; Throttle

Designator (Index Finger)

Qualifier (Left)

Effort (1)

Physical Descriptor [2]		; Stick

Designator (Hand)

Qualifier (Left)

Effort (0)

Physical Descriptor [3]		; Hat switch

Designator (Thumb)

Qualifier (Left)

Effort (1)

Physical Descriptor [4]		; Button 1 - Trigger

Designator (Index Finger)

Qualifier (Left)

Effort (0)

Physical Descriptor [5]		; Button 2

Designator (Thumb)

Qualifier (Left)

Effort (3)

Physical Descriptor [6]		; Button 3

Designator (Thumb)

Qualifier (Left)

Effort (2)

Physical Descriptor [7]		; Button 4

Designator (Thumb)

Qualifier (Left)

Effort (0)

}

These directions may appear counter-intuitive but they are consistent with the HID orientation conventions.

Ringback is a feature that a user could invoke when the destination is busy.  Once the destination hangs up its current call, the destination’s phone “places a call” to the original user.