Irps passed between the HID class driver and HID minidriver

version 0.2

All IRP-based communication between class and minidriver is via the following internal device control codes.    Any device control IRPs (internal or otherwise) received by the class driver but not understood by the class driver will be forwarded to the minidriver.

The only valid dispatch routine return codes from a minidriver are STATUS_PENDING or STATUS_SUCCESS.    The final result should be placed in the Status field of the irp’s IO_RESULT_BLOCK upon IRP completion.

When asked to place data in a buffer, the number of bytes the minidriver places into the buffer is min(bufferlength, bytesavailable).

IOCTL_HID_GET_DEVICE_DESCRIPTOR

Gets the raw HID description from a device.

Input:

irpSp->Parameters.DeviceIoControl.OutputBufferLength

size of the destination buffer

irp->UserBuffer

destination

Immediate return value:

STATUS_PENDING or STATUS_SUCCESS

Output (upon completion):

irp->IoStatus.Status

STATUS_SUCCESS

irp->UserBuffer

contains as much of the HID device description as would fit

irp->IoStatus.Information

number of bytes returned

OTHER:

?

IOCTL_HID_GET_REPORT_DESCRIPTOR

Gets the raw HID report description from a device.

Input:

irpSp->Parameters.DeviceIoControl.OutputBufferLength

size of the destination buffer

irp->UserBuffer

destination

Immediate return value:

STATUS_PENDING or STATUS_SUCCESS

Output (upon completion):

irp->IoStatus.Status

STATUS_SUCCESS:

irp->UserBuffer

contains as much of the HID report description as would fit

irp->IoStatus.Information

number of bytes returned

OTHER:

?

IOCTL_HID_READ_REPORT

Polled device:

Supplies a collection ID, forces a read of    the input report for that collection, returns a single input 	report.

Input:

irpSp->Parameters.DeviceIoControl.InputBufferLength

sizeof( ULONG )

irpSp->Parameters.DeviceIoControl.Type3InputBuffer

ULONG collection ID

irpSp->Parameters.DeviceIoControl.OutputBufferLength

size of a single input report for that collection

irp->UserBuffer - buffer to receive the input report

Immediate return value:

STATUS_PENDING or STATUS_SUCCESS

Output:

irp->IoStatus.Status

STATUS_SUCCESS:

irp->UserBuffer

contains an array of unparsed input reports

irp->IoStatus.Information

number of bytes returned, will be a multiple of the input report size

OTHER:

?

Interrupt device:

No collection ID supplied, returns the first report(s) that comes back from the device.      If no reports are    available, the IRP must be queued until a report or reports appear.    If a non-zero number of reports are available, the IRP will complete immediately (e.g. if a read with enough room for four reports is sent, and two reports are available, then those reports will be copied to the buffer and the IRP will complete.)

Input:

irpSp->Parameters.DeviceIoControl.InputBufferLength

0

irpSp->Parameters.DeviceIoControl.OutputBufferLength

size of the report buffer, must be a multiple of the unparsed input report size for that collection.

irp->UserBuffer

buffer to receive the input report array

Output:

irp->IoStatus.Status

STATUS_SUCCESS:

irp->UserBuffer

contains an array of unparsed input reports

irp->IoStatus.Information

number of bytes returned, will be a multiple of the input report size.

OTHER:

?

IOCTL_HID_WRITE_REPORT

Sends an output report or reports to a collection.

Input:

irpSp->Parameters.DeviceIoControl.InputBufferLength

equal to the size of a single write report descriptor

irpSp->Parameters.DeviceIoControl.Type3InputBuffer

pointer to the report descriptor to send to the device

Output:

irp->IoStatus.Status

STATUS_SUCCESS:

irp->IoStatus.Information

number of bytes written to the device

OTHER:

?

IOCTL_HID_GET_STRING_LENGTH

Given a code page and string ID (as found in an unparsed report descriptor), return the length of the resulting UNICODE string in bytes.

Input:

irpSp->Parameters.DeviceIoControl.InputBufferLength

set to sizeof( ULONG) * 2

irpSp->Parameters.DeviceIoControl.Type3InputBuffer

two ULONGs.    The first ULONG specifies the desired code page, the second contains the string ID of interest.

IrpSp->Parameters.DeviceIoControl.OutputBufferLength

set to sizeof( ULONG )

Irp->UserBuffer

pointer to ULONG to accept the number of bytes required to store the string

Output:

irp->IoStatus.Status

STATUS_SUCCESS:

irp->IoStatus.Information

number of bytes returned - should be sizeof( ULONG )

irp->UserBuffer

pointer to a ULONG that has been set to the number of bytes required to store the string.

OTHER

?

IOCTL_HID_GET_STRING

Given a code page and string ID (as found in an unparsed report descriptor), return the UNICODE string in bytes.

Input:

irpSp->Parameters.DeviceIoControl.InputBufferLength

set to sizeof( ULONG) * 2

irpSp->Parameters.DeviceIoControl.Type3InputBuffer

two ULONGs.    The first ULONG specifies the desired code page, the second contains the string ID of interest.

IrpSp->Parameters.DeviceIoControl.OutputBufferLength

the size of the output buffer, in bytes.    This will be equal to at least the value returned from IOCTL_HID_GET_STRING_LENGTH.

Irp->UserBuffer

pointer to destination string buffer.

Output:

irp->IoStatus.Status

STATUS_SUCCESS:

irp->IoStatus.Information

number of bytes returned

irp->UserBuffer

pointer to the resultant string.

OTHER

?

IOCTL_HID_OPEN_COLLECTION

A notification that the open count on this collection changed to non-zero.

NOT USED

IOCTL_HID_CLOSE_COLLECTION

A notification that the open count on this collection changed to zero

NOT USED

Irps used at the upper edge of the HID class driver

IOCTL_HID_GET_DRIVER_CONFIG

Fills and returns a HID_DRIVER_CONFIG structure with the current parameters associated with this handle.

Input:

irpSp->FileObject

file object representing the open instance

irpSp->Parameters.DeviceIoControl.OutputBufferLength

size of the destination buffer

irp->UserBuffer

pointer to receive the driver configuration

Immediate return value:

STATUS_PENDING or STATUS_SUCCESS

Output (upon completion):

irp->IoStatus.Status

STATUS_SUCCESS

STATUS_BUFFER_OVERFLOW

irp->UserBuffer

contains as much of the driver configuration structure as would fit

irp->IoStatus.Information

number of bytes returned

OTHER:

?

IOCTL_HID_SET_DRIVER_CONFIG

Modifies the processing parameters associated with this handle according to a HID_DRIVER_CONFIG structure passed in.

Input:

irpSp->FileObject

file object representing the open instance

irpSp->Parameters.DeviceIoControl.InputBufferLength

size of the driver configuration buffer

irpSp->Parameters.DeviceIoControl.Type3InputBuffer

pointer to the driver configuration buffer (HID_DRIVER_CONFIG)

Immediate return value:

STATUS_PENDING or STATUS_SUCCESS

Output (upon completion):

irp->IoStatus.Status

STATUS_SUCCESS

irp->IoStatus.Information

set to the number of bytes processed from the driver configuration buffer.

OTHER:

?

IOCTL_HID_FLUSH_QUEUE

Discards any waiting report descriptors in the given handle’s report queue.

Input:

irpSp->FileObject

file object representing the open instance

Immediate return value:

STATUS_PENDING or STATUS_SUCCESS

Output (upon completion):

irp->IoStatus.Status

STATUS_SUCCESS

OTHER:

?

IOCTL_HID_GET_COLLECTION_INFORMATION

Fills and returns a HID_COLLECTION_INFORMATION structure with the current parameters associated with this handle.

Input:

irpSp->FileObject

file object representing the open instance

irpSp->Parameters.DeviceIoControl.OutputBufferLength

size of the destination buffer

irp->UserBuffer

pointer to receive collection information.

Immediate return value:

STATUS_PENDING or STATUS_SUCCESS

Output (upon completion):

irp->IoStatus.Status

STATUS_SUCCESS

STATUS_BUFFER_OVERFLOW

irp->UserBuffer

contains as much of the collection information structure as would fit.

irp->IoStatus.Information

number of bytes returned

OTHER:

?

IOCTL_HID_GET_COLLECTION_DESCRIPTOR

Given an input buffer of sufficient size (see above), returns as much of the HIDP_COLLECTION_DESC structure as will fit.

Input:

irpSp->FileObject

file object representing the open instance

irpSp->Parameters.DeviceIoControl.OutputBufferLength

size of the destination buffer

irp->UserBuffer

pointer to receive the collection description.

Immediate return value:

STATUS_PENDING or STATUS_SUCCESS

Output (upon completion):

irp->IoStatus.Status

STATUS_SUCCESS

STATUS_BUFFER_OVERFLOW

irp->UserBuffer

contains as much of the collection description    as would fit.

irp->IoStatus.Information

number of bytes returned

OTHER:

?

IRP_MJ_CREATE

Opens a collection.

Input:

irpSp->FileObject

file object for the open

irpSp->FileObject->FileName

file name of the form “\nnnnnnnn”, where “nnnnnnnn” is a hex odinal representing the collection to open.

Immediate return value:

STATUS_PENDING or STATUS_SUCCESS

Output (upon completion):

irp->IoStatus.Status

STATUS_SUCCESS

OTHER

IRP_MJ_CLOSE

Closes a previously opened collection.

Input:

irpSp->FileObject

file object representing the open instance

Immediate return value:

STATUS_PENDING or STATUS_SUCCESS

Output (upon completion):

irp->IoStatus.Status

STATUS_SUCCESS:

OTHER:

IRP_MJ_READ

Reads n unparsed input report descriptors, where n is min( buffersize / reportsize, reportsavailable ).    Returns 0 bytes read only if the input buffer is smaller than reportsize.

Input:

irpSp->FileObject

file object representing the open instance

irpSp->Parameters.Read.Length

number of bytes to read

Irp->MdlAddress

pointer to a single MDL representing the read buffer.    Note that multiple MDLs are not supported, therefore the read buffer must be virtually contiguous

Immediate return value:

STATUS_PENDING or STATUS_SUCCESS

Output (upon completion):

irp->IoStatus.Status

STATUS_SUCCESS:

irp->IoStatus.Information

number of bytes read, must be a multiple of the size of the collection’s input report

irp->UserBuffer

pointer to the buffer to contain the intput report array.

STATUS_INVALID_BUFFER_SIZE

supplied buffer was not a multiple of the input report size for the given collection

OTHER:

IRP_MJ_WRITE

Writes n unparsed output report descriptors, where n is ( buffersize / reportsize )

Input:

irpSp->FileObject

file object representing the open instance

irpSp->Parameters.Write.Length

number of bytes to write.    Must be an even multiple of the size of the collection’s output report.

irp->UserBuffer

pointer to the buffer containing the output report array.

Immediate return value:

STATUS_PENDING or STATUS_SUCCESS

Output (upon completion):

irp->IoStatus.Status

STATUS_SUCCESS:

irp->IoStatus.Information

number of bytes written, must be a multiple of the size of the input report

STATUS_INVALID_BUFFER_SIZE

supplied buffer was not a multiple of the output report size for the given collection

OTHER: