WDM Streaming

Miniport Driver Model Specification

Draft version 0.10 – 4/25/97

- MICROSOFT CONFIDENTIAL

WARNING:  THIS SPECIFICATION IS FOR REVIEW ONLY, AND MAY CHANGE COMPLETELY.

The information contained in this document represents the current view of Microsoft Corporation of the issues discussed as of the date of publication. Because Microsoft must respond to changing market conditions, it should not be interpreted to be a commitment on the part of Microsoft, and Microsoft cannot guarantee the accuracy of any information presented after the date of publication.

This document is for informational purposes only. MICROSOFT MAKES NO WARRANTIES, EXPRESS OR IMPLIED, IN THIS DOCUMENT.

Microsoft Corporation may have patents or pending patent applications, trademarks, copyrights, or other intellectual property rights covering subject matter in this document. The furnishing of this document does not give you any license to the patents, trademarks, copyrights, or other intellectual property rights except as expressly provided in any written license agreement from Microsoft Corporation.

Microsoft does not make any representation or warranty regarding specifications in this document or any product or item developed based on these specifications. Microsoft disclaims all express and implied warranties, including but not limited to the implied warranties or merchantability, fitness for a particular purpose and freedom from infringement. Without limiting the generality of the foregoing, Microsoft does not make any warranty of any kind that any item developed based on these specifications, or any portion of a specification, will not infringe any copyright, patent, trade secret or other intellectual property right of any person or entity in any country. It is your responsibility to seek licenses for such intellectual property rights where appropriate. Microsoft shall not be liable for any damages arising out of or in connection with the use of these specifications, including liability for lost profit, business interruption, or any other damages whatsoever. Some states do not allow the exclusion or limitation of liability or consequential or incidental damages; the above limitation may not apply to you.

Microsoft, MS-DOS, Visual Basic, Win32, Windows, and Windows NT are registered trademarks and ActiveMovie, ActiveX, DirectDraw, Direct3D, DirectSound, and DirectX are trademarks of Microsoft Corporation in the United States and other countries.

© 1997 Microsoft Corporation. All rights reserved.

This document is provided under a Non-Disclosure Agreement (NDA). Do not redistribute.

1.	WDM Streaming Audio Miniport Drivers in Context	1

2.	Driver Classification:  Audio Miniports in Perspective	3

3.	COM in the Kernel	4

4.	Adapter Driver Construction	6

4.1	Startup Sequence	6

4.2	Subdevice Creation	7

5.	The Specific Miniport Interfaces	8

6.	Helper Objects	9

7.	Generic Interfaces	10

7.1	Miniport Generic Interfaces	10

7.2	Port Generic Interface	13

8.	Cyclic DMA Wave Input and Output	15

8.1	Miniport Interfaces	15

8.2	Port Driver Lower-Edge Interface	21

9.	PCI Scatter/Gather Wave Input and Output	26

9.1	Miniport Interfaces	26

9.2	Port Driver Lower-Edge Interface	32

10.	Static Wave Table Output	38

11.	Topology	39

11.1	Miniport Interface	40

11.2	Port Driver Lower-Edge Interface	47

12.	MIDI Input and Output	48

12.1	Miniport Interface	48

12.2	Port Driver Lower-Edge Interface	52

# WDM Streaming Audio Miniport Drivers in Context

This specification defines how audio hardware drivers for WDM Streaming systems are to be constructed.  WDM, the Windows Driver Model, is a robust model for driver implementation that will be supported by both Windows and Windows NT operating systems.  WDM Streaming is a media streaming architecture based on WDM.  This document concerns WDM Streaming drivers that communicate directly with audio adapter hardware.

In an effort to broaden the applicability of the model presented in this document, steps have been taken to insure that drivers written to meet this specification can potentially be used in the absence of WDM Streaming.  In spite of this, the specification retains its WDM Streaming orientation.

WDM Streaming presents drivers as filters through which media streams pass.  Filters have pins which can be connected together to form systems of filters.  Input pins consume media streams and output pins produce media streams.  In keeping with this model, audio adapters are manifested as filters.  Input pins on these filters consume audio streams and route them to the audio hardware.  Output pins on these filters produce audio streams from data produced by the hardware.

In order to make audio hardware drivers easier to write, the audio driver model isolates hardware interface issues from WDM Streaming filter implementation issues.  It accomplishes this by organizing the driver code into components that address these issues separately and by precisely defining the interfaces between these components.  All the components are referred to as ‘drivers’, although not all are recognized as such by the operating system:

- Miniport Driver  A miniport driver implements a function-specific interface for a specific function on a specific adapter (audio card).  For example, a particular miniport might implement a MIDI streaming interface for an FM synthesizer on a particular audio adapter.  Miniport drivers are actually part of an adapter driver, and are not recognized as drivers by the operating system.

- Adapter Driver  An adapter driver is essentially a container for all the miniports drivers associated with a given adapter.  It is recognized as a driver by the operating system, and is contained in its own .SYS file.  Adapter drivers consist of a set of miniport drivers and some additional code that addresses initialization issues.

- Port Driver  A port driver implements a WDM Streaming filter on behalf of a miniport.  It is responsible for the adapter-independent code that allows a miniport’s function-specific interface to be exposed to the system as a WDM Streaming filter.  Port drivers operate in the context of the port class driver.

- Port Class Driver  The port class driver is essentially a container for a set of port drivers.  At the request of the adapter driver, it binds together port drivers, miniport drivers and hardware resources to form complete subdevices.  It coordinates the operation of these subdevices to form a complete device corresponding to the audio adapter hardware.  It also provides a set of helper functions to adapter drivers and miniport drivers.  Although it exists as a .SYS file, it is not loaded like a driver.  Instead, it is loaded by the system because the adapter driver imports functions that the port class driver exports.

Audio miniport drivers control physical audio devices, providing a hardware-specific implementation of function-specific interfaces.  Each audio miniport driver interface specified in this document addresses a particular type of function on an audio adapter.  Typically, any given adapter will support multiple functions.  The adapter driver for a particular audio adapter will therefore expose multiple miniport interfaces.  In many cases, a given driver will expose more than one instance of a particular interface.

The following figure illustrates the relationships between these components:

This layered approach allows for the support of physical devices with a minimum of software development effort.  A hardware manufacturer that wants WDM Streaming support for a sound card goes through the following steps for each function the card supports:

- Determine if the function is hardware-compatible with hardware for which a miniport driver already exists.  If so, use the existing miniport driver to support the function.  If no such compatible hardware is found…

- Determine if the function is sufficiently similar to functions supported by existing port drivers.  If so, write a miniport driver to support that function, complying with the miniport interface specification associated with the identified port driver.  If no such port driver is found…

- Write a minidriver to support the function, complying with the interface specifications for the streaming class driver.

The focus of this document is on miniport driver interfaces and the development of adapter drivers.  This document does not address in detail the manner in which audio adapter functionality will be exposed in the form of WDM Streaming filters and pins.

This document includes drivers for all audio hardware whose registers are directly accessible to the system processor.  This includes all ISA/DMA, PCMCIA and PCI adapters.  Specifically excluded from the scope of this document are devices on external busses such as SCSI, USB and 1394.  In addition, this specification is not intended to address software emulation of audio devices.  Emulation should be accomplished at the WDM Streaming filter level.

# Driver Classification:  Audio Miniports in Perspective

In some ways, the configuration of audio driver components described above is peculiar when viewed in the context of ‘classic’ NT drivers.  Terms such as ‘miniport’, ‘port’ and ‘class’ have been borrowed from the NT driver nomenclature without necessarily being faithful to the models they represent.

There are five ‘classic’ terms of interest here:

- Miniport Driver  Traditionally, a miniport driver is a hardware-specific driver for an adapter that resides on a system bus (e. g, PCI or ISA).  A miniport has a single entry point, namely DriverEntry, and registers a table of functions with a port driver.  This table of functions serves as the upper-edge interface of the miniport.  A miniport is completely surrounded by a port driver in the sense that all calls to the miniport are made from the port driver and all calls out of the minidriver are to the port driver lower-edge interface.  Because of this, miniports a platform independent.

- Port Driver  A port driver surrounds a miniport, handling all system interface issues.  It handles requests from the system which is recasts as calls into the miniport’s function table.  It provides a library of support functions (the port’s lower-edge interface) to the miniport.

- Minidriver  A minidriver represents a piece of hardware on a bus.  It binds together the bus driver and one or more class drivers.  It uses the bus driver to communicate to the physical device over the bus, and the class drivers help the minidriver present the physical device to clients as a type of logical device.  In the NT and WDM environments, a minidriver typically receives requests in IRP form from class drivers and sends requests in IRP form to a bus driver.  An example of a minidriver that might bind to multiple class drivers is a minidriver for a CD-ROM drive on a 1394 bus.  The minidriver might bind to a file system class driver so the drive can be accessed as a file system and a Redbook audio class driver so that audio can be streamed from CDs.

- Bus Driver  A bus driver provides minidrivers with access to a physical bus.  The NT HAL is sometimes referred to as the ‘system bus driver’, meaning that it provides minidrivers with access to the system bus.

- Class Driver  A class driver implements behavior expected of a class of devices.  This eliminates the need for the duplication of functionality in hardware-specific drivers.  Class drivers are not bus-specific in any way, and so are not aware of resource issues (such as DMA and interrupts).

The audio miniports described here are similar to classic miniports with some exceptions.  Because multiple miniports addressing multiple functions can all be linked into a single adapter driver (and associated with a single device object), they do not implement DriverEntry and are not registered in the traditional miniport fashion.  Miniports do not rely entirely on their respective port drivers for support.  The port class driver provides some function-generic support on its lower edge.  Also, audio miniport developers that are willing to be WDM-dependent are free to call WDM functions.

The port drivers that support audio miniports fit the classic model fairly well except that they share the device object and are consequently instantiated somewhat differently.  Port drivers also have some characteristics of class drivers in that they implement behavior expected of a class of devices.  Unlike class drivers, they are not bus-independent.

The port class driver does not fit the class driver model particularly well.  Its job is to allow multiple port/miniport bindings to share a single device object.  The port class driver is also provides helper functionality to port and miniport drivers.

# COM in the Kernel

Miniport upper edge interfaces consist of function tables.  In traditional miniports, the function table is supplied to the port driver during registration.  Also during registration, the miniport tells the port driver the size of the context structure the miniport will required.  The port driver copies the function table to some private location, allocates the context structure and calls an initialization function in the function table, passing a pointer to the context structure.

Audio miniports use a similar approach with the added formalism of COM.  A function table is still used, but it is statically allocated and does not need to be copied by the port driver.  The port driver is also relieved of its context allocation responsibilities.  The miniport code provides a creation function, which allocates the context (‘object’) memory from a specified pool and installs a pointer to the function table into the context.  Because the function table pointer is always the first field in the context, the port driver needs only a context pointer and can access the function table through the context.

This approach was taken because COM supplies a solid, efficient, widely-understood formalism for creating abstracted objects.  The audio miniport model leverages industry experience with COM and the body of COM literature.  In addition, COM enhances code safety by isolating unsafe casts to a few functions.  Objects can be implemented and used in C or C++.  Assembly can also be used, but this hampers portability.

In user mode, COM objects are created using the function CoCreateInstance() or a similar function.  The use of such a function implies that the memory required for the object is allocated in a way that is unknown to the client.  In kernel mode, it is desirable to tightly control the allocation of memory.  Because of this, a different method of object creation is used.

The aspects of COM that are used in the audio driver model are the concept of the interface and the definition of the IUnknown interface.  Although there is a mechanism analogous to an in-process server, driver developers don’t need to deal with building such servers or accessing the registry.  Miniports are also officially excused from supporting aggregation, although the sample code demonstrates an easy way to support this.

By convention, the create function for a particular class of objects always takes the same form:

NTSTATUS

CreateMyObject

(

OUT PUNKNOWN *  Unknown,

IN  REFGUID     ClassId,

IN  PUNKNOWN    OuterUnknown  OPTIONAL,

IN  POOL_TYPE   PoolType

);

The first three parameters are identical to the parameters of COM’s CoCreateInstance().  A pointer to an IUnknown interface for the object is passed back through Unknown, the class GUID is passed as ClassId (only used if the create function creates objects of multiple classes), and OuterUnknown supplies the unknown interface for the object aggregating the new object (this is generally NULL).  The last parameter specifies the type of pool from which the object is to be allocated.

Another convention is to supply a New function for a class, which creates an object, initializes it, and passes back a pointer to the interface that the caller is likely to want.  Because the initialization parameters are class-specific, so is the prototype of the New function.

NTSTATUS

NewMyObject

(

OUT PMYINTERFACE *  FavoredInterfacePointer,

IN  PUNKNOWN        OuterUnknown    OPTIONAL,

IN  POOL_TYPE       PoolType,

//  more parameters

);

Such functions provide an easy way to instantiate (create and initialize) an object.

The sample code eases COM object implementation by providing a class that supports reference counting and aggregation.  The author of an object provides the function required to obtain interfaces on the object.  The helper class (CUnknown) is written in C++.  Objects implemented in C cannot use the helper class.

# Adapter Driver Construction

Driver support for a particular audio adapter takes the form of an adapter driver.  An adapter driver consists of some function-generic code (primarily for initialization) and a set of miniports that address specific functions on the adapter.  The hardware vendor supplies the function-generic code and those miniports that are not supplied by the system.

## Startup Sequence

Because the adapter driver is installed as a kernel mode driver service, the operating system loads the adapter driver and calls its DriverEntry routine.  The DriverEntry routine receives two parameters, both pointers, which it must not use directly.  DriverEntry calls the class driver function InitializeAdapterDriver passing the DriverEntry parameters and a pointer to the adapter driver’s add device function (MyAddDevice in this example).

NTSTATUS

DriverEntry

(

IN PVOID Context1,

IN PVOID Context2

)

{

return InitializeAdapterDriver(Context1,Context2,MyAddDevice);

}

InitializeAdaptorDriver installs the supplied add device function in the driver object’s driver extension, installs the class driver’s IRP handlers in the driver object itself, and returns control to DriverEntry.  DriverEntry returns control to the operating system.

In the process of enumerating the system busses, the operating system, encounters the adapter device, which is associated with the driver object.  It finds the pointer to MyAddDevice in the driver object’s driver extension and calls the function.  Two parameters are supplied, both pointers, which MyAddDevice must not use directly.  MyAddDevice calls the class driver function AddAdapterDevice passing the pointers it received and a pointer to the adapter driver’s start device function (MyStartDevice in this example).

NTSTATUS

MyAddDevice

(

IN PVOID Context1,

IN PVOID Context2

)

{

return AddAdapterDevice(Context1,Context2,MyStartDevice);

}

AddAdapterDevice creates a functional device object and associates it with the physical device object supplied by the system.  The new functional device object is created with an extension that the class driver uses to store the start device function pointer supplied by MyAddDevice. AddAdapterDevice then returns control to MyAddDevice, which returns control to the operating system.

The operating system determines what resources (interrupts, DMA channels and port addresses) will be assigned to the device and requests that the device be started.  The request handler in the class driver calls the adapter driver’s start device function, MyStartDevice.  It supplies MyStartDevice with the resource list and two void pointers that that establish context for calls back into the class driver.

MyStartDevice separates the resource list into the resources required for each miniport that must be started.  It then starts each miniport and returns control to the class driver, which completes the IRP and returns control to the operating system.

## Subdevice Creation

The term ‘subdevice’ is used to describe the binding of four things: a miniport, a port, a list of resources and a reference string.  The miniport and port are objects that expose the miniport interface and port interface respectively.  The list of resources is a list of those resources that the adapter driver has attributed to the subdevice.  The reference string is a name that is added to the device path name to specify a subdevice during filter creation.

The port class driver is not aware of the distinction between the port driver and the miniport driver.  It wants an object with an interface that will handle the requests generated by the system.  The port driver is such an object.  The port class driver is also unconcerned about resources, wanting only to bind the request handler (the port driver) to a reference string.  It is up to the adapter driver to bind the port, miniport and resources together and then call the port class driver to bind the port driver to the reference string.

The following code shows how this might be done:

// Instantiate the port using a function supplied by port class.

PPORT port = NewPort(PortClassId);

if (port)

{

// Create the miniport using its create function.

PUNKNOWN miniport;

ntStatus =

CreateMyMiniport

(

&miniport,

GUID_NULL,    // Class GUID not used.

NULL,         // No outer unknown.

NonPagedPool

);

if (NT_SUCCESS(ntStatus))

{

// Bind the port, miniport and resources.

ntStatus =

port->Init(Context1,miniport,ResourceList);

miniport->Release();

if (NT_SUCCESS(ntStatus))

{

// Hand the port driver and the reference string to the port class.

ntStatus =

RegisterSubdevice

(

Context1,

Context2,

Name,

port

);

}

}

port->Release();

}

# The Specific Miniport Interfaces

This document defines the following miniport driver interfaces:

- Cyclic DMA wave input and output.  This class of miniport drivers supports access to DMA-based wave I/O functions of ISA and other audio cards.  The interface addresses cyclic (autoinit) DMA buffers specifically, leaving streaming issues to be solved by the port driver.

- Scatter/gather wave input and output.  This class of miniport drivers supports access to wave audio functions such as the wave I/O function of a PCI audio card.  In this model, lists of wave audio buffers are logically appended to create a stream of wave audio.

- Static wave table output.  This class of miniport drivers supports access to wave table synthesizer voices such as the ones supplied by the Creative Labs SoundBlaster AWE32 adapter.  The interface allows for the transfer of static wave data to the adapter and for the manipulation of the voice parameters.

- Topology.  This class of miniport drivers supports access to the various controls (volume, equalization, hardware reverb) that audio adapters typically offer.  The interface concerns itself with the enumeration of components (known as nodes) in the topology, the discovery of their interconnections and the reading and writing of control parameters.

- MIDI input and output.  This class of miniport drivers supports access to MIDI-based functions like FM synthesizers and UART-based hardware MIDI interfaces.  Timing is handled by the port driver or some other component higher in the driver stack, allowing the miniport to concentrate on the immediate implementation or delivery of MIDI messages.  MIDI devices, such as MPU-401 coprocessors, which handle their own timing require a specialized port driver are not within the scope of this interface.

# Helper Objects

This section will describe the helper objects implemented by the port class.  Examples are the DMA channel and the resource list.

# Generic Interfaces

By definition, the audio miniport interfaces are specific to the functions they are intended to support.  In order to provide some degree of standardization, a generic interface is defined which, while not constituting a complete miniport interface, serves as a base from which other audio miniport interfaces are derived.  The generic miniport interface is called IMiniport.  IMiniport inherits from IPropertyTarget, which exposes support for properties that can be queried and set.

Port driver interfaces are standardized only in that they share a common initialization interface.  This interface is known as IPort.  All port drivers expose this interface, and all port driver lower-edge interfaces are derived from it.  Miniports do not call a port driver’s initialization function, but it is included in the lower edge to simplify the use and implementation of the port driver.

## Miniport Generic Interfaces

All miniport interfaces supply a property access function.  Some miniport interfaces defined in this document use the property function to manage aspects of the interface that are very complex or are likely to require future enhancements.  Using properties, vendors can extend miniport interfaces without conflicting with new standards or non-standard extensions.  Because WDM Streaming filters and pins expose properties that are much like miniport properties, miniport properties unknown to the port driver can be exposed to WDM Streaming clients.  This means that vendor-specific functionality can be exposed at the WDM Streaming level without the need for custom port drivers.

PnP and power management functions are supplied to control the state of the miniport during PnP driver stoppages and power transitions.

| Miniport Interface |  |  |
| --- | --- | --- |
| Interface | ### IPropertyTarget |  |
| Inheritance | #### IUnknown |  |
| Description | This interface allows an object to support properties. A single function allows properties to be queried and set. |  |
| Functions | NTSTATUS Property() ? PKSIDENTIFIER Property ? ULONG InstanceSize ? PVOID Instance ? ULONG ValueSize ? PVOID Value |  |

| IPropertyTarget |  |  |
| --- | --- | --- |
| Function | Property() |  Passive  DPC  IRQ |
| Description | This function accesses a property. |  |
| Parameters | PKSIDENTIFIER Property The property identifier, consisting of a GUID, which identifies the property set, a ULONG, which identifies the specific property within the set, and another ULONG which specifies the operation to be performed (e.g. get or set). |  In  Out  Optional |
|  | ULONG InstanceSize Size of the instance structure in bytes. |  In  Out  Optional |
|  | PVOID Instance A property-specific structure indicating the instance to which the requested property value applies. For example, if a property defines the volume associated with a channel in a mixer, the instance information might specify the channel for which volume information is being requested. |  In  Out  Optional |
|  | ULONG ValueSize The size in bytes of the buffer supplied by the caller to hold the property value. For property ‘get’ operations, the size must be at least the size of the property value. For property ‘set’ operations, the size must be exactly the size of the property value. The property function adjusts the value size if it is not exact. |  In  Out  Optional |
|  | PVOID Value The value of the property or a description of the property in the case of support queries. TODO: Explain this some more. |  In  Out  Optional |
| Return | NTSTATUS STATUS_SUCCESS indicates a successful outcome. Exceptional return values TBD. |  |

| Miniport Interface |  |  |
| --- | --- | --- |
| Interface | ### IMiniport |  |
| Inheritance | #### IPropertyTarget |  |
| Description | This is the generic interface supported by all miniports. By inheriting IPropertyTarget, it supports property access. In addition, two functions support PnP stop operations and power management. |  |
| Functions | NTSTATUS SetStopState() ? ULONG StopState |  |
|  | NTSTATUS SetPowerState() ? ULONG PowerState |  |

| IMiniport |  |  |
| --- | --- | --- |
| Function | SetStopState() |  Passive  DPC  IRQ |
| Description | This function changes the miniport’s stop state. |  |
| Parameters | ULONG StopState The new stop state to be assumed by the miniport: - StopStateNormal The normal state in which the miniport is not stopped and is not preparing to stop. - StopStateQuery A tentative state used to determine whether the miniport is prepared to stop operation. The function succeeds if operation can cease without ill effect. Between a successful transition to this state and the transition to the StopStateStop or StopStateCancel states, the miniport cannot initiate any operation that would compromise its ability to stop. - StopStateStop The stopped state. The miniport should shut down the hardware and free any system resources it has allocated. The miniport will not be called again without first being initialized. |  In  Out  Optional |
| Return | NTSTATUS STATUS_SUCCESS indicates a successful outcome. Exceptional return values TBD. |  |

| IMiniport |  |  |
| --- | --- | --- |
| Function | SetPowerState() |  Passive  DPC  IRQ |
| Description | This function changes the power state of the device supported by the miniport. When a transition occurs from the normal state to the power down state, the miniport must save the state of the hardware in system memory. When a transition occurs from the power down state to the normal state, the miniport must restore the state of the hardware. |  |
| Parameters | ULONG PowerState The power state that the miniport is to change to. A state value of 0 indicates that the device is to operate normally. A non-zero value indicates the device must save its hardware state information in preparation for power down. |  In  Out  Optional |
| Return | NTSTATUS STATUS_SUCCESS indicates a successful outcome. Exceptional return values TBD. |  |

## Port Generic Interface

All port drivers expose the generic port interface as part of their lower edge.  The adapter driver calls the initialization function on this interface.

| Port Interface |  |  |
| --- | --- | --- |
| Interface | ### IPort |  |
| Inheritance | #### IUnknown |  |
| Description | This interface allows for the initialization of port drivers. Its single function is called by the adapter driver during the starting of the device. |  |
| Functions | NTSTATUS Init() ? PVOID Context1 ? PUNKNOWN UnknownMiniport ? PRESOURCELIST ResourceList |  |

| IPort |  |  |
| --- | --- | --- |
| Function | Init() |  Passive  DPC  IRQ |
| Description | This function initializes the port driver. The miniport supplied by the second parameter must expose the miniport interface supported by the port driver, or the call will fail. The miniport is initialized by the port driver during this call. |  |
| Parameters | PVOID Context1 The first context parameter passed to the adapter driver’s start device function. |  In  Out  Optional |
|  | PUNKNOWN UnknownMiniport The IUnknown interface of the uninitialized miniport. This interface is queried for the specific miniport interface supported by the port driver, and the initialization function on that interface is called. |  In  Out  Optional |
|  | PRESOURCELIST ResourceList The resource list to be supplied to the miniport during initialization. The port driver is free to examine this resource list, but it will not be modified by the port driver. |  In  Out  Optional |
| Return | NTSTATUS STATUS_SUCCESS indicates a successful outcome. Exceptional return values TBD. |  |

# Cyclic DMA Wave Input and Output

This class of miniport drivers supports access to cyclic DMA-based wave audio functions.  The interface addresses cyclic buffers specifically, leaving streaming issues to be solved by the port driver.  Multiple channels are supported, each one unidirectional.

There are two interfaces that must be supported by the miniport.  The first provides for initialization of the miniport, the enumeration of channels and the creation of streams.  The second interface is the stream interface, which exposes most of the miniport’s functionality.

## Miniport Interfaces

The overall miniport interface has three functions:  one for initialization, one to enumerate channels and one to create a stream object.  The stream object interface (not derived from IMiniport) exposes the bulk of the miniport functionality.

| Miniport Interface |  |  |
| --- | --- | --- |
| Interface | ### IMiniportWaveCyclic |  |
| Inheritance | #### IMiniport |  |
| Description | This is the interface exposed to the port driver by the device-associated object that implements the miniport. It provides a function for initialization and a function for creating a miniport stream object. The stream object is associated with a pin on a filter. There is no interface for a filter-associated miniport object. |  |
| Functions | NTSTATUS Init() ? PRESOURCELIST ResourceList ? PPORTWAVECYCLIC Port |  |
|  | NTSTATUS GetChannelList() ? PULONG Count ? PCHANNELLISTENTRY *Entries |  |
|  | NTSTATUS NewStream() ? PMINIPORTWAVECYCLICSTREAM *Stream ? PUNKNOWN OuterUnknown ? POOL_TYPE PoolType ? ULONG Channel ? BOOLEAN Capture ? PWAVEFORMATEX DataFormat ? PWAVEPORTCYCLICDMACHANNEL DmaChannel |  |

| IMiniportWaveCyclic |  |  |
| --- | --- | --- |
| Function | Init() |  Passive  DPC  IRQ |
| Description | This function initializes the miniport. Initialization includes verification of the hardware using the resources specified in the resource list. |  |
| Parameters | PRESOURCELIST ResourceList List of the resources to be used by the hardware associated with this miniport. |  In  Out  Optional |
|  | PPORTWAVECYCLIC Port Pointer to the port driver’s callback interface. |  In  Out  Optional |
| Return | NTSTATUS STATUS_SUCCESS indicates a successful outcome. Exceptional return values TBD. |  |

| IMiniportWaveCyclic |  |  |
| --- | --- | --- |
| Function | GetChannelList() |  Passive  DPC  IRQ |
| Description | This function obtains a list of channels supported by the miniport. |  |
| Parameters | PULONG Count The number of elements in the channel list. |  In  Out  Optional |
|  | PCHANNELLISTENTRY *Entries Pointer to the list of channels, an array of *Count elements. This list may be paged, but must remain at the supplied address until the miniport’s reference count reaches zero. The channel list entry structure is defined as follows: typedef struct { ULONG DataRangesCount; PKSDATARANGE DataRanges; ULONG Instances; ULONG Flags; #define CHANNELLISTENTRYFLAG_RENDER 1 #define CHANNELLISTENTRYFLAG_CAPTURE 2 } CHANNELLISTENTRY, *PCHANNELLISTENTRY; The first two fields supply a list of data ranges indicating formats supported by the channel. For more information regarding the KSDATARANGE structure, consult the WDM Streaming specification. The Instances field indicates how many instances of the channel may be created. Typically, this value will be 1. The remaining field indicates whether the channel supports rendering or capture of audio data. |  In  Out  Optional |
| Return | NTSTATUS STATUS_SUCCESS indicates a successful outcome. Exceptional return values TBD. |  |

| IMiniportWaveCyclic |  |  |
| --- | --- | --- |
| Function | NewStream() |  Passive  DPC  IRQ |
| Description | This function creates a new instance of a logical stream associated with a specified physical channel. |  |
| Parameters | PMINIPORTWAVECYCLICSTREAM Stream The location at which the miniport deposits a pointer to the stream object’s IMiniportWaveCyclicStream interface. |  In  Out  Optional |
|  | PUNKNOWN OuterUnknown Pointer to the IUnknown interface of an object wishing to aggregate the stream object. Unless aggregation is desired, this parameter will be NULL. |  In  Out  Optional |
|  | POOL_TYPE PoolType The type of memory pool from which the object should be allocated. |  In  Out  Optional |
|  | ULONG Channel The number of the channel. Input and output channels are numbered independently starting at zero. The first input channel is numbered 0, and the first output channels is numbered 0. |  In  Out  Optional |
|  | BOOLEAN Capture TRUE for an input (capture) channel, FALSE for an output (playback) channel. |  In  Out  Optional |
|  | PKSDATAFORMAT DataFormat The format to use for this instance. See the WDM Streaming specification for more information on the type of this parameter. |  In  Out  Optional |
|  | PDMACHANNEL DmaChannel DMA channel object. |  In  Out  Optional |
| Return | NTSTATUS STATUS_SUCCESS indicates a successful outcome. Exceptional return values TBD. |  |

| Miniport Interface |  |  |
| --- | --- | --- |
| Interface | ### IMiniportWaveCyclicStream |  |
| Inheritance | #### IPropertyTarget |  |
| Description | This is the interface exposed to the port driver by the pin-associated object implemented by the miniport. It provides the functions required to manage a single stream of incoming or outgoing audio data. |  |
| Functions | NTSTATUS SetFormat() ? PKSDATAFORMAT Format |  |
|  | ULONG SetNoficationFrequency() ? ULONG Interval |  |
|  | NTSTATUS SetState() ? KSSTATE State |  |
|  | NTSTATUS GetPosition() ? PULONG Position |  |

| IMiniportWaveCyclicStream |  |  |
| --- | --- | --- |
| Function | SetFormat() |  Passive  DPC  IRQ |
| Description | This function changes the format associated with a stream. |  |
| Parameters | KSDATAFORMAT DataFormat The new format. |  In  Out  Optional |
| Return | NTSTATUS STATUS_SUCCESS indicates a successful outcome. Exceptional return values TBD. |  |

| IMiniportWaveCyclicStream |  |  |
| --- | --- | --- |
| Function | SetNotificationFrequency() |  Passive  DPC  IRQ |
| Description | This function sets the frequency at which notification interrupts are generated. |  |
| Parameters | ULONG Interval The interval between interrupts, expressed in frames. A frame is a set of samples corresponding to a moment in time. For example, for stereo channels, there are two samples (left and right) in a frame. |  In  Out  Optional |
| Return | ULONG The new notification frequency. |  |

| IMiniportWaveCyclicStream |  |  |
| --- | --- | --- |
| Function | SetState() |  Passive  DPC  IRQ |
| Description | This function starts playback or recording for the stream. The port driver will never call this function without first calling the pause function. Playback or recording starts at the current position in the DMA buffer. |  |
| Parameters | KSSTATE State The new state of the stream, one of: - KSSTATE_STOP - KSSTATE_ACQUIRE - KSSTATE_PAUSE - KSSTATE_RUN For the purposes of many miniports, KSSTATE_ACQUIRE and KSSTATE_PAUSE are indistinguishable. |  In  Out  Optional |
| Return | #### NTSTATUS STATUS_SUCCESS indicates a successful outcome. Exceptional return values TBD. |  |

| IMiniportWaveCyclicStream |  |  |
| --- | --- | --- |
| Function | GetPosition() |  Passive  DPC  IRQ |
| Description | This function gets the current position of the DMA read pointer for the stream. |  |
| Parameters | PULONG Position The position of the DMA read pointer. |  In  Out  Optional |
| Return | NTSTATUS STATUS_SUCCESS indicates a successful outcome. Exceptional return values TBD. |  |

## Port Driver Lower-Edge Interface

The port driver provides an interface to the miniport for instantiation of abstracted DMA channels and for interrupt notification.

| Port Interface |  |  |
| --- | --- | --- |
| Interface | ### IPortWaveCyclic |  |
| Inheritance | #### IPort |  |
| Description | This interface is supplied to the miniport for callbacks to the port driver. Notification is performed in the ISR. In the shared interrupt case, this may be done by the adapter driver. Slave and bus master DMA channels may also be instantiated on this interface. |  |
| Functions | void Notify() ? ULONG Channel |  |
|  | NTSTATUS NewSlaveDmaChannel() ? PDMACHANNEL *DmaChannel ? PUNKNOWN OuterUnknown ? POOL_TYPE PoolType ? PRESOURCELIST ResourceList ? ULONG DmaIndex ? ULONG MaximumLength ? BOOLEAN DemandMode ? DMA_SPEED DmaSpeed |  |
|  | NTSTATUS NewMasterDmaChannel() ? PDMACHANNEL *DmaChannel ? PUNKNOWN OuterUnknown ? POOL_TYPE PoolType ? PRESOURCELIST ResourceList ? ULONG DmaIndex ? ULONG MaximumLength ? BOOLEAN Dma32BitAddresses ? BOOLEAN Dma64BitAddresses ? DMA_SPEED DmaSpeed |  |

| IPortWaveCyclic |  |  |
| --- | --- | --- |
| Function | Notify() |  Passive  DPC  IRQ |
| Description | This function notifies the port driver that an interrupt indicating the progress of the DMA pointer has occurred. It should be called from the miniport’s interrupt service routine. This function is vital for accurate timing. Most miniport will call this function in response to a notification interrupt after having cleared the interrupt source. Although the miniport is free to use other methods to determine when to call this function, precise timing is important and should be maintained. When an ISR is installed, a context parameter (ServiceContext) is submitted along with the address of the ISR. When the interrupt occurs, the operating system calls the ISR, passing ServiceContext as one of the parameters of the ISR. It is up to the driver developer to determine what value ServiceContext will contain. It is recommended, though not required, that the ServiceContext be a pointer to the miniport object. The context will then allow access to any information required by the ISR. |  |
| Parameters | ULONG Channel The number of the channel for which notification is to occur. |  In  Out  Optional |
| Return | None. |  |

| IPortWaveCyclic |  |  |
| --- | --- | --- |
| Function | NewSlaveDmaChannel() |  Passive  DPC  IRQ |
| Description | This function creates a new instance of a slave DMA channel. |  |
| Parameters | PDMACHANNEL *DmaChannel The location at which the miniport deposits a pointer to the DMA channel object’s IDmaChannel interface. |  In  Out  Optional |
|  | PUNKNOWN OuterUnknown Pointer to the IUnknown interface of an object wishing to aggregate the DMA channel object. Unless aggregation is desired, this parameter will be NULL. |  In  Out  Optional |
|  | POOL_TYPE PoolType The type of memory pool from which the object should be allocated. |  In  Out  Optional |
|  | PRESOURCELIST ResourceList The resource list containing the resource which describes the DMA channel. |  In  Out  Optional |
|  | ULONG DmaIndex The index in the resource list of the DMA channel descriptor. This is used as a parameter to a Find…Entry() call performed on the resource list object. |  In  Out  Optional |
|  | ULONG MaximumLength The maximum length of the cyclic DMA buffer that will be associated with this channel. |  In  Out  Optional |
|  | BOOLEAN DemandMode Whether the device associated with the DMA channel supports demand mode. |  In  Out  Optional |
|  | DMA_SPEED DmaSpeed Compatible, TypeA, TypeB or TypeC. |  In  Out  Optional |
| Return | NTSTATUS STATUS_SUCCESS indicates a successful outcome. Exceptional return values TBD. |  |

| IPortWaveCyclic |  |  |
| --- | --- | --- |
| Function | NewMasterDmaChannel() |  Passive  DPC  IRQ |
| Description | This function creates a new instance of a bus master DMA channel. |  |
| Parameters | PDMACHANNEL *DmaChannel The location at which the miniport deposits a pointer to the DMA channel object’s IDmaChannel interface. |  In  Out  Optional |
|  | PUNKNOWN OuterUnknown Pointer to the IUnknown interface of an object wishing to aggregate the DMA channel object. Unless aggregation is desired, this parameter will be NULL. |  In  Out  Optional |
|  | POOL_TYPE PoolType The type of memory pool from which the object should be allocated. |  In  Out  Optional |
|  | PRESOURCELIST ResourceList The resource list containing the resource which describes the DMA channel. |  In  Out  Optional |
|  | ULONG DmaIndex The index in the resource list of the DMA channel descriptor. This is used as a parameter to a Find…Entry() call performed on the resource list object. |  In  Out  Optional |
|  | ULONG MaximumLength The maximum length of the cyclic DMA buffer that will be associated with this channel. |  In  Out  Optional |
|  | BOOLEAN Dma32BitAddresses Whether the device associated with the DMA channel supports 32 bit addresses. |  In  Out  Optional |
|  | BOOLEAN Dma64BitAddresses Whether the device associated with the DMA channel supports 64 bit addresses. |  In  Out  Optional |
|  | DMA_SPEED DmaSpeed Compatible, TypeA, TypeB or TypeC. |  In  Out  Optional |
| Return | NTSTATUS STATUS_SUCCESS indicates a successful outcome. Exceptional return values TBD. |  |

# PCI Scatter/Gather Wave Input and Output

This class of miniport drivers supports access to scatter/gather DMA-based wave audio functions.  The interface addresses physical mappings specifically, leaving streaming issues to be solved by the port driver.  Multiple channels are supported, each one unidirectional.

There are two interfaces that must be supported by the miniport.  The first provides for initialization of the miniport, the enumeration of channels and the creation of streams.  The second interface is the stream interface, which exposes most of the miniport’s functionality.

## Miniport Interfaces

The overall miniport interface has three functions:  one for initialization, one to enumerate channels and one to create a stream object.  The stream object interface (not derived from IMiniport) exposes the bulk of the miniport functionality.

| Miniport Interface |  |  |
| --- | --- | --- |
| Interface | ### IMiniportWavePci |  |
| Inheritance | #### IMiniport |  |
| Description | This is the interface exposed to the port driver by the device-associated object that implements the miniport. It provides a function for initialization, a function for enumerating channels and a function for creating a miniport stream object. The stream object is associated with a pin on a filter. There is no interface for a filter-associated miniport object. |  |
| Functions | NTSTATUS Init() ? PRESOURCELIST ResourceList ? PPORTWAVEPCI Port ? PDMACHANNEL *DmaChannel |  |
|  | NTSTATUS GetChannelList() ? PULONG Count ? PCHANNELLISTENTRY *Entries |  |
|  | NTSTATUS NewStream() ? PMINIPORTWAVEPCISTREAM *Stream ? PUNKNOWN OuterUnknown ? POOL_TYPE PoolType ? PPORTWAVEPCISTREAM PortStream ? ULONG Channel ? BOOLEAN Capture ? PKSDATAFORMAT DataFormat |  |

| IMiniportWavePci |  |  |
| --- | --- | --- |
| Function | Init() |  Passive  DPC  IRQ |
| Description | This function initializes the miniport. Initialization includes verification of the hardware using the resources specified in the resource list. |  |
| Parameters | PCM_RESOURCE_LIST ResourceList List of the resources to be used by the hardware associated with this miniport. |  In  Out  Optional |
|  | PPORTWAVEPCI Port Pointer to the port driver’s callback interface. |  In  Out  Optional |
|  | PDMACHANNEL *DmaChannel The DMA channel to be used by the miniport. This object is obtained through a call to the port driver’s lower edge. |  In  Out  Optional |
| Return | NTSTATUS STATUS_SUCCESS indicates a successful outcome. Exceptional return values TBD. |  |

| IMiniportWavePci |  |  |
| --- | --- | --- |
| Function | GetChannelList() |  Passive  DPC  IRQ |
| Description | This function obtains a list of channels supported by the miniport. |  |
| Parameters | PULONG Count The number of elements in the channel list. |  In  Out  Optional |
|  | PCHANNELLISTENTRY *Entries Pointer to the list of channels, an array of *Count elements. This list may be paged, but must remain at the supplied address until the miniport’s reference count reaches zero. The channel list entry structure is defined as follows: typedef struct { ULONG DataRangesCount; PKSDATARANGE DataRanges; ULONG Instances; ULONG Flags; #define CHANNELLISTENTRYFLAG_RENDER 1 #define CHANNELLISTENTRYFLAG_CAPTURE 2 } CHANNELLISTENTRY, *PCHANNELLISTENTRY; The first two fields supply a list of data ranges indicating formats supported by the channel. For more information regarding the KSDATARANGE structure, consult the WDM Streaming specification. The Instances field indicates how many instances of the channel may be created. The remaining field indicates whether the channel supports rendering or capture of audio data. |  In  Out  Optional |
| Return | NTSTATUS STATUS_SUCCESS indicates a successful outcome. Exceptional return values TBD. |  |

| IMiniportWavePci |  |  |
| --- | --- | --- |
| Function | NewStream() |  Passive  DPC  IRQ |
| Description | This function creates a new instance of a logical stream associated with a specified physical channel. |  |
| Parameters | PMINIPORTWAVEPCISTREAM Stream The location at which the miniport deposits a pointer to the stream object’s IMiniportWaveISAStream interface. |  In  Out  Optional |
|  | PUNKNOWN OuterUnknown Pointer to the IUnknown interface of an object wishing to aggregate the stream object. Unless aggregation is desired, this parameter will be NULL. |  In  Out  Optional |
|  | POOL_TYPE PoolType The type of memory pool from which the object should be allocated. |  In  Out  Optional |
|  | PPORTWAVEPCISTREAM PortStream Stream-associated callback interface to the port driver. |  In  Out  Optional |
|  | ULONG Channel The number of the channel. Input and output channels are numbered independently starting at zero. The first input channel is numbered 0, and the first output channels is numbered 0. |  In  Out  Optional |
|  | BOOLEAN Capture TRUE for an input (capture) channel, FALSE for an output (playback) channel. |  In  Out  Optional |
|  | PKSDATAFORMAT DataFormat The format to use for this instance. |  In  Out  Optional |
| Return | NTSTATUS STATUS_SUCCESS indicates a successful outcome. Exceptional return values TBD. |  |

| Miniport Interface |  |  |
| --- | --- | --- |
| Interface | ### IMiniportWavePciStream |  |
| Inheritance | #### IPropertyTarget |  |
| Description | This is the interface exposed to the port driver by the stream-associated object that implements the miniport. Most of the miniport functionality is exposed here. |  |
| Functions | NTSTATUS SetFormat() ? PKSDATAFORMAT DataFormat |  |
|  | NTSTATUS SetState() ? KSSTATE State |  |
|  | NTSTATUS GetPosition() ? PULONGLONG Position |  |
|  | NTSTATUS RevokeMappings() ? PVOID FirstTag ? PVOID LastTag ? PULONG MappingsRevoked |  |
|  | void MappingsAvailable() |  |

| IMiniportWavePciStream |  |  |
| --- | --- | --- |
| Function | SetFormat() |  Passive  DPC  IRQ |
| Description | This function sets the format of the stream. |  |
| Parameters | PKSDATAFORMAT DataFormat The new format of the stream. |  In  Out  Optional |
| Return | NTSTATUS STATUS_SUCCESS indicates a successful outcome. Exceptional return values TBD. |  |

| IMiniportWavePciStream |  |  |
| --- | --- | --- |
| Function | SetState() |  Passive  DPC  IRQ |
| Description | This function changes the state of the stream transport. |  |
| Parameters | KSSTATE State The new state of the stream, one of: - KSSTATE_STOP - KSSTATE_ACQUIRE - KSSTATE_PAUSE - KSSTATE_RUN For the purposes of many miniports, KSSTATE_ACQUIRE and KSSTATE_PAUSE are indistinguishable. |  In  Out  Optional |
| Return | NTSTATUS STATUS_SUCCESS indicates a successful outcome. Exceptional return values TBD. |  |

| IMiniportWavePciStream |  |  |
| --- | --- | --- |
| Function | GetPosition() |  Passive  DPC  IRQ |
| Description | This function gets the current position of the stream. |  |
| Parameters | PULONGLONG Position The current byte position of the stream. The miniport is required to maintain stream position based on the mappings it acquires, regardless of whether mappings are released or revoked. The position indicated by this function is the miniport’s best estimate of the byte position of the data currently in the DAC or ADC. The position is initially zero and is reset by a transition into the stopped state. |  In  Out  Optional |
| Return | NTSTATUS STATUS_SUCCESS indicates a successful outcome. Exceptional return values TBD. |  |

| IMiniportWavePciStream |  |  |
| --- | --- | --- |
| Function | RevokeMappings() |  Passive  DPC  IRQ |
| Description | This function revokes mappings that were previously obtained via GetMapping(). Mappings are revoked when a request (IRP) is cancelled and the memory previously mapped may not be available. |  |
| Parameters | PVOID FirstTag The tag associated with the first mapping that is being revoked. |  In  Out  Optional |
|  | PVOID LastTag The tag associated with the last mapping that is being revoked. |  In  Out  Optional |
|  | PULONG MappingsRevoked The number of mappings in the indicated interval that were actually mapped (not yet released). Due to synchronization issues, some of the revoked mappings may already have been released by the miniport. |  In  Out  Optional |
| Return | NTSTATUS STATUS_SUCCESS indicates a successful outcome. Exceptional return values TBD. |  |

| IMiniportWavePciStream |  |  |
| --- | --- | --- |
| Function | MappingsAvailable() |  Passive  DPC  IRQ |
| Description | This function indicates that new mappings are available. It will be called when new mappings become available after miniport requests for mappings via GetMapping() have been refused. |  |
| Parameters | None. |  |
| Return | None. |  |

## Port Driver Lower-Edge Interface

The port driver provides an interface to the miniport at the device level and at the stream level.  The interface at the device level allows only for the instantiation of the DMA channel returned by the miniport initialization function.  The interface at the stream level supplies mappings to the miniport stream.

| Port Interface |  |  |
| --- | --- | --- |
| Interface | ### IPortWavePci |  |
| Inheritance | #### IPort |  |
| Description | This is the device-associated callback interface implemented by the port driver and providing services to the miniport object. |  |
| Functions | NTSTATUS NewDmaChannel() ? PDMACHANNEL *DmaChannel ? PUNKNOWN OuterUnknown ? POOL_TYPE PoolType ? ULONG MaximumLength ? BOOLEAN Dma32BitAddresses ? BOOLEAN Dma64BitAddresses ? DMA_SPEED DmaSpeed |  |

| IPortWavePci |  |  |
| --- | --- | --- |
| Function | NewDmaChannel() |  Passive  DPC  IRQ |
| Description | This function creates a new instance of a bus master DMA channel. |  |
| Parameters | PDMACHANNEL *DmaChannel The location at which the miniport deposits a pointer to the DMA channel object’s IDmaChannel interface. |  In  Out  Optional |
|  | PUNKNOWN OuterUnknown Pointer to the IUnknown interface of an object wishing to aggregate the DMA channel object. Unless aggregation is desired, this parameter will be NULL. |  In  Out  Optional |
|  | POOL_TYPE PoolType The type of memory pool from which the object should be allocated. |  In  Out  Optional |
|  | ULONG MaximumLength The maximum length of the cyclic DMA buffer that will be associated with this channel. |  In  Out  Optional |
|  | BOOLEAN Dma32BitAddresses Whether the device associated with the DMA channel supports 32 bit addresses. |  In  Out  Optional |
|  | BOOLEAN Dma64BitAddresses Whether the device associated with the DMA channel supports 64 bit addresses. |  In  Out  Optional |
|  | DMA_SPEED DmaSpeed Compatible, TypeA, TypeB or TypeC. |  In  Out  Optional |
| Return | NTSTATUS STATUS_SUCCESS indicates a successful outcome. Exceptional return values TBD. |  |

| Port Interface |  |  |
| --- | --- | --- |
| Interface | ### IPortWavePciStream |  |
| Inheritance | #### IUnknown |  |
| Description | This is the stream-associated callback interface implemented by the port driver and providing services to miniport stream objects. |  |
| Functions | NTSTATUS GetMapping() ? PVOID Tag ? PPHYSICAL_ADDRESS PhysicalAddress ? PULONG ByteCount ? PULONG Flags |  |
|  | NTSTATUS ReleaseMapping() ? PVOID Tag |  |
|  | NTSTATUS TerminatePacket() |  |

| IPortWavePciStream |  |  |
| --- | --- | --- |
| Function | GetMapping() |  Passive  DPC  IRQ |
| Description | This function obtains a mapping from the port driver and associates a tag with the mapping. Mappings obtained through this function must be released using ReleaseMapping() unless they are revoked by the port driver. Mappings are revoked by a call to the stream’s RevokeMappings() function. |  |
| Parameters | PVOID Tag The tag to be associated with the mapping. This tag may be used in subsequent RevokeMappings() calls to identify the mapping for revocation. |  In  Out  Optional |
|  | PPHYSICAL_ADDRESS PhysicalAddress The physical address of the mapping. |  In  Out  Optional |
|  | PULONG ByteCount The number of bytes in the mapping. |  In  Out  Optional |
|  | PULONG Flags A non-zero flag value indicates that the mapping acquired in this call is the last mapping in a packet. This flag recommends that the hardware interrupt the miniport when it is done with this mapping. In response to the interrupt, the miniport would obtain new mappings to deliver to the hardware. The miniport is not obligated to use this suggestion. |  In  Out  Optional |
| Return | NTSTATUS STATUS_SUCCESS indicates a successful outcome. Exceptional return values TBD. |  |

| IPortWavePciStream |  |  |
| --- | --- | --- |
| Function | ReleaseMapping() |  Passive  DPC  IRQ |
| Description | This function releases mappings previously obtained through GetMapping(). |  |
| Parameters | PVOID Tag The tag of the mapping to be released. |  In  Out  Optional |
| Return | NTSTATUS STATUS_SUCCESS indicates a successful outcome. Exceptional return values TBD. |  |

| IPortWavePciStream |  |  |
| --- | --- | --- |
| Function | TerminatePacket() |  Passive  DPC  IRQ |
| Description | This function terminates the packet currently being mapped. This is used primarily for capture situations in which packets need to end at specific points in the stream (e.g. for video frames). |  |
| Parameters | None. |  |
| Return | NTSTATUS STATUS_SUCCESS indicates a successful outcome. Exceptional return values TBD. |  |

# Static Wave Table Output

This class of miniport drivers supports access to wavetable synthesizer voices.  It is intended for devices that provide on-board articulation functions that can be manipulated by system software.

The specification of this interface has been temporarily withdrawn.  Subsequent revisions of this document will include this interface.

# Topology

This class of miniport drivers supports access to the various controls (volume, equalization, hardware reverb) that audio adapters typically offer.  It does this by exposing a topology consisting of a set of nodes and their interconnections.  A node can have any number of properties, one of which may be the control value.

Controls are organized in a hierarchy to allow arbitrary subdivision of the control graph.  A top-level control, designated by the identifier 0, contains, directly or indirectly, all other controls in the graph.

Nodes that have control values may have any type of value that can be supported by the property mechanism.  Typically, a node will have at most one control value.  For example, a volume node would have a value indicating its current volume setting.  Volume and pan would be exposed as separate nodes.

Nodes are connected to one another by entities called connections.  Connections are directed, referring to a from node and a to node.  A connection also specifies a pin number for both of its associated nodes.  Pins are not explicitly supported as entities in the topology, but the pin numbering in the connection entities allows inputs and outputs with distinct functions to be differentiated.  For example, a state variable filter might have three output pins, one each for high, middle and low frequencies.  Pin numbering allows clients of the miniport to determine which connections are associated with which pins.

A distinguished node identification is used to represent the topology’s external connections.  In the WDM Streaming world, the pins on this ‘filter node’ are the pins on the filter implemented by the topology port driver.  These pins do not allow connection or support data flow, but instead represent physical connections of the adapter hardware.  Some of the filter pins represent connections internal to the system, such as the point at which analog data from a DAC enters the topology.  Other filter pins represent external physical connections, such as the line out jack on an audio adapter.

Node pin numbering alone does not allow the client to determine the function of the pins.  Although the miniport identifies the type of a node (by means of a GUID), it offers no description of the node type and no description of the pins supported by the node type.  The specification for how this information is discovered is left to other documents.

NOTE:  The set of node types and associated properties is detailed in a document entitled WDM Audio Driver Design Notes and Reference.

This miniport interface also supports the delivery of unsolicited control value changes from the miniport to the port driver.  This feature allows for devices that can be physically manipulated by the user.  External changes to node control values are indicated by a notification call to the lower edge of the port driver.

The topology miniport interface relies on a descriptor for describing connections between nodes:

#define FILTER_NODE ((ULONG)-1)

typedef struct

{

ULONG	FromNode;

ULONG	FromNodePin;

ULONG	ToNode;

ULONG	ToNodePin;

} TOPOLOGY_CONNECTION;

This structure is used to describe a connection between two pins in a topology.  A given pin may take part in multiple connections.

| Field | Description |
| --- | --- |
| FromNode | Identifier of the node which produces information for this connection, or FILTER_NODE if the pin is on the WDM Streaming filter. |
| FromNodePin | Identifier of the connected pin on the ‘from’ node. |
| ToNode | Identifier of the node which consumes information for this connection, or FILTER_NODE if the pin is on the WDM Streaming filter. |
| ToNodePin | Identifier of the connected pin on the ‘to’ node. |

## Miniport Interface

| Miniport Interface |  |  |
| --- | --- | --- |
| Interface | ### IMiniportTopology |  |
| Inheritance | #### IMiniport |  |
| Description | This is the interface exposed to the port driver by the device-associated object that implements the miniport. It provides functions for initialization of the miniport, the enumeration of pins, nodes and connections and the acquisition of control change information. |  |
| Functions | NTSTATUS Init() ? PRESOURCELIST ResourceList ? PPORTTOPOLOGY Port |  |
|  | NTSTATUS GetCounts() ? PULONG PinCount ? PULONG NodeCount ? PULONG ConnectionCount |  |
|  | NTSTATUS GetPins() ? ULONG First ? ULONG Requested ? PULONG Delivered ? PTOPOLOGY_PIN Buffer |  |
|  | NTSTATUS GetNodes() ? ULONG First ? ULONG Requested ? PULONG Delivered ? GUID *Buffer |  |
|  | NTSTATUS GetConnections() ? ULONG First ? ULONG Requested ? PULONG Delivered ? PTOPOLOGY_CONNECTION Buffer |  |
|  | NTSTATUS ReadControlChanges() ? ULONG Requested ? PULONG Delivered ? PULONG Buffer |  |
|  | NTSTATUS FlushControlChanges() |  |

| IMiniportTopology |  |  |
| --- | --- | --- |
| Function | Init() |  Passive  DPC  IRQ |
| Description | This function initializes the miniport.. |  |
| Parameters | PRESOURCELIST ResourceList List of the resources to be used by the hardware associated with this miniport. |  In  Out  Optional |
|  | PPORTTOPOLOGY Port Pointer to the port driver’s callback interface. |  In  Out  Optional |
| Return | NTSTATUS STATUS_SUCCESS indicates a successful outcome. Exceptional return values TBD. |  |

| IMiniportTopology |  |  |
| --- | --- | --- |
| Function | GetCounts() |  Passive  DPC  IRQ |
| Description | This function gets counts for the various entities provided by the miniport. |  |
| Parameters | PULONG PinCount The number of pins. |  In  Out  Optional |
|  | PULONG NodeCount The number of nodes. |  In  Out  Optional |
|  | PULONG ConnectionCount The number of connections. |  In  Out  Optional |
| Return | NTSTATUS STATUS_SUCCESS indicates a successful outcome. Exceptional return values TBD. |  |

| IMiniportTopology |  |  |
| --- | --- | --- |
| Function | GetPins() |  Passive  DPC  IRQ |
| Description | This function gets pin descriptors from the miniport. |  |
| Parameters | ULONG First The identifier of the first pin to get a descriptor for. |  In  Out  Optional |
|  | ULONG Requested The number of pin descriptors requested. |  In  Out  Optional |
|  | PULONG Delivered The number of pin descriptors delivered. |  In  Out  Optional |
|  | PTOPOLOGY_PIN Buffer The buffer in which pin descriptors are to be deposited. This buffer must be sufficiently large to contain all the pin descriptors requested. Entries in this buffer have the following format: typedef struct { PKSDATAFORMAT Format; KSPIN_DATAFLOW DataFlow; } TOPOLOGY_PIN, *PTOPOLOGY_PIN; The first field indicates the format of the pin. The second indicates the direction that information flows through the pin. The WDM Streaming specification provides more information on both of the types used in TOPOLOGY_PIN. The format structures referenced from entries in the buffer may be paged but must remain at the indicated addresses until the miniport’s reference count drops to zero. |  In  Out  Optional |
| Return | NTSTATUS STATUS_SUCCESS indicates a successful outcome. Exceptional return values TBD. |  |

| IMiniportTopology |  |  |
| --- | --- | --- |
| Function | GetNodes() |  Passive  DPC  IRQ |
| Description | This function gets node type GUIDs from the miniport. |  |
| Parameters | ULONG First The identifier of the first node to get a type GUID for. The range of node identifiers runs from 0 to the number of nodes minus one. |  In  Out  Optional |
|  | ULONG Requested The number of node type GUIDs requested. |  In  Out  Optional |
|  | PULONG Delivered The number of node type GUIDs delivered. |  In  Out  Optional |
|  | GUID *Buffer The buffer in which node type GUIDs are to be deposited. This buffer must be sufficiently large to contain all the node type GUIDs requested. |  In  Out  Optional |
| Return | NTSTATUS STATUS_SUCCESS indicates a successful outcome. Exceptional return values TBD. |  |

| IMiniportTopology |  |  |
| --- | --- | --- |
| Function | GetConnections() |  Passive  DPC  IRQ |
| Description | This function gets connection descriptors from the miniport. |  |
| Parameters | ULONG First The index of the first connection to get a descriptor for. The range of connection indexes runs from 0 to the number of connections minus one. |  In  Out  Optional |
|  | ULONG Requested The number of connection descriptors requested. |  In  Out  Optional |
|  | PULONG Delivered The number of connection descriptors delivered. |  In  Out  Optional |
|  | PTOPOLOGY_CONNECTION Buffer The buffer in which connection descriptors are to be deposited. This buffer must be sufficiently large to contain all the connection descriptors requested. Entries in the buffer have the following structure: typedef struct { ULONG FromNode; ULONG FromPin; ULONG ToNode; ULONG ToPin; } TOPOLOGY_CONNECTION, *PTOPOLOGY_CONNECTION; The From… pair of fields indicates the pin data flows from in the connection. The To… pair of fields indicates the pin data flows to in the connection. Nodes are numbered contiguously starting at zero. The distinguished node number (ULONG)-1 represents the filter itself. The interpretation of node pin numbers is dependent on the node type. The filter pin numbers are indices into the pin array. |  In  Out  Optional |
| Return | NTSTATUS STATUS_SUCCESS indicates a successful outcome. Exceptional return values TBD. |  |

| IMiniportTopology |  |  |
| --- | --- | --- |
| Function | ReadControlChanges() |  Passive  DPC  IRQ |
| Description | This function reads control value changes induced externally. It is called during the port driver’s DPC to determine what control value changes occurred due to an external influence. For example, if the miniport controls a motorized studio mixer, rotation of the mixer knobs by the user would cause external control value changes to be reported. |  |
| Parameters | ULONG Requested The number of value changes requested. |  In  Out  Optional |
|  | PULONG Delivered The number of control value changes delivered. |  In  Out  Optional |
|  | PULONG Buffer Node identifiers for the controls that have changed values. |  In  Out  Optional |
| Return | NTSTATUS STATUS_MORE_ENTRIES indicates a successful outcome and that more value changes await subsequent calls to this function. STATUS_SUCCESS indicates a successful outcome and that all value changes have been read. STATUS_NO_MORE_ENTRIES indicates that no value changes currently exist. Exceptional return values TBD. |  |

| IMiniportTopology |  |  |
| --- | --- | --- |
| Function | FlushControlChanges() |  Passive  DPC  IRQ |
| Description | This function discards external control value changes so they need not be read. |  |
| Parameters | None. |  |
| Return | NTSTATUS STATUS_SUCCESS indicates a successful outcome. Exceptional return values TBD. |  |

## Port Driver Lower-Edge Interface

| Port Interface |  |  |
| --- | --- | --- |
| Interface | ### IPortTopology |  |
| Inheritance | #### IPort |  |
| Description | This is the interface exposed by the port driver to the device-associated object that implements the miniport. It provides a function to notify the port driver that control changes are available to be read. |  |
| Functions | void NotifyControlChanges() |  |

| IPortTopology |  |  |
| --- | --- | --- |
| Function | NotifyControlChanges() |  Passive  DPC  IRQ |
| Description | This function notifies the port driver of the arrival of external control value changes. It is typically called from an interrupt service routine (ISR). When an ISR is installed, a context parameter (ServiceContext) is submitted along with the address of the ISR. When the interrupt occurs, the operating system calls the ISR, passing ServiceContext as one of the parameters of the ISR. It is up to the driver developer to determine what value ServiceContext will contain. It is recommended, though not required, that the ServiceContext be the miniport object pointer. The context will then contain any information required by the ISR. |  |
| Parameters | None. |  |
| Return | None. |  |

# MIDI Input and Output

This miniport definition is intended for MIDI devices of the UART variety:  that is, devices that do not have their own timing hardware.  The sound port handles all the timing issues in both directions, and relies on the miniport to promptly move data on and off the adapter in response to the calls listed below.

## Miniport Interface

| Miniport Interface |  |  |
| --- | --- | --- |
| Interface | ### IMiniportMidi |  |
| Inheritance | #### IMiniport |  |
| Description | This is the interface exposed to the port driver by the device-associated object that implements the miniport. It provides functions for initialization of the miniport and the reading and writing of MIDI data. |  |
| Functions | NTSTATUS Init() ? PRESOURCELIST ResourceList ? PPORTMIDI Port |  |
|  | NTSTATUS DeferredRead() ? ULONG Channel ? PVOID BufferAddress ? ULONG Length ? PULONG BytesRead |  |
|  | NTSTATUS Flush() ? ULONG Channel ? PULONG BytesFlushed |  |
|  | NTSTATUS Write() ? ULONG Channel ? PVOID BufferAddress ? ULONG Length ? PULONG BytesRead |  |

| IMiniportMidi |  |  |
| --- | --- | --- |
| Function | Init() |  Passive  DPC  IRQ |
| Description | This function initializes the miniport. |  |
| Parameters | PRESOURCELIST ResourceList List of the resources to be used by the hardware associated with this miniport. |  In  Out  Optional |
|  | PPORTMIDI Port Pointer to the port driver’s callback interface. |  In  Out  Optional |
| Return | NTSTATUS STATUS_SUCCESS indicates a successful outcome. Exceptional return values TBD. |  |

| IMiniportMidi |  |  |
| --- | --- | --- |
| Function | DeferredRead() |  Passive  DPC  IRQ |
| Description | This function supplies incoming MIDI data to the port driver. It is generally called in response to an interrupt generated by the MIDI input device indicating the arrival of new data. |  |
| Parameters | ULONG Channel The number of the channel to read from. |  In  Out  Optional |
|  | PVOID BufferAddress Requested MIDI data is deposited here. |  In  Out  Optional |
|  | ULONG Length The maximum number of bytes of MIDI data to read. |  In  Out  Optional |
|  | PULONG BytesRead The actual number of bytes read. This number will be Length or the number of bytes currently available, whichever is less. |  In  Out  Optional |
| Return | NTSTATUS STATUS_SUCCESS indicates a successful outcome and that no more data is available to be read. Miniports that cannot easily determine whether more data is available may return STATUS_SUCCESS_MORE_ENTRIES whenever BytesRead is equal to Length. STATUS_NO_MORE_ENTRIES indicates that no data is currently available to be read. Exceptional return values TBD. |  |

| IMiniportMidi |  |  |
| --- | --- | --- |
| Function | Flush() |  Passive  DPC  IRQ |
| Description | This function discards incoming MIDI data from the device. It is used when data is available for reading, but all outstanding read requests have been satisfied. |  |
| Parameters | ULONG Channel The number of the channel to flush. |  In  Out  Optional |
|  | PULONG BytesFlushed The number of bytes that were discarded. The port driver may pass a NULL pointer if it does not require this information to be passed back. |  In  Out  Optional |
| Return | NTSTATUS STATUS_SUCCESS indicates a successful outcome. Exceptional return values TBD. |  |

| IMiniportMidi |  |  |
| --- | --- | --- |
| Function | Write() |  Passive  DPC  IRQ |
| Description | This function supplies outgoing MIDI data to the miniport. When the number of bytes written by the call is less than the indicated length, the port driver will call this function again to write the remaining data. The call should not block on the transmission of data. |  |
| Parameters | ULONG Channel The number of the channel to write to. |  In  Out  Optional |
|  | PVOID BufferAddress Pointer to a buffer containing MIDI data to be written. |  In  Out  Optional |
|  | ULONG Length The maximum number of bytes of MIDI data to write. |  In  Out  Optional |
|  | PULONG BytesWritten The actual number of bytes written. |  In  Out  Optional |
| Return | NTSTATUS STATUS_SUCCESS indicates a successful outcome. Exceptional return values TBD. |  |

## Port Driver Lower-Edge Interface

| Port Interface |  |  |
| --- | --- | --- |
| Interface | ### IPortMidi |  |
| Inheritance | #### IPort |  |
| Description | This is the interface exposed by the port driver to the device-associated object that implements the miniport. It provides a function to notify the port driver that data is available to be read. |  |
| Functions | void NotifyIncoming() ? ULONG Channel |  |

| IPortMidi |  |  |
| --- | --- | --- |
| Function | NotifyIncoming() |  Passive  DPC  IRQ |
| Description | This function notifies the port driver of the arrival of MIDI data to be read. It is typically called from an interrupt service routine (ISR). When an ISR is installed, a context parameter (ServiceContext) is submitted along with the address of the ISR. When the interrupt occurs, the operating system calls the ISR, passing ServiceContext as one of the parameters of the ISR. It is up to the driver developer to determine what value ServiceContext will contain. It is recommended, though not required, that the ServiceContext be the miniport object pointer. The context will then contain any information required by the ISR. |  |
| Parameters | ULONG Channel The number of the channel on which data is available to be read. |  In  Out  Optional |
| Return | None. |  |