Program:        pciexp.exe

Author:         NCR MPD

Purpose:        53C8xx Test Software

This software was written by NCR Microelectronics to develop and

test new products.  NCR assumes no liability for its use.  This

software is released to the public domain to illustrate certain

programming techniques for the 53c800 family.

INTRODUCTION

This program provides and example of how to "correctly" setup

and use and 53C8xx as a SCSI driver.  The program inits the chip and

an interrupt service routine to handle interrupt from of chip.  The

example script is a general purpose target driven script.  This

allows virtually any SCSI command to be executed with the same

script.

PROGRAM LAYOUT

The program is contained in four modules:

main.c          Contains all the code that inits the C8xx and the

hardware.  All low-level communication with the chip

is contained in this module.

commands.c	Contains the code to setup the tables and start the

script to perform different SCSI commands.

os.c		A simple menu driven program that calls the commands

defined in commands.c.  A this level all SCSI details

and hardware is hidden.

tools.c		Misc. software tools that are used by the rest of the

program.

pci.c           Contains PCI specific routines

general.ss	This SCSI SCRIPTS used by this program.  This script

is a totally target driven script that can handle

many different SCSI commands.

These modules are used to provide progressive levels of

abstraction from the hardware. Most of the functions in the first two

modules are contained in the device driver for a SCSI device.

DESIGN DETAILS

The script that is executed by this program is totally target

driven.  The program just inits the locations of the command, data,

and message buffers and starts the script.  The script performs

whatever functions the target wishes.  In this way, any command can

be executed by just putting that command in the command buffer and

setting up the data buffer correctly for that command.

All of the data is passed to the script using the

table-indirect addressing mode of the C8xx.  The DSA is initialized to

point at a table of counts and addresses to be used in the script

instructions.  The offsets into that table for the different buffers

are stored in the script.

SUPPORT FOR SCATTER/GATHER

This program was not intended as an example on how to handle

Scatter/Gather type files requests. (Where many different pieces of

memory need to be written end-for-end to the disk)  However in

solving another problem, I provided one way this kind of transfer can

be done.

This feature started out with the question "How do I handle

a drive that disconnects between each block on multiblock reads?"  In

other words if I request 10 blocks from the disk in one read command

and the drive sends the first 4 blocks, disconnects to seek to a new

track, reconnects, and then sends the rest of the data, what is going

to keep the script from getting a phase mismatch in the middle of

the one block move?

The first solution that comes to mind is to check the phase

after every block transferred. This works but how do you continue the

transfer where it was left off?  All we can tell in the script is that

the current phase is DATA_IN and we should jump back to the data move

routines.

The solution used in this program is to have a separate table

entry for every block to be transferred.  After each block the script

modifies its offset in the table to point to the next table entry.

This way each block that is read/written can go to a different

address and even be a different length.

Since we are now able to read data from many different places

to write to the disk or read from the disk and write to several

different places, this script can solve several scatter/gather

problems.

#