Writing SCSI Miniport Drivers for Windows 95

#### Microsoft Windows 95

Device Driver Developer Support Team

(OEM/IHV)

We Develop Developers

| FAQ Number: | 1004 | Date Created: | 20 October 1997 |
| --- | --- | --- | --- |
|  |  | Last Updated: | 25 November 1997 |

Copyright © 1998 Microsoft Corporation.

##### Table of Contents

1.  Background	2

2.  How SCSI fits into the Windows 95 I/O Subsystem	2

2.1  General	2

2.2  Windows NT IOCTL unavailable with Windows 95.	5

2.3  Older Windows NT miniport drivers	5

2.4  Unraveling the Scatter Gather Descriptor (SGD) Confusion	6

2.5  BufferAccessScsiPortControlled Flag Information.	6

2.6  Debugging Techniques	8

2.7  Format of a SCSI PNP ID	8

2.8  How to uniquely identify a specific SCSI adapter in a DCB	9

2.9  Dynamic device removal/installation (hot-swapping)	9

2.10  ScsiPortSetBusDataByOffset	9

2.11  On obtaining both an I/O range and a memory window from within a SCSI miniport:	10

2.12  Mapping between physical and linear (logical) memory	10

3.  Additional Differences in SCSI between Windows 95 and Windows NT	10

3.1  Plug and Play	10

3.2  Using the ATAPI miniport driver sample located in the Windows NT DDK	10

4.  Miscellaneous Questions	12

5.  IOP Structures	16

6.  Additional Reference Material	25

7.  List of Knowledge Base Articles for Windows 95 SCSI Miniports	25

8.  List of Knowledge Base Articles for Windows NT SCSI Miniports	25

# 1.  Background

This document supplements the SCSI Miniport documentation in the Windows 95 DDK (STORAGE.DOC).

Additional information about this topic can be found in the documents listed in Section 6, Additional Reference Material.

Sections 7 and 8 contain relevant Knowledge Base articles, which can also be found in the MSDN Library CD and Microsoft’s web site (http://www.microsoft.com).

# 2.  How SCSI fits into the Windows 95 I/O Subsystem

## 2.1  General

Figure 1 - The Windows 95 Storage Device Device Driver Model (on the following page) describes the overall structure of device drivers that are used to implement local storage devices.  Note that each module is numbered, for ease of reference.

Figure 1 - The Windows 95 Storage Device Device Driver Model

The following table describes the modules relevant to SCSI Miniport developers:

| Module(s) | ###### Comments |
| --- | --- |
| 4, 5, 11, 12, 18 | These modules implement the ASPI specification. ASPI is designed to provide low level communications (at the SCSI Request Block, or SRB, level) between applications and SCSI miniport devices or IDE ATAPI devices. |
| 14 | The Volume Tracker is used to accommodate removable media devices, including SCSI devices. |
| 15 | DISKTSD.VXD assigns drive letter(s) to disk-type storage devices. |
| 16 | CDTSD.VXD assigns drive letter(s) to CDROM storage devices. |
| 17 | RMM.VXD is used to direct I/O to real-mode driver(s), for when there is no functioning 32-bit protected mode device driver for the device. |
| 20 | DISKVSD.VXD is the disk device SCSI’izer, amending its IOP into a full blown SCSI request (complete with SCSI Request Block). |
| 21 | CDVSD.VXD is the CD-ROM device SCSI’izer, amending its IOP into a full blown SCSI request (complete with SCSI Request Block). |
| 23 | SCSI1HLP.VXD inspects CD-ROM I/O requests and compares the request against the manufacturer and model of CD-ROM. It modifies the request if needed, to accommodate special requirements, or correct unusual behavior, of certain CD-ROM devices. In some cases, SCSI1HLP will force SCSI 1 protocol behavior. SCSI1HLP also “helps” ATAPI CD-ROM devices. |
| 25 | ESDI_506.PDR handles IDE drives and ATAPI devices. ATAPI CDROM and similar ATA devices appear to IOS as if they are SCSI devices, i.e. the IOP packet includes a SCSI Request Block (SRB). |
| 26 | SCSIPORT.PDR supplies the interface to 1 or more SCSI Miniport device drivers. SCSIPORT implements the Windows NT miniport standard, orchestrates the loading and initialization of miniport drivers, exports a number of services available to Miniport device drivers, and coordinates I/O between IOS and the miniport driver. |
| 29 | The Windows NT Device Driver Kit supplies a sample ATAPI.SYS miniport driver. This driver sample is binary compatible with Windows 95. If used, it is renamed to ATAPI.MPD and placed into the windows\system\iosubsys directory. This miniport driver can be used to replace much of the functionality of ESDI_506.PDR. |
| 30 | SCSI Miniport drivers commonly interface SCSI adapter cards (and the attached SCSI devices) to the rest of the system. |
| 31 | Miniport drivers can also be used to interface Parallel-port based devices such as external tape drives and external hard drives, or to interface unusual hardware, making the device appear as a SCSI device. |

The Windows 95 Device Driver Kit documentation (STORAGE.DOC) discusses in more detail how a SCSI miniport interfaces with the rest of the system.

Here is a brief summary of the SCSI Miniport initialization sequence:

- IOS calls the miniport's DriverEntry routine as a result of Configuration Manager devnode enumeration.

- The miniport calls ScsiPortInitialize (for each supported bus)

- SCSIPORT.PDR calls IOS_Register as a result of each received ScsiPortInitialize

- IOS_Register presents an AEP_initialize message to SCSIPORT.  SCSIPORT creates a DDB, reads miniport ASCII configuration info from registry, then calls the miniport's HwFindAdapter routine (e.g. FindController). This should occur for each bus.

- IOS_Register creates a temporary "inquiry" DCB and issues an AER_Device_Inquiry for each supported LUN number etc.

- Each instance of step 5 causes SCSIPORT to create an IOP, for a SCSI_PASS_THROUGH inquiry command, and sends it to the miniport’s StartIo  routine.

- For each inquiry, the miniport either succeeds or fails.  If it succeeds, SCSIPORT sets up the DCB_product_id, DCB_vendor_id and DCB_rev_level into the  DCB and returns AEP_success. to IOS.

Steps 2-7 occur for each bus type supported.

The miniport interface is the same between Windows NT and Windows 95, and hence the Windows NT DDK’s ATAPI miniport sample, when compiled under NT, works under Windows 95.  There is a strict miniport protocol, designed to be portable (binary code compatible) between systems.  This protocol is violated, however, if there are embedded Windows 95 VxDCall(s) or embedded Windows NT system calls within the SCSI miniport driver.  Unfortunately, there are a few cases under Windows 95 where VxDCall(s) are necessary, thus breaking binary compatibility.  For example, Windows 95’s ScsiPortGetDeviceBase function fails to return a linear address (as described in KB article Q169584).  This means that the miniport must use a Windows 95 VMM function _MapPhysToLinear to map the physical address to a linear address.

## 2.2  Windows NT IOCTL unavailable with Windows 95.

Windows NT uses a miniport IOCTL interface that's not available with Windows 95.

Windows NT allows you to send IOCTL's to SCSI miniports, using CreateFile, and a filespec such as  “\\.\Scsi0” and “\\.\c:”.  A sample application that uses this technique is found in the Windows NT DDK: \ddk\src\storage\class\spti.

Windows 95 does not support this Windows NT IOCTL technique.

If you want to issue private commands to a (Windows 95) miniport, you can send a SCSI request to it using ASPI16 or ASPI32 (which uses the IOS “SCSI passthrough” message structure).  Create a unique function code in the CDB being sent to it, that your miniport can interpret as a custom command. There is sample code in the Windows 95 DDK (\DDK\Block\SAMPLES\WNASPI32, documentation is in \DDK\Docs\DESGUIDE\STORAGE.DOC) which performs simple drive inquiry functions. ASPI is implemented using WINASPI.DLL (for 16 bit apps) or WNASPI32.DLL (32 bit apps) at Ring 3.  These DLL's talk to (ring 0) APIX.VXD located in the IOS layered hierarchy.  APIX, located at layer 11 of IOS, injects IOR_SCSI_PASS_THROUGH commands into IOS.

Another possible communications technique under Windows 95 is to write a "helper" Vendor Supplied Driver (VSD) that communicates with the user application via (Windows 95) DeviceIoControl(), behaving similarly to APIX.VXD (used by ASPI).  The VSD could setup its own private IOP pointing to a private SRB containing the appropriate SRB_FUNCTION_IO_CONTROL function.  The VSD could then issue an ILB_internal_request, which sends the IOP request down to the miniport driver.

There is a sample passthrough VSD, available from the Windows 95 DDK Support Team, bundled in the following file: PASSTHRU.ZIP.  This sample demonstrates how to build the necessary SCSI Passthrough IOP packet.  The sample will require changes, for example, in its IOCTL handler which currently does nothing but will need to be modified to forward requests using ILB_internal_request.

An example of a Win32 application communicating with a generic Windows 95VXD using CreateFile() and DeviceIoControl() is in the Windows 95 DDK, \DDK\Base\SAMPLES\ASYNCW32. This sample, coupled with the Passthrough sample, can be used to create the desired “helper” VSD.

## 2.3  Older Windows NT miniport drivers

Older miniport drivers written for Windows NT do not include Plug and Play information and, therefore, will not perform well on Windows 95.

## 2.4  Unraveling the Scatter Gather Descriptor (SGD) Confusion

There are two different types of Scatter Gather Descriptors; linear (a.k.a. logical) and physical.

Walt Oney's book Systems Programming for Windows 95 discusses this issue somewhat, on pages 542-543.

Linear SGDs

The linear SGD structure is defined in \ddk\inc32\blockdev.h.

A linear SGD consists of a DWORD count followed by a DWORD linear pointer.  The count is always assumed to be a block count within SCSIPORT's transfer routines (such as ScsiPortReadPortBufferUchar).  Linear SGDs are terminated with a zero length SGD element, but you should use IOR_xfer_count for total length info (this field contains the sector count if IORF_CHAR_COMMAND is not set).  The IOP's IOR_buffer_ptr points to a normal linear memory buffer if (IOR_flags & IORF_SCATTER_GATHER)==FALSE.  Otherwise, IOR_buffer_ptr points to a simple list of linear SGD's as defined in blockdev.h.

Physical SGDs

The physical SGD structure is defined in \ddk\inc32\sgd.h.

The physical SGD structure consists of a DWORD physical pointer followed by a DWORD count. (the ordering is reverse that of the linear SGD structure).  If the port driver demanded the physical SGD creation service then the DCB_dmd_phys_sgd bit will be set within DCB_dmd_flags.

Inside SCSIPORT.PDR, the  ILB_int_io_criteria_rtn is called before forwarding any read/write to the miniport driver. ILB_int_io_criteria_rtn will create physical SGDs if DCB_dmd_phys_sgd is set.  In which case, the IOP's IOR_sgd_lin_phys field must point to a valid memory buffer, designed to accommodate the max number of physical SGDs (DCB_max_sg_elements), remembering that each SGD is 8 bytes long.  It would be safe to assign (17*8) bytes of memory for IOR_sgd_lin_phys since the maximum number of SGDs is 17.

To obtain an updated copy of STORAGE.DOC (containing an expanded explanation of ILB_int_io_criteria_rtn), contact the Windows 95 DDK Technical Support Team in Microsoft Technical Support.

## 2.5  BufferAccessScsiPortControlled Flag Information.

The KB article Q116450, BufferAccessScsiPortControlled Flag Information, describes a mechanism in Windows 95 to improve miniport performance (throughput).

Under Windows 95, if the miniport driver does not report that it supports scatter-gather, then it will only see read requests for one sector at a time.

The ScatterGather flag is used to determine if the miniport can handle more than one SGD.  If you do not set this flag to true, then SCSIPORT.PDR will  assume that the number of SGDs is 1.  Typically this will result in your  miniport only getting I/O requests for 1 sector at a time (at least for CD-ROM  drives).

This is obviously a problem for PIO devices, which do not typically support the concept of scatter-gather.  In these cases it is possible to force SCSIPORT.PDR to emulate scatter-gather on behalf of your device.  This is done by setting  BufferAccessScsiPortControlled=TRUE in the PORT_CONFIGURATION_INFORMATION structure in your SCSI miniport.

Please note that this field was originally only documented in the Windows NT 3.5x DDK (it was accidentally omitted from the Win95 DDK documentation).  It is however correctly defined in the Windows 95 DDK include files.  Also please be aware that it is very important to *not* touch the data buffer directly in your miniport when this flag is set; instead you should only use ScsiPort functions to access the buffer.  This is due to the fact that the buffer will not be a linearly contiguous block, but instead will be a list of linear scatter-gather descriptors.

The Master field of Port_Configuration_Information is used to determine if system DMA is required.  If SCSIPORT.PDR thinks system DMA is required, it  will allocate a 64K physically contiguous block of memory for use as a DMA  buffer, and transfer lengths will be limited to 64K.  Please set the flag  Master=TRUE if you do support bus-mastering.

When using BufferAccessScsiPortControlled:

- PIO should work fine, in fact that's what BufferAccessScsiPortControlled is designed for.  Bus-mastering devices should not set this field to TRUE.

- Never modify the SRB's DataTransferLength or DataBuffer.   The ScsiPort{Read,Write}... functions actually scan through the chained list of  SRB's in the system, matching the I/O range against DataBuffer / DataTransferLength of each pending SRB.  When found, if  IOR_flags.IORF_SCATTER_GATHER is set, IOR_buffer_ptr points to a list of  scatter-gather descriptors, and the routine does "emulation" of scatter-gather  data transfer, moving data to or from the scatter-gather memory areas.  If  IORF_SCATTER_GATHER is false, DataBuffer is used directly (it is assumed to be  a linear address).  Note that within an IOP’s SRB, there is a pointer to its corresponding IOP (SrbIopPointer) which can be used to inspect IOP/IOR fields such as DataBuffer, IOR_flags etc., when debugging.

- ALWAYS use the ScsiPort{Read,Write}Buffer... functions when accessing your buffer.

Here is a brief description of what ScsiPortReadPortBufferUchar (and its generic counterparts) do:

- Scan SCSIPORT's active SRB list, examining the .DataBuffer and .DataTransferLength fields. Attempt to find a buffer that "contains" the memory address requested.  If fails, perform the desired transfer between buffer and port, without using IOP / SRB info at all.  Then quits.

- Given the SRB, obtain the .SrbIopPointer.  Use this to inspect for (IOR_flags & IORF_SCATTER_GATHER).  If this bit is clear, perform the desired transfer between buffer and port, without using IOP / SRB info at all.  Then quits.

- Here we know we are using linear SGD's.  Perform the desired transfer between buffer and port, using the linear SGD's to reference the scattered chunks of memory. When the count has been exhausted, quit.  Note that care must be taken to not request too many bytes, since you will run off the end of the SGD list (the code doesn't appear to audit for a zero length SGD element).

## 2.6  Debugging Techniques

Use the following debug techniques to facilitate SCSI Miniport development:

- If you are having problems with your miniport being loaded, or to check for import problems (for example, see Knowledge Base article Q160667 Miniport Driver Fails Using Certain ScsiPort APIs), you can copy the debug binary file vxdldr.vxd into the windows\system\vmm32\ path, and restart the debugger.  Errors will appear in the debug monitor window.

- You can start up Win 95 in “Logged” mode (touch the F8 key just as Windows 95 starts, then select “Logged” mode from the Startup Menu.  After the system has booted up, inspect \BOOTLOG.TXT to confirm that your miniport and associated VxD’s are getting loaded

- Using the debugger, set a breakpoint at the miniport’s DriverEntry to confirm that the driver is getting the initial Inquiry command.

- When an IOS device driver fails to load, information about that failure can be found in a file named IOS.LOG.  If this file exists on your system, check its contents.

- The file IOS.INI contains two sections; the “safe” driver list ([SafeList]) and the “CD ROM unsafe” driver list ([CDUnsafe]). The [SafeList] section contains a list of real mode drivers that can safely be bypassed by their 32-bit protected mode counterparts.  The [CDUnsafe] section contains a list of real mode CD ROM drivers that cannot be safely bypassed by their 32-bit protected mode counterparts.  If the real-mode driver is not listed in [SafeList] then your protected mode SCSI miniport driver won’t run. 

Read KB Article Q130179, “Troubleshooting MS-DOS Compatibility Mode on Hard Disks”.

- Use the SCSIPORT.PDR debug binary.  Any debug error or warning messages emitted by SCSIPORT to the debugger can help identify problems.

- You can use DebugPrint statements inside Windows 95 miniport drivers.  Viewing DebugPrint messages from a SCSI miniport under Windows 95 requires the installation of the debug version of VMM.VXD, IOS.VXD, and SCSIPORT.PDR.  These debug components are shipped with the Windows 95 DDK in the DDK\DEBUG directory.  When you install the Win95 DDK, the setup utility will create a DDKDEBUG.BAT file in the DDK\DEBUG directory.  You can install the debug components by running 'DDKDEBUG SET', and remove them by running 'DDKDEBUG RESTORE'.  Once you have installed the debug DDK components, you should see your DebugPrint statements when you run a Windows system debugger (either WDEB386, or SoftIce/W).

## 2.7  Format of a SCSI PNP ID

Under Windows 95, the SCSI Plug and Play ID is a 25-byte field structured as follows:

| Offset | Length | Field name |
| --- | --- | --- |
| 0 | 8 | Vendor ID |
| 8 | 16 | Product ID |
| 24 | 1 | Product revision (first byte only) |

An example of a SCSI id follows:

HP______C3323A__________5

## 2.8  How to uniquely identify a specific SCSI adapter in a DCB

A custom VSD, installed within the layered hierarchy of IOS, can uniquely identify a specific SCSI adapter by inspecting the following DCB fields:  DCB_port_name, DCB_scsi_hba, and DCB_bus_number.

## 2.9  Dynamic device removal/installation (hot-swapping)

SCSIPORT.PDR, working in concert with IOS, supports hot swapping.  In order to detect hardware changes, a custom-written IOS layer VxD can periodically check for hardware insertion or removal.  When either event is detected, the custom VxD can call CONFIGMG_Reenumerate_Devnode() at AppyTime.  This will cause SCSI device reenumeration.  SCSI device reenumeration issues a SCSI Device Inquiry command to all devices associated with SCSI miniports, to detect arrival or disappearance of SCSI devices.

IOS will detect new devices and devices which have disappeared, and issue the corresponding system messages to make the operating system aware of the device change(s).

If you are developing your own ATAPI miniport driver, you can take the necessary actions within the miniport driver itself when the hardware is inserted or removed.  Modify the driver to make sure it completes pending I/O that has become stuck because the hardware was unplugged (or while in the process of plugging the device in).  This will avoid causing the system to hang waiting for the orphaned IOP(s) to complete.

In order to locate the devnode associated with the hot-swappable device, use the setupx API DiGetClassDevices to find all instances of the class you are interested in (SCSI or ESDI).

## 2.10  ScsiPortSetBusDataByOffset

This function is intended only for use when first initializing the bus. If you wish to read or write PCI Configuration Space after initialization, refer to Knowledge Base article Q140730 for more information.

This function was missing from the original Windows 95 DDK documentation but can be found in the original header file \DDK\BLOCK\INC\SRB.H.

In SCSIPORT.PDR, this routine actually is simply a wrapper to the following call: CONFIGRET CONFIGMG_Call_Enumerator_Function(DEVNODE dnDevNode, ENUMFUNC  PCI_ENUM_FUNC_SET_DEVICE_INFO, ULONG Offset, PFARVOID Buffer, ULONG Length,  ULONG 0).

ScsiPortSetBusDataByOffset automatically accommodates non-DWORD granular boundaries by selectively preceding and/or postpending I/O accesses with byte or word  granular accesses to PCI config space.  Thus you can write one BYTE, one WORD or one DWORD, with the correct I/O data width being issued to the PCI chip.

## 2.11  On obtaining both an I/O range and a memory window from within a SCSI miniport:

If a device has both I/O and memory windows, SCSIPORT will only populate the ACCESS_RANGE array with the I/O windows.  There is an open bug report on this problem, so hopefully it will be fixed in future versions of Windows 95.

Currently the only work-around for this problem is to read the PCI configuration space directly to get addresses of the memory windows.

## 2.12  Mapping between physical and linear (logical) memory

The following SCSIPORT option flags deal with the usage of physical memory and virtual memory:

###### Master

If true, during SCSIPORT's CONFIG_DCB, it sets a ceiling on DCB_max_sg_elements to 17.  Also if Dma32BitAddresses=0, it sets DCB_dmd_small_memory (used if  total memory accessible by bus master device is 16MB or less), and if applicable to the bus type, calls VDMAD_Virtualize_Channel.

###### MapBuffers

The purpose of this flag is to notify the SCSIPORT manager that all data buffer addresses need to be mapped to virtual addresses, in order to allow the miniport driver access to them.

###### NeedPhysicalAddresses

This flag causes the following DCB demand bit to be set: DCB_dmd_phys_sgd.  When SCSIPORT prepares an IOP, it calls the IOS’s io criteria routine (ILB_int_io_criteria_rtn).  If the DCB demand flag DCB_dmd_phys_sgd is set, ILB_int_io_criteria_rtn performs the service of converting the data buffer (or linear SGD list) pointed to by the (linear) IOR_buffer_pointer, into a physical SGD list pointed to by  IOR_sgd_lin_phys.  See the section in this article titled Unraveling the Scatter Gather Descriptor (SGD) Confusion, for more information.

# 3.  Additional Differences in SCSI between Windows 95 and Windows NT

First refer to the Knowledge Base articles dealing with implementation differences between Windows 95 and Windows NT.  Some of these articles are listed at the end of this document.

## 3.1  Plug and Play

The only major difference between the two is support for Plug and Play under Windows 95.  In Windows 95 the PORT_CONFIGURATION_INFO structure will already be populated with the system resources (I/O and memory ranges, DMA channel, etc.) assigned to the controller.  In this case the miniport is expected to not “sniff” for the controller.

You might consider setting flags in your code to accomplish conditional compilation (to accommodate both Windows NT and Windows 95).

## 3.2  Using the ATAPI miniport driver sample located in the Windows NT DDK

A common issue deals with the Windows 95 IDE/ATAPI port driver (ESDI_506.PDR).  Microsoft does not supply the source code ESDI_506.PDR.  However, Microsoft DOES supply the source code for an ATAPI miniport driver in the Windows NT DDK. This driver may be used to replace the functionality of ESDI_506.  Windows NT miniport drivers are generally binary compatible with Windows 95 (there are minor differences, for example, Windows NT supports miniport iocontrol commands that are not supported by Windows 95).

SCSI miniport driver devices support dynamic re-enumeration (hot-swapping), while the ESDI_506.PDR driver does not.  If you want to accommodate “hot” device removal and reinstallation, you can modify the miniport driver to detect the addition and removal of the  hardware, and call CM_Reenumerate_Devnode(), to force the system to re-enumerate the bus (which detects both removal and arrival of devices).  Part of the IOS SCSI reenumeration process is to issue device inquiries to identify new device(s) and obtain the SCSI device's hardware ID.  The .INF files are scanned to find  a hardware ID match.  If a match is found, the "new hardware" popup menu is  avoided, and the process automatically completes.

Here is the general process for setting up the Windows NT ATAPI miniport sample into Windows 95:

- Inspect the file named scsi.inf located in the <windows directory>\inf\ path (or do a grep for "mpd" in the inf\ path).  You'll find two different types of entry, here are some examples:

CopyFiles=@ncr53C9x.mpd

...

HKR,,PortDriver,,ncr53C9x.mpd

The first item causes the miniport driver (.mpd) to be copied to the \<windows directory>\system\iosubsys\ path. The second item adds the name of the miniport driver to your entry in the registry.  You need to amend scsi.inf to contain your new atapi.mpd, or create your own INF file.

- You can test your miniport driver by manually copying it to the \iosubsys\ path (renaming your binary file atapi.sys to atapi.mpd), and going into the registry, renaming ESDI_506.PDR to the name of your miniport driver (atapi.mpd).

- For Windows 95, you don't need any other Windows NT files.  Windows 95 uses its own port driver, SCSIPORT.PDR to interface your miniport driver.

# 4.  Miscellaneous Questions

Q: While executing code in the HwInterrupt() function of my miniport, if I do something that causes another h/w interrupt, will the O/S call my HwInterrupt() function again?

A: VPICD will actually mask off the interrupt at the PIC before it calls the installed interrupt handler (SCSIPORT.PDR, in this case).  This means that your HwInterrupt() procedure will not be re-entered if it causes an interrupt to occur.  To be safe, make the assumption that HwInterrupt() would be called again after you return from HwInterrupt().  This would occur after SCSIPORT.PDR has physically unmasked the interrupt at the PIC,  since your device is still asserting an interrupt to the PIC and the PIC was EOI'd by VPICD before the original HwInterrupt() call.

It should also be noted that SCSIPORT.PDR executes an STI before calling HwInterrupt(), so other interrupts will continue to be serviced while HwInterrupt() is executing.

###### Q: How can I send a user-defined parameter string to the miniport?

A:  See the following Knowledge Base article for the answer to the above question: Q140728   AdapterSettings Entry for SCSI Miniport under Windows 95.

As an example, the AddReg section for the NCRC810 (found in  \windows\inf\scsi.inf) looks like this:

[NCRC810.reg]

HKR,,PortDriver,,ncrc810.mpd

If you wished to add an AdapterSettings string, you would simply do this:

[NCRC810.reg]

HKR,,PortDriver,,ncrc810.mpd

HKR,,AdapterSettings,,"This is the Adapter Settings string passed to HwFindAdapter"

This string would be added to the software branch of the registry for this device.  If you want to the AdapterSettings string to be added to the  device's key in the hardware branch of the registry, you can specify a  Hardware Section in your INF to do this.  This is useful if you want  end-users to be able to modify the string via the 'Settings' property page  for the device.

To add a Hardware Section, you simply add a ".HW" extension to the install section name used for your device.  For example, the NCRC810's install  section looks like this:

[NCRC810]

CopyFiles=@ncrc810.mpd

AddReg=IOS,NCRC810.reg

UpdateInis=DoubleBuffer

You create a separate hardware section in your INF by adding a .HW extension to the name of your install section.  You can then specify a AddReg  section that adds the AdapterSettings subkey:

[NCRC810.HW]

AddReg=NCRC810.HW.reg

[NCRC810.reg]

HKR,,AdapterSettings,,"This is the Adapter Settings string passed to

HwFindAdapter"

Q: My SCSI miniport supports Auto Request Sense.  What SCSI status should be returned to SCSIPORT -- the status of the original command that failed (check  condition) or the status of the request sense that just happened (good  status)?  Currently, they are returning good status.  Seems to work fine on Windows NT and Windows 95 for the most part.  However, on Windows 95 when using ASPI we are seeing a good status returned to the application, but they think they should see a check condition.

A: Once the request  sense has successfully completed, you should return an SrbStatus of  SRB_STATUS_ERROR and SRB_STATUS_AUTOSENSE_VALID, and a ScsiStatus of  SCSISTAT_CHECK_CONDITION.

###### Q: Can I assign a floppy disk drive letter to a SCSI Miniport driver?

A:  Under Windows 95, a miniport driver, by itself, is not capable of assigning to drive A or B.  A miniport drive can be assigned as "removable", but not as a "floppy disk".

The actual drive letter assignment is performed by the "Disk TSD" (DISKTSD.VXD) in the middle layer of the (Windows 95 OSR2) IOS hierarchy.  Each time it  receives the AEP_CONFIG_DCB message, it inspects the DCB.  If it is a hard disk or floppy disk (which includes floptical), it inserts itself into the IOS  "calldown chain" and assigns a drive letter using IOS'  ISP_ASSOCIATE_DCB  service.

This service inspects real-mode drivers; the real-mode drive letter assignment affects the assignment of drive letters in protected mode. DISKTSD also handles the situation where a single floppy disk drive appears as both drive A and drive B.

If the floppy disk device is an IDE LS120 (or any floptical type) driver, used with Windows 95 version B (OSR2) or higher, note that DISKTSD checks the DCB field DCB_device_flags2 to detect this by inspecting the following bit: DCB_DEV2_IDE_FLOPTICAL_BIT. This bitmask flag is not defined in the original Windows 95 DDK; its value is 0x00000040.  If it is set, it indicates the device is a floptical device.  DISKTSD then issues an IOR_COMPUTE_GEOM to the device, in order to obtain the correct disk geometry information.

Miniport drivers communicate through the scsiport port driver (SCSIPORT.PDR).  Unfortunately, the miniport interface does not provide the miniport with information  such as DCB_DEV2_IDE_FLOPTICAL_BIT and other such internals.

If you need to reassign drive letters, and you want to keep using your miniport driver (instead of writing your own port driver), you might try writing a custom TSD (type-specific driver) that is  installed into the layered hierarchy of IOS (like DISKTSD.VXD, but placed  "below" DISKTSD, closer to port drivers).

The custom TSD could be used to change drive letter assignments.  Perform similarly to DISKTSD (respond to AEP_CONFIG_DCB messages).

- In general, use the ISP_GET_DCB service to locate devices.  Supply the desired drive letter as input.  Returns a pointer to the corresponding DCB.

- If inspecting a floppy disk drive A, check its DCB to see if it is used as both Drive A and Drive B (check DCB_dmd_do_a_b_toggling).   If it is set,  clear it.  This will allow you to use drive B.

- Use the ISP_DISASSOC_DCB to "disassociate" the drive letter (for example drive B)

- Locate the drive letter currently assigned to your drive (using ISP_GET_DCB).

- Use the ISP_ASSOCIATE_DCB to assign your drive to drive B.

- Use the ISP_DISASSOC_DCB to deassign the former drive letter of your drive (for example drive D)

A sample driver is located at \DDK\Block\SAMPLES\VSD\HELPVSD\ in the Windows 95 DDK.  It is referred to as a "VSD" which stands for "Vendor Supplied Device".   A TSD is a VSD that has the special function in the IOS layered hierarchy of  assigning drive letters (among other things).  The sample's  vsd_config_device() entry point is where DCB(s) need to be inspected and  modified as required.  The compiled TSD belongs in the  \<windows>\system\iosubsys\ directory (along with other IOS layer drivers).

The DDK contains documentation describing how to setup the compile environment.  The easiest and quickest technique is to use the batch file DDKENV.BAT to  establish your environment (DDKENV 32 BLOCK), then change to the source code  directory, then run "nmake".

An excellent introduction to IOS layered device drivers is contained in the following book: Systems Programming for Windows 95, by Walter Oney (Microsoft Press).  A CD containing sample code is included.

###### Q: How does Windows 95’s ScsiPortGetPhysicalAddress work?

This SCSIPORT service is used to enable the miniport driver to obtain a physical address, given a data buffer’s linear address.  Under Windows 95 (NOT Windows NT), the process is as follows:

- If (Srb != NULL) && (VirtualAddress lies within the Srb's DataBuffer) scan the corresponding IOP's IOR_sgd_lin_phys field to obtain the desired SG element and physical offset within that element.  The returned length is the number of bytes from the offset within the SG element, to the end of the SG element.

(else)

- If (VirtualAddress lies within the IOP Memory Pool)
The physical address is obtained by finding the offset within the memory pool and adding IDA_ios_mem_phys to the offset value.  The returned Length is  ALWAYS 4096.

(else)

- _CopyPageTable(VirtualAddress/4096, 1, Pte, 0).
PhysAddr = (Pte & 0xFFFFF000) + (VirtualAddress & 0xFFF).
Pte is the Page Table Entry as described in the Pentium Processor Users Manual Volume 3.
The returned Length is ALWAYS 4096.

Based on the above info, one cannot always rely on the returned Length containing the correct length; it may be longer than the actual buffers.

In general, if the SRB pointer supplied is good, and the VirtualAddress is within range of the SRB's DataBuffer, taking into account DataTransferLength, then the returned length makes sense (within the physical SGD associated with VirtualAddress).  Otherwise, the returned length field will contain a fixed 4096 byte value which probably shouldn't be used when performing total length calculations.

Q:  How do I interpret the “problem number” listed in the Device Manager, when a device is not working properly?

A:  The problem number displayed  is actually a configmg problem number, and can be found in CONFIGMG.H in the  Windows 95 DDK.  (the CM_PROB_xxx error codes).  Be sure to keep in mind that Device Manager displays this number in decimal, while CONFIGMG.H #defines are  in hex.

# 5.  IOP Structures

The following figure illustrates the structure of a typical “extended IOP” used when performing SCSI operations. Drivers that talk directly to SCSI devices (SCSI Passthrough) need to create this structure. Note that the ordering of certain items within the extended IOP, such as the SGD list and the sense info, may vary.  For example CDVSD locates the PORT_SRB, SRB extension and sense info into the DCB expansion area, not at the end of the IOP packet as shown below.

| IOP contents | Component size | Comments |
| --- | --- | --- |
| DCB expansion area | DCB_expansion_length |  |
| IOP/IOR | Size IOP | The combined IOP+IOR structure |
| Physical SGD list | DCB_max_sg_elements * 8 | IOR_sgd_lin_phys points to this list. Required when target hardware requires physical addresses (such as a bus mastering device). |
| PORT_SRB | size PORT_SRB | Contains base SRB plus miniport SRB extension |
| SRB extension | DCB_srb_ext_size | Contains Adapter port extension (APEXT) plus per-miniport-adapter memory area (miniport-requested per-request state info) |
| Sense info buffer | VSD_REQ_SENSE_SIZE | Set the SRB's .SenseInfoBufferLength to this size |

typedef struct _IOP {

| Element name | Offset | Description |
| --- | --- | --- |
| ULONG IOP_physical | 00 | physical address of IOP. |
| ULONG IOP_physical_dcb | 04 | pointer to physical dcb. Filled in by IOS_SendCommand or ILB_Internal_Request |
| ULONG IOP_original_dcb | 08 | ptr to dcb designated by ior. Filled in by IOS_SendCommand (address of logical DCB) |
| USHORT IOP_timer | 0C | rcb current timeout value. Filled in by IOS_SendCommand. Default value = 15 (7.5 seconds) |
| USHORT IOP_timer_orig | 0E | rcb original timeout value. Filled in by IOS_SendCommand. Default value = 15 (7.5 seconds) |
| ULONG IOP_calldown_ptr | 10 | ptr to next calldown routine (structure type DCB cd_entry) Filled in by IOS_SendCommand. |
| ULONG IOP_callback_ptr | 14 | ptr to current callback address. |
| ULONG IOP_voltrk_private | 18 | for use by volume tracking. |
| ULONG IOP_Thread_Handle | 1C | contains the handle of the thread in whose context this IO originated. This is inited in IOS_SendCommand. Not used if called via ILB_Internal_Request. |
| ULONG IOP_srb | 20 | used by SCSI'izers (or by SCSI Passthrough requests) to pass SRB pointer to next layer. Valid if (IOR_flags & IORF_SRB_VALID) set |
| ULONG IOP_reserved[2] | 24 | reserved for future use - must be zero. |
| IOP_callback_entry IOP_callback_table[IOP_CALLBACK_TABLE_DEPTH] | 2C | RCB_CallBackTable. |
| BYTE IOP_format_head | 5C | fields for low level format */ |
| BYTE IOP_format_xfer_rate | 5D |  |
| USHORT IOP_format_track | 5E |  |
| ULONG IOP_format_num_sectors | 60 |  |
| IOR IOP_ior |  | i/o request descriptor (which follows) |

typedef struct _IOR {

| ULONG IOR_next | 64/00 | client link for BCB's (MBZ for IORF_VERSION_002) |
| --- | --- | --- |
| USHORT IOR_func | 68/04 | function to be performed - see defines below. |
| USHORT IOR_status | 6A/06 | request status - see defines below. |
| ULONG IOR_flags | 6C/08 | request control flags – see defines below. |
| CMDCPLT IOR_callback | 70/0C | address to call request back to if IORF_SYNC_COMMAND is not set (Used in internal IOS_SendCommand routine) |
| ULONG IOR_start_addr[2] | 74/10 | volume relative starting addr. if IORF_LOGICAL_START_SECTOR is set. physical if not set. |
| ULONG IOR_xfer_count | 7C/18 | number of sectors to process if IORF_CHAR_COMMAND is not set, or # of bytes if it is set. MUST be set to zero if no data xfer. Must be less than 65536 if CD-ROM. |
| ULONG IOR_buffer_ptr | 80/1C | BlockDev client buffer pointer. Contains pointer to data buffer or to null terminated list of linear sgd's depending on IORF_SCATTER_GATHER. Undefined if no data transfer. |
| ULONG IOR_private_client | 84/20 | BlockDev/IOS client reserved. |
| ULONG IOR_private_IOS | 88/24 | reserved space for IOS. |
| ULONG IOR_private_port | 8C/28 | private area for port driver. |
| union urequestor_usage _ureq | 90/2C | requestor usage area, also used for IOCTL's. SCSIPORT uses this to point to SRB. |
| ULONG IOR_req_req_handle | A4/40 | requestor provided request handle. often is a pointer to this ior or its containing iop. pushed on the stack by IOS before IOR_callback is called |
| ULONG IOR_req_vol_handle | A8/44 | requestor provided media handle designating the media to perform the function on (VRP). |
| ULONG IOR_sgd_lin_phys | AC/48 | pointer to first physical sgd, as contrasted with IOR_buffer_ptr, which points to the logical sgds this is either a linear or phys address, depending on the needs of the drivers, as indicated via the DCB demand bits. |
| UCHAR IOR_num_sgds | B0/4C | number of phys sgd's pointed to by IOR_sgd_lin_phys |
| UCHAR IOR_vol_designtr | B1/4D | numeric representation of the drive letter designating the volume to perform the function on (c: = 2). Set this to DCB_unit_number. |
| USHORT IOR_ios_private_1 | B2/4E | reserved by IOS to audit alignment. Currently used only within the ILB_io_criteria_rtn. |
| ULONG IOR_reserved_2[2] | B4/50 | reserved for internal use |

} IOR, *PIOR;

} IOP, *pIOP;

typedef struct IOP_callback_entry { /* */

| ULONG IOP_CB_address | 00 | call back address |
| --- | --- | --- |
| ULONG IOP_CB_ref_data | 04 | pointer to callback ref data |

} IOP_callback_entry;

typedef struct _SCSI_REQUEST_BLOCK {

| USHORT Length | 0 | IN: size SCSI_REQUEST_BLOCK |
| --- | --- | --- |
| UCHAR Function | 2 | IN: function (See Table 1 - SRB Functions) |
| UCHAR SrbStatus | 3 | OUT: Status from HBA (See Table 4 - SRB Status). |
| UCHAR ScsiStatus | 4 | OUT: Status from HBA (see Table 3 - SCSI bus status codes) |
| UCHAR PathId | 5 | IN: set to DCB_bus_number (geometry) |
| UCHAR TargetId | 6 | IN: set to DCB_scsi_target_id |
| UCHAR Lun | 7 | IN: set to DCB_scsi_lun |
| UCHAR QueueTag | 8 | IN: 0. Used by SCSIPORT to enqueue the SRB on the adapter's input queue. Also see MultipleRequestPerLu in HW_INITIALIZATION_DATA structure. |
| UCHAR QueueAction | 9 | IN: 0 (Not used in SCSIPORT) |
| UCHAR CdbLength | a | IN: Length of Cdb e.g. 6 for Inquiry |
| UCHAR SenseInfoBufferLength | b | IN: Length of SenseInfoBuffer. Postcall: miniport driver updates this field if it supports auto sense request (transfers sense-request info). |
| ULONG SrbFlags | c | IN; see Table 2 - SRB Flag Bits. e.g. SRB_FLAGS_DATA_IN + SRB_FLAGS_DISABLE_DISCONNECT |
| ULONG DataTransferLength | 10 | IN:Size of DataBuffer, e.g. SIZE_OF_INQUIRYDATA. OUT: miniport driver updates this field if an under-run or over-run occurs. Caution: this field, combined with DataBuffer is used by Windows 95 ScsiPort(xxx)Buffer(xxx) (SGD emulation) routines when searching for the SRB corresponding to the memory buffer address desired. In such cases, if this SRB is being accessed by the miniport driver, never modify these fields. |
| ULONG TimeOutValue | 14 | IN: Must set this pre-request, if SCSI PASSTHROUGH. SCSIPORT puts this into IOP_timer and IOP_timer_orig (500ms intervals) |
| PVOID DataBuffer | 18 | IN: Point to mem created using the ILB_Service_rtn()ISP_Alloc_Mem, e.g. SIZE_OF_INQUIRYDATA etc. SCSIPORT sets this to the value in IOP_ior.buffer_ptr, if SRB_flags indicate data transfer in or out. |
| PVOID SenseInfoBuffer | 1c | IN: Pointer to Sense info buffer; which is typically located just past our SrbExtension. Make large enough to accommodate worst-case Sense data from drive. Set to null to disable autosense. |
| struct _SCSI_REQUEST_BLOCK *NextSrb | 20 | IN: set to 0. SCSIPORT uses this field to link the enqueued SRB’s in a chain (see QueueTag above) |
| PVOID OriginalRequest | 24 | IN: 0. Not used by SCSIPORT. |
| PVOID SrbExtension | 28 | IN: Pointer to extension area (just past this SRB, and the _PORT_SRB extension). Length of this extension area is DCB_srb_ext_size. Setup by ScsiPortInitialize() to supply miniport requested/defined per-request state information storage. |
| ULONG QueueSortKey | 2c | Used in SCSIPORT as a temporary to hold a pointer to an APE structure if paging drive is having problems and drive retries are required. |
| UCHAR Cdb[16] | 30 | IN: Contains desired CDB to issue to HBA. Actual command length is CdbLength. See Table 5 - SCSI CDB Operation Codes. If communicating with an ATAPI device (through ESDI_506.PDR), length can be set to (a max of) 12, even if an actual command uses fewer bytes. If communicating with a SCSI device, the length must correspond to the expected length of the command. |

} SCSI_REQUEST_BLOCK, *PSCSI_REQUEST_BLOCK;

typedef struct _PORT_SRB { /* Miniport SRB extension */

| SCSI_REQUEST_BLOCK BaseSrb | (40h bytes) | _PORT_SRB IS AN EXPANSION OF SCSI_REQUEST_BLOCK |
| --- | --- | --- |
| PVOID SrbIopPointer | 40 | IN: Pointer to the IOP |
| SCSI_REQUEST_BLOCK *SrbNextSrb | 44 |  |
| SCSI_REQUEST_BLOCK *SrbNextActiveSrb | 48 | IN: Allows SCSIPORT to keep a list of all active SRBs (SRBs that have been submitted to a miniport and are still under its control). |
| UCHAR SrbRetryCount | 4C |  |
| UCHAR Filler[3] | 4D |  |

} PORT_SRB;

Table 1 - SRB Functions

| SRB_FUNCTION_EXECUTE_SCSI | 0x00 | A SCSI device I/O request should be executed on the target logical unit. |
| --- | --- | --- |
| SRB_FUNCTION_CLAIM_DEVICE | 0x01 | The port driver processes this request without calling the miniport driver. |
| SRB_FUNCTION_IO_CONTROL | 0x02 | (Supported in Windows NT IOCTL mechanism). The request is an I/O control request. The SRB DataBuffer points to an SRB_IO_CONTROL header followed by the data area. The value in DataBuffer can be used by the driver, regardless of the value of MapBuffers field. If the HBA miniport driver supports this request, it should execute the request and notify the OS-specific port driver when it has completed it, using the normal mechanism of ScsiPortNotification with RequestComplete and NextRequest. Only the Function, SrbFlags, TimeOutValue, DataBuffer, DataTransferLength and SrbExtension are valid. |
| SRB_FUNCTION_RECEIVE_EVENT | 0x03 | (Defined for future versions) The HBA should be prepared to receive an Asynchronous Event Notification from the addressed target. The DataBuffer indicates where the data should be placed. |
| SRB_FUNCTION_RELEASE_QUEUE | 0x04 | The port driver processes this request without calling the miniport driver. |
| SRB_FUNCTION_ATTACH_DEVICE | 0x05 | The port driver processes this request without calling the miniport driver. |
| SRB_FUNCTION_RELEASE_DEVICE | 0x06 | The port driver processes this request without calling the miniport driver. |
| SRB_FUNCTION_SHUTDOWN | 0x07 | The system is being shut down. The request is passed to the miniport driver if CachesData was set to TRUE in the PORT_CONFIGURATION_INFORMATION data. The miniport driver can receive several of these notifications before all system activity is actually stopped; however, the last shutdown notification will occur after the last start I/O. Only the Function, PathId, TargetId and Lun fields are valid. |
| SRB_FUNCTION_FLUSH | 0x08 | The miniport should flush any cached data for the device. The request is passed to the miniport driver if CachesData is TRUE in the PORT_CONFIGURATION_INFORMATION data. Only the Function, PathId, TargetId and Lun fields are valid. |
| SRB_FUNCTION_ABORT_COMMAND | 0x10 | A SCSIMESS_ABORT message should be sent to cancel the request pointed to by the NextSrb field. If this is tagged queue request, then a SCSIMESS_ABORT_WITH_TAG message should be used. If the indicated request has been completed, this request should be completed normally. Only the Function, PathId, TargetId, Lun, and NextSrb fields are valid. |
| SRB_FUNCTION_RELEASE_RECOVERY | 0x11 | (Defined for future versions) A SCSIMESS_RELEASE_RECOVERY message should be sent to the target controller. Only the Function, PathId, TargetId and Lun fields are valid. |
| SRB_FUNCTION_RESET_BUS | 0x12 |  |
| SRB_FUNCTION_RESET_DEVICE | 0x13 | (Defined for future versions) The SCSI target controller should be reset using the SCSIMESS_BUS_DEVICE_RESET message. The HBA miniport driver should complete any active requests for the target controller. Only the Function, TargetId and PathId fields are valid. |
| SRB_FUNCTION_TERMINATE_IO | 0x14 | (Defined for future versions) A SCSIMESS_TERMINATE_IO_PROCESS message should be sent to cancel the request pointed to by the NextSrb field. If the indicated request has already completed, this request should be completed normally. Only the Function, PathId , TargetId, Lun, and NextSrb fields are valid. |
| SRB_FUNCTION_FLUSH_QUEUE | 0x15 | The port driver processes this request without calling the miniport driver. |
| SRB_FUNCTION_REMOVE_DEVICE | 0x16 | ? |

Table 2 - SRB Flag Bits

| SRB_FLAGS_QUEUE_ACTION_ENABLE | 0x00000002 |  |
| --- | --- | --- |
| SRB_FLAGS_DISABLE_DISCONNECT | 0x00000004 |  |
| SRB_FLAGS_DISABLE_SYNCH_TRANSFER | 0x00000008 |  |
| SRB_FLAGS_BYPASS_FROZEN_QUEUE | 0x00000010 |  |
| SRB_FLAGS_DISABLE_AUTOSENSE | 0x00000020 |  |
| SRB_FLAGS_DATA_IN | 0x00000040 | Note 1 |
| SRB_FLAGS_DATA_OUT | 0x00000080 | Note 1 |
| SRB_FLAGS_NO_DATA_TRANSFER | 0x00000000 |  |
| SRB_FLAGS_NO_QUEUE_FREEZE | 0x00000100 |  |
| SRB_FLAGS_UNSPECFIED_DIRECTION | (SRB_FLAGS_DATA_IN ¦ SRB_FLAGS_DATA_OUT) |  |
| SRB_FLAGS_UNSPECIFIED_DIRECTION | (SRB_FLAGS_DATA_IN ¦ SRB_FLAGS_DATA_OUT) |  |
| SRB_FLAGS_ADAPTER_CACHE_ENABLE | 0x00000200 |  |
| SRB_FLAGS_BIT_IS_ACTIVE | 16 |  |
| SRB_FLAGS_IS_ACTIVE | 1 << SRB_FLAGS_BIT_IS_ACTIVE |  |
| SRB_FLAGS_ALLOCATED_FROM_ZONE | 0x00020000 |  |
| SRB_FLAGS_SGLIST_FROM_POOL | 0x00040000 |  |

Misc notes:

1. When doing SCSI_PASS_THROUGH, if both SRB_FLAGS_DATA_IN and SRB_FLAGS_DATA_OUT are clear, SCSIPORT will set DataBuffer to null before calling the port driver.

Table 3 - SCSI bus status codes

| SCSISTAT_GOOD | 0x00 |  |
| --- | --- | --- |
| SCSISTAT_CHECK_CONDITION | 0x02 | If SCSIPORT detects this, it issues a request sense command if autosense not supported |
| SCSISTAT_CONDITION_MET | 0x04 |  |
| SCSISTAT_BUSY | 0x08 |  |
| SCSISTAT_INTERMEDIATE | 0x10 |  |
| SCSISTAT_INTERMEDIATE_COND_MET | 0x14 |  |
| SCSISTAT_RESERVATION_CONFLICT | 0x18 |  |
| SCSISTAT_COMMAND_TERMINATED | 0x22 |  |
| SCSISTAT_QUEUE_FULL | 0x28 |  |

Table 4 - SRB Status

| SRB_STATUS_PENDING | 0x00 | Set to this value before initiating a SCSI command. |
| --- | --- | --- |
| SRB_STATUS_SUCCESS | 0x01 |  |
| SRB_STATUS_ABORTED | 0x02 |  |
| SRB_STATUS_ABORT_FAILED | 0x03 |  |
| SRB_STATUS_ERROR | 0x04 |  |
| SRB_STATUS_BUSY | 0x05 |  |
| SRB_STATUS_INVALID_REQUEST | 0x06 |  |
| SRB_STATUS_INVALID_PATH_ID | 0x07 |  |
| SRB_STATUS_NO_DEVICE | 0x08 |  |
| SRB_STATUS_TIMEOUT | 0x09 |  |
| SRB_STATUS_SELECTION_TIMEOUT | 0x0A |  |
| SRB_STATUS_COMMAND_TIMEOUT | 0x0B |  |
| SRB_STATUS_MESSAGE_REJECTED | 0x0D |  |
| SRB_STATUS_BUS_RESET | 0x0E |  |
| SRB_STATUS_PARITY_ERROR | 0x0F |  |
| SRB_STATUS_REQUEST_SENSE_FAILED | 0x10 | If HBA failed to supply sense. “Device is really hosed”. |
| SRB_STATUS_NO_HBA | 0x11 |  |
| SRB_STATUS_DATA_OVERRUN | 0x12 |  |
| SRB_STATUS_UNEXPECTED_BUS_FREE | 0x13 |  |
| SRB_STATUS_PHASE_SEQUENCE_FAILURE | 0x14 |  |
| SRB_STATUS_BAD_SRB_BLOCK_LENGTH | 0x15 |  |
| SRB_STATUS_REQUEST_FLUSHED | 0x16 |  |
| SRB_STATUS_INVALID_LUN | 0x20 |  |
| SRB_STATUS_INVALID_TARGET_ID | 0x21 |  |
| SRB_STATUS_BAD_FUNCTION | 0x22 |  |
| SRB_STATUS_ERROR_RECOVERY | 0x23 |  |
| (The following are status masks) |  |  |
| SRB_STATUS_QUEUE_FROZEN | 0x40 |  |
| SRB_STATUS_AUTOSENSE_VALID | 0x80 | After SCSI command completed, indicates sense info is available. |

Use the following macro to look only at status codes:

#define SRB_STATUS(Status) (Status & ~(SRB_STATUS_AUTOSENSE_VALID | SRB_STATUS_QUEUE_FROZEN))

Table 5 - SCSI CDB Operation Codes

| SCSIOP_TEST_UNIT_READY | 0x00 |  |
| --- | --- | --- |
| SCSIOP_REZERO_UNIT | 0x01 |  |
| SCSIOP_REWIND | 0x01 |  |
| SCSIOP_REQUEST_BLOCK_ADDR | 0x02 |  |
| SCSIOP_REQUEST_SENSE | 0x03 |  |
| SCSIOP_FORMAT_UNIT | 0x04 |  |
| SCSIOP_READ_BLOCK_LIMITS | 0x05 |  |
| SCSIOP_REASSIGN_BLOCKS | 0x07 |  |
| SCSIOP_READ6 | 0x08 |  |
| SCSIOP_RECEIVE | 0x08 |  |
| SCSIOP_WRITE6 | 0x0A |  |
| SCSIOP_PRINT | 0x0A |  |
| SCSIOP_SEND | 0x0A |  |
| SCSIOP_SEEK6 | 0x0B |  |
| SCSIOP_TRACK_SELECT | 0x0B |  |
| SCSIOP_SLEW_PRINT | 0x0B |  |
| SCSIOP_SEEK_BLOCK | 0x0C |  |
| SCSIOP_PARTITION | 0x0D |  |
| SCSIOP_READ_REVERSE | 0x0F |  |
| SCSIOP_WRITE_FILEMARKS | 0x10 |  |
| SCSIOP_FLUSH_BUFFER | 0x10 |  |
| SCSIOP_SPACE | 0x11 |  |
| SCSIOP_INQUIRY | 0x12 |  |
| SCSIOP_VERIFY6 | 0x13 |  |
| SCSIOP_RECOVER_BUF_DATA | 0x14 |  |
| SCSIOP_MODE_SELECT | 0x15 |  |
| SCSIOP_RESERVE_UNIT | 0x16 |  |
| SCSIOP_RELEASE_UNIT | 0x17 |  |
| SCSIOP_COPY | 0x18 |  |
| SCSIOP_ERASE | 0x19 |  |
| SCSIOP_MODE_SENSE | 0x1A |  |
| SCSIOP_START_STOP_UNIT | 0x1B |  |
| SCSIOP_STOP_PRINT | 0x1B |  |
| SCSIOP_LOAD_UNLOAD | 0x1B |  |
| SCSIOP_RECEIVE_DIAGNOSTIC | 0x1C |  |
| SCSIOP_SEND_DIAGNOSTIC | 0x1D |  |
| SCSIOP_MEDIUM_REMOVAL | 0x1E |  |
| SCSIOP_READ_CAPACITY | 0x25 |  |
| SCSIOP_READ | 0x28 |  |
| SCSIOP_WRITE | 0x2A |  |
| SCSIOP_SEEK | 0x2B |  |
| SCSIOP_LOCATE | 0x2B |  |
| SCSIOP_WRITE_VERIFY | 0x2E |  |
| SCSIOP_VERIFY | 0x2F |  |
| SCSIOP_SEARCH_DATA_HIGH | 0x30 |  |
| SCSIOP_SEARCH_DATA_EQUAL | 0x31 |  |
| SCSIOP_SEARCH_DATA_LOW | 0x32 |  |
| SCSIOP_SET_LIMITS | 0x33 |  |
| SCSIOP_READ_POSITION | 0x34 |  |
| SCSIOP_COMPARE | 0x39 |  |
| SCSIOP_COPY_COMPARE | 0x3A |  |
| SCSIOP_WRITE_DATA_BUFF | 0x3B |  |
| SCSIOP_READ_DATA_BUFF | 0x3C |  |
| SCSIOP_READ_LONG | 0x3E |  |
| SCSIOP_CHANGE_DEFINITION | 0x40 |  |
| SCSIOP_READ_SUB_CHANNEL | 0x42 |  |
| SCSIOP_READ_TOC | 0x43 |  |
| SCSIOP_READ_HEADER | 0x44 |  |
| SCSIOP_PLAY_AUDIO_10 | 0x45 |  |
| SCSIOP_PLAY_AUDIO_MSF | 0x47 |  |
| SCSIOP_PLAY_TRACK_INDEX | 0x48 |  |
| SCSIOP_PLAY_TRACK_RELATIVE | 0x49 |  |
| SCSIOP_PAUSE_RESUME | 0x4B |  |
| SCSIOP_LOG_SELECT | 0x4C |  |
| SCSIOP_LOG_SENSE | 0x4D |  |
| SCSIOP_PLAY_AUDIO | 0xA5 |  |
| SCSIOP_READ_CDDA | 0xD8 |  |
| SCSIOP_DENON_EJECT_DISC | 0xE6 | The following are DENON CD ROM specific |
| SCSIOP_DENON_STOP_AUDIO | 0xE7 |  |
| SCSIOP_DENON_PLAY_AUDIO | 0xE8 |  |
| SCSIOP_DENON_READ_TOC | 0xE9 |  |
| SCSIOP_DENON_READ_SUBCODE | 0xEB |  |

Miscellaneous CDB information

Table 6. - Mode Sense/Select page constants.

|  |  | Device Types |  |
| --- | --- | --- | --- |
| MODE_PAGE_ERROR_RECOVERY | 0x01 | DTCO |  |
| MODE_PAGE_DISCONNECT | 0x02 | DTPCSOM |  |
| MODE_PAGE_FORMAT_DEVICE | 0x03 | D |  |
| MODE_PAGE_RIGID_GEOMETRY | 0x04 | DP |  |
| MODE_PAGE_FLEXIBILE | 0x05 | DP |  |
| MODE_PAGE_VERIFY_ERROR | 0x07 | DCO |  |
| MODE_PAGE_CACHING | 0x08 | DCO |  |
| MODE_PAGE_PERIPHERAL | 0x09 | DTPCSOM |  |
| MODE_PAGE_CONTROL | 0x0A | DTPCSOM |  |
| MODE_PAGE_MEDIUM_TYPES | 0x0B | DTC |  |
| MODE_PAGE_NOTCH_PARTITION | 0x0C | D |  |
| MODE_SENSE_RETURN_ALL | 0x3f | DTPCSOM | For mode sense; read all device pages. |
| MODE_SENSE_CURRENT_VALUES | 0x00 | DTPCSOM | Vendor specific |
| MODE_SENSE_CHANGEABLE_VALUES | 0x40 |  |  |
| MODE_SENSE_DEFAULT_VAULES | 0x80 |  |  |
| MODE_SENSE_SAVED_VALUES | 0xc0 |  |  |
| MODE_PAGE_DEVICE_CONFIG | 0x10 |  |  |
| MODE_PAGE_MEDIUM_PARTITION | 0x11 |  |  |

DCB (Device Control Block) Data Structure

typedef struct	_DCB_cd_entry {

| PVOID DCB_cd_io_address | addr of request routine |
| --- | --- |
| ULONG DCB_cd_flags | demand bits - as defined below |
| ULONG DCB_cd_ddb | driver's DDB pointer |
| ULONG DCB_cd_next | pointer to next cd entry |
| USHORT DCB_cd_expan_off | offset of expansion area used by the request routine |
| UCHAR DCB_cd_layer_flags | flags for layer's use |
| UCHAR DCB_cd_lgn | load group number |

} DCB_cd_entry, *pDCB_cd_entry;

typedef struct	_DCB {

typedef struct	_DCB_COMMON {

| ULONG DCB_physical_dcb | 00 | DCB for physical device |
| --- | --- | --- |
| ULONG DCB_expansion_length | 04 | total length of IOP extension filled in by IOS (excludes IOP size) |
| PVOID DCB_ptr_cd | 08 | pointer to calldown list |
| ULONG DCB_next_dcb | 0C | link to next DCB |
| ULONG DCB_next_logical_dcb | 10 | pointer to next logical dcb for physical device |
| BYTE DCB_drive_lttr_equiv | 14 | drive number (A: = 0, etc.) set up by iosserv during logical device associate processing. |
| BYTE DCB_unit_number | 15 | either physical drive number(sequential drive number or'd with 80h) or unit number within tsd. set up for disk physical dcb's. set up by tsdpart for disk logical dcb's. Set up by tsdaer for cdrom physical dcb's. |
| USHORT DCB_TSD_Flags | 16 | Flags for TSD |
| ULONG DCB_vrp_ptr | 18 | pointer to VRP for this DCB |
| ULONG DCB_dmd_flags | 1C | demand bits of the topmost layer |
| ULONG DCB_device_flags | 20 | was BDD_Flags |
| ULONG DCB_device_flags2 | 24 | second set of general purpose flags |
| ULONG DCB_Partition_Start | 28 | partition start sector |
| ULONG DCB_track_table_ptr | 2C | pointer for the track table buffer for ioctls |
| ULONG DCB_bds_ptr | 30 | DOS BDS corresp. to this DCB (logical DCB's only) |
| ULONG DCB_Reserved1 | 34 | reserved |
| ULONG DCB_Reserved2 | 38 | reserved |
| BYTE DCB_apparent_blk_shift | 3C | Log of apparent_blk_size |
| BYTE DCB_partition_type | 3D | partition type |
| USHORT DCB_sig | 3E | padding and signature |
| BYTE DCB_device_type | 40 | Device Type |
| ULONG DCB_Exclusive_VM | 41 | handle for exclusive access to this device |
| UCHAR DCB_disk_bpb_flags | 45 | bpb flags see defines below |
| UCHAR DCB_cAssoc | 46 | count of logical drives associated with this logical DCB |
| UCHAR DCB_Sstor_Host | 47 | This field indicates a sstor host volume |
| USHORT DCB_user_drvlet | 48 | contains the userdriveletter settings else ff |
| USHORT DCB_Reserved3 | 4A | reserved |
| ULONG DCB_Reserved4 | 4C | reserved |

} DCB_COMMON, *PDCB_COMMON;

(DCB ends here if it is a logical (non-physical) DCB)

| ULONG DCB_max_xfer_len | 50 | maximum transfer length |
| --- | --- | --- |
| ULONG DCB_actual_sector_cnt [2] | 54 | number of sectors as seen below the tsd. |
| ULONG DCB_actual_blk_size | 5C | actual block size of the device as seen below the tsd. |
| ULONG DCB_actual_head_cnt | 60 | number of heads as seen below the tsd. |
| ULONG DCB_actual_cyl_cnt | 64 | number of cylinders as seen below the tsd |
| ULONG DCB_actual_spt | 68 | number of sectors per track as seen below the tsd. |
| PVOID DCB_next_ddb_dcb | 6C | link to next DCB on DDB chain. |
| PVOID DCB_dev_node | 70 | pointer to dev node for this device |
| BYTE DCB_bus_type | 74 | Type of BUS, see below |
| BYTE DCB_bus_number | 75 | channel (cable) within adapter |
| UCHAR DCB_queue_freeze | 76 | queue freeze depth counter. If nonzero indicates frozen. Tested but not set by IOS services’ Dequeue_Iop() (Unless there are BYPASS_QUEUE IOP’s, Dequeue_IOP is disabled). |
| UCHAR DCB_max_sg_elements | 77 | max # of logical and/or physical s/g elements. Set initially by port, but may be MORE RESTRICTIVELY updated by other layers |
| UCHAR DCB_io_pend_count | 78 | indicates number of requests pending for this DCB (VOLUME TRACKING LAYER USE ONLY) |
| UCHAR DCB_lock_count | 79 | depth counter for LOCK MEDIA commands (VOLUME TRACKING LAYER USE ONLY) |
| USHORT DCB_SCSI_VSD_FLAGS | 7A | Flags for SRB builder |
| BYTE DCB_scsi_target_id | 7C | SCSI target ID |
| BYTE DCB_scsi_lun | 7D | SCSI logical unit number |
| BYTE DCB_scsi_hba | 7E | adapter number relative to port driver |
| BYTE DCB_max_sense_data_len | 7F | Maximum sense Length |
| USHORT DCB_srb_ext_size | 80 | miniport srb extension length |
| BYTE DCB_inquiry_flags[8] | 82 | Device Inquiry Flags |
| BYTE DCB_vendor_id[8] | 8A | Vendor ID string |
| BYTE DCB_product_id[16] | 92 | Product ID string |
| BYTE DCB_rev_level[4] | A2 | Product revision level |
| BYTE DCB_port_name[8] | A6 |  |
| UCHAR DCB_current_unit | AE | used to emulate multiple logical devices with a single physical device |
| ULONG DCB_blocked_iop | AF | pointer to requests for an inactive volume (VOLUME TRACKING LAYER USE ONLY) |
| ULONG DCB_vol_unlock_timer | B3 | unlock timer handle |
| UCHAR DCB_access_timer | B7 | used to measure time between accesses |
| UCHAR DCB_Vol_Flags | B8 | Flags for Volume Tracking volume tracking use only |
| BYTE DCB_q_algo | B9 | queuing algorithm index – 0=FIFO, 1=SORTED |
| BYTE DCB_unit_on_ctl | BA | relative device number on ctlr (0-based) |
| ULONG DCB_Port_Specific | BB | bytes for PORT DRIVER use |
| ULONG DCB_spindown_timer | BF | timer for drive spin down |
| DCB_BLOCKDEV DCB_bdd |  |  |

typedef struct	_DCB_BLOCKDEV { /* */

| Extension for INT13h drives (BDD) |  | The initial set of fields below should not be re-ordered or modified in anyway as the offset and the size of fields have to match those in BLOCKDEV.H for COMPATIBILITY!!! |
| --- | --- | --- |
| ULONG DCB_BDD_Next | C3 |  |
| BYTE DCB_BDD_BD_Major_Version | C7 | INIT <BDD_BD_Major_Ver> |
| BYTE DCB_BDD_BD_Minor_Version | C8 | INIT <BDD_BD_Minor_Ver> |
| BYTE DCB_BDD_Device_SubType | C9 |  |
| BYTE DCB_BDD_Int_13h_Number | CA |  |
| ULONG DCB_BDD_flags | CB | was BDD_Flags |
| ULONG DCB_BDD_Name_Ptr | D3 |  |
| ULONG DCB_apparent_sector_cnt[2] | D7 | num. of secs as seen by tsd and above |
| ULONG DCB_apparent_blk_size | DF | blk size of dev. as seen by tsd and above |
| ULONG DCB_apparent_head_cnt | E3 | num. of heads as seen by tsd and above |
| ULONG DCB_apparent_cyl_cnt | E7 | num. of cyls as seen by tsd and above |
| ULONG DCB_apparent_spt | EB | num. of secs/trk as seen by tsd and above |
| ULONG DCB_BDD_Sync_Cmd_Proc | EF |  |
| ULONG DCB_BDD_Command_Proc | F3 |  |
| ULONG DCB_BDD_Hw_Int_Proc | F7 | INIT <0> |
| ULONG DCB_BDP_Cmd_Queue_Ascending | FB | Ptrs to queued IOP’s. Each IOP uses its IOR_next field as a chaining pointer. |
| ULONG DCB_BDP_Cmd_Queue_Descending | FF | Ptrs to queued IOP’s. Each IOP uses its IOR_next field as a chaining pointer. |
| ULONG DCB_BDP_Current_Flags | 103 |  |
| ULONG DCB_BDP_Int13_Param_Ptr | 107 |  |
| ULONG DCB_BDP_Current_Command | 10B |  |
| ULONG DCB_BDP_Current_Position[2] | 10F | Used to remember prior sector number (used by enqueue_iop to determine whether to queue ascending or descending) |
| ULONG DCB_BDP_Reserved[5] | 117 |  |
| ULONG DCB_fastdisk_bdd | 12B | set for DCBs created when a fastdisk regs with the blockdev BDD for it else 0 |
|  | 12F | End BlockDev compatibility |

} DCB_BLOCKDEV, *PDCB_BLOCKDEV;

} DCB, *PDCB;

// Structure appended to physical DCB if cdrom device

typedef struct	_DCB_cdrom {

| DCB DCB_filler_2 |  | standard dcb header |
| --- | --- | --- |
| ULONG DCB_cdrom_Partition_Start |  | partition start sector |
| ULONG DCB_cdrom_Partition_End |  | partition end sector |
| UCHAR DCB_cd_ls_ft |  | first track number in the last sess |
| ULONG DCB_TOC[CDROM_MAX_TOC / 4 + 1] |  | cdrom toc buffer |
| ULONG DCB_cd_mode_sense_buf[CDROM_MAX_MODE_BUF /4 + 1] |  | cdrom mode |
| UCHAR DCB_cd_first_session |  | index number of first session |
| UCHAR DCB_cd_last_session |  | index of last session on disc |
| ULONG DCB_play_resume_start |  | re-start address when paused (LBA) |
| ULONG DCB_play_resume_stop |  | end of re-started play (LBA) |
| ULONG DCB_play_status |  | last command play status |
| ULONG DCB_cd_device_flags |  | flags indicating the audio support capabilities of the device |
| UCHAR DCB_cd_fs_lt |  | last track in the first session |
| UCHAR DCB_cd_bobbit_pt |  | indicates the track where a CDPLUS disc is chopped off to prevent data tracks from showing up in the table of contents |
| ULONG DCB_cd_fs_lo |  | lead out address of first session |
| ULONG DCB_cd_last_session_start |  | LBA start address of data area in the last session on a multi-session disc |
| ULONG DCB_cd_current_block_size |  | current block size selected by the mode command |
| UCHAR DCB_cd_vol_map[8] |  | current volume / channel mapping |
| ULONG DCB_cd_current_command |  | current command |
| ULONG DCB_cd_queue_head |  | head of current command queue |
| ULONG DCB_cd_reserved |  | reserved - MBZ |

} DCB_cdrom, *PDCB_cdrom;

// Structure appended to physical DCB if disk device

//  NOTE: Any changes in this structure must also be reflected in cfr.h. See iosbid.asm in inquiry_type_table.

typedef struct	_DCB_disk {

| DCB DCB_filler_1 | standard dcb header |
| --- | --- |
| USHORT DCB_write_precomp | Write Precompensation |
| ULONG DCB_disk_tsd_private | private area for tsd. used to store b: BDS ptr for single flp |

// To support Japanese 3mode floppy driver this is defined to add an extra

// field to the end of DCB_disk.

typedef struct	_DCB_floppy { /* */

| DCB_disk DCB_filler_3 | standard dcb header |
| --- | --- |
| USHORT DCB_3mode_Flags | 3mode floppy flag |

} DCB_floppy, *pDCB_floppy;

} DCB_disk, *pDCB_disk;

typedef struct _DCB_disk_geometry {

| ULONG sector_cnt[2] | number of sectors |
| --- | --- |
| ULONG blk_size | actual block size of the device as seen below the tsd. |
| ULONG head_cnt | number of heads as seen below the tsd. |
| ULONG cyl_cnt | number of cylinders as seen below the tsd. |
| ULONG spt | number of sectors per track as seen below the tsd. |

} DCB_disk_geometry, *pDCB_disk_geometry;

# 6.  Additional Reference Material

The following reference material can be useful to SCSI Miniport developers:

- Microsoft Windows 95 Device Driver Kit (supplied to MSDN subscribers)

- Microsoft Windows NT Device Driver Kit (supplied to MSDN subscribers).  Contains SCSI miniport information relevant to Windows 95 SCSI miniport development as well.

- Systems Programming for Windows 95, by Walter Oney (Microsoft Press).
See his web site for errata: http://www.oneysoft.com.

- The SCSI Bus and IDE Interface, by Friedhelm Schmidt (Addison-Wesley) ISBN 0-201-42284-0.

- Microsoft support web site:
http://www.microsoft.com/support/

- For Windows 95 DDK samples & tools:
http://www.microsoft.com/support/products/developer/ddk_hardware/WinDDK/default.htm

# 7.  List of Knowledge Base Articles for Windows 95 SCSI Miniports

Q116450   BufferAccessScsiPortControlled Flag Information

Q140728   AdapterSettings Entry for SCSI Miniport under Windows 95

Q140730   Accessing PCI Device's Configuration Space from Device Driver

Q140732   PRB: Windows 95 Doesn't Load SCSI Miniport Driver

Q160667   Miniport Driver Fails Using Certain ScsiPort APIs

Q163358   PRB: DDKDEBUG.BAT Copies Some Drivers to the Wrong Directory

Q169584   BUG: Win95: ScsiPortGetDeviceBase Does Not Return Linear Address

# 8.  List of Knowledge Base Articles for Windows NT SCSI Miniports

Q113706   How to Pass Parameters to a SCSI Miniport via the Registry

Q126369   FIX: Large Transfers Via SCSI Passthrough May Crash System

Q137247   IOCTL_SCSI_MINIPORT and IOCTL_SCSI_PASS_THROUGH Limitations

Q140269   PRB: SCSI Miniport Adapter-Specific Parameter May Be Incorrect

Q140610   BUG: SCSI Miniport Parms Not Parsed on Multi PCI Bus Systems

Q140268   SCSI Miniport's Use of the SRB DataBuffer