Symbios Logic Validation software (TAZ)

Core Code Documentation

Table of Contents

1. Overview of core functions

1.1. Generic PCI functions (pci.c)

1.2. Generic Low-Level Functions (gen_tool.c):

1.3. 8XX Specific PCI Functions 8xx_pci.c:

1.4. 8XX Specific Functions (8xx_tool.c):

1.5. SCSI Specific functions (8xx_scsi.c):

2. Data structures and Macros

2.1. VALTYPES.H

2.1.1. #defines

2.2. PCI.H

2.2.1. #defines

2.2.2. Structures/types

2.3. GEN_TOOL.H

2.3.1. #defines

2.4. 8XX_PCI.H

2.4.1. #defines

2.5. 8XX_TOOL.H

2.5.1. #defines

2.5.2. Structures/types

2.6. 8XX_SCSI.H

2.6.1. #defines

2.6.2. Structures/types

2.6.2.1. maint_table_offsets

2.6.2.2. rw_table_offsets

2.6.2.3. scsimaint

2.6.2.4. scsirw

2.6.2.5. scsidev_record

3. Function descriptions and examples

3.1. FILE: PCI.C

3.1.1. Function: PCI_FindSYMDevice

3.1.2. Function: PCI_SetConfigRegister

3.1.3. Function: PCI_GetConfigRegister

3.1.4. Function: PCI_GetPCIBIOSVersion

3.1.5. Function: PCI_check_pci_bios

3.2. FILE: GEN_TOOL.C

3.2.1. Function: getPhysAddr

3.2.2. Function: IORead8

3.2.3. Function: IOWrite8

3.2.4. Function: IORead32

3.2.5. Function: IOWrite32

3.2.6. Function: ByteAlignBuffer

3.2.7. Function: CacheAlignBuffer

3.2.8. Function: InitBuffer

3.2.9. Function: Get_Time

3.2.10. Function: HasTimeElapsed

3.2.11. Function: WriteLog

3.3. FILE: GEN_TOOL.H

3.3.1. Macro Function: RMWon

3.3.2. Macro Function: RMWoff

3.4. FILE: 8XX_PCI.C

3.4.1. Function: C8XX_DevID2Str

3.4.2. Function: C8XX_BuildPCIDeviceList

3.4.3. Function: C8XX_ChoosePCIDevice

3.4.4. Function: GetMemBase

3.4.5. Function: GetIOBase

3.4.6. Function: GetRAMBase

3.4.7. Function: PCI_print_config

3.5. FILE: 8XX_TOOL.C

3.5.1. Function: init_8XX

3.5.2. Function: align_script

3.5.3. Function: PollISTAT

3.5.4. Function: CopyToScRam

3.5.5. Function: WriteScRam

3.5.6. Function: ReadScRam

3.5.7. Function: WriteLongScRam

3.5.8. Function: ReadLongScRam

3.5.9. Function: GenRandSG

3.5.10. Function: C8XX_GetChipType

3.5.11. Function: C8XX_SetBurstSize

3.6. FILE: 8XX_TOOL.H

3.6.1. Macro Function: iowriteDSP

3.7. FILE: 8XX_SCSI.C

3.7.1. Function: init_gpscripts

3.7.2. Function:  init_scsi

3.7.3. Function:  do_tur

3.7.4. Function:  do_RqSense

3.7.5. Function:  do_Inquiry

3.7.6. Function:  do_RdCap

3.7.7. Function:  do_StartStopUnit

3.7.8. Function:  do_SyncNeg

3.7.9. Function:  do_WideNeg

3.7.10. Function: do_Read6

3.7.11. Function: do_Read10

3.7.12. Function: do_Write6

3.7.13. Function: do_Write10

3.7.14. Function: do_rw

3.7.15. Function: Fill_SG_Entries

3.7.16. Function: HandleDataInPM

3.7.17. Function: HandleDataOutPM

3.7.18. Function: Fail_SCSI

3.7.19. Function: Bitbucket

3.8. FILE: VALIDATE.C

3.8.1. Function: main

Revision History:

Document Rev.		Date					Covers Core Version

1.0				11/95					1.01.00

2.0				3/96					1.02.00

# Overview of core functions

## Generic PCI functions (pci.c)

*PCI_FindSYMDevice - Determines if a given Symbios device is present in the system

PCI_SetConfigRegister - Writes the config register of a given Symbios device

PCI_GetConfigRegister - Read the config register of a given Symbios device

PCI_GetPCIBIOSVersion - Returns the Version of the PCI system bios in the system

*PCI_check_pci_bios - Checks for PCI system bios and prints out info on it

## Generic Low-Level Functions (gen_tool.c):

getPhysAddr - Converts Virtual to Physical address

IORead8 - Read a chip register (1-byte)

IOWrite8 - Writes a chip register (1-byte)

IORead32 - Read a Dword chip register (4-bytes)

IOWrite32 - Write a Dword chip register (4-bytes)

ByteAlignBuffer - Aligns a buffer to a particular byte boundary

CacheAlignBuffer - Aligns a buffer to a particular cache boundary

InitBuffer - Initializes a given buffer to a known state

Get_Time - Gets the current time from CMOS

HasTimeElapsed - Used to implement time-out mechanisms by using Get_Time

WriteLog - Used to write to a log file or standard out depending on command line switches

RMWon - Turns a bit or bits on in a register

RMWoff - Turns a bit or bits off in a register

## 8XX Specific PCI Functions 8xx_pci.c:

C8XX_DevID2Str - Converts a given device id/rev id to a printable device name string

*C8XX_GetMemBase - Gets the register memory base address of the 8xx part

*C8XX_GetIOBase - Gets the register IO base address of the 8xx part

*C8XX_GetRAMBase - Gets the Script RAM memory base address of the 8xx part

*C8XX_BuildPCIDeviceList - Builds a list of 8XX devices found in the system

*C8XX_ChoosePCIDevice - Allows the user to choose a specific 8xx part to test

*PCI_print_config - Prints out config information to the screen or the log file

## 8XX Specific Functions (8xx_tool.c):

init_8XX - Initializes chip to a known configuration

IOWriteDSP - Specialized IO Write to start script operation

PollISTAT - Polls for an interrupt and returns the type and vector

GenRandSG - Generates a random Scatter Gather List

CopyToScRAM - Copies Script to Script RAM

ReadScRAM - Reads a byte from Script RAM

WriteScRAM - Writes a byte to Script RAM

ReadLongScRAM - Reads a Dword from Script RAM

WriteLongScRAM - Writes a Dword to Script RAM

## SCSI Specific functions (8xx_scsi.c):

do_TUR - Does a Test Unit Ready and returns status

do_RqSense - Does a Request Sense

do_RdCap - Does a Read Capacity

do_Inquiry - Does an Inquiry

do_SyncNeg - Does a Synchronous Negotiation

do_WideNeg - Does a wide negotiation

do_Read6 - Does a 6 byte Read, TCQ, Sync., Wide & Disc. supported

do_Write6 - Does a 6 byte Write, TCQ, Sync., Wide & Disc. supported

do_Read10 - Does a 10 byte Read, TCQ, Sync., Wide & Disc. supported

do_Write10 - Does a 10 byte Write, TCQ, Sync., Wide & Disc. supported

* - Generally these functions should not be called by the test writer as they are specific tools used by the core code and are of little or no use to the test writer

# Data structures and Macros

## VALTYPES.H

### #defines

VCHAR - char

VBYTE - char

VUBYTE - unsigned char

VINT - int

VUINT - unsigned int

VLONG - long

VULONG - unsigned long

## PCI.H

### #defines

PCI_BIOS_INT - System Bios Interrupt Vector (0x1A)

PCI_FUNCTION_ID - Function for PCI system bios call (0xB1)

PCI_SYM_VENDOR_ID - Symbios Logic’s vendor ID (0x1000)

PCI_BIOS_PRESENT - Used to make PCI bios present call (0x01)

PCI_FIND_DEVICE - Used to make PCI find device call (0x02)

PCI_READ_CONFIG_DWORD - Used to make PCI read config dword call (0x0A)

PCI_WRITE_CONFIG_DWORD - Used to make PCI write config dword call (0x0D)

PCI_DEVICE_FOUND - Returned by FindSYMDevice indicating that the requested PCI device was found

PCI_NO_DEVICE_FOUND - Returned by FindSYMDevice indicating that the requested PCI device was not found

PCI_BIOS_REV_1X - Returned by PCI_GetPCIBIOSVersion to indicate the PCI bios version is 1.X

PCI_BIOS_REV_2X - Returned by PCI_GetPCIBIOSVersion to indicate the PCI bios version is 2.X

PCI_UNKNOWN_BIOS - Returned by PCI_GetPCIBIOSVersion to indicate the PCI bios version is unknown

PCI_NO_BIOS - Returned by PCI_GetPCIBIOSVersion to indicate that a PCI bios is not present in the system

PCI Configuration Register dword offsets

PCI_CONFIG_REG_VENID - Vendor ID

PCI_CONFIG_REG_DEVID - Device ID

PCI_CONFIG_REG_CMD - Command register

PCI_CONFIG_REG_STAT - Status register

PCI_CONFIG_REG_REVID - Revision ID

PCI_CONFIG_REG_CLASS - Class code register

PCI_CONFIG_REG_CLS - Cache line size register

PCI_CONFIG_REG_LAT - Latency timer

PCI_CONFIG_REG_HDRT - Header type

PCI_CONFIG_REG_BIST - Built in self test

PCI_CONFIG_REG_CIS - Card Info Serv. ptr.

PCI_CONFIG_REG_SUBV - Subsystem vend ID

PCI_CONFIG_REG_SUBID - Subsystem dev ID

PCI_CONFIG_REG_ROM - Exp. ROM BAR

PCI_CONFIG_REG_INTL - Interrupt line reg

PCI_CONFIG_REG_INTP - Interrupt pin register

PCI_CONFIG_REG_MGNT - Min grant register

PCI_CONFIG_REG_MLAT - Max latency register

### Structures/types

pcidev_record - Type used to hold PCI config information.  This structure is returned by PCI_FindSYMDevice.  This structure is passed to the TEST function for the device being tested.

typedef struct {

VUINT  bus_num;

VUINT  device_num;

VUINT  function;

VUINT  device_id;

VUINT  device_index;

VUBYTE rev_id;

VUBYTE CLS;

VUBYTE latency;

VUINT  command;

VULONG base_addr1;

VULONG base_addr2;

VULONG base_addr3;

VULONG base_addr4;

VULONG base_addr5;

VULONG base_addr6;

VULONG CIS;

VUINT  sub_id;

VUINT  sub_ven_id;

VULONG ROM_base_addr;

VUBYTE intl;

VUBYTE intp;

} pcidev_record;

*bus_num - the bus number the PCI device resides on

*device_num - the PCI device number of the PCI device

*function - the function number of the PCI device

*device_id - the device ID of the PCI device

*device_index - the index number of the PCI device

*rev_id - the revision id of the PCI device

*CLS - the cache line size register of the PCI device

*latency - the latency timer of the PCI device

*command - the command register of the PCI device

base_addr1 - BAR1 (8XX Register Memory base)

*base_addr2 - BAR2 (8XX Register IO base)

*base_addr3 - BAR3 (8XX Script RAM base 825A/875 only)

base_addr4 - BAR4

base_addr5 - BAR5

base_addr6 - BAR6

CIS - Cardbus CIS pointer

sub_id - Subsystem ID of the PCI Device

sub_ven_id - Subsystem vendor ID of the PCI device

*ROM_base_addr - ROM base address of the PCI device

*intl - Interrupt line of the PCI device

*intp - Interrupt pin of the PCI device

* - These variables are currently filled in by the Core code when initializing PCI devices

Note that if a PCI config register is changed then the corresponding information in this data structure should be changed also

pci_bios_info - structure returned by PCI_GetPCIBIOSVersion.  Contains information about PCI system bios.

typedef struct {

VUBYTE access_mech;

VUINT interface_lvl;

VUBYTE last_bus;

} pci_bios_info;

access_mech - Bit 0 set, Hardware mech #1 supported/ Bit 1 set, Hardware mech #2 supported

interface_lvl - Upper byte = major version, lower byte = minor version

last_bus - Last PCI bus number in system

## GEN_TOOL.H

### #defines

LOG_BASIC - Used in Writelog function to indicate a basic log entry

LOG_DETAIL - Used in Writelog function to indicate a detail log entry

LOG_PASS - Used in Writelog function to indicate a PASSing test log entry

LOG_FAIL - Used in Writelog function to indicate a FAILing test log entry

## 8XX_PCI.H

### #defines

Device/Version IDs

C810_DEVICE_ID

C810A_DEVICE_ID

C810_VERSION_ID

C810A_VERSION_ID

C810ALV_VERSION_ID

C810AP_DEVICE_ID

C810AP_VERSION_ID

C815_DEVICE_ID

C815_VERSION_ID

C820_DEVICE_ID

C820_VERSION_ID

C825_DEVICE_ID

C825A_DEVICE_ID

C825_VERSION_ID

C825A_VERSION_ID

C860_DEVICE_ID

C860LV_DEVICE_ID

C860_VERSION_ID

C860LV_VERSION_ID

C875_DEVICE_ID

C875_VERSION_ID

C885_DEVICE_ID

C885_VERSION_ID

C895_DEVICE_ID

C895_VERSION_ID

C896_DEVICE_ID

C896_VERSION_ID

NULL_DEVICE_ID

Return values from functions

C8XX_GOOD - Returned by any function in C8XX_PCI.C which successfully completed

C8XX_NO_BASE - Returned by C8XX_GetIOBase, C8XX_GetMEMBase and C8XX_GetRAMBase to indicate that no base address was assigned to the respective register

8XX Base address offsets

C8XX_CONFIG_REG_IOB - Register IO Base address config register offset

C8XX_CONFIG_REG_MEMB - Register Memory Base address config register offset

C8XX_CONFIG_REG_RAMB - Script RAM Memory Base address config register offset

## 8XX_TOOL.H

### #defines

Return values from functions

C8XX_GOOD - Returned by any function in C8XX_TOOL.C which successfully completed

C8XX_COPYCOMPLETE - Internal return code used when copying to and from script RAM, will not be returned by test accessible function

C8XX_POLLTO - Returned by PollISTAT to indicate no chip interrupt was detected in the allotted time

C8XX_RAMCOPYFAILURE - Returned by CopyToScRam to indicate the script copy to Script RAM failed

C8XX_RAMWRITEFAILURE - Returned by WriteScRam and WriteLongScRam to indicate the write to the script RAM failed

C8XX_RAMREADFAILURE - Returned by ReadScRam and ReadLongScRam to indicate the Read of the script RAM failed

53C8XX Interrupt bits

C8XX_INT_DIP - DMA interrupt pending (ISTAT)

C8XX_INT_SIP - SCSI interrupt pending (ISTAT)

C8XX_INT_MDPE - Master parity error (DSTAT)

C8XX_INT_BF - Bus Fault (DSTAT)

C8XX_INT_ABRT - Scripts aborted (DSTAT)

C8XX_INT_SSI - Script Step interrupt (DSTAT)

C8XX_INT_SIR - Script Interrupt instruction (DSTAT)

C8XX_INT_MA - Phase Mismatch (SIST)

C8XX_INT_CMP - Function Complete (SIST)

C8XX_INT_SEL - Selected (SIST)

C8XX_INT_RSL - Reselected (SIST)

C8XX_INT_SGE - SCSI Gross error (SIST)

C8XX_INT_UDC - Unexpected disconnect (SIST)

C8XX_INT_RST - SCSI Reset received (SIST)

C8XX_INT_PAR - SCSI Parity error (SIST)

C8XX_INT_STO - SCSI Selection time-out (SIST)

C8XX_INT_GEN - General purpose timer int (SIST)

C8XX_INT_HTH - Handshake to Handshake int (SIST)

53C8XX Device listing for the c8xxdev_record.device field

C8XX_53C810

C8XX_53C810A

C8XX_53C810ALV

C8XX_53C815

C8XX_53C825

C8XX_53C825A

C8XX_53C860

C8XX_53C860LV

C8XX_53C875

C8XX_53C885

C8XX_53C895

C8XX_53C896

C8XX_UNKNOWN_DEVICE

Feature masks for c8xxdev_record below

C8XX_WIDE_SCSI - Device supports Wide SCSI transfers

C8XX_ROM_IF - Device has ROM interface

C8XX_DIFF_SCSI - Device supports Differential SCSI

C8XX_BRST_OP - Device supports burst op code fetch

C8XX_PREFETCH - Device supports Script Prefetch

C8XX_SRAM - Device has Scripts RAM

C8XX_CACHE - Device supports PCI Cache commands

C8XX_SSAID - Device supports SSAID register

C8XX_EX_TIMER - Device supports expanded H2H and Gen timers

C8XX_EX_SLPAR - Device supports expanded SLPAR

C8XX_EN_RMOVE - Device supports Enhanced register move

C8XX_IRQ_DIS - Device supports IRQ disable

C8XX_LD_STR - Device supports Load Store script instruction

C8XX_FAST20 - Device supports Fast-20

C8XX_32SCRATCH - Device supports additional 32 scratch registers

C8XX_CLK_DBLR - Device has SCSI clock doubler

C8XX_FAST40 - Device supports Fast-40

C8XX_64BITPCI - Device supports 64-bit pci

Burst Sizes for Feature sets below and for calls to C8XX_SetBurstSize

C8XX_BURST_2

C8XX_BURST_4

C8XX_BURST_8

C8XX_BURST_16

C8XX_BURST_32

C8XX_BURST_64

C8XX_BURST_128

Feature info for all devices for c8xxdev_record

C8XX_53C8XX_FEAT - Device features (as defined above)

C8XX_53C8XX_FIFO_A - Small (compatible) fifo size

C8XX_53C8XX_FIFO_B - Large (incompatible) fifo size

C8XX_53C8XX_BRST_A - Max burst at Fifo size A

C8XX_53C8XX_BRST_B - Max burst at Fifo size B

C8XX_53C8XX_OFFSET - Max SCSI Sync offset

53C8XX register set

All registers are #defined as they appear in the 53C8XX data manuals

### Structures/types

ti_entry - Used for creating Script table indirect entries.

typedef struct {

VULONG count;

VULONG address;

} ti_entry;

count - Either a byte count in the Block Move Table Indirect entry or Selection information in a Select Table Indirect entry.  The select information format is as follows (0x) SCNTL3:SDID:SXFER:00.

address - This is the destination/source data address in a Block Move Table Indirect entry.  This field is not used in a Select Table Indirect entry and must be set to 0.

c8xxdev_record - Used to store device feature information

typedef struct {

VINT device;

VINT fifo_size_a;

VINT fifo_size_b;

VINT max_burst_a;

VINT max_burst_b;

VINT max_offset;

VULONG features;

} c8xxdev_record;

device - This is the type of device being tested.  This value is #defined for all known devices above

features - indicates the particular features that a 8xx device supports

The bit definitions are as follows:

Bit 0 - Device supports Wide SCSI data transfers

Bit 1 - Device has a back end ROM interface

Bit 2 - Device supports differential

Bit 3 - Device supports burst op code fetch

Bit 4 - Device supports instruction prefetch

Bit 5 - Device has Scripts RAM

Bit 6 - Device supports Cache Line Size and Cache 			Commands

Bit 7 - Device supports SSAID register

Bit 8 - Device supports expanded timers

Bit 9 - Device supports expanded SLPAR register

Bit 10 - Device supports enhanced register move script 			instruction

Bit 11 - Device supports IRQ disable

Bit 12 - Device supports LOAD/STORE script instructions

Bit 13 - Device supports Fast-20 transfers

Bit 14 - Device supports additional 32 Scratch registers

Bit 15 - Device supports Clock Doubler

Bit 16 - Device supports Fast-40 transfers

The remaining bit definitions are reserved for future use.

fifo_size_a - This is the power up fifo size of the device

fifo_size_b - If the device supports a larger programmable fifo size This will contain that size.  It will be zero if the device does not support another fifo size;

max_burst_a - This is the maximum burst size that the device will support given that the fifo size is at fifo_size_a above

max_burst_b - This is the maximum burst size that the device will support given that the fifo size is at fifo_size_b above.  If the device does not support fifo_size_b then this will be zero

max_offset - This is the maximum synchronous offset the device will support

## 8XX_SCSI.H

### #defines

Core script information

MAINT_TABLE_SIZE - Script maint table indirect size

RW_TABLE_SIZE - Script RW table indirect size

MAX_SG_ENTRIES - Maximum SG entries script can service at one time

INITSCSI_MAX_RETRIES - Maximum retries to clear a check condition in the init_scsi routine

Phases of the scsi bus as reported by socl and sbcl

SCSI_MSGO_PHASE

SCSI_MSGI_PHASE

SCSI_STATUS_PHASE

SCSI_CMD_PHASE

SCSI_DATAI_PHASE

SCSI_DATAO_PHASE

Return values from functions

SCSI_GOOD - Returned by any function in 8XX_SCSI.C which completes successfully

SCSI_SELECTTO - Returned by any of the do_ functions where there was a selection time-out

SCSI_MSGREJECT - Returned by do_syncneg or do_wideneg to indicate that the device has rejected the sync or wide negotiation attempt

SCSI_COMMANDFAILED - Returned by any of the do_ functions where a SCSI command failed but the SCSI bus is no longer in use

SCSI_CON_COMMANDFAILED - Returned by any of the do_ functions where a SCSI command failed and the target that was being accessed is still connected to the bus.

SCSI_ILLEGAL_INT - Returned by any of the do_ maintenance functions (do_tur, do_rqsense, do_inquiry, do_rdcap, do_syncneg, and do_wideneg) where an illegal interrupting condition occurred

SCSI_POLLTO - Returned by any of the do_ maintenance functions where there was no interrupt in the specified time-out period

SCSI_UNKNOWN - Returned by any of the do_ maintenance functions where there was an unknown error condition.

### Structures/types

#### maint_table_offsets

Enumerated list for use with the core maintenance table indirect script.  Should not be used by the test function.

enum maint_table_offsets {

MAINT_SCSI_ID = 0,

MAINT_MSGO_BUF,

MAINT_CMD_BUF,

MAINT_STAT_BUF,

MAINT_MSGI_BUF,

MAINT_EMSGI_BUF,

MAINT_DATA_BUF

};

#### rw_table_offsets

Enumerated list for use with the core read/write table indirect script.  Should not be used by the test function.

enum rw_table_offsets {

RW_SCSI_ID = 0,

RW_ID_BUF,

RW_CMD_BUF,

RW_STAT_BUF,

RW_MSGI_BUF,

RW_DATA_BUF1,

RW_DATA_BUF2,

RW_DATA_BUF3,

RW_DATA_BUF4,

RW_DATA_BUF5,

RW_DATA_BUF6,

RW_DATA_BUF7,

RW_DATA_BUF8,

RW_DATA_BUF9,

RW_DATA_BUF10,

RW_DATA_BUF11,

RW_DATA_BUF12,

RW_DATA_BUF13,

RW_DATA_BUF14,

RW_DATA_BUF15,

RW_DATA_BUF16

};

#### scsimaint

Structure required by the maintenance scsi functions (do_tur, do_rqsense, do_inquiry, do_readcap, do_syncneq, and do_wideneg) All items marked input must be filled in depending on the function called.

typedef struct {

VULONG sel_info;          	/* input/output */

VULONG data_length;    	/* input/output */

VUBYTE msgo_buf[6];   	/* input */

VUBYTE cmd_buf[12];   	/* output */

VUBYTE stat_buf;       	/* output */

VUBYTE msgi_buf;       	/* output */

VUBYTE emsgi_buf[4];   	/* output */

VUBYTE *data_buf;      	/* input/output */

} scsimaint;

sel_info - A select table indirect entry which contains the appropriate information for the device being selected.  This is required to be filled in for all maintenance functions.  This field is modified by the do_syncneg and do_wideneg to reflect the proper sxfer and scntl3 values for the respective negotiation that took place.

data_length - This field must be filled in to indicate the amount of data desire for the do_rqsense and do_inquiry functions.  Upon return from those functions this field contains the actual amount of data that was sent from the target during the command.

msgo_buf - Array item 0 of this field must be filled in with the appropriate identify message (0x8X or 0xCX).  The remaining array items can be filled in with a tag message or in the case of do_syncneg or do_wideneg must be filled in with the appropriate negotiation messages.  Tag messages are not allowed when doing sync or wide negotiation.

cmd_buf - This array field is filled in by the maintenance function.  It is not necessary to fill in this field.  Upon return from the function the actual scsi command used will be in this array.

stat_buf - This field is filled in by the maintenance function.  Upon return from the function the status byte received from the target will be located here.

msgi_buf - This field is filled in by the maintenance function.  Upon return from the function the msg in byte from the target at the completion of the command will be stored here.

emsgi_buf - This array field is filled in by the do_syncneg and do_wideneg with the return negotiation message.  It is not necessary to use this information to program the chip for wide or sync operation as the do_wideneg and do_syncneg functions perform this operation and place the appropriate values in the sel_info field above.

data_buf - This pointer must be assigned to a valid data buffer when executing a do_rqsense, do_inquiry, or do_rdcap function.  The buffer must be large enough to hold the maximum amount of data requested (i.e. it must be at least “data_length” in size or 10 bytes in the case of a read capacity)

#### scsirw

Structure required by the read/write scsi functions (do_read6, do_read10, do_write6, do_write10, and do_rw) All items marked input must be filled in.

typedef struct {

ti_entry *sg_entry;     		/* input */

VULONG sel_info;        		/* input */

VULONG lba;             		/* input */

VULONG blk_count;       		/* input */

VINT id_count;          		/* input */

VINT sg_count;          		/* input */

VUBYTE enable_auto_rqs;	/* input */

VUBYTE id_buf[3];       		/* input */

VUBYTE cmd_buf[12];		/* output */

VUBYTE sense_length;		/* output */

VUBYTE sense_buf[18];		/* output */

VUBYTE IO_major_status;	/* output */

VUBYTE IO_minor_status;	/* output */

VUBYTE Xfer_status;		/* output */

VUBYTE stat_buf;        		/* output */

VUBYTE msgi_buf;        		/* output */

VUBYTE resel_msg_buf[3];	/* output */

} scsirw;

sg_entry - This is a pointer to a scatter gather list.  This pointer must be assigned to a valid scatter gather list.  Note that even if scatter gather is not used (i.e. only on data Table indirect entry) this pointer must be assigned to that single entry as this is the sole mechanism for passing byte count and address information to the function.

sel_info - This is the standard selection table indirect entry.  It must be filled in with the appropriate sync, wide and destination id information in the correct format.  See the ti_entry data structure description for the Select Table Indirect entry format.

lba - This is the starting Logical Block Address on the for the read or write transfer.  This value should not exceed the maximum LBA as reported by the target device.

blk_count - This is the block count for the read or write transfer.  This value plus the LBA must also not exceed the maximum LBA as reported by the target device.

id_count - This is the number of initial message bytes that should be sent to the target device.  It will generally be 1 or 3, 1 indicating only an identify message should be sent and 3 indicated an identify plus tag message should be sent.

sg_count - This is the number of scatter gather entries for this IO.  Note that non scatter gather IO’s should initialize this value to one as all IO’s are considered to be scatter gather, non-scatter gather IO’s will just have an sg_count of 1.

enable_auto_rqs - Enable Auto Request Sense.  If the IO completes with a check condition the a request sense will be issued automatically if this flag is set.  Otherwise no request sense will be issued.

id_buf - This field should be filled in with the identify message for the IO and optionally a tag message for tagged command queuing.

cmd_buf - This field hold the actual command of the read or write.  It is not necessary to fill in this field except for the do_rw function.  Upon return from the read or write function this command will contain the command sent to the target device.  If the do_rw function is being called this field must be filled in with a read or write type command that is compatible with the do_rw function.

sense_length - If the IO completed with a check condition then this field will contain the number of request sense byte in the sense buffer

sense_buf - If the IO completes with a check condition then this buffer will contain the sense data from an auto request sense.

IO_major_status - This contains the major status of the IO on return. This indicatates the general cause of a failure if there is in fact a failure. See the above list of status values for all possible return values.

IO_minor_status - This contains the minor status of the IO on return. This indicates the specific cause of a failure if thare is in fact a failure. See the above list of status values for all possible return values.

Xfer_status - This status byte indicates if the transfer completed or not. See the above list for Xfer_status values.

stat_buf - Upon successful return this field will contain the status byte from the target at the end IO.

msgi_buf - Upon successful return this field will contain message in byte from the target at the end of the IO.

resel_msg_buf - This field is used to hold the reselection identify message and option tag from the target device after a reselection.  Upon successful return of a disconnecting IO this field will contain the last identify and tag message the target device sent.  If the IO is not disconnecting this field is undefined.

#### scsidev_record

This structure is returned as a linked list by the init_scsi routine.  It contains information about each device that was found on the SCSI bus.

typedef struct _scsidev_record  {

VUBYTE id_type;

VUBYTE flags;                    /* 0-sync,1-wide,2-cmdq,3-remove */

VULONG sel_info;

VULONG maxlba;

VULONG blksize;

struct _scsidev_record *nxt_dev;

} scsidev_record;

id_type - This field contains the target id of the device found and the type of the device as report in the inquiry data.

flags - This field contains certain flags indicating certain capabilities of the device.  Bit 0 indicates sync capability, bit 1 indicates wide capability, bit 2 indicates tag command queuing capability, and bit 3 indicates if the medium is removable.  Note that these flags only indicate capability, they do not indicate the current state of the device.

sel_info - This is the standard selection table indirect entry.  It must be filled in with the appropriate sync, wide and destination id information in the correct format.  See the ti_entry data structure description for the Select Table Indirect entry format.

maxlba - This is the Maximum Logical Block address allowed by the device as reported by the Read Capacity command.

blksize - This is the Block size of the device in bytes as reported in the Read Capacity command.

nxt_dev - This is a pointer to the next device in the linked list.

# Function descriptions and examples

## FILE: PCI.C

### Function: PCI_FindSYMDevice

Prototype: VINT PCI_FindSYMDevice(pcidev_record *PCIDevice)

Purpose : To determine if a given Symbios Logic device is installed in

the PCI system.

Input: Structure pcidev_record with the following entries filled in:

device_id       The device id being searched for

device_index    The index of the device being searched for

Output: This function fills in the rest of the PCIDevice data structure

if a device is found and returns PCI_DEVICE_FOUND.

If no device was found with that ID and INDEX then

PCI_NO_DEVICE_FOUND is returned with the PCIDevice structure

unmodified.

Assumptions: None

Other functions called: 	PCI_GetPCIBIOSVersion to make sure there's a PCI BIOS

PCI_GetConfigRegister to fill in the device rev id

Calling Example:

/* look for devices */

while (ID_Array[i] != NULL_DEVICE_ID)

{

PCIDevice[NumPCIDevices]->device_id = ID_Array[i];

PCIDevice[NumPCIDevices]->device_index = 0;

/* find all devices with a particular device id */

while (PCI_FindSYMDevice(PCIDevice[NumPCIDevices]) != PCI_NO_DEVICE_FOUND)

{

NumPCIDevices++;

/* did we run out of space to store devices, if yes return */

if (NumPCIDevices == Max_PCIDevices) return(NumPCIDevices+1);

PCIDevice[NumPCIDevices]->device_index =

PCIDevice[NumPCIDevices-1]->device_index+1;

}

i++;

}

### Function: PCI_SetConfigRegister

Prototype: void PCI_SetConfigRegister(pcidev_record *PCIDevice, VUINT offset, VULONG value)

Purpose: To write a particular device's configuration register

Input: pcidev_record data structure indicating the device to write to

offset - the dword offset in config space to write

value - the value to write to the config register

Output: Nothing

Assumptions: None

Restrictions: Only dword writes to config space are allowed

Other functions called: PCI_GetPCIBIOSVersion to make sure there's a PCI BIOS

Calling Example:

value = 0x11223344;

offset = 0x0C;

PCI_SetConfigRegister(PCIDevice, offset, value);

### Function: PCI_GetConfigRegister

Prototype: VULONG PCI_GetConfigRegister(pcidev_record *PCIDevice, VUINT offset)

Purpose: To read a particular devices configuration register

Input: pcidev_record data structure indicating the device to read from

offset - the dword offset in config space to read

Output: Dword config register value

Assumptions: None

Restrictions: Only dword reads to config space are allowed

Other functions called: PCI_GetPCIBIOSVersion to make sure there's a PCI BIOS

Calling Example:

offset = 0x0C;

value = PCI_GetConfigRegister(PCIDevice, offset);

### Function: PCI_GetPCIBIOSVersion

Prototype: VUINT PCI_GetPCIBIOSVersion(pci_bios_info *pcibios)

Purpose: To determine what (if any) PCI BIOS version is out there

Input: pointer to pci_bios_info data structure

If this pointer is null then no attempt is made to fill in the

structure (can be used for just assuring the existance of a PCI BIOS),

if it is a valid pointer then info on the PCI BIOS is returned.

This includes the Interface Version, Access Mechanism's supported

and the last pci bus in the system.

Output: 	PCI_BIOS_REV_1X - PCI BIOS version 1.x found

PCI_BIOS_REV_2X - PCI BIOS version 2.x found

PCI_UNKNOWN_BIOS - PCI BIOS found but unknown version

PCI_NO_BIOS - No PCI BIOS present in the system

If pcibios != NULL then BIOS info is filled in and returned in the

structure.

Assumptions: None

Restrictions: None

Other functions called: None

Calling Example:

CALLED WITH POINTER TO PCI_BIOS_INFO STRUCTURE

/* Check for PCI bios existance */

if (PCI_GetPCIBIOSVersion(pcibios) == PCI_NO_BIOS) {

WriteLog(LOG_FAIL, LOG_DETAIL, "\nNo PCI Bios found!!!", NULL);

exit(1);

}

CALLED WITHOUT POINTER TO PCI_BIOS_INFO STRUCTURE

pci_version = PCI_GetPCIBIOSVersion(NULL);

/* make sure there is a PCI bios present */

if (pci_version == PCI_NO_BIOS) {

return(0xFFFFFFFFl);

}

### Function: PCI_check_pci_bios

Prototype: void PCI_check_pci_bios(pci_bios_info *pcibios)

Purpose: To check for the existance of a PCI system bios and print

out information about that bios.

Input: pointer to pci_bios_info data structure

Output: If no PCI bios is found this function prints an error and

exits the entire program.  If a PCI bios is found then

the information in the pcibios data structure is filled

in via a call to PCI_GetPCIBIOSVersion and is printed.

Assumptions: A valid pointer to the pcibios structure is passed

into the function.

Restrictions: None

Other functions called: 	PCI_GetPCIBIOSVersion to determine the

existance of a PCI bios and fill in the

pcibios data structure

Writelog to output information to a file

or the screen depending on command line

switches

sprintf to format a string for Writelog

Calling Example:

PCI_check_pci_bios(&pcibios);

## FILE: GEN_TOOL.C

### Function: getPhysAddr

Prototype: VULONG getPhysAddr(void far * addr)

Purpose: To determine the physical address of a given pointer

Input: far pointer to a data buffer

Output: Physical address of the pointer

Assumptions: Executable is currently running in DOS real mode.

Restrictions: This function will not work properly unless it is

running in DOS real mode.  This does not include

a Windows DOS shell or any other type of DOS

shell running under a protected mode Operating system

Other functions called: none

Calling Example:

rw_buffer_table[RW_ID_BUF].address = getPhysAddr(rw->id_buf);

### Function: IORead8

Prototype: VUBYTE IORead8(VULONG IO_Addr)

Purpose: To read a byte from an io port

Input: IO address of byte to be read

Output: byte read from io port

Assumptions: That the IO port actually exists

Restrictions: Although IO_Addr is defined as a VULONG it must not

exceed 16 bits in length as this is the maximum

IO address the X86 archtecture can produce

Other functions called: inportb to read the io port

Calling Example:

data = IORead8(PCIDevice->base_addr2+ISTAT);

### Function: IOWrite8

Prototype: void IOWrite8(VULONG IO_Addr, VUBYTE value)

Purpose: To write a byte out to an IO port

Input: Value to be written and IO port address

Output: None

Assumptions: That the IO port actually exists

Restrictions: Although IO_Addr is defined as a VULONG it must not

exceed 16 bits in length as this is the maximum

IO address the X86 archtecture can produce

Other functions called: outportb to write to the io port

Calling Example:

data = 0x25;

IOWrite8(PCIDevice->base_addr2+DCNTL, data);

### Function: IORead32

Prototype: VULONG IORead32(VULONG IO_Addr)

Purpose: To read a dword (32 bits) from an io port

Input: IO address of dword to be read

Output: dword read from io port

Assumptions: That the IO port actually exists

Restrictions: Although IO_Addr is defined as a VULONG it must not

exceed 16 bits in length as this is the maximum

IO address the X86 archtecture can produce

Other functions called: none

Calling Example:

long_data = IORead32(PCIDevice->base_addr2+DSPS);

### Function: IOWrite32

Prototype: void IOWrite32(VULONG IO_Addr, VULONG value)

Purpose: To write a dword (32 bits) out to an IO port

Input: Value to be written and IO port address

Output: None

Assumptions: That the IO port actually exists

Restrictions: Although IO_Addr is defined as a VULONG it must not

exceed 16 bits in length as this is the maximum

IO address the X86 archtecture can produce

Other functions called: none

Calling Example:

long_data = 0x11223344;

IOWrite32(PCIDevice->base_addr2+SCRATCHA, long_data);

### Function: ByteAlignBuffer

Prototype: void far *ByteAlignBuffer(void far *buffer, VINT alignment)

Purpose: To align a buffer to a given byte alignment

Input: Pointer to buffer to be aligned. Buffer should be 3 bytes

larger than it's desired size.

Desired alignment of the given buffer.

0 = dword (32-bit) aligned

1 = byte (8-bit) aligned

2 = word (16-bit) aligned

3 = 3byte (24-bit) aligned

Output: Pointer to requested buffer alignment.  The pointer will

be as close to the buffer start address as possible.

Assumptions: Executable is currently running in DOS real mode.

Restrictions: This function will not work properly unless it is

running in DOS real mode.  This does not include

a Windows DOS shell or any other type of DOS

shell running under a protected mode Operating system

Other functions called: FP_SEG to get buffer segment

FP_OFF to get buffer offset

MK_FP to create a new far pointer to the

requested buffer alignment

Calling Example:

VUBYTE buffer[43];

buffer = ByteAlignBuffer(buffer, 0);	/* align buffer to dword boundary */

### Function: CacheAlignBuffer

Prototype: void far *CacheAlignBuffer(void far *buffer, VINT cls, VINT offset)

Purpose: To align a buffer to a given cache alignment

Input: pointer to data buffer to be aligned.  Buffer should be a

cache line size larger than the desired buffer size.

The cache line size to be aligned to.  This does not have

to be the cache line size of the system.

An offset from the cache line boundary

Output: A pointer to the cache line boundary plus the offset based

on the provided cache line size.  The pointer will be as

close to the begining of the buffer as possible.

Assumptions: Executable is currently running in DOS real mode.

Restrictions: This function will not work properly unless it is

running in DOS real mode.  This does not include

a Windows DOS shell or any other type of DOS

shell running under a protected mode Operating system

Other functions called: FP_SEG to get buffer segment

FP_OFF to get buffer offset

MK_FP to create a new far pointer to the

requested buffer alignment

Calling Example:

VUBYTE buffer[160];

buffer = CacheAlignBuffer(buffer, 8, 0); 	/* align buffer to 8 dword cache boundary */

### Function: InitBuffer

Prototype: void InitBuffer(VUBYTE far *buffer, VUINT size, VINT operation,VUBYTE fill_val)

Purpose: To initialize a buffer to a known value

Input: Far pointer to a byte data buffer.

The size of the data buffer.

The fill/increment value for the buffer

The operation to be performed on the buffer

0 - fill buffer with fill value

1 - fill buffer with incrementing pattern, increment by

the fill value and start at 0.

2 - fill buffer with a random pattern.

Output: Initialized buffer

Assumptions: none

Restrictions: Buffer size cannot exceed 64k

Other functions called: none

Calling Example:

VUBYTE buffer[43];

InitBuffer(buffer, sizeof(buffer), 0, 0x5A);	/* fill buffer with 0x5A’s */

### Function: Get_Time

Prototype: VULONG Get_Time(void)

Purpose: Gets the current timer tick value from the system bios

data area.  The timer ticks 18.2 times every second.

Input: none

Output: Current value of the timer

Assumptions: None

Restrictions: None

Other functions called: MK_FP to create a pointer to the timer tick

memory location

enable to enable interrupts

disable to disable interrupts

Calling Example:

time = Get_Time();

### Function: HasTimeElapsed

Prototype: VINT HasTimeElapsed(VULONG StartTime, VINT Seconds)

Purpose: To determine if a given amount of time in seconds has elapsed

Input: A start time for a given time period to be measured

The number of seconds to have elapsed

Output: True (1) if the number of seconds has elapsed

False (0) if not

Assumptions: The system timers is ticking at 18.2 ticks per second.

To reduce complexity the ticks per second is rounded up

to 19.  This results in slightly higher actual elapsed

time periods but it is felt this will not pose a

substantial problem as this function will be mainly

used as a watchdog type timer.

Restrictions: None

Other functions called: Get_Time to get the current time.

Calling Example:

start_time = Get_Time();

if (HasTimeElapsed(start_time, 5)) {

break;

}

### Function: WriteLog

Prototype: VINT WriteLog( VINT P_F, VINT B_D, VCHAR *Str1, VCHAR *Str2)

Purpose: To write a standard string out to an output file. If

the log file is an actual file (not stdout) then the

file is opened appended to and closed to prevent any

loss of information do to some hardware failure.

Input: P_F - indicates a passing or failing log entry

(The macros LOG_PASS and LOG_FAIL in gen_tool.h can be

used for this parameter)

B_D - indicates a basic or detail log entry

(The macros LOG_BASIC and LOG_DETAIL in gen_tool.h can be

used for this parameter)

Str1 - Test name in a basic log entry & entire output string

in a detail entry.

Str2 - Parameter list in a basic entry, not used in detail entry

logfilename_ptr - This is a global pointer that points to

the name of the log file name.  If it is null

then the strings above are written to stdout

Output:  Given the two parameter P_F and B_D four different strings

are written to either the log file or stdout or both.  The

following chart spells out the variations.

LOG_BASIC, LOG_PASS - "PASS Test:(Str1) Par:(Str2)\n"

LOG_BASIC, LOG_FAIL - "FAIL Test:(Str1) Par:(Str2)\n"

LOG_DETAIL, LOG_PASS - "(Str1)"

LOG_DETAIL, LOG_FAIL - "(Str1)"

(Note that the final option (DETAIL, FAIL) is written

to both the log file and stderr if necessary, all

other options go to just the log file or stdout)

Assumptions: logfilename_ptr is properly assigned

Restrictions: Str1 and Str2 must not contain the newline symbol (\n)

Other functions called: fopen to open the log file (if not stdout)

fclose to close the log file (if not stdout)

fprintf to print the strings

Calling Example:

sprintf(string,"Device ID: %02X", PCIDevice->device_id);

WriteLog(LOG_PASS, LOG_DETAIL, string, NULL);

## FILE: GEN_TOOL.H

### Macro Function: RMWon

Prototype: RMWon (reg, mask)

Purpose: Used to turn on a bits in an 8 bit register

Input: IO port address and mask indicating which bits to turn on

1 indicating turn bit on and 0 indicating leave bit alone.

Output: None

Assumptions: That the IO port actually exists

Restrictions: Although IO_Addr is defined as a VULONG it must not

exceed 16 bits in length as this is the maximum

IO address the X86 archtecture can produce

Other functions called: IOWrite8 and IORead8 to write and read to the io port

Calling Example:

RMWon(PCIDevice->base_addr2+DMODE, 0xC0);

### Macro Function: RMWoff

Prototype: RMWoff (reg, mask)

Purpose: Used to turn off a bits in an 8 bit register

Input: IO port address and mask indicating which bits to turn off

1 indicating turn bit off and 0 indicating leave bit alone.

Output: None

Assumptions: That the IO port actually exists

Restrictions: Although IO_Addr is defined as a VULONG it must not

exceed 16 bits in length as this is the maximum

IO address the X86 archtecture can produce

Other functions called: IOWrite8 and IORead8 to write and read to the io port

Calling Example:

RMWoff (PCIDevice->base_addr2+DMODE, 0xC0);

## FILE: 8XX_PCI.C

### Function: C8XX_DevID2Str

Prototype: VCHAR *C8XX_DevID2Str(VINT id, VUBYTE rev_id)

Purpose : Returns 53C8XX Device Name string given a 53C8XX Device ID

Input:  Valid 53C8XX device ID

Output:	Descriptor string of 53C8xx device

"Unknown Symbios SCSI Device" if the ID pass in is not known

Assumptions: None

Other functions called: None

Calling Example:

sprintf(string,"\nDevice Name: %s", C8XX_DevID2Str(PCIDevice->device_id,

PCIDevice->rev_id));

### Function: C8XX_BuildPCIDeviceList

Prototype: VINT C8XX_BuildPCIDeviceList(pcidev_record *PCIDevice[], VINT Max_PCIDevices)

Purpose : Builds a list of 53C8XX devices found in the system

Input:  Pointer to an array of pcidev_record structures and a limit

Max_PCIDevices indicating how big the pcidev_record array is.

Output:	This function fills the PCIDevice data structure by calling

the PCI_FindSYMDevice function for all possible ID's.

If Max_PCIDevices is reached then the function returns immediately

with a return value = Max_PCIDevices, otherwise it returns the

number of devices found.

Assumptions: That enough PCIDevice structures have been allocated to fill in

Max_PCIDevices structures

Other functions called: PCI_FindSYMDevice

Calling Example:

NumPCIDevices = C8XX_BuildPCIDeviceList(&PCIDevice_ptr, MAX_DEVICES);

### Function: C8XX_ChoosePCIDevice

Prototype: VINT C8XX_ChoosePCIDevice(pcidev_record *PCIDevice[], VINT NumPCIDevices)

Purpose : Allows the selection of a particular 8xx device in the system

Displays all devices and prompts for a selection

Input:  Array of pcidev_records (for all devices in the system) and the

actual number of devices in the system

Output:	Returns the index of the selected device used to index into the

PCIDevice array

Assumptions: That NumPCIDevices gives the proper number of devices in the

PCIDevice array

Other functions called: C8XX_DeviceID2Str

Calling Example:

SelPCIDevice = C8XX_ChoosePCIDevice(&PCIDevice_ptr, NumPCIDevices);

### Function: GetMemBase

Prototype: VUINT C8XX_GetMemBase(pcidev_record *PCIDevice)

Purpose : Fills in the PCIDevice structure with the Memory Base address

of the 53C8XX device

Input: Pointer to Structure pcidev_record PCIDevice which indicates which

device to get the Memory base address for

Output: Fills in the PCIDevice->base_addr1 field and returns C8XX_GOOD status

if the value read is non-zero.  Returns C8XX_NO_BASE if the value

read is zero

Assumptions: Pointer to device structure is valid.

Other functions called: PCI_GetConfigRegister

Calling Example:

if (C8XX_GetMemBase(PCIDevice) != C8XX_GOOD)

{

WriteLog(LOG_FAIL, LOG_DETAIL, "\nError: device not mapped to Mem space!!", NULL);

WriteLog(LOG_FAIL, LOG_DETAIL, "Cannot proceed with tests!!", NULL);

exit(1);

}

### Function: GetIOBase

Prototype: VUINT C8XX_GetIOBase(pcidev_record *PCIDevice)

Purpose : Fills in the PCIDevice structure with the IO Base address

of the 53C8XX device

Input: Pointer to Structure pcidev_record PCIDevice which indicates which

device to get the IO base address for

Output: Fills in the PCIDevice->base_addr2 field and returns C8XX_GOOD status

if the value read is non-zero.  Returns C8XX_NO_BASE if the value

read is zero

Assumptions: Pointer to device structure is valid.

Other functions called: PCI_GetConfigRegister

Calling Example:

if (C8XX_GetIOBase(PCIDevice) != C8XX_GOOD)

{

WriteLog(LOG_FAIL, LOG_DETAIL, "\nError: device was not mapped to IO space!!", NULL);

WriteLog(LOG_FAIL, LOG_DETAIL, "Cannot proceed with tests!!", NULL);

exit(1);

}

### Function: GetRAMBase

Prototype: VUINT C8XX_GetRAMBase(pcidev_record *PCIDevice)

Purpose : Fills in the PCIDevice structure with the SCRIPT RAM Base address

of the 53C825A or 875 device

Input: Pointer to Structure pcidev_record PCIDevice which indicates which

device to get the SCRIPT RAM base address for

Output: Fills in the PCIDevice->base_addr3 field and returns C8XX_GOOD status

if the value read is non-zero.  Returns C8XX_NO_BASE if the value

read is zero

Assumptions: Pointer to device structure is valid and the device is a 53C825A

or a 53C875.

Other functions called: PCI_GetConfigRegister

Calling Example:

if (C8XX_GetRAMBase(PCIDevice) != C8XX_GOOD)

{

WriteLog(LOG_FAIL, LOG_DETAIL, "\nWarning: device's Script RAM not mapped!!", NULL);

}

### Function: PCI_print_config

Prototype: void PCI_print_config(pcidev_record *PCIDevice)

Purpose : To output configurtion information about a PCI device

Input: Pointer to Structure pcidev_record PCIDevice.  This structure

contains the config information to be output.

Output: PCI config information either to the screen or to a file

Assumptions: none

Other functions called: sprintf to format string for Writelog

WriteLog to output information to file or screen

Calling Example:

PCI_print_config(PCIDevice);

## FILE: 8XX_TOOL.C

### Function: init_8XX

Prototype: void init_8XX(pcidev_record *PCIDevice)

Purpose: To initialize a 53C8xx device to a known state

Input: A pointer to the pcidev_record structure.

The base_addr2 and device_id fields of this structure

are used in this routine.

Output: Nothing

Assumptions: SCSI Scripts are currently not executing

Restrictions:  None

Other functions called: IOWrite8 to write initialization values to

chip registers

RMWon to turn on single bits in registers

WriteLog to output exactly what is being

initialized to a log file

sprintf to format a string for WriteLog

delay to hold the SCSI bus reset assertion

Calling Example:

init_8XX(PCIDevice);

### Function: align_script

Prototype: VULONG *align_script(VULONG *original_script, VUINT size)

Purpose: To align a SCSI Script to a dword boundary

Input: A pointer to the original script array

The size of the script array

Output: A pointer to an aligned script array.  If the original

array is already aligned then no memory is allocated.

If not, new memory is allocated and aligned and the

script is copied into this aligned memory and a pointer

to this array is returned. If memory cannot be allocated

an error is printed and the program will exit.

Assumptions: none

Restrictions:  none

Other functions called: FP_OFF to get offset of script array

malloc to allocate a new array if necessary

_fmemcpy to copy script array

printf to print error message if memory

cannot be allocated

Calling Example:

script = align_script(MAINT_SCRIPT, sizeof(MAINT_SCRIPT));

/* did we get the memory, if it was necessary to relocate the script */

if (script == NULL) return(SCSI_COMMANDFAILED);

### Function: PollISTAT

Prototype: VINT PollISTAT(VULONG IO_Base, VULONG *vector, VUBYTE *dstat,

VUBYTE *istat, VUINT *sist, VUINT timeout)

Purpose: Poll the ISTAT register until an interrupt is detected or

a timeout occurs.  This function will clear all interrupts

including stacked ones.

Input: The IO Base address of the 53C8XX part

Pointers to variables to hold the values of dstat, istat, sist

and an interrupt vector

A timeout period in seconds, a timeout of 0 equals no timeout.

Output: If a timeout occurs and no interrupt is detected then the

function returns C8XX_POLLTO.

If an interrupt is detected then the dstat, istat, sist and

if necessary vector variables are filled in and the function

returns C8XX_GOOD.

Assumptions:   none

Restrictions:  none

Other functions called: IORead8 to read the ISTAT register and

dstat register

IORead32 to read the vector regiser and

the sist register

Get_Time to get the current time

HasTimeElapsed to determine if the timeout

has expired

Calling Example:

/* wait for interrupt to complete */

int_return = PollISTAT(PCIDevice->base_addr2, &vector, &dstat, &istat, &sist, 2);

### Function: CopyToScRam

Prototype: VINT CopyToScRam (pcidev_record *PCIDevice, VULONG far *script, VUINT size)

Purpose: Copy a script to script ram.  This function uses a memory

to memory move to copy the script to script ram

Input: A pointer to the pcidev_record data structure

The base_addr2 and 3 fields are used in this structure

A pointer to the script to be copied

The size of the script being copied

Output: C8XX_GOOD upon successful completion of the copy

C8XX_RAMCOPYFAILURE upon failure of the copy

Assumptions: SCSI Scripts are currently not executing

The device being accessed currently has scripts ram

available.

Restrictions:  Do not call this function while scripts are executing

Other functions called: Bytealignbuffer to align the mem to mem

move script

iowriteDSP to start the mem to mem script

pollISTAT to wait for an interrupt for

completion of mem to mem move

Calling Example:

ret = CopyToScRam(PCIDevice, script, 512);

if (ret == C8XX_RAMCOPYFAILURE) {

printf("Ram copy failed\n");

}

### Function: WriteScRam

Prototype: VINT WriteScRam(pcidev_record *PCIDevice, VUBYTE byte, VINT offset)

Purpose: write a byte to script ram.  This function uses a memory

to memory move to write to script ram

Input: A pointer to the pcidev_record data structure

The base_addr2 and 3 fields are used in this structure

The value of the byte to be written

The offset in scripts ram where the byte is to be written

Output: C8XX_GOOD upon successful completion of the write

C8XX_RAMWRITEFAILURE upon failure of the write

Assumptions: SCSI Scripts are currently not executing

The device being accessed currently has scripts ram

Restrictions:  Do not call this function while scripts are executing

Other functions called: Bytealignbuffer to align the mem to mem

move script

iowriteDSP to start the mem to mem script

pollISTAT to wait for an interrupt for

completion of mem to mem move

Calling Example:

WriteScRam(PCIDevice, 0, offset);

### Function: ReadScRam

Prototype: VINT ReadScRam(pcidev_record *PCIDevice, VUBYTE *byte, VINT offset)

Purpose: read a byte from script ram.  This function uses a memory

to memory move to read from script ram

Input: A pointer to the pcidev_record data structure

The base_addr2 and 3 fields are used in this structure

A pointer to the variable to recieve the byte

The offset of the byte to be read

Output: C8XX_GOOD upon successful completion of the read

C8XX_RAMREADFAILURE upon failure of the read

Assumptions: SCSI Scripts are currently not executing

The device being accessed currently has scripts ram

Restrictions:  Do not call this function while scripts are executing

Other functions called: Bytealignbuffer to align the mem to mem

move script

iowriteDSP to start the mem to mem script

pollISTAT to wait for an interrupt for

completion of mem to mem move

Calling Example:

ReadScRam(PCIDevice, &byte, offset);

### Function: WriteLongScRam

Prototype: VINT WriteLongScRam(pcidev_record *PCIDevice, VULONG dword, VINT offset)

Purpose: write a dword to script ram.  This function uses a memory

to memory move to write to script ram

Input: A pointer to the pcidev_record data structure

The base_addr2 and 3 fields are used in this structure

The value of the dword to be written

The offset in scripts ram where the dword is to be written

Output: C8XX_GOOD upon successful completion of the write

C8XX_RAMWRITEFAILURE upon failure of the write

Assumptions: SCSI Scripts are currently not executing

The device being accessed currently has scripts ram

Restrictions:  Do not call this function while scripts are executing

Other functions called: Bytealignbuffer to align the mem to mem

move script

iowriteDSP to start the mem to mem script

pollISTAT to wait for an interrupt for

completion of mem to mem move

Calling Example:

WriteLongScRam(PCIDevice, 0x11223344, offset);

### Function: ReadLongScRam

Prototype: VINT ReadLongScRam(pcidev_record *PCIDevice, VULONG *dword, VINT offset)

Purpose: read a dword from script ram.  This function uses a memory

to memory move to read script ram

Input: A pointer to the pcidev_record data structure

The base_addr2 and 3 fields are used in this structure

A pointer to the variable to recieve the dword

The offset of the dword to be read

Output: C8XX_GOOD upon successful completion of the read

C8XX_RAMREADFAILURE upon failure of the read

Assumptions: SCSI Scripts are currently not executing

The device being accessed currently has scripts ram

Restrictions:  Do not call this function while scripts are executing

Other functions called: Bytealignbuffer to align the mem to mem

move script

iowriteDSP to start the mem to mem script

pollISTAT to wait for an interrupt for

completion of mem to mem move

Calling Example:

ReadLongScRam(PCIDevice, &dword, offset);

### Function: GenRandSG

Prototype: VINT GenRandSG(VUBYTE far *buf, VUINT size, ti_entry *sg_list, VINT num_entries)

Purpose: Generate a random scatter gather list.  The list is generated

in scripts table indirect format.  The function generates

random entries by dividing the buffer up into equal slices

and then generating a random number within each slice.  The

byte count for entry x then is equal to to the distance

to the slice boundary for entry x-1 plus some random number

between 0 and the slice.  This allows for relatively equal

distribution of scatter gather entries while maintaining a

relatively high amount of randomness.

Input: A pointer to the buffer for which the list is to be generated

The size of the buffer

A pointer to a scatter gather table (table indirect entries)

The number of scatter gather entries desired

Output: This function always returns C8XX_GOOD

Assumptions: The number of entries is <= the size of the buffer

The function randomize was called prior to calling this

function

Restrictions: The number of entries must be <= the size of the buffer

Only call randomize once during the execution of the code

(i.e. do not repeatedly call randomize as this will

cause not random numbers to be generated)

Other functions called: getPhysAddr to get the physical address of the

buffer

rand to generate random numbers

Calling Example:

vubyte buffer[16000];

ti_entry sg_list[10];

GenRandSG(buffer, sizeof(buffer), sg_list, 10);

### Function: C8XX_GetChipType

Prototype: VINT C8XX_GetChipType (pcidev_record *PCIDevice, c8xxdev_record *C8XXDevice)

Purpose: Return the device type that is being tested

Input:  Pointer to PCIDevice data structure and Pointer to C8XXDevice data

structure

Output: Filled in C8XXDevice data structure if the device is recognized and C8XX_GOOD

is returned.

If not, C8XXDevice is unknown and UNKNOWN_DEVICE_TYPE is returned.

Assumptions: none

Restrictions: none

Other functions called: none

Calling Example:

c8xxdev_record C8XXDevice;

/* make sure this is a device we recognize */

if (C8XX_GetChipType(PCIDevice, &C8XXDevice) != C8XX_GOOD)

return(NULL);

### Function: C8XX_SetBurstSize

Prototype: void C8XX_SetBurstSize(pcidev_record *PCIDevice, VINT burst)

Purpose: Set the burst size of the device being tested

Input: A pointer to the PCIDevice data structure and the desired burst

size.  Burst size macros C8XX_BURST_N should be used for the

burst parameter to insure compatibility.

Output: Sets the burst size for the device pointed to by PCIDevice

Assumptions: That the burst is valid for this device

Restrictions: It would not be wise to call this function with a burst

size that is invalid for the device.

Other functions called: RMWoff/on to set and clear bits in regs

Calling Example:

C8XX_SetBurstSize(PCIDevice, C8XX_BURST_16);

## FILE: 8XX_TOOL.H

### Macro Function: iowriteDSP

Prototype: iowriteDSP(IO_Base, value)

Purpose: To write to DSP register to start script execution

Input: The IO Base address of the 53C8XX part

The value to be written to the DSP

Output: Nothing

Assumptions: SCSI Scripts are currently not executing

Restrictions:  Do not call this macro while scripts are executing

Other functions called: IOWrite32 to write the DSP

Calling Example:

iowriteDSP(PCIDevice->base_addr2, getPhysAddr(script));

## FILE: 8XX_SCSI.C

### Function: init_gpscripts

Prototype: VINT init_gpscripts(void)

Purpose : To initialize the maintenance and read/write scripts.  This

function is called by the core validation code and need not

be called by the test writer.

Input: Nothing

Output: On successful alignment and script initialization the function

returns SCSI_GOOD.  If there are any memory allocation problems

then the function returns SCSI_COMMANDFAILED.

Assumptions: none

Other functions called: malloc to allocate memory for ti tables

ByteAlignBuffer to align ti tables

align_script to ensure script dword alignment

sizeof to get size of script arrays

getPhysAddr to initialize script buffers.

Calling Example:

if (init_gpscripts() != SCSI_GOOD)

### Function:  init_scsi

Prototype: scsidev_record *init_scsi(pcidev_record *PCIDevice, VINT do_sync, VINT do_wide)

Purpose : Find and initialize all scsi devices on the bus optionally

negotiating sync and wide.

Input:  Pointer to pcidev_record structure to indicate the PCI device

whose scsi bus to initialize.

Flag do_sync to indicate whether or not to do sync negotiation

or not (0 don't do sync neg., 1 do sync neg.)  The function

will negotiate for the fastest the PCI device can transfer data.

Flag do_wide to indicate whether or not to do wide negotiation

or not (0 don't do wide neg., 1 do wide neg.)

Output: A pointer to a linked list of scsidev_record strutures.  Each

entry in the list represents a device located on the scsi bus.

All important information about the device is contained in this

structure.  See 8xx_scsi.h for a detailed description of the

scsidev_record structure.

Assumptions:  none

Restrictions: If a device hangs the bus during this function it will

no longer function properly.

Other functions called: SCSI routines are called in this order:

do_tur, do_rqsense, do_inquiry, do_rdcap

and optionally do_wideneg and do_syncneg

IORead8 to read chip registers

malloc to get memory for scsi device entry

sizeof to get size of scsi device record

Notes: This function will attempt to clear all check conditions prior

to continuing to look at a device.  This includes clearing power on

reset or issuing a start unit to spin the device up.

This function will reset the SCSI bus.

This function will only do wide negotiation if requested, the drive

supports it and the chip supports it.

If the 8xx device being used is not recognized this function will

return NULL.

This function will not scan the device's scsi id, it will skip

that id.

Calling Example:

scsidev_record *SCSIDevice;

SCSIDevice = NULL;

SCSIDevice = init_scsi(PCIDevice, 0, 0);

if (SCSIDevice == NULL) printf("No SCSI devices found...\n");

else {

current_dev = SCSIDevice;

while (current_dev != NULL) {

printf("id_type: %02x\n",current_dev->id_type);

printf("flags: %02x\n",current_dev->flags);

printf("Sel info: %08lx\n", current_dev->sel_info);

printf("maxlba: %08lx\n", current_dev->maxlba);

printf("block size: %08lx\n", current_dev->blksize);

current_dev = current_dev->nxt_dev;

}

}

### Function:  do_tur

Prototype: VINT do_TUR(pcidev_record *PCIDevice, scsimaint *maint)

Purpose :  To execute a Test Unit Ready to a device on the SCSI bus

Input:  A pointer to the pcidev_record data structure and the scsimaint

data structure.  The following variables in the scsimaint structure

must be initialized for do_tur to function properly:

sel_info must have a valid table indirect selection entry

remember ordering of the dword is (SCNTL3|SDID|SXFER|00)

All other variables in the structure need not be initialized.

Output: Upon successful completion do_tur returns SCSI_GOOD.

Upon an ISTAT polling timeout (no interrupt) do_tur returns SCSI_POLLTO

Upon a selection timeout do_tur returns SCSI_SELECTTO

Upon any other type of interrupt do_tur returns SCSI_ILLEGAL_INT

Upon any other error condition do_tur returns SCSI_UNKNOWN

The following variables in the scsimaint data structure will be

modified:

cmd_buf will contain the TUR command

msgo_buf[0] will contain the identify message 0x80

stat_buf will contain the status from the target

msgi_buf will contain the final message in byte from the target

Assumptions: none

Restrictions: Disconnects are not allowed with this command.

Other functions called:  IOWriteDSP to start the Script engine

IOWrite32 to initialize the DSA register

pollISTAT to look for an interrupt

getPhysAddr to patch TI entries

Fail_SCSI to clean up on an error condition

Calling Example:

if ((PCIDevice->device_id == C875_DEVICE_ID) ||

(PCIDevice->device_id == C860_DEVICE_ID)) {

maint.sel_info = 0x35000000l;

}

else {

maint.sel_info = 0x33000000l;

}

ret = do_tur(PCIDevice, &maint);

if (ret == SCSI_SELECTTO) printf("Select Timeout\n");

else if (ret == SCSI_GOOD) {

printf("Command completed status = %x\n", maint.stat_buf);

}

### Function:  do_RqSense

Prototype: VINT do_RqSense(pcidev_record *PCIDevice, scsimaint *maint)

Purpose :  To execute a Request Sense or Inquiry to a device on the SCSI bus

Input:  A pointer to the pcidev_record data structure and the scsimaint

data structure.  The following variables in the scsimaint structure

must be initialized for do_RqSense to function properly:

sel_info must have a valid table indirect selection entry

remember ordering of the dword is (SCNTL3|SDID|SXFER|00)

data_length must be set to the desired amount of bytes to be

recieved (Note this can be set to 0xFF to receive the maximum

bytes the drive will give)

data_buf must be initialized to point to a valid data buffer

this buffer must be at least 'data_length' in size

All other variables in the structure need not be initialized.

The Rqsns_or_Inquiry variable is set to the command byte of one

or the other commands.

Output: Upon successful completion do_rqsense returns SCSI_GOOD.

Upon an ISTAT polling timeout (no interrupt) do_rqsense returns SCSI_POLLTO

Upon a selection timeout do_rqsense returns SCSI_SELECTTO

Upon any other type of interrupt do_rqsense returns SCSI_ILLEGAL_INT

Upon any other error condition do_rqsense returns SCSI_UNKNOWN

The following variables in the scsimaint data structure will be

modified:

cmd_buf will contain the Request Sense or Inquiry command

msgo_buf[0] will contain the identify message 0x80

data_buf will be filled with the Rq Snse or Inquiry data

data_length will reflect the actual amount of data received

stat_buf will contain the status from the target

msgi_buf will contain the final message in byte from the target

Assumptions: none

Restrictions: Disconnects are not allowed with this command.

Other functions called:  IOWriteDSP to start the Script engine

IOWrite32 to initialize the DSA register

pollISTAT to look for an interrupt

getPhysAddr to patch TI entries

Fail_SCSI to clean up on an error condition

Calling Example:

VUBYTE test2[0xFF];

if ((PCIDevice->device_id == C875_DEVICE_ID) ||

(PCIDevice->device_id == C860_DEVICE_ID)) {

maint.sel_info = 0x35000000l;

}

else {

maint.sel_info = 0x33000000l;

}

maint.data_length = 0xff;

maint.data_buf = test2;

ret = do_RqSense(PCIDevice, &maint);

if (ret == SCSI_SELECTTO) printf("Select Timeout\n");

else if (ret == SCSI_GOOD) {

printf("Command completed status = %x\n", maint.stat_buf);

printf("Sense data:\n");

for (i=0; i<maint.data_length; i++) {

printf("%02x ", test2[i]);

if (!((i+1)%8)) printf("\n");

}

}

### Function:  do_Inquiry

Prototype: VINT do_Inquiry(pcidev_record *PCIDevice, scsimaint *maint)

Purpose :  To execute an Inquiry to a device on the SCSI bus

Input:  A pointer to the pcidev_record data structure and the scsimaint

data structure.  The following variables in the scsimaint structure

must be initialized for do_inquiry to function properly:

sel_info must have a valid table indirect selection entry

remember ordering of the dword is (SCNTL3|SDID|SXFER|00)

data_length must be set to the desired amount of bytes to be

recieved (Note this can be set to 0xFF to receive the maximum

bytes the drive will give)

data_buf must be initialized to point to a valid data buffer

this buffer must be at least 'data_length' in size

All other variables in the structure need not be initialized.

Output: Upon successful completion do_inquiry returns SCSI_GOOD.

Upon an ISTAT polling timeout (no interrupt) do_inquiry returns SCSI_POLLTO

Upon a selection timeout do_inquiry returns SCSI_SELECTTO

Upon any other type of interrupt do_inquiry returns SCSI_ILLEGAL_INT

Upon any other error condition do_inquiry returns SCSI_UNKNOWN

The following variables in the scsimaint data structure will be

modified:

cmd_buf will contain Inquiry command

msgo_buf[0] will contain the identify message 0x80

data_buf will be filled with the Inquiry data

data_length will reflect the actual amount of data received

stat_buf will contain the status from the target

msgi_buf will contain the final message in byte from the target

Assumptions: none

Restrictions: Disconnects are not allowed with this command.

Other functions called:  IOWriteDSP to start the Script engine

IOWrite32 to initialize the DSA register

pollISTAT to look for an interrupt

getPhysAddr to patch TI entries

Fail_SCSI to clean up on an error condition

Calling Example:

VUBYTE test2[0xFF];

if ((PCIDevice->device_id == C875_DEVICE_ID) ||

(PCIDevice->device_id == C860_DEVICE_ID)) {

maint.sel_info = 0x35000000l;

}

else {

maint.sel_info = 0x33000000l;

}

maint.data_length = 0xff;

maint.data_buf = test2;

ret = do_Inquiry(PCIDevice, &maint);

if (ret == SCSI_SELECTTO) printf("Select Timeout\n");

else if (ret == SCSI_GOOD) {

printf("Command completed status = %x\n", maint.stat_buf);

printf("Inquiry data:\n");

for (i=0; i<maint.data_length; i++) {

printf("%02x ", test2[i]);

if (!((i+1)%8)) printf("\n");

}

}

### Function:  do_RdCap

Prototype: VINT do_RdCap(pcidev_record *PCIDevice, scsimaint *maint)

Purpose :  To execute a Read Capicity to a device on the SCSI bus

Input:  A pointer to the pcidev_record data structure and the scsimaint

data structure.  The following variables in the scsimaint structure

must be initialized for do_RdCap to function properly:

sel_info must have a valid table indirect selection entry

remember ordering of the dword is (SCNTL3|SDID|SXFER|00)

data_buf must be initialized to point to a valid data buffer

this buffer must be at least 8 bytes in size

All other variables in the structure need not be initialized.

Output: Upon successful completion do_RdCap returns SCSI_GOOD.

Upon an ISTAT polling timeout (no interrupt) do_RdCap returns SCSI_POLLTO

Upon a selection timeout do_RdCap returns SCSI_SELECTTO

Upon any other type of interrupt do_RdCap returns SCSI_ILLEGAL_INT

Upon any other error condition do_RdCap returns SCSI_UNKNOWN

The following variables in the scsimaint data structure will be

modified:

cmd_buf will contain the Read Capacity command

msgo_buf[0] will contain the identify message 0x80

data_buf will be filled with the 8 bytes of Read Capacity data

stat_buf will contain the status from the target

msgi_buf will contain the final message in byte from the target

Assumptions: none

Restrictions: Disconnects are not allowed with this command.

Other functions called:  IOWriteDSP to start the Script engine

IOWrite32 to initialize the DSA register

pollISTAT to look for an interrupt

getPhysAddr to patch TI entries

Fail_SCSI to clean up on an error condition

Calling Example:

VUBYTE test2[0xFF];

if ((PCIDevice->device_id == C875_DEVICE_ID) ||

(PCIDevice->device_id == C860_DEVICE_ID)) {

maint.sel_info = 0x35000000l;

}

else {

maint.sel_info = 0x33000000l;

}

maint.data_buf = test2;

ret = do_RdCap(PCIDevice, &maint);

if (ret == SCSI_SELECTTO) printf("Select Timeout\n");

else if (ret == SCSI_GOOD) {

printf("Command completed status = %x\n", maint.stat_buf);

printf("Read Capacity data:\n");

for (i=0; i<8; i++) {

printf("%02x ", test2[i]);

if (!((i+1)%8)) printf("\n");

}

}

### Function:  do_StartStopUnit

Prototype: VINT do_StartStopUnit(pcidev_record *PCIDevice, scsimaint *maint)

Purpose :  To execute a Start or Stop Unit to a device on the SCSI bus

Input:  A pointer to the pcidev_record data structure and the scsimaint

data structure.  The following variables in the scsimaint structure

must be initialized for do_tur to function properly:

cmd_buf[4] must contain a valid start or stop command.

sel_info must have a valid table indirect selection entry

remember ordering of the dword is (SCNTL3|SDID|SXFER|00)

All other variables in the structure need not be initialized.

Output: Upon successful completion do_StartStopUnit returns SCSI_GOOD.

Upon an ISTAT polling timeout (no interrupt) do_StartStopUnit

returns SCSI_POLLTO

Upon a selection timeout do_StartStopUnit returns SCSI_SELECTTO

Upon any other type of interrupt do_StartStopUnit returns SCSI_ILLEGAL_INT

Upon any other error condition do_StartStopUnit returns SCSI_UNKNOWN

The following variables in the scsimaint data structure will be

modified:

cmd_buf will contain the Start or Stop unit command

msgo_buf[0] will contain the identify message 0x80

stat_buf will contain the status from the target

msgi_buf will contain the final message in byte from the target

Assumptions: none

Restrictions: Disconnects are not allowed with this command.

Other functions called:  IOWriteDSP to start the Script engine

IOWrite32 to initialize the DSA register

pollISTAT to look for an interrupt

getPhysAddr to patch TI entries

Fail_SCSI to clean up on an error condition

Calling Example:

maint.sel_info = 0x05050000

maint.cmd_buf[4] = 0x01;

if (do_StartStopUnit(PCIDevice, &maint) != SCSI_GOOD)

error = 1;

### Function:  do_SyncNeg

Prototype: VINT do_SyncNeg(pcidev_record *PCIDevice, scsimaint *maint)

Purpose :  To perform sync negotiation via a tur to a device on the SCSI bus

Input:  A pointer to the pcidev_record data structure and the scsimaint

data structure.  The following variables in the scsimaint structure

must be initialized for do_SyncNeg to function properly:

sel_info must have a valid table indirect selection entry

remember ordering of the dword is (SCNTL3|SDID|SXFER|00)

msgo_buf[4] must have the Sync Negotiation period as defined by

the SCSI spec

msgo_buf[5] must have the Sync Neqotiation offset as defined by

the SCSI spec

All other variables in the structure need not be initialized.

Output: Upon successful completion do_SyncNeg returns SCSI_GOOD.

Upon a Message Reject of the negotiation do_SyncNeg returns SCSI_MSGREJECT

Upon an ISTAT polling timeout (no interrupt) do_SyncNeg returns SCSI_POLLTO

Upon a selection timeout do_SyncNeg returns SCSI_SELECTTO

Upon any other type of interrupt do_SyncNeg returns SCSI_ILLEGAL_INT

Upon any other error condition do_SyncNeg returns SCSI_UNKNOWN

The following variables in the scsimaint data structure will be

modified:

cmd_buf will contain the TUR command

msgo_buf[0] will contain the identify message 0x80

msgo_buf[1-3] will contain the sync neg message [0x01,0x03,0x01]

emsgi_buf[0-3] will contain the sync neg reply from the target

stat_buf will contain the status from the target

msgi_buf will contain the final message in byte from the target

sel_info SCNTL3 an SXFER values will be modified to meet the

clock divider requirement of the negotiated sync offset and

transfer rate

Assumptions: none

Restrictions: Disconnects are not allowed with this command.

Remember negotiation ordering should always be Wide then Sync

as Wide negotiation resets Sync values to Async.

Other functions called:  IOWriteDSP to start the Script engine

IOWrite32 to initialize the DSA register

pollISTAT to look for an interrupt

getPhysAddr to patch TI entries

Fail_SCSI to clean up on an error condition

Calling Example:

maint.msgo_buf[4] = 0x19;

maint.msgo_buf[5] = 0x08;

ret = do_SyncNeg(PCIDevice, &maint);

if (ret == SCSI_SELECTTO) printf("Select Timeout\n");

else if (ret == SCSI_GOOD) {

printf("Command completed status = %x\n", maint.stat_buf);

printf("Sync values from drive:");

for (i=0; i<4; i++) {

if (!((1+i)%8)) printf("\n");

printf("%02x ", maint.emsgi_buf[i]);

}

}

else if (ret == SCSI_MSGREJECT) printf("Sync Message Reject\n");

### Function:  do_WideNeg

Prototype: VINT do_WideNeg(pcidev_record *PCIDevice, scsimaint *maint)

Purpose :  To perform wide negotiation via a tur to a device on the SCSI bus

Input:  A pointer to the pcidev_record data structure and the scsimaint

data structure.  The following variables in the scsimaint structure

must be initialized for do_WideNeg to function properly:

sel_info must have a valid table indirect selection entry

remember ordering of the dword is (SCNTL3|SDID|SXFER|00)

msgo_buf[4] must have the Wide Negotiation value as defined by

the SCSI spec (0=8-bit, 1=16-bit, 2=32-bit)

All other variables in the structure need not be initialized.

Output: Upon successful completion do_WideNeg returns SCSI_GOOD.

Upon a Message Reject of the negotiation do_WideNeg returns SCSI_MSGREJECT

Upon an ISTAT polling timeout (no interrupt) do_WideNeg returns SCSI_POLLTO

Upon a selection timeout do_WideNeg returns SCSI_SELECTTO

Upon any other type of interrupt do_WideNeg returns SCSI_ILLEGAL_INT

Upon any other error condition do_WideNeg returns SCSI_UNKNOWN

The following variables in the scsimaint data structure will be

modified:

cmd_buf will contain the TUR command

msgo_buf[0] will contain the identify message 0x80

msgo_buf[1-3] will contain the wide neg message [0x01,0x02,0x03]

emsgi_buf[0-2] will contain the wide neg reply from the target

stat_buf will contain the status from the target

msgi_buf will contain the final message in byte from the target

sel_info SCNTL3 values will be modified to so that the EWS bit

is set appropriately.  SXFER values will be reset to 0x00

so that the chip will be reset to async as required in SCSI.

Assumptions: none

Restrictions: Disconnects are not allowed with this command.

Remember negotiation ordering should always be Wide then Sync

as Wide negotiation resets Sync values to Async.

do_wide will reset sync values (in SCNTL3 and SXFER) as

required in the scsi spec.

Other functions called:  IOWriteDSP to start the Script engine

IOWrite32 to initialize the DSA register

pollISTAT to look for an interrupt

getPhysAddr to patch TI entries

Fail_SCSI to clean up on an error condition

Calling Example:

maint.msgo_buf[4] = 0x01;

ret = do_WideNeg(PCIDevice, &maint);

if (ret == SCSI_SELECTTO) printf("Select Timeout\n");

else if (ret == SCSI_GOOD) {

printf("Command completed status = %x\n", maint.stat_buf);

printf("Wide values from drive:");

for (i=0; i<3; i++) {

if (!((i+1)%8)) printf("\n");

printf("%02x ", maint.emsgi_buf[i]);

}

}

else if (ret == SCSI_MSGREJECT) printf("Wide Message Reject\n");

### Function: do_Read6

Prototype: VINT do_Read6(pcidev_record *PCIDevice, scsirw *rw)

Purpose : To execute a 6 byte read to a SCSI device

Input: A pointer to the pcidev_record structure and a pointer to the

scsirw data structure.  The follow variables in the scsirw

structure must be initialized:

sel_info must have a valid table indirect selection entry

remember ordering of the dword is (SCNTL3|SDID|SXFER|00)

lba must be initialized to a valid target logical block address

blk_count must have a valid block count for the read

id_count must be 1 or 3 depending on whether or not tagged

command queuing is used (1 for no TCQ 3 for TCQ)

id_buf[0-2] must be initilized with a valid identify message and

optionally a 2 byte TCQ message

sg_count must be initialized to the number of scatter gather entries

NOTE: If scatter gather is not being used this variable should

be set to 1.

sg_entry should point to a valid scatter gather list in table

indirect format.  NOTE: If scatter gather is not being used

this pointer should point to the single table indirect

entry for the io.  The scatter gather list pointed to by

sg_entry should match sg_count in length exactly.

All other variables in the structure need not be initialized.

Output:  do_Read6 returns the return value from the do_rw function.

please see the do_rw function for a detailed description of

return values.

do_Read6 modifies the following variables in the scsirw data structure:

cmd_buf is set to the appropriate read command given the

settings of lba and blk_count.

Other variables are modified by the do_rw function.  Please see that

function description for a listing of other variables modified.

Assumptions:  None

Restrictions: None

Other functions called:  do_rw to actually execute the read io to the

target.

Calling Example:

rw.sel_info = maint.sel_info;

rw.id_count =3;

rw.id_buf[0] = 0xC0;

rw.id_buf[1] = 0x20;

rw.id_buf[2] = 0x35;

rw.sg_entry = sg_list;

rw.sg_count = 10;

rw.blk_count = 32;

rw.lba = 0;

GenRandSG(test3, rw.blk_count*512, sg_list, rw.sg_count);

printf("Read6, LBA: %lx, #SG: %d, #Blk: %d\n", rw.lba, rw.sg_count, rw.blk_count);

ret = do_Read6(PCIDevice, &rw);

if (ret != SCSI_GOOD)

{

printf("Read6 FAIL LBA: %lx, #SG: %d, #Blk: %d\n", rw.lba, rw.sg_count, rw.blk_count);

}

### Function: do_Read10

Prototype: VINT do_Read10(pcidev_record *PCIDevice, scsirw *rw)

Purpose : To execute a 10 byte read to a SCSI device

Input: A pointer to the pcidev_record structure and a pointer to the

scsirw data structure.  The follow variables in the scsirw

structure must be initialized:

sel_info must have a valid table indirect selection entry

remember ordering of the dword is (SCNTL3|SDID|SXFER|00)

lba must be initialized to a valid target logical block address

blk_count must have a valid block count for the read

id_count must be 1 or 3 depending on whether or not tagged

command queuing is used (1 for no TCQ 3 for TCQ)

id_buf[0-2] must be initilized with a valid identify message and

optionally a 2 byte TCQ message

sg_count must be initialized to the number of scatter gather entries

NOTE: If scatter gather is not being used this variable should

be set to 1.

sg_entry should point to a valid scatter gather list in table

indirect format.  NOTE: If scatter gather is not being used

this pointer should point to the single table indirect

entry for the io.  The scatter gather list pointed to by

sg_entry should match sg_count in length exactly.

All other variables in the structure need not be initialized.

Output:  do_Read10 returns the return value from the do_rw function.

please see the do_rw function for a detailed description of

return values.

do_Read10 modifies the following variables in the scsirw data structure:

cmd_buf is set to the appropriate read command given the

settings of lba and blk_count.

Other variables are modified by the do_rw function.  Please see that

function description for a listing of other variables modified.

Assumptions:  None

Restrictions: None

Other functions called:  do_rw to actually execute the read io to the

target.

Calling Example:

rw.sel_info = maint.sel_info;

rw.id_count =3;

rw.id_buf[0] = 0xC0;

rw.id_buf[1] = 0x20;

rw.id_buf[2] = 0x35;

rw.sg_entry = sg_list;

rw.sg_count = 10;

rw.blk_count = 32;

rw.lba = 0;

GenRandSG(test3, rw.blk_count*512, sg_list, rw.sg_count);

printf("Read10, LBA: %lx, #SG: %d, #Blk: %d\n", rw.lba, rw.sg_count, rw.blk_count);

ret = do_Read10(PCIDevice, &rw);

if (ret != SCSI_GOOD)

{

printf("Read10 FAIL LBA: %lx, #SG: %d, #Blk: %d\n", rw.lba, rw.sg_count, rw.blk_count);

}

### Function: do_Write6

Prototype: VINT do_Write6(pcidev_record *PCIDevice, scsirw *rw)

Purpose : To execute a 6 byte write to a SCSI device

Input: A pointer to the pcidev_record structure and a pointer to the

scsirw data structure.  The follow variables in the scsirw

structure must be initialized:

sel_info must have a valid table indirect selection entry

remember ordering of the dword is (SCNTL3|SDID|SXFER|00)

lba must be initialized to a valid target logical block address

blk_count must have a valid block count for the write

id_count must be 1 or 3 depending on whether or not tagged

command queuing is used (1 for no TCQ 3 for TCQ)

id_buf[0-2] must be initilized with a valid identify message and

optionally a 2 byte TCQ message

sg_count must be initialized to the number of scatter gather entries

NOTE: If scatter gather is not being used this variable should

be set to 1.

sg_entry should point to a valid scatter gather list in table

indirect format.  NOTE: If scatter gather is not being used

this pointer should point to the single table indirect

entry for the io.  The scatter gather list pointed to by

sg_entry should match sg_count in length exactly.

All other variables in the structure need not be initialized.

Output:  do_Write6 returns the return value from the do_rw function.

please see the do_rw function for a detailed description of

return values.

do_Write6 modifies the following variables in the scsirw data structure:

cmd_buf is set to the appropriate write command given the

settings of lba and blk_count.

Other variables are modified by the do_rw function.  Please see that

function description for a listing of other variables modified.

Assumptions:  None

Restrictions: None

Other functions called:  do_rw to actually execute the write io to the

target.

Calling Example:

rw.sel_info = maint.sel_info;

rw.id_count =3;

rw.id_buf[0] = 0xC0;

rw.id_buf[1] = 0x20;

rw.id_buf[2] = 0x35;

rw.sg_entry = sg_list;

rw.sg_count = 10;

rw.blk_count = 32;

rw.lba = 0;

GenRandSG(test3, rw.blk_count*512, sg_list, rw.sg_count);

printf("Write6, LBA: %lx, #SG: %d, #Blk: %d\n", rw.lba, rw.sg_count, rw.blk_count);

ret = do_Write6(PCIDevice, &rw);

if (ret != SCSI_GOOD)

{

printf("Write6 FAIL LBA: %lx, #SG: %d, #Blk: %d\n", rw.lba, rw.sg_count, rw.blk_count);

}

### Function: do_Write10

Prototype: VINT do_Write10(pcidev_record *PCIDevice, scsirw *rw)

Purpose : To execute a 10 byte write to a SCSI device

Input: A pointer to the pcidev_record structure and a pointer to the

scsirw data structure.  The follow variables in the scsirw

structure must be initialized:

sel_info must have a valid table indirect selection entry

remember ordering of the dword is (SCNTL3|SDID|SXFER|00)

lba must be initialized to a valid target logical block address

blk_count must have a valid block count for the write

id_count must be 1 or 3 depending on whether or not tagged

command queuing is used (1 for no TCQ 3 for TCQ)

id_buf[0-2] must be initilized with a valid identify message and

optionally a 2 byte TCQ message

sg_count must be initialized to the number of scatter gather entries

NOTE: If scatter gather is not being used this variable should

be set to 1.

sg_entry should point to a valid scatter gather list in table

indirect format.  NOTE: If scatter gather is not being used

this pointer should point to the single table indirect

entry for the io.  The scatter gather list pointed to by

sg_entry should match sg_count in length exactly.

All other variables in the structure need not be initialized.

Output:  do_Write10 returns the return value from the do_rw function.

please see the do_rw function for a detailed description of

return values.

do_Write10 modifies the following variables in the scsirw data structure:

cmd_buf is set to the appropriate write command given the

settings of lba and blk_count.

Other variables are modified by the do_rw function.  Please see that

function description for a listing of other variables modified.

Assumptions:  None

Restrictions: None

Other functions called:  do_rw to actually execute the write io to the

target.

Calling Example:

rw.sel_info = maint.sel_info;

rw.id_count =3;

rw.id_buf[0] = 0xC0;

rw.id_buf[1] = 0x20;

rw.id_buf[2] = 0x35;

rw.sg_entry = sg_list;

rw.sg_count = 10;

rw.blk_count = 32;

rw.lba = 0;

GenRandSG(test3, rw.blk_count*512, sg_list, rw.sg_count);

printf("Write10, LBA: %lx, #SG: %d, #Blk: %d\n", rw.lba, rw.sg_count, rw.blk_count);

ret = do_Write10(PCIDevice, &rw);

if (ret != SCSI_GOOD)

{

printf("Write10 FAIL LBA: %lx, #SG: %d, #Blk: %d\n", rw.lba, rw.sg_count, rw.blk_count);

}

### Function: do_rw

Prototype: VINT do_rw(pcidev_record *PCIDevice, scsirw *rw)

Purpose : To execute a read or write type command to a SCSI device

Input: A pointer to the pcidev_record structure and a pointer to the

scsirw data structure.  The follow variables in the scsirw

structure must be initialized:

sel_info must have a valid table indirect selection entry

remember ordering of the dword is (SCNTL3|SDID|SXFER|00)

id_count must be 1 or 3 depending on whether or not tagged

command queuing is used (1 for no TCQ 3 for TCQ)

id_buf[0-2] must be initilized with a valid identify message and

optionally a 2 byte TCQ message

sg_count must be initialized to the number of scatter gather entries

NOTE: If scatter gather is not being used this variable should

be set to 1.

sg_entry should point to a valid scatter gather list in table

indirect format.  NOTE: If scatter gather is not being used

this pointer should point to the single table indirect

entry for the io.  The scatter gather list pointed to by

sg_entry should match sg_count in length exactly.

cmd_buf must be initialized with a valid read or write type command

enable_auto_rqs must be initialized to either 0 to disable auto

request sense or 1 (non-zero) to enable auto req. sense.

All other variables in the structure need not be initialized.

Output: Upon successful completion do_rw returns SCSI_GOOD.

Upon an ISTAT polling timeout (no interrupt) do_rw returns SCSI_POLLTO

Upon a selection timeout do_rw returns SCSI_SELECTTO

Upon a phase mismatch during Status, Command or Message In

phase do_rw returns SCSI_COMMANDFAILED or SCSI_CON_COMMANDFAILED

depending on the state of the scsi bus after trying to clear it

up with the bitbucket routine in the Fail_SCSI function

Upon any SCSI interrupts other that Phase Mismatch or Select TO

do_rw returns SCSI_COMMANDFAILED or SCSI_CON_COMMANDFAILED

Upon any DMA interrupt other than SIR do_rw will return

SCSI_COMMANDFAILED or SCSI_CON_COMMANDFAILED

Upon any DMA SIR interrupt other than Command_Complete, Need_More_SG

or SWIDE_byte_moved vectors do_rw will return SCSI_COMMANDFAILED

or SCSI_CON_COMMANDFAILED

Upon a SCSI CHECK CONDITION status do_rw will return

SCSI_COMMANDFAILED or SCSI_CON_COMMANDFAILED

The following variables in the scsirw data structure are modified by

the do_rw function:

The data buffers pointed to by the sg_entry TI entries

will be filled with the data from the target on

a read operation.

stat_buf will contain the status from the target

msgi_buf will contain the final message in byte from the target

resel_msg_buf will contain the final reselection identify and

TCQ message (if used) if disconnects were allowed on

this IO.

If auto request sense is enabled and a SCSI CHECK CONDITION

occurs during the IO then the sense_length will contain

the number of valid sense bytes and sense_buffer will

contain the sense data from the request sense.

Three other Status values, IO_Major_Status, IO_Minor_Status and

Xfer_status will be filled in with various status values.

The values are #defined in 8XX_SCSI.H. The following is a

description on how those statuses will be filled in:

Xfer_status will be a 1 if all data was transferred or a 0 if

all data was not transferred.  This is independant of whether

or not the IO completed succesfully or not.

Upon an Illegal SCSI interrupt:

IO_Major_Status will contain SCSI_ILLEGAL_SCSI_INTERRUPT

IO_Minor_Status will indicate the specific interrupt

Upon an Illegal DMA interrupt:

IO_Major_Status will contain SCSI_ILLEGAL_DMA_INTERRUPT

IO_Minor_Status will indicate the specific interrupt

Upon an Illegal SCRIPT interrupt:

IO_Major_Status will contain SCSI_ILLEGAL_SCRIPT_INTERRUPT

IO_Minor_Status will indicate the specific interrupt

Upon a succesful SCSI read or write:

IO_Major_Status will contain SCSI_COMMAND_COMPLETE

IO_Minor_Status will contain SCSI_NO_STATUS

Upon a SCSI check condition status:

IO_Major_Status will contain SCSI_CHECK_CONDITION

IO_Minor_Status will indicate whether or not the request

sense data is valid.  If enable_auto_rqs is 0 then this

will always indicate invalid sense data.

All of the conditions below will only have the IO_Major_Status

set.  IO_Minor_Status will be set to SCSI_NO_STATUS.

Upon a interrupt polling time out:

IO_Major_Status will contain SCSI_CHECK_CONDITION

Upon a SCSI Select Time Out:

IO_Major_Status will contain SCSI_SELECTTO

Assumptions:  That the byte counts pointed to be sg_entry add up to the

blk_count * the Block Size (i.e. the number of bytes requested

from the target = the number of bytes to be moved by the part)

If this is not the case then the IO will die a miserable death

Restrictions: This function is designed to do read or write type io's to

SCSI block devices only.  The author does not gaurantee it's

functionality if any other type of scsi command is used.

Other functions called: IORead8/32 and IOWrite32 to Read and Write chip registers

IOWriteDSP to start Scripts engine

getPhysAddr to patch TI entries for IO

HandleDataInPM to clean up after a Data In phase mismatch

HandleDataOutPM to clean up after a Data Out phase mismatch

Fill_SG_Entries to set up scatter gather entries for

the io

PollISTAT to look for interrupts during the IO

Fail_SCSI to clean up on any failing condition.

Calling Example:

rw.sel_info = maint.sel_info;

rw.id_count =3;

rw.id_buf[0] = 0xC0;

rw.id_buf[1] = 0x20;

rw.id_buf[2] = 0x35;

rw.sg_entry = sg_list;

rw.sg_count = 10;

rw.blk_count = 32;

rw.lba = 0;

GenRandSG(test3, rw.blk_count*512, sg_list, rw.sg_count);

printf("Write10, LBA: %lx, #SG: %d, #Blk: %d\n", rw.lba, rw.sg_count, rw.blk_count);

rw.cmd_count = 0x0000000A;

/* set up the command */

rw->cmd_buf[0] = 0x28;

rw->cmd_buf[1] = 0x00;

rw->cmd_buf[2] = (VUBYTE) ((rw.lba >> 24) & 0xFF);

rw->cmd_buf[3] = (VUBYTE) ((rw.lba >> 16) & 0xFF);

rw->cmd_buf[4] = (VUBYTE) ((rw.lba >> 8) & 0xFF);

rw->cmd_buf[5] = (VUBYTE) rw.lba;

rw->cmd_buf[6] = 0x00;

rw->cmd_buf[7] = (VUBYTE) (rw.blk_count >> 8);

rw->cmd_buf[8] = (VUBYTE) rw.blk_count;

rw->cmd_buf[9] = 0x00;

return(do_rw(PCIDevice, rw));

### Function: Fill_SG_Entries

Prototype: void Fill_SG_Entries(pcidev_record *PCIDevice, scsirw *rw, VINT *NextSGEntry)

Purpose : To set up Scatter Gather entries in the RW script.  The

Script can handle up to MAX_SG_ENTRIES scatter gather entries

before interrupting.  This routine is called initially to set

up up to 16 entries in the Script and is called again

for each 16 entries there are from there on.

Input:  A pointer the pcidev_record and scsirw structures.  And

a pointer to an integer indicating the next Scatter Gather

entry to be set up.

Output: The rw_buffer_table scatter gather entries are initialized for

the current set of scatter gather entries.

NextSGEntry is set to the next SG entry number that is not set up

if more than 16 entries were left to be serviced.

It is set to -1 if no more entries are to be serviced.

Assumptions:  None

Restrictions: None

Other functions called: RMWon/off to set or clear the bit in the SBR

register that the script checks to determine

if there are more scatter gather entries to

be serviced.

Calling Example:

/* do the initial set up of the Scatter Gather entries */

NextSGEntry = 0;

Fill_SG_Entries(PCIDevice, rw, &NextSGEntry);

### Function: HandleDataInPM

Prototype: static void HandleDataInPM(pcidev_record *PCIDevice, VINT *Current_SG_Entry)

Purpose : To handle clean up after a Phase Mismatch during Data In phase

Input:  A pointer the pcidev_record and a pointer to a variable which

will indicate the Scatter Gather entry that was executing when

the PM occured, this is needed by the do_re function if there

was a byte in the SWIDE register.

Output: Current_SG_Entry is filled in with the SG entry that was being

serviced.

Assumptions:  That a phase mismatch has actully occured during data in.

Restrictions: None

Other functions called: IORead32 to read chip info

iowriteDSP to start the script

Calling Example:

HandleDataInPM(PCIDevice, &Current_SG_Entry);

### Function: HandleDataOutPM

Prototype: static void HandleDataOutPM(pcidev_record *PCIDevice)

Purpose : To handle clean up after a Phase Mismatch during Data Out phase

Input:  A pointer the pcidev_record.

Output: None

Assumptions:  That a phase mismatch has actully occured during data out.

Restrictions: None

Other functions called: IORead32/8 to read chip info

RMWon to set bits in chip regiters

iowriteDSP to start the script

Calling Example:

HandleDataOutPM(PCIDevice);

### Function: Fail_SCSI

Prototype: VINT Fail_SCSI(pcidev_record *PCIDevice, VUBYTE istat, VUBYTE dstat,

VUINT sist, VULONG vector, VINT fail_type)

Purpose : To fail a SCSI io and attempt to clear the scsi bus to a bus

free condition

Input:  A pointer to the pcidev_record data structure.  The interrupt

registers so that they can be printed out.  The failure type

which is one of the following:

SCSI_ILLEGAL_INT - an illegal interrupting condition

SCSI_POLLTO - a interrupt polling timeout has occured

Output: Fail_SCSI returns SCSI_COMMANDFAILED if the chip is not

connected to the SCSI bus upon exit of the routine

Fail_SCSI returns SCSI_CON_COMMANDFAILED if the chip is

connected to the SCSI bus upon exit of the routine.

Assumptions: Scripts are not currently running at the time

Restrictions: This routine should only be called in catastophic failure

conditions.

Other functions called:  BitBucket to attempt to clear the scsi bus to

bus free.

IORead8 to check the connected status of the chip

printf to print the error and interrupt regs

Calling Example:

return(Fail_SCSI(PCIDevice, istat, dstat, sist, vector, SCSI_ILLEGAL_INT));

### Function: Bitbucket

Prototype: VINT Bitbucket(pcidev_record *PCIDevice)

Purpose : To clear any addtional bytes from the SCSI bus and a

failing io situation.

Input: A pointer to the pcidev_record structure.

Output:  If it is possible to clear the scsi bus to a bus free

condition then SCSI_COMMANDFAILED is returned.

If it is not possible to clear the scsi bus to a bus free

condition then SCSI_CON_COMMANDFAILED is returned.

Assumptions: That scripts are not currently running

Restrictions: This function can only attempt to clear the scsi bus

if it is currently in a scsi input phase (i.e. STATUS,

MSG_IN, or DATA_IN) if it is not in one of those phases

then this routine cannot attempt to clear the bus.

Other functions called:  RMWon to clear the fifos

IORead8 to check connected status

PollISTAT to look for an interrupt from the chip

Calling Example:

/* attempt to clear the scsi bus to bus free */

return(Bitbucket(PCIDevice));

## FILE: VALIDATE.C

### Function: main

Prototype: void main(VINT argc, VCHAR **argv)

Purpose : To initialize devices and standard data structures.  It

also calls the parsecmdline function to parse the command

line options and lastly calls the test function to execute

the specific test implemented.

Input: Argc and argv which contain the command line parameters

Output: Nothing

Assumptions: none

Restrictions: none

Other functions called: strncpy to copy the default file name to a string

WriteLog to output various device information

sprintf to format strings for WriteLog

fprintf to output things only to the screen

exit to exit the program on errors

The following initialization routines are called in this order:

parsecmdline to parse command line parameters

pci_check_pci_bios to ensure the existance of

a pci bios

C8XX_BuildDeviceList to build a list of C8XX

devices in the system

C8XX_ChoosePCIDevice to allow the user to

select a specific pci device to be

used for testing

C8XXGetChipType to get chip specific info

C8XX_GetIOBase to get the IO Base address of

a 53C8XX device

C8XX_GetRAMBase to get the Script RAM Base

Address of a 53C8XX device

PCI_print_config to output the config registers

of a specific device.

init_8XX to initialize a 53C8XX device to a

known state

test to execute the programs specific chip test