# AVC Part1 - Core Services for Routing and Quality of Service Management

## AVC Routing and quality of service management - Introduction

Creation of a real-time conference will require communication of real-time media Streams between disparate nodes. From a real-time application’s perspective a logical connection between the media endpoints of those nodes must be set up. This logical connection is termed a Route. A Route has one source endpoint and may have one or more destination endpoints. The application requesting a Route need have no knowledge of intermediate physical routing or stream processing to connect the media endpoints.

The AVC Routing and Quality of Service Management (ARQM) provides a service for Applications to establish these Routes. When requesting for a Route to be created, the application will specify not only the source and destination nodes, but a Quality of Service (QoS) associated with that Route. The QoS will be relevant to the media type and will be defined in a generic, network independent manner. For example, QoS associated with a Route intended for a video stream will have associated with it elements such as frame rate, resolution, delay and jitter; however it does not require definition of specific algorithms such as H.261 or network dependent elements such as H0 channels. This method of QoS definition allows the Applications to be network independent. ARQM does not preclude applications which have knowledge of the network and node characteristics from specifying explicit QoS related parameters  (e.g. H.261, CIF, 15 frames/second).

The topology used to achieve the Route may involve media processing elements such as transcoders, mixers and switches. The inclusion of such devices will depend on the requested QoS and the capabilities of the conference nodes. The Route request may also include processing or routing  preferences.

ARQM utilises the services of RTLM to obtain information relating to Route establishment. Having established any necessary processing elements, ARQM will use the services of RTLM to establish the physical routing between those nodes.

In the provision  of the AVC Routing Services, ARQM will need to arbitrate different requests for the media Routes. Such arbitration is necessary since in many cases the network and node capabilities will not support all Routes QoS’s or all Routes being present at the same time. Also to be considered are conference requirements such as the priorities of the various media streams or constraints such as cost.  To allow for this, AVC includes parameters for input into the QoS arbitration process, however the precise algorithm for arbitrating the QoS is not the subject of standardisation. It should be noted that the arbitration considers not only requests from the application, but also dynamic changes of the network. Thus depending on the network type and the conference operation, the QoS arbitration will involve dynamic adjustment.  Finally, establishment and management may take place in either a centralised or distributed manner.

With regard to bandwidth allocated to T.120 data, ARQM is not responsible for its creation or deletion, however ARQM has control over the T.120 bandwidth once established. Particularly where the T.120 bandwidth is multiplexed with real-time media, then management of the  T.120 bandwidth partition is a necessary part of the overall conference QoS arbitration. Where change to the T.120 partition is required, ARQM directly accesses the Network Specific Mapping (T.NSM) to control the network multiplex. Utilisation of bandwidth within the T.120 partition is out of  the scope of ARQM.

### Terminology

ARQM relies on the nomenclature of Routes and Pipelines as described in T.ARCH (the top level document for Real-time Audio-Visual Extensions to T.120). The reader should refer to that document for details.

### ARQM  Operation

Prior to a conference, a desired 'start up mode'  (ie. the audio, video and data settings) may be specified to ARQM via local access which could involve local communication from a Reservation application. The ‘start-up mode’ essentially comprises the required interconnects of media between endpoint nodes and their associated QoS. Additionally processing elements for streams may also be specified.

ARQM will attempt to establish  the 'start up mode' though this will  also be dependent on the  capabilities of the conference nodes and links. These capabilities will be obtained from the RTLM database.

Subsequent to initial establishment, should the equilibrium of the conference change either due to requests for new or modified Routes from the real-time applications, or the network service change (with ARQM being informed by RTLM), then ARQM will be responsible for further arbitration to establish the new equilibrium.

ARQM entities will communicate to each other via an MCS channel dedicated for ARQM. This will enable information such as requests for Routes, Route responses, information pertaining to nodes and ARQM  commands to be conveyed.

Having arbitrated various requests, (and hence a mode of operation for the conference), the realising of the Route(s) and associated QoS will be performed by utilising T.RTLM Pipeline services.

The model of ARQM is built on the concept of allowing the real-time applications to be network independent thus allowing establishment and control of conferences across multiple network types. Key to this model is the generic way that QoS is defined. As well as QoS, guidelines are required for the ARQM arbitrator. Such guidelines include ie. Priority (ie. ranking)  of one Route against another.

Examples of how real-time applications utilise AVC Routing services are provided in Appendix A.

### ARQM control of arbitration mechanisms

In general, the arbitration of the mode of the conference by ARQM will depend on:

- Capabilities of all nodes and their associated network links

- Requests for media Routes and the (range of) QoS acceptable for that Route

- The relative Priorities of Routes

- Whether the conference is conducted

When a Route is created, part of the QoS specification includes the minimum, preferred and maximum acceptable values of  QoS for that Route. If there are several Routes, then the Priority determines which Route will be the first removed or reinstated. The Route Priorities may be set by a Conference Convenor, and may take into account factors such as cost, the type of media to be deployed and the desired user characteristics of the conference -though these aspects not subject to standardisation.

The arbitration of Route requests by ARQM will take into account  both the  requested Route QoSs and the Route Priorities. The precise  algorithm used for arbitration is however not within the scope of T.AVC.

#### Non-conducted Conference

During a non-conducted conference, request for a change in the Route Priority is not permitted by anything other than the conference Convenor.

#### Conducted Conference

In the conducted conference, then the Conductor will also have the ability to set a Route’s Priority. In addition, permission for a Route request must be obtained from the Conductor. The Conductor may make any Route request he desires.

### Utilisation of RTLM Pipeline service

ARQM will use the following RTLM Pipeline primitives in establishing and maintaining media Routes:

- RTLM-Create-Pipeline

- RTLM-Destroy-Pipeline

- RTLM-Pipeline-Add-Node

- RTLM-Pipeline-Remove-Node

- RTLM-Pipeline-Pipeline-Information

- RTLM-Pipeline-Routing-Options

### Utilisation of RTLM ROSTER Primitives

ARQM utilises RTLM’s database. As well as information pertaining to Link and node capabilities, ARQM will also be able to create entries pertaining to the Routes that it creates.

The information held in the database related to the Route includes its QoS, any processing elements required and the Pipelines chosen to achieve the Route.  Each Route also as associated with it Priority information allowing it to be ranked against other Routes.

The following RTLM primitives are used by ARQM for Roster access:

- RTLM-Roster-Register-Route

- RTLM-Roster-Delete-Route

- RTLM-Roster-Monitor

- RTLM-Retrieve-Entry

### Usage of MCS channels

One MCS channel is utilised:

- ARQMChannel

The ARQM channel is used for both issuing of requests for desired Routes as well as for a ARQM to instruct nodes as to the permitted mode to operate in.

## Route QOS

When creating or modifying a Route, the QoS of that Route is specified in  a generic form.  The purpose of generic AVC QoS parameters is to allow specification of the target 'form factor' of the various media types applicable to each AVC Routing. Note that this is different to absolute 'quality' which is difficult to define since this is (a) subjective and (b) open to implementation (eg. different implementations of H.261 at CIF resolution will not show the same results).

Generic  (and expandable) definition of parameters for QoS, enable the real time applications to be network independent. It is  ARQM’s responsibility  to arbitrate between different requests, and then to effect the resultant Routes via distributed ARQM entities (ultimately utilising T.RTLM). Note that in this process, ARQM  would translate the requests to something which was appropriate to the networks involved. For example a request for Audio QoS 7KHz Wideband from ARQM may be translated to a  request for G.722 before being passed to T.RTLM. The precise mapping chosen by an implementation of ARQM is outside the scope of ARQM.

Section 1.2.1 addresses how ranges of QoS may be specified, whilst Section 1.2.2 describes how the QoS parameters for each media type are defined.

### QoS Ranges

Each Route QoS requires the specification of 3 associated values:

Maximum QoS (MaxQOS), Preferred QoS (PrefQOS), Minimum QoS (MinQOS)

#### Maximum QoS

This specifies the maximum QoS for the particular Route, and is defined in terms of the generic ‘form factor’  parameters defined later in this section.

This parameter thus identifies those media Routes which may be given an increased QoS and the maximum which should be permitted. Its value must be set equal to or greater than the Preferred QoS value.

#### Preferred QoS

This specifies the desired QoS for the particular Route, and is defined in terms of the generic ‘form factor’  parameters. For Routes where the QoS may be varied up or down, then this value represents  the target average QoS.

#### Minimum QoS

This information may either, be defined as ‘0’ or be defined in terms of the generic ‘form factor’  parameters, where the QoS is less than PrefQOS or where MinQOS = PrefQOS

The implications of the Preferred and Minimum values are as follows:

| Minimum QoS Value | Description |
| --- | --- |
| 0 < MinQOS < PrefQOS | If a Route is permitted to have its QoS degraded, this field specifies the minimum QoS for that particular Route. |
| MinQOS = PrefQOS | There is no minimum acceptable value, the QoS of the Route must therefore not be modified below the Preferred QoS |
| MinQOS = 0 | The QoS of this Route may be degraded to 0, in other words, it may be removed if it is necessary to provide greater bandwidth for other media. |

### Media QoS Parameters

Definition of QoS for each media uses the parameters defined in Sections 1.2.2.1-1.2.2.3, whilst definitions are covered in Section 1.2.2.4.

#### Audio QoS

- On/off

- Quality

- Lossless

- Transparent ie. with transparent loss

- 7KHz Wideband

- PCM Narrowband

- Sub-PCM

- Artificially generated - Vocoder quality

- Projection

- Mono

- Stereo

- Delay

- ms

- Jitter

- ms

- LevelOfGuarantee

- Guaranteed

- Predictive

- Best Effort

- Route Synchronisation

- Associated Route ID + relative delay(ms)

#### Video QoS

- On/off

- Resolution 			(pixels)

- Frame rate			(frames/sec)

- Projection 			(2D, 3D, VR)

- Colour mode			Monochrome/Colour

- Delay			ms

- Jitter				ms

- LevelOfGuarantee 		Guaranteed, Predictive or Best Effort

- Route Synchronisation		Associated Route ID + relative delay(ms)

#### Data QoS

The Data QoS is applicable to Real Time Data Routes:

- TokenRate 			Bytes/sec

- TokenBucketSize 		Bytes

- PeakBandwidth 		Bytes/sec

- Latency 			Microseconds

- DelayVariation 		Microseconds

- LevelOfGuarantee 		Guaranteed, Predictive or Best Effort

- Route Synchronisation 		Associated Route ID + relative delay(ms)

#### Definitions

LevelOfGuarantee	This is the level of service being negotiated for.  Three levels of service are defined: Guaranteed, Predictive, and Best Effort.

The reason for defining both predictive and guaranteed service is that predictive services may achieve substantially better performance given the same level of network resource usage, while guaranteed service provides the mathematical level of certainty needed by selected applications. Specific providers may implement none, one, or both of these services.

Best effort service should be always supported.

GuaranteedService	A service provider supporting guaranteed service implements a queuing algorithm which isolates the flow from the effects of other flows as much as possible, and guarantees the flow the ability to propagate data at least TokenRate for the duration of the connection. If the sender sends faster than that rate, the network may delay or discard the excess traffic. If the sender does not exceed TokenRate over time, then latency is also guaranteed. This service is designed for applications which require a precisely known quality of service but would not benefit from better service, such as real-time control systems.

PredictiveService	A service provider supporting predictive service guarantees the flow the ability to propagate data at least TokenRate for the duration of the connection. If the sender sends faster than that rate, the network may delay or discard the excess traffic. The delay limit is not guaranteed (occasional packets may take longer than specified), but is generally highly reliable. This service is designed for applications that can accommodate or adapt to some variation in service quality, such as video service.

BestEffortService	A service provider supporting best effort service, at minimum, takes the flow specification as a guideline and makes reasonable efforts to maintain the level of service requested, however without making any guarantees whatsoever.

TokenRate/TokenBucketSize	A Token bucket model is used to specify the rate at which permission to send traffic accrues. The value -1 in these variables indicates that no rate limiting is in force.  The TokenRate is expressed in bits per second, and the TokenBucketSize in octets.

The concept of the token bucket is a bucket which has a maximum volume (token size) and continuously fills at certain rate (token rate). If the “bucket” contains sufficient credit, the application may send data; if it does, it reduces the available credit by that amount. If sufficient credits are not available, the application must wait or discard the extra traffic.

If an application has been sending at a low rate for a period of time, it clearly may send a large burst of data all at once until it runs out of credit.  Having done so, it must limit itself to sending at TokenRate until its data burst is exhausted.  Equally clearly, if it transmits at a rate below TokenRate for a period of time, it accrues the right to send a burst of data.

In video applications, the TokenRate is typically the average bit rate peak to peak, and the TokenBucketSize is the largest typical frame size. In constant rate applications, the TokenRate is equal to the PeakBandwidth, and the TokenBucketSize is chosen to accommodate small variations.

PeakBandwidth	This field, expressed in bytes/second, limits how fast bits may be sent back to back from the application.  Some intermediate systems can take advantage of this information resulting in a more efficient resource allocation.

Latency	Latency is the maximum acceptable delay between transmission of a bit by the sender and its receipt by the intended receiver(s), expressed in microseconds. The precise interpretation of this number depends on the level of guarantee specified in the QoS request.

DelayVariation	This field is the difference, in microseconds, between the maximum and minimum possible delay that a packet will experience.  This value is used by applications to determine the amount of buffer space needed at the receiving side in order to restore the original data transmission pattern.

Route Synchronisation	This parameter specifies the relative delay permissible between this Route and a single other Route. (Note that the ‘association’ of Routes is defined via T.RTLM. This parameter just specifies the degree of delay of those Routes.)

## Route Priority

As part of ARQM’s arbitration of Routes requests, the relative of importance of the Routes needs to be defined. This information is  particularly important where the bandwidth is limited and where the different media types are multiplexed within the same network link.

Priority is defined as a number in the range 0-65535 and is meaningful across the media types (ie. a video Route can be ranked against a data Route). 0 is the highest priority and 65535 the lowest. Each real-time Application may request the relative priority of Routes which it sources, however across the conference, the relative Priority of the Routes may only be set by the conference Convenor or via a conducting node. Additionally,  the Convenor may override the request for local priority.

The effect of prioritisation is not to explicitly set the conference QoS, but to provide guidance for ARQM when servicing the Route requests from real-time applications.

In any situation, if there is a need to remove a particular media Route or to reduce its QoS, then this Priority value provides guidance as to which Route should be first removed or degraded. Similarly, when bandwidth becomes available, this Priority provides guidance as to which Route should first be reinstated or upgraded.

When bandwidth becomes available, the general order of QoS reinstatement and upgrade will be as follows:

- Reinstate all media Routes to their Preferred QoS,

- Upgrade Routes from their Preferred QoS to Maximum QoS  starting with the highest priority Routes

Note that Priorities can only be assigned prior to a conference if the conference configuration i.e. participants and all nodes involved are pre-allocated e.g. via a Reservation System. Otherwise, Routes will be created dynamically. In this instance, if required, it is likely that the Conference Convenor  or Conductor would gain access to the Route information as the conference is established, and at that point allocate Priority settings to the Routes in order to gain control over the mode of the conference eg. whether the conference is ‘data centric’ or ‘video centric’.  During a  conference,  a request to ARQM for a change to the Route Priority is only valid when the request is made by the convenor or a conducting node.

Examples of usage and effect of media priority are given in Appendix B.

## AVC Routing Service Definition

AVC Routing service  allows nodes to request media Routes between specified media endpoints. Routes are unidirectional and reliance is made on transmitters to initiate the Route request. The AVC Routing service supports the following primitives:

- AVC-Route-Create

- AVC-Route-Delete

- AVC-Route-Modify

- AVC-Route-List

- AVC-Route-Query

- AVC-Route-Monitor

- AVC-T120BW-Modify

### AVC-Route-Create

The AVC-Route-Create primitive allows a node to request a Route and specify the QoS requirements for the Route.  A Route is created by the node that intends to source a media stream on the channel.

An application wishing to create a new Route, issues an AVC-Route-Create request to its local AVC Provider which assesses the request to determine if it can be accommodated locally. If the QoS requirements cannot be accommodated then the request may be rejected immediately.   Otherwise, there is a need to communicate with other AVC Providers to determine whether the request can be fulfilled, and if so, to indicate to the requesting application the level of QoS that can be offered.

The action taken by the AVC Provider is determined by the mode of operation selected for the conference.  If the conference is configured for centralised management, the AVC Provider issues an AVC-Route-Create request to the Top AVC Provider.  The Top AVC Provider then processes the request which will involve mapping the Routes to T.RTLM Pipelines.  Requests to T.RTLM will be made for these Pipelines. The response includes the number of nodes which have not achieved the ‘preferred’ QoS (since depending on the Route requirements it may not be necessary to have all parts of the Route with the same QoS). This allows the Application to find out the explicit QoS possible for all nodes on the Route via the AVC-Route-Query primitive. On receipt of the AVC-Route-Create response, the local AVC Provider forwards an AVC-Route-Create confirm to the requesting application.  If the result is positive, streams may be submitted to the Route until an AVC-Route-Delete indication is received. A Route Create indication is sent to nodes specified as sinks.

If the conference is configured for distributed management, the local AVC Provider forwards the AVC-Route- Create request to the AVC Provider at the nearest MCU node. Typically, this would be the immediately superior node within the T.120 hierarchy.  If this node does not have MCU functionality, the request is passed up the tree until it reaches an MCU node.  All MCUs are viewed together as a single entity known as the ‘Virtual MCU’.

All nodes within the ‘Virtual MCU’ have a local copy of the capabilities of nodes , which enables them to determine independently (i.e. without consulting other nodes) whether a Route create request can be fulfilled.  The AVC Provider may consult with peer AVC Providers before reaching a decision.  If the request is accepted, the AVC Provider shall ensure that all necessary Pipelines have been configured to support the new Route before responding to the AVC Provider that issued the AVC-Create-Route request.  This ensures that the new Route is available for use as soon as the requesting application receives a positive AVC-Create-Route confirm from its local AVC Provider.  All AVC Providers within the Virtual MCU are notified of the successful creation of a new Route as are all AVC Providers at nodes that are part of the  new Route.

Note that since the QoS of each Route includes the minimum and maximum acceptable level, it is possible that different media endpoints may be permitted to have different QoS’s within the constraints set for each Route. If the same level of QoS is required at each media endpoint then ‘Uniform’ Route qualifier  should be specified.

The Route Create request may optionally include specification of type of network elements (eg. switch, mixer, transcoder) permitted or desired in the Route. If known, the Route request may also specify explicitly network elements nodes to be included in the Route. This could be useful, for example, where a knowledgeable application is aware of routing issues, or desires the characteristics of a particular network element.

Note that only the Route Creator or the Conference Conductor may subsequently delete this Route.

Figure 8.1  AVC-Route-Create sequence of primitives - Centralised/Distributed cases

AVC-Route-Create Primitive

| Parameter | Request | Indication | Confirm |
| --- | --- | --- | --- |
| Request Handle | M |  | M(=RQ) |
| Route Identifier |  | M(=CF) | M |
| Route Endpoints | M | M(=CF) | M |
| Media Type | O | O(=) |  |
| Route QoS | M | M(=CF) | C |
| Route QoS Uniformity | O | M(=CF) | M |
| Local Route Priority | O | O(=CF) | C |
| Network element list | O | O(=CF) | C |
| Associated Routes | O | O(=) | C |
| Result |  |  | M |

Request Handle: Locally unique number to allow the requesting node to match responses.

Route Identifier: A globally unique number identifying the Route to which the primitive refers.  Constructed by the AVC Provider at the requesting node from the node’s GCC User ID and a locally unique number. A text description is also included.

Route Endpoints: A list of media endpoints that will act as sinks for the Route.

Media Type: audio, video, real time data

Route QoS:  Desired Quality of Service is included in the request. Values for the Minimum, Maximum as well as the Preferred QoS must be provided. This is specified using the generic QoS parameters, though optionally, explicit values may also be included. The actual QoS offered by the Virtual MCU is provided in the confirm.

Route QoS Uniformity: This indicates whether the QoS must be uniform to all nodes on the Route. Absence of this parameter allows the Route QoS to be different at various nodes, provided it is within the limits of QoS requested.

Local Route Priority: This specifies the relative Priority of this Route against others sourced by this node. The achieved relative priority of the local Routes is returned in the confirm. Note, this is not the same as the conference wide priority.

Network Element List: This comprises a list of types of elements which are acceptable or required in the Route (this may include no processing elements). Optionally, explicit processing nodes may be specified. Absence of this parameter indicates that the requesting application does not care what or which elements are in the Route.

Associated Routes: A list of Routes with which the new Route is to be associated.  The list identifies any Routes to which the new Route is to be synchronised, and specifies the maximum relative latency between the Routes.  (This parameter should specify a descriptor to apply meaning to the association e.g. two audio Routes used to distribute a stereo signal, a video Route & its associated audio Route(s).)

Result: Indicates whether the request was accepted or rejected, and if rejected, the reason why. It contains one of the following possible results: successful, invalid nodes, QoS not attainable, media not supported by nodes, processing elements not available.

### AVC-Route-Delete

The AVC-Route-Delete primitive is used to remove a Route that was previously created using the AVC-Route-Create request.  All records of the Route are removed. and all nodes were identified with media endpoint sinks are notified. If the source of a Route leaves the conference, the Route ceases to exist.  In this case the Top AVC Provider issues an AVC-Route-Delete indication to all members of the Route.

Only the creator of the Route or Conference Conductor or AVC is permitted to delete it.

Figure 8.2 AVC-Route-Delete sequence of primitives - Creator/conductor initiated

Figure 8.3  AVC-Route-Delete sequence of primitives -AVC Provider/Convener initiated

AVC-Route-Delete Primitive

| Parameter | Request | Indication | Confirm |
| --- | --- | --- | --- |
| Request Handle | M |  | M(=RQ) |
| Route Identifier | M | M(=) | M(=) |
| Result |  |  | M |
| Reason |  | M | M |

Request Handle: Locally unique number to allow the requesting node to match responses.

Route Identifier: A globally unique number identifying the Route to which the primitive refers.  Constructed by the AVC Provider at the requesting node from the node’s GCC User ID and a locally unique number. A text description is also included.

Result: Indicates successful, invalid Route, not Route creator/conductor/convenor

Reason: user initiated, Route purged.

### AVC-Route-Modify

This primitive is used to change any aspect of the Route. This includes the Route QoS and the associated media endpoints which sink the media stream. A change in the Route source media endpoint is not permitted as this constitutes a new Route.

Figure 8.4 AVC-Route-Modify sequence of primitives Centralised/Distributed management cases

AVC-Route-Modify Primitive

| Parameter | Request | Indication | Confirm |
| --- | --- | --- | --- |
| Request Handle | M |  | M(=RQ) |
| Route Identifier |  | M(=CF) | M |
| Route Endpoints | M | M(=CF) | M |
| Media Type | O | O(=) |  |
| Route QoS | M | M(=CF) | C |
| Route QoS Uniformity | O | M(=CF) | M |
| Local Route Priority | O | O(=CF) | C |
| Network element list | O | O(=CF) | C |
| Associated Routes | O | O(=) | C |
| Result |  |  | M |

Request Handle: Locally unique number to allow the requesting node to match responses.

Route Identifier: A globally unique number identifying the Route to which the primitive refers.  Constructed by the AVC Provider at the requesting node from the node’s GCC User ID and a locally unique number. A text description is also included.

Route Endpoints: A list of media endpoints that will act as sinks for the Route.

Media Type: audio, video, real time data

Route QoS:  Desired Quality of Service is included in the request. Values for the Minimum, Maximum as well as the Preferred QoS must be provided. This is specified using the generic QoS parameters, though optionally, explicit values may also be included. The actual QoS offered by the Virtual MCU is provided in the confirm.

Route QoS Uniformity: This indicates whether the QoS must be uniform to all nodes on the Route. Absence of this parameter allows the Route QoS to be different at various nodes, provided it is within the limits of QoS requested.

Local Route Priority: This specifies the relative Priority of this Route against others sourced by this node. The achieved relative priority of the local Routes is returned in the confirm. Note, this is not the same as the conference wide priority.

Network Element List: This comprises a list of types of elements which are acceptable or required in the Route (this may include no processing elements). Optionally, explicit processing nodes may be specified. Absence of this parameter indicates that the requesting application does not care what or which elements are in the Route.

Associated Routes: A list of Routes with which the new Route is to be associated.  The list identifies any Routes to which the new Route is to be synchronised, and specifies the maximum relative latency between the Routes.  (This parameter should specify a descriptor to apply meaning to the association e.g. two audio Routes used to distribute a stereo signal, a video Route & its associated audio Route(s).)

Result: Indicates whether the request was accepted or rejected, and if rejected, the reason why. It contains one of the following possible results: successful, invalid nodes, QoS not attainable, media not supported by nodes, processing elements not available.

### AVC-Route-List

This primitive allows an application to find out what Routes are established.

Figure 8.5  AVC-Route-List sequence of primitives

AVC-Route-List  Primitive

| Parameter | Request | Indication | Confirm |
| --- | --- | --- | --- |
| Request Handle | M |  | M(=RQ) |
| Route ID(s) |  |  | M |
| Result |  |  | M |

Request Handle: Locally unique number to allow the requesting node to match responses.

Route  Identifier(s): This is a list of the Routes present returned in the Confirm.

Result: Indicates whether the request was accepted or rejected, and if rejected, the reason why. It contains one of the following possible results: successful, failure.

### AVC-Route-Query

This primitive allows an application to obtain detailed information about particular Routes. The information on a particular Route will  comprise the nodes on that Route (source and sink(s) and network elements) and the QoS of the Pipelines associated with those nodes. The Priority of Route is also revealed.

Figure 8.6  AVC-Route-Query  sequence of primitives

AVC-Route-Query Primitive

| Parameter | Request | Indication | Confirm |
| --- | --- | --- | --- |
| Request Handle | M |  | M(=RQ) |
| Route ID | M |  | M(=RQ) |
| Route Details |  |  | M |
| Result |  |  | M |

Request Handle: Locally unique number to allow the requesting node to match responses.

Route  Identifier(s): This is a list of the Routes for which details are required. AVC-T120 is a special identifier to obtain details of the T.120 bandwidth.

Route Details:  This is a list of the details of requested Routes Each entry in this list contains the Route ID, source/destination nodes, Min QoS, Max. QoS, Preferred QoS, achieved QoS for each end Node, Route Priority. Processing elements in the Route are also revealed. If the Route ID is the special ID of AVC-T120, then the Route details will return the bandwidth allocated to T.120 data.

Result: Indicates whether the request was accepted or rejected, and if rejected, the reason why. It contains one of the following possible results: successful, Invalid Routes with list of Routes which were invalid

### AVC-Route-Monitor

This primitive allows an application to monitor changes to (a) an individual Route (b) a selection of Routes or (c) all Routes. If registered for Route monitoring, the requester will receive an indication  when ever the relevant Routes change. This may be as a result of a request for a Route change or because the Route has been modified as a result of changes to another Route.

Figure 8.7 AVC-Route-Monitor sequence of primitives

AVC-Route-Monitor Primitive

| Parameter | Request | Indication | Confirm |
| --- | --- | --- | --- |
| Request Handle | M |  | M(=RQ) |
| List of Route(s) | M |  | M(=RQ) |
| Route Details |  | M |  |
| Result |  |  | M |

Request Handle: Locally unique number to allow the requesting node to match responses.

Route  Identifier List: This is a list of the Routes which should be monitored.

Route Details:  This is a list of the details of requested Routes Each entry in this list contains the Route ID, source/destination nodes, Min QoS, Max. QoS, Preferred QoS, achieved QoS for each end Node, Route Priority. Processing elements in the Route are also revealed. If the Route ID is the special ID of AVC-T120, then the Route details will return the bandwidth allocated to T.120 data

Result: Indicates whether the request was accepted or rejected, and if rejected, the reason why. It contains one of the following possible results: successful, Invalid Routes with list of Routes which were invalid

### AVC-T120BW-modify

This primitive allows applications (typically a node controller) to modify the bandwidth available to T.120 data. ARQM needs to be aware of and control of any changes in the overall T.120 bandwidth since this may have an effect on the QoS of real-time Routes. Any nodes on Routes affected and any nodes monitoring the Routes will be made aware via AVC-Route-Modify and AVC-Route-Monitor indications.

Figure 8.8  AVC-T120BW-Modify sequence of primitives

AVC- T120BW -Modify Primitive

| Parameter | Request | Indication | Confirm |
| --- | --- | --- | --- |
| Request Handle | M |  | M(=RQ) |
| T.120 Bandwidth | M |  | M |
| Link ID(s) | O | O(=RQ) | O(=RQ) |
| Result |  |  | M |

Request Handle: Locally unique number to allow the requesting node to match responses.

T.120 Bandwidth: This specifies the bandwidth required for T.120 data. The Confirm contains the achieved bandwidth.

Link ID(s): Optionally, specific Link ID(s) may be defined for which the bandwidth is to be applied. If omitted then the bandwidth will be applied to all end-point nodes.

Result: Indicates whether the request was accepted or rejected, and if rejected, the reason why. It contains one of the following possible results: successful, Bandwidth not attainable,