| Question: 10/8 INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- |
| TELECOMMUNICATION STANDARDISATION SECTOR STUDY PERIOD 1993 - 1996 | COM 8-__ February 1996 Original: English 31 July, 2026 |

Geneva 5th - 16th February 1996

STUDY GROUP 8 – CONTRIBUTION

| SOURCE: | BT |
| --- | --- |

| CONTACT: | Trevor Peers, BT Labs, Martlesham Heath, IPSWICH, Suffolk, UK IP5 7RE. | Tel: +44 1473 647697 Fax: +44 1473 643791 E-Mail: peers_t_w@bt-web.bt.co.uk |  |
| --- | --- | --- | --- |
| TITLE: | Draft recommendation T.132 - Real Time Link Management |  |  |

___________________

REAL TIME LINK MANAGEMENT

Draft Recommendation T.132

# Summary

This recommendation defines a service to manage the configuration of network connections used to distribute real time streams in a conferencing environment  It provides mechanisms to manipulate communication channels to deliver such streams across heterogeneous networks at a specified Quality of Service.   The service is provided in a manner that is independent of the underlying networks.

Draft Recommendation T.132

REAL TIME LINK MANAGEMENT

CONTENTS

Summary

1. Scope

2. Normative References

3. Definitions

4. Abbreviations

5. Conventions

6. Real Time Stream Control Architecture

6.1. RTLM System Model for a Conference Node

6.2. RTLM Communication Channels

6.2.1. RTLM Broadcast Channel

6.2.2. MCU Control Channel

6.2.3. MCU Communication Channel

7. T.132 Modes of Operation

8. Creation and Termination of an RTLM Session

9. Real Time Link Negotiation

9.1. Arbitration of Real Time Link Capabilities

9.1.1. Link Capabilities

9.1.2. Node Capabilities

9.1.3. Capability Table

9.1.4. Capability Descriptors

9.2. Changing Real Time Link Capabilities

9.3. Setting Real Time Link Configuration

10. Pipeline Management

10.1. RTLM-Create-Pipeline

10.2. RTLM-Destroy-Pipeline

10.3. RTLM-Pipeline-Add-Sink-Node

10.4. RTLM-Pipeline-Remove-Sink-Node

10.5. RTLM-Pipeline-Modify

10.6. Requesting a Stream

10.7. RTLM-Pipeline-Information

10.8. RTLM-Pipeline-Routing-Options

11. RTLM Database

11.1. Routing Roster

11.1.1. RTLM-Roster-Monitor

11.1.2. RTLM-Roster-Retrieve-Entry

11.1.3. RTLM-Roster-Register-Path

11.1.4. RTLM-Roster-Delete-Path

11.2. Capability Roster

11.2.1. RTLM-Retrieve-Node-Capability

11.2.2. RTLM-Node-Monitor

Annex I

REAL TIME LINK MANAGEMENT

# 1 Scope

The Real time Audio-Visual Control Architecture is based upon the provision of a Real Time Link Management Service (RTLM) as defined in this recommendation.  This service supports the establishment of multiple logical communication channels which allow real time streams to be routed from a source to one or more destinations.  RTLM facilitates the manipulation of multipoint communication channels for real time streams by providing a routing overlay on the conference network topology.  It uses a distributed management architecture which scales for use in environments with large numbers of end points.

This recommendation makes provision for the management and control of real time streams, independent of content.  As such it provides support for manipulation of real time data streams in addition to audio and video streams.  The content of such real time data streams is beyond the scope of this recommendation.

FIGURE 1/T.132

Audio-visual control architecture

T.132 provides the following features:

- A real time link capability negotiation mechanism, which determines the transmission capabilities of each real time link in the conference and media types (e.g. audio, video) which can be conveyed over each real time link.  The real time link negotiation is performed when the RTLM infrastructure is established, but may also be carried out at any subsequent time if there is a change of capabilities.  This enables RTLM to build up a map of a routing infrastructure describing the connectivity of bandwidth between nodes in the conference.  Overlaid on the routing infrastructure is the capability infrastructure which describes the media types that can be conveyed on each real time link.  This includes media specific encodings (e.g video compression algorithms) and will support dependent capabilities.  The routing infrastructure is held at all switching nodes (MCUs) within the conference, with each MCU required to maintain only that portion of the capability infrastructure that applies to its immediately connected real time links.  A sample real time link capability record is illustrated in figure 2.

- Inheritance of existing connections and configurations from network specific real time control protocols.

- A real time stream transport and routing service, to a specified Quality of Service (QoS).  The pipeline management primitives are similar to the channel management primitives provided by MCS and support:

- Pipeline creation (defining pipeline parameters)

- Pipeline destruction (de-activating the pipeline & removal of all associated records)

- Adding nodes to a pipeline

- Removing nodes from a pipeline

A routing roster which allows nodes to register an interest in specific media pipelines in order that they may receive notification of any changes (e.g. in routing, quality of service) applied to those pipelines.

The RTLM routing infrastructure is derived from the T.120 conference topology and may have a different topology for each media type if the real time streams are routed independently.  The RTLM routing infrastructure is not required to map directly onto the T.120 topology and allows each node to have a direct connection to any other node within the conference.  This recommendation imposes no requirement for media streams between adjacent nodes to share the same physical link, although this would typically be the case.  Similarly, the T.120 data channel to control the real time stream infrastructure may be routed independently.

# 2 Normative References

The following Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid.  All Recommendations and other references are subject to revision: All users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below.  A list of the currently valid ITU-T Recommendations is regularly published.

–	ITU-T Recommendation T.120 (1996), Transmission Protocols for Multipoint Data Conferencing

–	ITU-T Recommendation T.122 (1993), Multipoint Communication Service for audiographics and audiovisual conferencing service definition.

–	ITU-T Recommendation T.123 (1994), Protocol Stack for Audiographics and Audiovisual Teleconference Applications.

–	ITU-T Recommendation T.124 (1995), Generic Conference Control.

–	ITU-T Recommendation T.125 (1994), Multipoint Communication Service protocol specification.

# 3 Definitions

For the purposes of this Recommendation, the following definitions apply:

Adjacent Node: An adjacent node is a node that forms one of a pair of nodes that are connected and thus associated by a real time link.

Application Protocol: Any standard or non-standard protocol specification that makes use of T.120 services.

Application Protocol Entity: The instantiation of an Application Protocol in a terminal or MCU.

Application Protocol Session: A group of peer Application Protocol Entities communicating with each other.

Controllable Device: A device that has the capability to be controlled remotely in a conference environment.

Communications Port: A physical interface at a node that possesses AVC media capabilities and is able to provided a connection to a real time link.

Device: An entity which provides a specific function, or set of functions, to the conference. Some devices such as cameras and microphones are capable of generating a source to the conference. Other devices provide control and processing functions for streams.

Inter-MCU-Link: A real time link that interconnects two adjacent MCU nodes.

MCU (Multipoint Control Unit): A device used to interconnect multiple terminals and/or further MCUs to allow multiple participants to be connected to single  conference.

Multipoint Service: One or more functions that are provided and managed by MCUs.

Node: Any entity in the network that runs the T.120/T.132 protocols. This includes both terminal equipment and network elements such as routers and bridges (otherwise known as MCUs).

Pipe: A communications path within a real time link that carries a single stream between adjacent nodes.

Pipeline: A collection of connected pipes used to convey a real time stream from its source to one or more destinations.

Real Time Link: A connection between two adjacent nodes used for the transfer of real time information in a single direction.  A real time link may provide one or more communication paths (termed pipes), each of which conveys a single stream between the two nodes.

Source: The origin of a real time media stream.

Stream: A flow of real time information of a specific media type (e.g. audio) and format (e.g. G.722) from a single source to one or more destinations.

Video Processor: A device that modifies the content of video streams, e.g. a transcoder, continuous presence processor.

Virtual MCU: Term used to describe the collection of MCUs within a real time conference which present a single logical interface to terminals.

User Application: The instantiation of an Application Protocol in a terminal or MCU.

# 4 Abbreviations

APE			Application Protocol Entity

AVC		Audio-Visual Control

ATM		Asyncronous Transfer Mode

B-ISDN		Broadband Integrated Services Digital Network

GCC		Generic Conference Control

ISDN		Integrated Services Digital Network

LAN		Local Area Network

MCS		Multipoint Communication Service

MCU		Multipoint Control Unit

RTLM		Real Time Link Management

PDU		Protocol Data Unit

PSDN		Packet Switched Digital Network

PSTN		Public Switched Telephone Network

TCP			Transmission Control Protocol

MCS	Multipoint Communication Service

MCU	Multipoint Control Unit

PDU	Protocol Data Unit

SAP	Service Access Point

# 5	Conventions

The primitive parameters of the abstract services defined in this Recommendation use the following key:

M:		parameter is mandatory

C:		parameter is conditional

O:		parameter is optional

Blank:	parameter is absent

(=):		value of the parameter is identical to the value of the corresponding parameter of the preceding primitive, where preceding is defined relative to the order:  request, indication, response, confirm.

(=RQ):	value of the parameter is identical to the value of the corresponding parameter in a preceding primitive, where RQ = request, IN = indication, RS = response, and CF = confirm.

Primitives are categorised in up to four types: Request, Indication, Response, and Confirm.  Some primitives support all of these types, while others do not.  These four types are defined as follows:

Request primitive:	Those that are sourced from a Node Controller or Application Protocol Entity to initiate a certain action.

Indication primitive:	Those that are sourced from an RTLM Provider either as a result of a Request primitive, or as a result of an RTLM initiated action.

Response primitive:	Those that are sourced from a Node Controller or Application Protocol Entity in response to an Indication primitive which is defined to require a response.

Confirm primitive:	Those that are sourced from an RTLM Provider as a result of a Response primitive.

# 6 Real Time Stream Control Architecture

The Audio-Visual Service Architecture is built up from a collection of elements to provide an unambiguous nomenclature for controlling a conference.  This section defines the elements of the model.

The logical connection between two adjacent nodes that is used to convey real time information in a single direction between those nodes is termed a real time link.  At least two links are required to support bi-directional communications between adjacent nodes, one (or more) in each direction.  Real time information conveyed on a real time link may include any combination of video, audio or real time data streams.  In this context, signals used to control the real time link and data conveyed in the T.120 data channel are not considered to be real time data streams and are thus not part of the real time link.

FIGURE 3/T.132

Single unicast real time link supporting unidirectional communication between adjacent nodes

FIGURE 4/T.132

Two unicast real time links supporting bidirectional communication between adjacent nodes

FIGURE 5/T.132

Multicast real time link

Multiple real time links between adjacent nodes are permitted if there are multiple independent physical connections between those nodes.  If a number of connections between two nodes are managed as a single entity, for the purposes of this recommendation they are considered to be a single real time link.  For example, multiple basic rate ISDN connections may be managed as a single entity using H.221.

There are two types of real time link defined: Unicast Real Time Link and Multicast Real Time Link.

Unicast Real Time (URT) Link: a direct connection between two nodes, allowing unidirectional real time communication between those nodes.  This is the type of link encountered in switched network environments (ISDN, PSTN).

Multicast Real Time (MRT) Link: a connection between one node and a group of commonly connected nodes on the same network.  It is defined to allow the mapping of a multicast topology (e.g Local Area Network) into the RTLM environment.  The characteristics of the multicast real time link are determined by the node sourcing the media stream (source node).  All other nodes on the real time link are receivers (sink nodes).

FIGURE 6/T.132

Example of T.132 network topology

A real time link may be subdivided into a number of pipes.  A pipe is used to convey a single uni-directional flow of information of a specific media type (audio, video or real time data) between the two nodes.  Each independent information flow requires its own pipe.  So, a basic full duplex audio-video call would require two unicast real time links, each containing two pipes, one pipe for video and one for audio.

Note that a pipe is equivalent to a unidirectional logical channel as defined in H.245.

The number of pipes within a real time link and the transmission characteristics of those pipes can be varied dynamically throughout the course of a conference.  There is no requirement for symmetry within a real time link, thus the number of pipes per media type may differ.

FIGURE 7/T.132

Typical real time link

FIGURE 8/T.132

Full duplex audio-video communications

A stream is a uni-directional flow of real time information from a specified origin to one or more destinations. The logical communication path used to transport a stream end to end is termed a pipeline.  Stream processing (e.g. transcoding) is not permitted within a pipeline, so the information delivered to the destinations is identical in all respects to that submitted at the stream origin.  The pipeline itself is comprised of one or more pipes, each of which is used to convey the stream over a single link between the source and its destination(s).

FIGURE 9/T.132

Example of a Pipeline

The origin of a stream is termed its source.  A source may either be a physical device such as a camera or microphone, or it may be some other entity that is introduced into the conference such as an 'off-air' signal.  The destination of a stream is termed a sink.

## 7 RTLM System Model for a Conference Node

The figure below illustrates the relationship between RTLM and the components it interacts with to realise management of real time streams.  The Node Controller is the entity responsible for creating and terminating real time stream control sessions and specifying the conditions under which RTLM will operate.  Application Protocol Entities (APEs) use the services of RTLM to create and manipulate real time streams.  Typically, user applications would access RTLM services through an AVC Service Provider APE (e.g. Multimedia Network Services).  Communication between RTLM Providers is achieved using the services of MCS.  The Network Specific Multiplex Control provides capability information to the RTLM Provider and is used to implement link configuration commands issued by the RTLM Provider.

RTLM maintains a database for core AVC information (capabilities and configurations) that may be of use to entities (AVC Service Providers) that make use of its services to deliver enhanced audio-visual control services.  It is recognised that different AVC services require access to different levels of information, but maintaining separate databases would result in duplication of information and increase the potential for conflicts.

FIGURE 10/T.132

System model showing RTLM Service Access Points

RTLM is accessed by both the Node Controller and APEs via Control and User Service Access Points (SAPs) respectively.  The method of establishing these RTLM SAPs is a local matter.

## 8 RTLM Communication Channels

For a multipoint session T.132 utilises 3 MCS static channels to provide management and control services for real time channels:-

·	RTLM Broadcast Channel

·	MCU Control Channel

·	MCU Communication Channel

For a point-to-point session only the RTLM Broadcast Channel will be utilised.

### 9 RTLM Broadcast Channel

The RTLM Broadcast Channel is joined by all RTLM capable terminals and all MCUs.  It is used for broadcasting information to all RTLM capable nodes.

### 10 MCU Control Channel

The  MCU Control Channel is joined by all MCUs participating in a T.120 conference.  It shall not be joined or used by terminals. Its use is reserved for MCU specific communications.  MCUs providing time critical management functions are permitted to use this channel at Top priority (i.e. the priority that is normally reserved for GCC management  functions).

### 11 MCU Communication Channel

The MCU Communication Channel is also joined by all MCUs participating in a T.120 conference. This channel may not be joined by terminals but in a multipoint session they may send messages to it in order to communicate with the conference MCUs.  It provides a single channel for communication to MCUs that is independent of the number of MCUs and the conference topology and effectively allows terminals to see the MCU network as a single ‘virtual’  MCU.

FIGURE 11/T.132

RTLM communication channels in a point-to-point environment

FIGURE 12/T.132

RTLM communication channels in a multipoint environment

# 12 T.132 Modes of Operation

The mode of operation of a T.132 session is determined at by the conference creator at conference creation time.  It may be possible for a node with appropriate authority (e.g. chairman or convenor) to subsequently change some of the parameters.

The following criteria may be specified:

- Initial conference configuration: this defines whether conference start-up conditions are inherited from the low level network specific negotiation mechanisms or whether they are to be defined by T.132.  In the latter case, the conference creator specifies the real time streams that need to be established and provides an indication of their status i.e. whether the streams need to exist for the duration of the conference and whether they may be overidden by requests to create new streams or requests to modify the transmission characteristics of other streams.  For example, the creator may specify an audio stream and video stream linking all participants and require that the audio stream be maintained throughout the conference, but allow the video stream to be pre-empted if the need arises (e.g. to allow a fast file transfer).

- A weighting factor for specific streams, in order to determine which streams may be pre-empted in the event of traffic on a real time link exceeding its transmission capacity.

- Whether centralised or distributed arbitration of real time link capabilities and configuration is to be employed.

- Use of enhancements which take the facilities offered by the lower transport layers and provide additional functionality e.g. superimposing a multiplex to allow multiple streams to be routed over a single lower layer pipeline to support continuous presence.

- Status/privileges assigned to specific roles (conductor/convenor/presentor/participant etc)

# 13 Creation and Termination of an RTLM Session

An RTLM session may exist for the entire duration of a T.120 conference, or for only part of the conference.  Use of RTLM is determined by the conference creator who specifies one of the following options:

- Use of RTLM mandatory for the duration of the conference.

- Use of RTLM prohibited.

- Use of RTLM permitted.  In this case, an RTLM session is not established automatically at conference create time, but may be established subsequently by an authorised node.  The conference creator designates who has the privilege to create and terminate RTLM sessions (i.e. convenor, conductor etc).

Note that audio and video communications are permitted in the absence of an RTLM session, subject to being authorised by the conference creator.

For an RTLM session to be successfully established, RTLM must be present at the Top Provider node and all MCUs within the conference.  There is no requirement for RTLM to be present at all terminals within the conference, although its absence is likely to give reduced functionality.  MCUs may choose to proxy RTLM functionality for non-RTLM capable terminals; how this is achieved is outside the scope of this recommendation.

The conference creator specifies the start up conditions and restrictions to be applied to an RTLM session in the GCC-Conference-Create primitive.  This may include an indication of which media types may be used in the conference (i.e. audio, video, real time data).  Extensions to this primitive are required to support RTLM parameters.  Other nodes are notified of the use of RTLM through the GCC Conference Roster.  Nodes joining the conference examine the Conference Roster and if use of RTLM is either permitted or mandated they shall join the appropriate RTLM communication channels.

If the conference creator has permitted but not mandated use of RTLM, an RTLM session may be created by an authorised node issuing an RTLM-Session-Initiate request primitive to all RTLM capable nodes within the conference on the RTLM Broadcast Channel (beware - the late joiner problem exists here).  Each node receiving this primitive must then follow the real time link negotiation process described in the next section.

FIGURE 13/T.132

RTLM-Session-Initiate - sequence of primitives

RTLM sessions created using the RTLM-Session-Initiate primitive may be subsequently terminated by an authorised node issuing an RTLM-Session-Terminate request primitive to all RTLM Providers on the RTLM Broadcast Channel.  Termination of an RTLM session does not necessarily imply the termination of any real time streams being managed by RTLM; it simply means that control of such streams reverts to the underlying network specific control mechanisms (e.g. H.245).  Any command received on an RTLM control channel subsequent to the receipt of an RTLM-Session-Terminate primitive shall be ignored, unless it is an RTLM-Session-Initiate.

FIGURE 14/T.132

RTLM-Session-Terminate - sequence of primitives

# 14 Real Time Link Negotiation

An RTLM session is comprised of a number of nodes that are interconnected in a given configuration, by real time links. Nodes may have differing media capabilities depending on their rôle within the session. The networks and bandwidth available to nodes may also differ, thus real time links within a session which are on different networks will inherently have different capabilities.  In some cases it may be necessary to try to establish a common session wide capability, but in many cases this may unnecessarily diminish the communication capability of the session.

T.132 provides mechanisms which allow for negotiation of real time link capabilities and configuration of real time links on an individual basis, and also provides the potential for central arbitration.  Real time link negotiation is most efficiently done in a distributed way, with adjacent nodes determining a common capability for the real time link.

A real time link provides a connection between adjacent nodes for the transfer of real time information.  In order to define a conference wide real time link negotiation mechanism, there is a need for real time links and their associated nodes to be uniquely identified.  A unicast real time link is identified by its two endpoints (using the GCC User IDs of the respective nodes), direction (up or down) and an optional locally unique number to distinguish multiple real time links between the same pair of nodes.  In many cases, the RTLM hierarchy will mirror the T.120 hierarchy; in such cases the GCC Conference Roster directly provides the GCC User ID of the adjacent higher node in the T.120 hierarchy.  Where real time links exist between nodes which are not adjacent in the T.120 hierarchy, sufficient information (e.g. phone number) needs to be provided in the conference roster to allow the GCC User ID to be correlated with a specific physical site.

For the purposes of capability negotiation, the status of each node at the end of a unicast real time link is determined from its position in the T.120 hierarchy.  The node lower in the T.120 hierarchy is termed the lower node; it is subordinate to the node at the other end of the real time link (the higher node).  If a unicast real time link exists between nodes which are not directly connected in the T.120 hierarchy, then the lower node shall be the node with the lower numerical value of GCC User ID.

In the case of a multicast real time link the node which is sourcing the real time information is designated as the higher node; all other nodes on the multicast real time link are designated lower nodes.  If the higher node leaves the conference, the multicast real time link ceases to exist (from a T.120 perspective, the link will also cease to exist if a sink node on the link which has a higher position in the T.120 hierarchy than the source node, leaves the conference); other nodes may join and leave the multicast real time link at will without affecting the integrity of the real time link, although of course the real time link capability may change.  A multicast real time link is identified  by the GCC User ID of the higher node and an optional locally unique number to distinguish between multiple multicast real time links.

FIGURE 15/T.132

Relationship between multicast real time links & T.120 hierarchy

Real time link capability negotiation may be a multi-stage process.  When a physical real time link is established, a point-to-point capability exchange typically occurs at the multiplex control level, for example H.245 for PSTN unicast real time links.  The results of these negotiations are inherited by T.132 and form the basis for subsequent real time link negotiations.  If the low level capability exchange process enables all nodes on a real time link to determine completely the capabilities of the link, then the initial RTLM real time link negotiation is not required.  Subsequent operations (as defined in §9.1) which allow the link capabilities to propagate through the conference are still necessary.  Each Network Specific Mapping document shall specify whether the RTLM link negotiation method is required for its respective network.

When a node joins a conference, it consults the GCC Conference Roster to determine whether it is the higher or lower node for each of the unicast real time links it is connected to as described above.  For each of those real time links where it is lower in the real time link hierarchy, the node sends its capabilities to the corresponding higher node via an RTLM-Announce-Node-Capability PDU.  The higher node is responsible for combining its own capabilities with those of the lower node in order to determine the capabilities of the unicast real time link.

FIGURE 16/T.132

RTLM-Announce-Node-Capability - sequence of primitives

A different approach is adopted for multicast real time links, allowing a number of different possibilities:

- The conference creator may specify that there is no negotiation on multicast real time links.  The characteristics of such links are then determined by their respective source nodes.

- The conference creator may allow negotiation on multicast real time links.  In this case, nodes wishing to join the multicast real time link send their capabilities to the source node which may choose to take account of, or ignore, the capabilities of any sink node when determining the characteristics of the multicast real time link.

Capabilities are divided into two types:

- real time link transmission characteristics which quantify the transmission capabilities of a node when connected to the real time link e.g. bandwidth, latency.  Observe that transmission capacity may be network limited or node limited.  This capability quantifies the amount of traffic a node can handle.  The actual transmission capacity available at any instance may be less than the values specified in this field due to network conditions (e.g. other traffic on a LAN, PSTN modems training down to a lower data rate).

- media specific characteristics which describe the ability of a node to process real time streams of a specific format, for example the ability to transmit or receive an H.261 video stream.  This field may specify the transmission requirements (e.g. bandwidth) of each media format supported.

FIGURE 17/T.132

Example of real time link capability record

These capabilities are handled separately, allowing multiple real time links to exist between adjacent nodes without requiring duplication of media specific characteristics which need only be forwarded on establishing the first real time link between adjacent nodes.  Any real time links established subsequently between the same pair of nodes only require the node transmission characteristics for the new real time link to be passed to the higher node.

## 15 Arbitration of Real Time Link Capabilities

Once the higher node has determined a common set of capabilities for the real time link, subsequent operations are dependent on the mode of the conference as defined at conference creation time.  If the conference is configured for distributed arbitration, decisions on real time link capabilities and configurations may be made locally; if the conference is configured for centralised arbitration, a single entity (the Top RTLM Provider) may intervene to impose real time link capabilities and configurations.  The Top RTLM Provider is at the same node as the Top GCC Provider and Top MCS Provider.

If the conference is configured for distributed arbitration, the higher node informs the lower node of the real time link capability by sending an RTLM-Announce-Link-Capability PDU comprising both real time link transmission characteristics and common media formats to the GCC User ID channel of the lower node.  It also forwards an RTLM-Announce-Link-Capability PDU to the MCU broadcast channel containing only the real time link transmission characteristics.  This enables the switching nodes within the conference to build up a map of the routing infrastructure i.e. the availability of bandwidth between any collection of endpoints, without having to be aware of the media specific capabilities of each node.  This map is termed the routing roster.

If the conference is configured for centralised arbitration, once the higher node has compiled the common real time link capability, it forwards the information to the RTLM Provider at the highest node in the T.120 hierarchy (the Top RTLM Provider) using the RTLM-Announce-Link-Capability PDU.  The Top RTLM Provider may then choose to arbitrate on a set of real time link records for the session.  Any algorithms used in the arbitration process are considered to be outside the scope of this recommendation.

The Top RTLM Provider may choose to arbitrate on all real time link configurations or may delegate authority to subordinate nodes.  In the former case, varying degrees or intervention are possible:  the Top Provider may simply choose to provide each node with a new capability record derived centrally by the arbitration, allowing nodes the flexibility to operate as they choose within the constraints defined by the capability record.  Alternatively, it may impose an authoritarian regime, explicitly configuring each real time link within the conference and giving no scope for nodes to adjust their configurations, other than via requests to the Top Provider.  It is left to the Top RTLM Provider to determine if it is acceptable to allow each real time link to be configured at its maximum capability (determined by the local negotiation between adjacent nodes), or if there are over-riding factors that require session wide provision.

FIGURE 18/T.132

RTLM-Announce-Link-Capability - sequence of primitives

Essentially, the Top Provider has three options available to it in managing the real time link infrastructure:-

- Allow each real time link to operate within its locally determined capabilities.

- Provide each node with a new real time link capability record determined centrally by the arbitration process.

- Provide each node with a precise real time link configuration.

Observe that RTLM does not inherently mandate any requirements for real time link symmetry, although restrictions may be imposed by the underlying networks.

The Top RTLM Provider may issue an RTLM-Announce-Link-Capability PDU either to acknowledge acceptance of the locally negotiated real time link capabilities or to impose a session requirement on real time links either collectively or individually, as required.  An RTLM-Announce-Link-Capability PDU containing only real time link transmission characteristics is sent to the MCU Communication Channel to enable switching hubs to build up a picture of the routing infrastructure (i.e. update the routing roster).

The Top RTLM Provider may also issue configuration commands using the RTLM-Set-Link-Configuration request in order to inform either, each pair of (real time link adjacent) nodes of a real time link requirement, or all nodes (by broadcast) of a common requirement.  Observe that real time link configuration is limited in scope to the partitioning of bandwidth between real time streams and does not address the bandwidth requirements of T.120 data.  The partitioning of bandwidth between T.120 data and real time streams on networks where there is a network multiplex (e.g. H.221 on ISDN, H.223 on PSTN) is managed externally to RTLM (by the multiplex manager defined in T.AVC).  RTLM is notified of changes in bandwidth available to real time streams by the underlying network specific multiplex control.

TABLE 1/T.132.

RTLM-Announce-Node-Capability Primitive

| Parameter | Request | Indication |
| --- | --- | --- |
| Real Time Link Identifier - Higher Node | M | M(=) |
| Real Time Link Identifier - Lower Node | C | C(=) |
| List of Link Transmission Records | O | O(=) |
| Number of Up Audio Streams | M | M(=) |
| Number of Down Audio Streams | M | M(=) |
| Number of Up Video Streams | M | M(=) |
| Number of Down Video Streams | M | M(=) |
| Number of Up Real Time Data Streams | M | M(=) |
| Number of Down Real Time Data Streams | M | M(=) |
| Node Capability Table | O | O(=) |

Real Time Link Identifier (Higher Node): The GCC User ID of the higher of the two nodes forming a unicast real time link, or the source node in a multicast real time link.

Real Time Link Identifier (Lower Node): The GCC User ID of the lower of the two nodes forming a unicast real time link.  This parameter is omitted for a multicast real time link.

List of Link Transmission Records:  A list specifying the transmission characteristics of each link between the lower node and upper node.

Number of Up Audio Streams: the maximum number of audio streams that the lower node can send to the higher node simultaneously.

Number of Down Audio Streams: the maximum number of audio streams that the lower node can receive from the higher node simultaneously.

Number of Up Video Streams: the maximum number of video streams that the lower node can send to the higher node simultaneously.

Number of Down Video Streams: the maximum number of video streams that the lower node can receive from the higher node simultaneously.

Number of Up Real Time Data Streams: the maximum number of real time data streams that the lower node can send to the higher node simultaneously.

Number of Down Real Time Data Streams: the maximum number of real time data streams that the lower node can receive from the higher node simultaneously.

Node Capability Table:  Contains capability records for each media format supported by the node.  It may optionally include a list of valid operating configurations (i.e. combinations of media formats that can be supported simultaneously).

### 16 Link Capabilities

TABLE 2/T.132.

Content of each link transmission record

| Parameter | Request | Indication |
| --- | --- | --- |
| Network Type (e.g. PSTN, ISDN) | M | M(=) |
| Direction | O | O(=) |
| Real Time Link Handle | O | O(=) |
| Mean Real Time Link Bandwidth (bits/s) | O | O(=) |
| Peak Real Time Link Bandwidth (bits/s) | O | O(=) |
| Default Real Time Link Bandwidth (bits/s) | O | O(=) |
| Mean Latency (ms) | O | O(=) |
| Latency variation (ms) | O | O(=) |

Network Type:Identifies the type of physical network employed by the real time link e.g. 	ISDN, PSTN and the transport protocol used over that network (e.g. H.221, H.223).

Direction: Up if the link goes from the lower node to the upper node, Down if the link goes from the upper node to the lower node.  Omitted for multicast real time links.

Real Time Link Handle: A locally unique (amongst the nodes joined to the real time link) number provided by the lower node to distinguish multiple real time links between it and a given higher node.  This may be, for example, a port number.  In the case of a unicast real time link this number is provided by the lower node;  in the case of a multicast real time link this number is provided by the source node (as each lower node could assign a different number.  (This implies lower nodes need to identify multicast real time links and their sources before starting the negotiation process: this information may be gleaned from the RTLM routing roster....)

Mean Real Time Link Bandwidth: The average bandwidth available from the lower node to the upper node, where relevant.

Peak Time Link Bandwidth: The maximum bandwidth available between the lower node and the upper node.  For switched networks the bandwidth available within a real time link may be changed by adding dropping connections (e.g. adding/releasing B channels in an ISDN call).

Default Real Time Link Bandwidth: The bandwidth initially available between the lower node and the upper node on establishing the real time link.  This term only applies to switched networks which may increase the bandwidth available to the real time link by adding further connections between the two nodes.

Mean Latency: Specifies the average delay between transmission of a bit by the node at one end of the real time link and its receipt by the node at the other end of the real time link.

Latency variation: The difference between the maximum and minimum possible delays that a bit will experience when traversing the real time link.  This may be used to determine the amount of buffer space required at the receiving side in order to restore the original signal.

Use of the parameters in the link transmission record is specified in the appropriate Network Specific Mapping document.  Some parameters may not be relevant to specific networks and may thus be omitted.

### 17 Node Capabilities

Node capabilities characterise a node’s abilities to process (encode and send, receive and decode or both) specific media formats; essentially they define the modes of operation a node can support.  Node capabilities are not tied to a specific link, though if multiple links exist between adjacent nodes it is possible that some modes may be incompatible with one or more of those links.

For each media type (audio, video or real time data), a node declares the maximum number of streams it can handle simultaneously in each direction.  In this context up refers to streams flowing from the lower node in the link towards the upper node; down refers to streams from the upper node to the lower node.  The node declares the media formats it can support in a media capability table, indicating whether it is capable of transmitting, receiving or transmitting and receiving each media format.

Observe that in practice it may not be a possible for a node to accomodate the number of streams indicated in these fields simultaneously due to local constraints (e.g. processing power).  Nodes may be able to send and receive a wide variety of different media formats, but may only be able to process a limited subset of those media formats at any given instance due to resource limitations (e.g. processing power).  A node may optionally choose to indicate in the media capability table which media formats (or operating modes) can be supported simultaneously.  The construction of the media capability table is described below (extract from H.245 with minor amendments) :

TABLE 3/T.132.

Media capability record (Capability Descriptor)

| Parameter | Request | Indication |
| --- | --- | --- |
| Capability Record Identifier | M | M(=) |
| Media Type | M | M(=) |
| Media Format | M | M(=) |
| Transmit/Receive Capability | M | M(=) |
| Media Format Specific Parameters | O | O(=) |
| Bandwidth Range | O | O(=) |

Capability Record Identifier: Locally unique number used to reference this particular capability record

Media Type: Audio, Video or Real Time Data.

Media Format: Identifies the specific media encoding format e.g. G.728, H.261.

Transmit/Receive Capability: Indicates whether the terminal is capable of transmitting, receiving or transmitting and receiving the specified media format.

Media Format Specific Parameters:  These characterise the negotiable capabilities available to a specific media format and may include parameters such as video frame rate and resolution.

Bandwidth Range:  Upper and lower bounds on the bandwidth required by the encoding format.

A node assigns a locally unique identifier to each media specific format.  For example, G.723 audio, G.728 audio, and CIF H.263 video would each be assigned separate numbers.

These capability identifiers are grouped into AlternativeCapabilitySet structures. Each AlternativeCapabilitySet indicates that the terminal is capable of operating in exactly one mode listed in the set. For example, an AlternativeCapabilitySet listing {G.711, G.723, G.728} means that the node can operate in any one of those audio modes, but not more than one.

These AlternativeCapabilitySet structures are grouped into simultaneousCapabilities structures. Each simultaneousCapabilities structure indicates a set of modes the node is capable of using simultaneously.  For example, a simultaneousCapabilities structure containing the two AlternativeCapabilitySet structures {H.261, H.263} and {G.711, G.723, G.728} means that the node can operate either of the video codecs simultaneously with any one of the audio codecs.  The simultaneousCapabilities set {{H.261}, {H.261, H.263}, {G.711, G.723, G.728} } means the terminal can operate two video channels and one audio channel simultaneously: one video channel per H.261, another video channel per either H.261 or H.263, and one audio channel per either G.711, G.723, or G.728.

Note: the actual capabilities stored in the capabilityTable are often more complex than presented here. For example, each H.263 capability indicates details including ability to support various picture formats at given minimum picture intervals, and ability to use optional coding modes.

The node's total capabilities are described by a set of CapabilityDescriptor structures, each of which is a single simultaneousCapabilities structure and a capabilityDescriptorNumber. By sending more than one CapabilityDescriptor, the node may signal dependencies between operating modes by describing different sets of modes which it can simultaneously use. For example, a terminal issuing two CapabilityDescriptor structures, one {{H.261, H.263}, {G.711, G.723, G.728} } as in the previous example, and the other { {H.262}, {G.711} }, means the terminal can also operate the H.262 video codec, but only with the low-complexity G.711 audio codec.

Nodes may dynamically add capabilities during a communication session by issuing additional CapabilityDescriptor structures, or remove capabilities by sending revised CapabilityDescriptor structures.  All nodes shall transmit at least one CapabilityDescriptor structure.

### 18 Capability Table

A capability table is a numbered list of capabilities. A node shall be capable of everything that it lists in its capability table, but shall not necessarily be capable of simultaneously performing more than one of them.

A NodeCapabilitySet may contain zero or more CapabilityTableEntrys.  At the start, no table entries are defined. When a CapabilityTableEntry is received, it replaces the previously received CapabilityTableEntry with the same CapabilityTableEntryNumber.  A CapabilityTableEntry without a Capability may be used to remove the previously received CapabilityTableEntry with the same CapabilityTableEntryNumber.

### 19 Capability Descriptors

CapabilityDescriptors are used to indicate a node's capability to transmit and receive.  Each CapabilityDescriptor provides an independent statement about the node's capabilities.

capabilityDescriptorNumber is used to number CapabilityDescriptors.  If a node has a preference for the mode it would like to transmit or receive, and wishes to express this when transmitting its capabilities, it may do so by giving CapabilityDescriptors that relate to its preferred mode or modes small values of capabilityDescriptorNumber.

simultaneousCapabilities is a set of AlternativeCapabilitySet.  It is used to list the simultaneous capabilities of the node.

An AlternativeCapabilitySet is a sequence of CapabilityTableEntryNumbers.  Only those CapabilityTableEntrys that have been defined shall be present in an AlternativeCapabilitySet, although it is possible to define CapabilityTableEntrys and refer to them in the same TerminalCapabilitySet.  If a node has a preference for the mode it would like to transmit or receive, and wishes to express this when transmitting its capabilities, it may do so by listing elements in AlternativeCapabilitySets in order of decreasing preference.

A node shall be capable of simultaneously performing any one capability from each AlternativeCapabilitySet listed in simultaneousCapabilities.

At least one capability descriptor shall have the following structure: there shall be at least one AlternativeCapabilitySet containing only capabilities of a single medium type for each medium type that the node can support.  This is to ensure that the remote node can select a mode of transmission that includes at least one instance of each medium type that the receiver can support.

Note: a repetition of a capability in an AlternativeCapabilitySet is redundant and conveys no further information, while the repetition of a capability in different AlternativeCapabilitySets in the same CapabilityDescriptor indicates the possibility of an additional, simultaneous, instance of the particular capability.

Note: nodes that can not vary the allocation of resources can indicate their capability completely by use of a single CapabilityDescriptor.

## 20 Changing Real Time Link Capabilities

A node may change its capabilities at any time by sending a RTLM-Announce-Node-Capability PDU to the higher node(s) on all affected real time links.  The process followed by the higher node is identical to that at the initial establishment of the real time link.  If the conference is configured for distributed arbitration, the node shall also inform lower nodes on affected real time links using the RTLM-Announce-Link-Capability PDU.

Nodes shall accept and act upon commands received from any node higher in the RTLM hierarchy.

## 21 Setting Real Time Link Configuration

The RTLM-Set-Link-Configuration primitive is used by the RTLM Top Provider to impose real time link configurations on the session.  It may be broadcast to the session, or to any pair of adjacent nodes connected by a real time link.  It may also be used by the higher node of an adjacent pair to define a required real time link configuration

TABLE 4/T.132

RTLM-Set-Link-Configuration Primitive

| Parameter | Request | Indication |
| --- | --- | --- |
| Network Type (e.g. PSTN, ISDN) | M | M(=) |
| Real Time Link Identifier - Higher Node | M | M(=) |
| Real Time Link Identifier - Lower Node | M | M(=) |
| Direction | O | O(=) |
| Real Time Link Handle | O | O(=) |
| Accept Locally Negotiated Capabilities Flag | M | M(=) |
| Permit Local Real Time Link Configuration Flag | M | M(=) |
| Mean Real Time Link Bandwidth (bits/s) | O | O(=) |
| Peak Real Time Link Bandwidth (bits/s) | O | O(=) |
| Default Real Time Link Bandwidth (bits/s) | O | O(=) |
| Mean Latency (ms) | O | O(=) |
| Latency variation (ms) | O | O(=) |
| Number of Audio Streams | M | M(=) |
| Number of Video Streams | M | M(=) |
| Number of Real Time Data Streams | M | M(=) |
| List of Pipe Records | O | O(=) |
| Node Capability Table | O | O(=) |

Network Type:Identifies the type of physical network employed by the real time link e.g. 	ISDN, PSTN and the transport protocol used over that network.

Real Time Link Identifier (Higher Node): The GCC User ID of the higher of the two nodes forming a unicast real time link, or the source node in a multicast real time link.

Real Time Link Identifier (Lower Node): The GCC User ID of the lower of the two nodes forming a unicast real time link.  This parameter is omitted for a multicast real time link.

Direction: Up if the link goes from the lower node to the upper node, Down if the link goes from the upper node to the lower node.  Omitted for multicast real time links.

Real Time Link Handle: A locally unique (amongst the nodes joined to the real time link) number provided by the lower node to distinguish multiple real time links between it and a given higher node.  This may be, for example, a port number.  In the case of a unicast real time link this number is provided by the lower node;  in the case of a multicast real time link this number is provided by the source node (as each lower node could assign a different number.  (This implies lower nodes need to identify multicast real time links and their sources before starting the negotiation process: this information may be gleaned from the RTLM routing roster....)

Accept Local Capabilities Flag: Flags for audio, video and real time data telling the receiver that real time link capability resulting from the local negotiation can be used.

Accept Local Configuration Flag: Flags for audio, video and real time data telling the receiver that real time link configuration can be negotiated locally.

Mean Real Time Link Bandwidth: The average bandwidth available from the lower node to the upper node.

Peak Time Link Bandwidth: The maximum bandwidth available between the lower node and the upper node.  For switched networks the bandwidth available within a real time link may be changed by adding dropping connections (e.g. adding/releasing B channels in an ISDN call).

Default Real Time Link Bandwidth: The bandwidth initially available between the lower node and the upper node on establishing the real time link.  This term only applies to switched networks which may increase the bandwidth available to the real time link by adding further connections between the two nodes.

Mean Latency: Specifies the average delay between transmission of a bit by the node at one end of the real time link and its receipt by the node at the other end of the real time link.

Latency variation: The difference between the maximum and minimum possible delays that a bit will experience when traversing the real time link.  This may be used to determine the amount of buffer space required at the receiving side in order to restore the original signal.

Number of Audio Streams: the maximum number of audio streams that the real time link can handle simultaneously.

Number of Video Streams: the maximum number of video streams that the real time link can handle simultaneously.

Number of Real Time Data Streams: the maximum number of real time data streams that the real time link can handle simultaneously.

List Of Pipe Records:  Contains QoS parameters for each pipe that is currently in use and identifies the pipeline and path associated with the pipe, where appropriate.  To minimise network traffic, only those pipe records whose characteristics are being modified should be included.

Node Capability Table:  Contains capability records for each media format supported by the node.  It may optionally include a list of valid operating configurations (i.e. combinations of media formats that can be supported simultaneously).

FIGURE 19/T.132

RTLM-Set-Link-Configuration - sequence of primitives

# 22 Pipeline Management

Once the capability negotiation is complete it is possible for nodes to request the creation of pipelines to support the distribution of real time streams.

The creator of a pipeline may designate that it wishes to own the pipeline.  Nodes may join the pipeline at the discretion of the owner.  Ownership lapses if the pipeline creator leaves the conference.  If a pipeline is unowned, it may be freely joined by any node, subject to QoS constraints.   Typically, the owner of a pipeline would be a source node or a node within the Virtual MCU.

## 23 RTLM-Create-Pipeline

The RTLM-Create-Pipeline primitive allows a node to request the creation of a new pipeline and specify the QoS requirements for the pipeline.  Typically, a pipeline would be opened by the node that intends to source a stream on the pipeline, although it may be opened by a remote node.

An application wishing to create a new pipeline issues an RTLM-Create-Pipeline request to its local RTLM Provider.  If the requesting node is included as either the source or a sink for the new pipeline, the request is assessed by the local RTLM Provider to determine if it can be accomodated on any of its local real time links.  If the local link(s) cannot accomodate the QoS parameters specified for the pipeline without affecting existing pipelines then the request may be rejected immediately.   Otherwise, there is a need to communicate with other RTLM Providers to determine whether the request can be fulfilled, and if so, to indicate to the requesting application the level of QoS that can be offered.

The action taken by the RTLM Provider is determined by the mode of operation selected for the conference.  If the conference is configured for centralised management, the RTLM Provider issues an RTLM-Create-Pipeline request to the Top RTLM Provider.  The Top RTLM Provider then processes the request and responds accordingly.  On receipt of the RTLM-Create-Pipeline response, the local RTLM Provider forwards an RTLM-Create-Pipeline confirm to the requesting application.  If the result is positive, streams may be submitted to the pipeline until an RTLM-Destroy-Pipeline indication is received.

If the conference is configured for distributed management, the local RTLM Provider forwards the RTLM-Create-Pipeline request to the RTLM Provider at the nearest node within the ‘Virtual MCU’.  (Typically, this would be the immediately superior node within the T.120 hierarchy.  If this node does not have MCU functionality, the request is passed up the tree until it reaches an MCU node.)

All nodes within the ‘Virtual MCU’ have a local copy of the routing roster, which enables them to determine independently (i.e. without consulting other nodes) whether a pipeline create request can be fulfilled.  The RTLM Provider may consult with peer RTLM Providers before reaching a decision.  If the request is accepted, the RTLM Provider shall ensure that all necessary real time links have been configured to support the new pipeline before responding to the RTLM Provider that issued the RTLM-Create-Pipeline request.  This ensures that the new pipeline is available for use as soon as the requesting application receives a positive RTLM-Create-Pipeline confirm from its local RTLM Provider.  All RTLM Providers within the Virtual MCU are notified of the successful creation of a new pipeline as are all RTLM Providers at nodes that are joined to the new pipeline.

Observe that the creator of a pipeline specifies the nodes to which the pipeline is to be routed.  Those endpoints will be automatically added to the real time pipeline (subject to QoS) constraints; if they do not wish to receive any real time streams distributed on that pipeline they must explicitly leave the pipeline by issuing an RTLM-Pipeline-Remove-Node request.  If a node subsequently decides to re-join the pipeline, it must issue an RTLM-Pipeline-Add-Node request.

A pipeline will automatically cease to exist if its source leaves the conference.

FIGURE 20/T.132

RTLM-Create-Pipeline - sequence of primitives: centralised management case

TABLE 5/T.132

RTLM-Create-Pipeline Primitive

| Parameter | Request | Indication | Confirm |
| --- | --- | --- | --- |
| Request Handle | M |  | M(=RQ) |
| Pipeline Identifier |  | M(=CF) | M |
| Sinks | M | M(=CF) | M |
| Media Type | O | O(=) |  |
| Ownership Flag | M | M(=CF) | M |
| Pipeline QoS | M | M(=CF) | C |
| Associated Pipelines | O | O(=) | C |
| Result |  |  | M |

Request Handle: Locally unique number to allow the requesting node to match responses.

Pipeline Identifier: A globally unique number identifying the pipeline to which the primitive refers. Constructed from the GCC User ID of the source node and a locally unique number.

Source: The node acting as source for the pipeline.

Sinks: A list of nodes that will act as sinks for the pipeline.

Media Type: audio, video, real time data

Ownership Flag: Set if the pipeline creator wishes to assert ownership.

Pipeline QoS:  Desired Quality of Service is included in the request.  The actual QoS offered by the Virtual MCU is provided in the confirm.

Associated Pipelines: A list of pipelines with which the new pipeline is to be associated.  The list identifies any pipelines to which the new pipeline is to be synchronised, and specifies the maximum relative latency between the pipelines.  (This parameter should specify a descriptor to apply meaning to the association e.g. two audio pipelines used to distribute a stereo signal, a video pipeline & its associated audio pipeline(s).)

Result:  Indicates the level of service the Virtual MCU can provide for the pipeline.  It contains one of the following results: service unavailable, guaranteed service, predictive service, best effort service.

FIGURE 21/T.132

RTLM-Create-Pipeline - sequence of primitives: distributed management case

## 24 RTLM-Destroy-Pipeline

The RTLM-Destroy-Pipeline primitive is used to remove a real time pipeline that was previously created using the RTLM-Create-Pipeline request.  All records of the pipeline are removed, and all nodes within the pipeline are notified as is any node that registered an interest in the pipeline.   If the source of a pipeline leaves the conference, the pipeline ceases to exist.  In this case the Top RTLM Provider issues an RTLM-Destroy-Pipeline indication to all members of the pipeline.

FIGURE 22/T.132

RTLM-Destroy-Pipeline - sequence of primitives: Pipeline Owner initiated

FIGURE 23/T.132

RTLM-Destroy-Pipeline - sequence of primitives: RTLM Provider initiated

TABLE 6/T.132

RTLM-Destroy-Pipeline Primitive

| Parameter | Request | Indication | Confirm |
| --- | --- | --- | --- |
| Request Handle | M |  | M(=RQ) |
| Pipeline Identifier | M | M(=) | M(=) |
| Result |  |  | M |
| Reason |  | M | M |

Request Handle: Locally unique number to allow the requesting node to match responses.

Pipeline Identifier: A globally unique number identifying the pipeline to which the primitive refers.

Result:  Indicates the level of service the Virtual MCU can provide for the pipeline.  It contains one of the following results: successful, invalid pipeline, not pipeline owner.

Reason: user initiated, pipeline purged.

## 25 RTLM-Pipeline-Add-Sink-Node

The RTLM-Pipeline-Add-Sink-Node primitive is issued by to add a new sink node wishing to an existing pipeline.  If the pipeline is owned, the request is directed to the pipeline owner, who may choose to grant or refuse the request.  If the owner grants permission, the request may ultimately be rejected by the Virtual MCU if the QoS criteria cannot be fulfilled.  If the pipeline is unowned, the request is arbitrated by the nearest RTLM Provider within the Virtual MCU (distributed management case) or the Top RTLM Provider (centralised management case).

FIGURE 24/T.132

RTLM-Pipeline-Add-Node - sequence of primitives: centralised management case

FIGURE 25/T.132

RTLM-Pipeline-Add-Node - sequence of primitives: distributed management case

TABLE 7/T.132

RTLM-Pipeline-Add-Sink-Node Primitive

| Parameter | Request | Indication | Response | Confirm |
| --- | --- | --- | --- | --- |
| Request Handle | M | M(=) | M | M(=) |
| Pipeline Identifier | M | M(=) | M | M(=) |
| Node to be Added | M | M(=) | M | M(=) |
| Result |  |  | M | M(=) |

Request Handle: Locally unique number to allow the requesting node to match responses.

Pipeline Identifier: A globally unique number identifying the pipeline to which the primitive refers.

Node to be Added: GCC User ID of node that is to be added to the pipeline.

Result:  Indicates the level of service the Virtual MCU can provide for the pipeline.  It contains one of the following results: guaranteed service, predictive service, best effort service, service unavailable, invalid pipeline, rejected by owner.

## 26 RTLM-Pipeline-Remove-Sink-Node

The RTLM-Pipeline-Remove-Sink-Node primitive is used to remove a node from a pipeline.  Observe that this may or may not result in the removal of one or more pipes from the pipeline.  No action is required by the owner of the pipeline.  The RTLM-Pipeline-Remove-Sink-Node indication primitive is issued by the Top RTLM Provider if a node is forced to be removed from a pipeline (e.g. due to network problems).

FIGURE 26/T.132

RTLM-Pipeline-Remove-Node - sequence of primitives: centralised management case

FIGURE 27/T.132

RTLM-Pipeline-Remove-Node - sequence of primitives: distributed management case

FIGURE 28/T.132

RTLM-Pipeline-Remove-Node - sequence of primitives: RTLM Provider initiated

TABLE 8/T.132

RTLM-Pipeline-Remove-Node Primitive

| Parameter | Request | Indication | Confirm |
| --- | --- | --- | --- |
| Pipeline Identifier | M | M(=) | M(=) |
| Node to be Removed | M | M(=) | M(=) |
| Result |  |  | M |

Pipeline Identifier: A globally unique number identifying the pipeline to which the primitive refers.

Node to be Removed: GCC User ID of node that is to be removed from the pipeline.

Result: Successful, Invalid Pipeline, Invalid Node.

## 27 RTLM-Pipeline-Modify

The RTLM-Pipeline-Modify primitive is used to modify the QoS characteristics of a pipeline.  If the pipeline is owned, then the pipeline owner is the only node permitted to modify the characteristics.  Any node is permitted to modify the characteristics of an unowned pipeline.

FIGURE 29/T.132

RTLM-Modify-Pipeline - sequence of primitives: centralised management case

FIGURE 30/T.132

RTLM-Modify-Pipeline - sequence of primitives: distributed management case

TABLE 9/T.132

RTLM-Modify-Pipeline Primitive

| Parameter | Request | Indication | Confirm |
| --- | --- | --- | --- |
| Request Handle | M |  | M(=RQ) |
| Pipeline Identifier |  | M(=CF) | M |
| Pipeline QoS | M | M(=CF) | C |
| Result |  |  | M |

Request Handle: Locally unique number to allow the requesting node to match responses.

Pipeline Identifier: A globally unique number identifying the pipeline to which the primitive refers.

Pipeline QoS:  Desired Quality of Service is included in the request.  The actual QoS offered by the Virtual MCU is provided in the confirm.

Result:  Indicates the level of service the Virtual MCU can provide for the pipeline.  It contains one of the following results: not owner, service unavailable, guaranteed service, predictive service, best effort service.

## 28 Requesting a Stream

A node wishing to receive a stream from a particular source first issues a request to the node originating the stream (specifying whether it requires exclusive control of the source e.g. for remote camera control).  The sourcing node may/may not be able to supply the stream due to local constraints (e.g. number of video inputs, processing power) or may choose that it does not wish to supply the requested stream.  The stream may already be being sent to other nodes; if so the sourcing node responds identifying the pipeline on which the stream may be received.  The requesting node may then issue a request to the virtual MCU to join the indicated pipeline.  If the stream is not currently being made available to the conference, the sourcing node requests the creation of a new pipeline, specifying the requesting node (and any other node(s) it may choose) as the endpoint(s).

## 29 RTLM-Pipeline-Information

This primitive allows the owner of a pipeline to broadcast information about the current content of a real time pipeline (i.e. the stream).

FIGURE 31/T.132

RTLM-Pipeline-Information - sequence of primitives

TABLE 10/T.132

RTLM-Pipeline-Information Primitive

| Parameter | Request | Indication |
| --- | --- | --- |
| Pipeline Identifier | M | M(=) |
| Media Type | M |  |
| Media Format | O | O(=) |
| Source Identifier | O | O(=) |
| Stream Descriptor | O | O(=) |

Pipeline Identifier: A globally unique number identifying the pipeline to which the primitive refers.

Media Type: audio, video, real time data

Media Format: Identifies the specific media encoding format e.g. G.728, H.261.

Source Identifier: A globally unique number identifying the device or entity that is sourcing the stream.

Stream Descriptor: A text string to describe the stream content.

## 30 RTLM-Pipeline-Routing-Options

This primitive allows an AVC Service Provider to request a list of possible pipeline routings between a source node and one or more destination nodes and thence to select an appropriate pipeline configuration based on its knowledge of the capability of the endpoints and any intermediate processing nodes.  The requesting entity may optionally specify the QoS and hop limit required for the pipeline and an upper limit on the number of possible routings to be returned.

If the QoS and hop limit requirements cannot be fulfilled, but a physical routing for the pipeline is possible, RTLM will return a best effort routing, indicating the criteria that cannot be met.

FIGURE 32/T.132

RTLM-Pipeline-Routing-Options - sequence of primitives: centralised management case

FIGURE 33/T.132

RTLM-Pipeline-Routing-Options - sequence of primitives: distributed management case

TABLE 11/T.132

RTLM-Pipeline-Routing-Options Primitive

| Parameter | Request | Confirm |
| --- | --- | --- |
| Request Handle | M | M(=RQ) |
| Source | M |  |
| Sinks | M |  |
| Media Type | O |  |
| Pipeline QoS | O |  |
| Hop Limit | O |  |
| Route Ordering Criteria | O |  |
| Route Limit | O |  |
| Routing List |  | C |
| Result |  | M |

Request Handle: Locally unique number to allow the requesting node to match responses.

Source: The node that acts as the pipeline source.

Sinks: A list of nodes that will act as sinks for the pipeline.

Media Type: audio, video, real time data

Pipeline QoS:  Desired Quality of Service for the pipeline.

Hop Limit:  Maximum number of hops between the source node and any sink node

Route Selection Criterion: Preferred criterion for selection of routes.  Options available are:

minimise number of hops between source and sink nodes

maximise pipeline QoS

Route Limit: Maximum number of routes to be returned.

Routing List: A list of pipeline routings that meet the requested pipeline QoS and hop limit requirements and which are available at the time the request was processed.  One or more of these routings may subsequently become unavailable due to competing pipeline requests.  Each pipeline routing comprises a list of real time links and an indication of whether the QoS is guaranteed, predictive or best effort.  Pipeline routings are arranged in descending order of suitability according to the route selection criterion specified.

Result:  Indicates whether a pipeline can be established.  It contains one of the following results: successful, no routing available, requested QoS unavailable, requested hop limit unavailable.

# 31 RTLM Database

RTLM maintains a database for core AVC information (capabilities and configurations) that may be of use to entities (AVC Service Providers) that make use of its services to deliver enhanced audio-visual control services.  It is recognised that different AVC services require access to different levels of information (for example a path routing service may only require access to pipeline information, whereas a video switching service would require direct access to pipe information).  Maintaining different subsets of information within each of the AVC Service Providers would be inefficient (because of duplication of information and the volume of information being passed between RTLM and each of the AVC Service Providers) and error prone (due to the possibility of the databases getting out of sync.)  RTLM therefore provides a single database to service all AVC entities and regulates write access in order to minimise database conflicts.  RTLM will allow AVC Service specific information to be maintained in the database, to avoid the need to establish separate databases.  For example, RTLM allows paths to be registered to support the provision of a path routing service.

To minimise traffic, clients of RTLM are not automatically notified of changes to the database.  They can, however, request automatic updates on selected portions of the database through an RTLM monitoring service.

## 32 Routing Roster

Each RTLM Provider within the Virtual MCU maintains a database of the current configuration of links within a conference, termed the routing roster.  For each active communications path, a list of the component pipelines is maintained.  Similarly, for each pipeline, a list of the component pipes is maintained.

This information may be accessed by AVC Service Providers that make use of the services of RTLM.  For example, a video switching service needs to identify the pipes used to convey the streams it wishes to switch.  Entries may be referenced by path, pipeline or pipe.  Pipeline and pipe entries may only modified indirectly by making use of the RTLM pipeline management primitives.  Path entries may be modified directly by MNS Providers to indicate use of network elements (e.g. transcoders) within a path.

Cross-referencing of records should be supported e.g. a retrieved pipe record should identify the pipeline and path it belongs to.

Nodes may register an interest in specific paths, pipelines, pipes or links in order that they may receive notification of any changes (e.g. in path routing & quality of service, link configuration) that occur.

Since the routing roster is maintained at all nodes within the virtual MCU, routing roster requests should be directed to the nearest RTLM Provider within the Virtual MCU.

TABLE 12/T.132.

Content of routing roster path entry

| Parameter |  |
| --- | --- |
| Path Identifier | M |
| Media Type | M |
| Stream Descriptor | O |
| Pipeline List | M |
| Network Element List | O |
| Path QoS Characteristics | O |
| List of Associated Paths | O |

Path Identifier: A globally unique number identifying the path.

Media Type: audio, video, real time data

Source Identifier: A globally unique number identifying the device or entity that is sourcing the stream.

Stream Descriptor: A text string to describe the stream content.

Pipeline List: List of pipelines used to make up the path.

Network Element List:  List of network elements within the path (e.g. transcoders), used to connect pipelines with different characteristics (e.g different video format).

Path QoS Characteristics: Either expressed explicitly in terms of media format and transmission requirements or  in ‘form factor’ notation.

TABLE 13/T.132.

Content of routing roster pipeline record

| Parameter |  |
| --- | --- |
| Pipeline Identifier | M |
| Media Format | M |
| Pipe List | M |
| Pipeline QoS Characteristics | O |

Pipeline Identifier: A globally unique number identifying the pipeline.

Media Format: Identifies the specific media encoding format e.g. G.728, H.261.

Pipe List: List of pipes used to make up the pipeline.

Pipeline QoS Characteristics:

TABLE 14/T.132.

Content of routing roster pipe record

| Parameter |  |
| --- | --- |
| Pipe Identifier | M |
| Pipe QoS Characteristics | O |

Pipe Identifier: A globally unique number identifying the pipe (and thus the link of which it forms a part).

FIGURE 34/T.132

Relationship between  path, pipeline, link and pipe records

The following RTLM Database Management primitives are provided for the routing roster:

RTLM-Roster-Monitor

RTLM-Roster-Retrieve-Entry

RTLM-Roster-Register-Path

RTLM-Roster-Delete-Path

### 33 RTLM-Roster-Monitor

RTLM provides a mechanism for monitoring changes to path, pipeline, pipe and link attributes.  By default this information is only provided automatically to nodes within the Virtual MCU in order that they can maintain an up-to-date routing roster.  The RTLM-Roster-Monitor request primitive may be used to enable/disable monitoring of a specific path, pipeline, pipe, or link. Monitoring may be enabled/disabled for multiple entities of the specified type in a single request (e.g. to enable monitoring of all video pipelines) by qualifying the request appropriately.  While enabled, the requesting entity is notified of all changes to the specified roster entries via the RTLM-Roster-Monitor indication primitive.  For example, if monitoring is enabled for a pipeline, an indication will be generated as a result of a node joining or leaving the pipeline,  modification of the QoS characteristics of the pipeline, or closing of the pipeline.

Permission to issue the RTLM-Roster-Monitor request primitive may be restricted by the conference convenor and/or conductor.

FIGURE 35/T.132

RTLM-Roster-Monitor - sequence of primitives

TABLE 15/T.132

RTLM-Roster-Monitor Primitive

| Parameter | Request | Indication | Confirm |
| --- | --- | --- | --- |
| Request Handle | M |  | M(=RQ) |
| Roster Key | O | M | O(=RQ) |
| Enable/Disable | M |  | M(=RQ) |
| Media type | O | O | O(=RQ) |
| Roster Entry |  | M |  |
| Result |  |  | M |

Request Handle: Locally unique number to allow the requesting node to match confirms.

Roster Key: Specifies the record(s) to be monitored.  A Roster Key consists of two components, one (Entity Type) which identifies the type of record to be monitored (path, pipeline, pipe or link) and the other which uniquely identifies the specific entity to be monitored.  If the latter is omitted, monitoring will be enabled/disabled for all entities of the specified type.   The media type parameter may be used to provide further qualification of the entries to be monitored.

Enable/Disable: TRUE to deliver indications of changes, FALSE to suppress indications.

Media Type: Audio, Video or Real Time Data

Roster Entry: Describes the entity whose attributes have changed

Result: Successful, insufficient privilege, entry not found.

### 34 RTLM-Roster-Retrieve-Entry

The RTLM-Roster-Retrieve-Entry request primitive may be used to determine the contents of a single routing roster entry.  The primitive may be issued at any time.

FIGURE 36/T.132

RTLM-Roster-Retrieve-Entry - sequence of primitives

TABLE 16/T.132

RTLM-Roster-Retrieve-Entry Primitive

| Parameter | Request | Confirm |
| --- | --- | --- |
| Request Handle | M | M(=) |
| Roster Key | O | O(=) |
| Roster Entry |  | C |
| Result |  | M |

Request Handle: Locally unique number to allow the requesting node to match confirms.

Roster Key: Specifies the record(s) to retrieve.  A Roster Key consists of two components, one (Entity Type) which identifies the type of record to be monitored (path, pipeline, pipe or link) and the other which uniquely identifies the specific entity to be monitored.  Both components must be provided (unless we wish to allow retrieval of multiple entries with a single request).

Roster Entry: Describes the entity whose attributes have changed

Result: Successful, insufficient privilege, entry not found.

### 35 RTLM-Roster-Register-Path

The RTLM-Roster-Register-Path request primitive is intended for use by an AVC Provider that offers a high-level path routing service.  In order to establish a logical path between a source and one or more destinations it is necessary to create one or more pipelines.  This primitive allows an AVC Provider to register the pipelines that make up a path, and identify any associated network elements that may be required within the path.  The primitive may be issued at any time.  If an entry already exists in the routing roster for the specified path identifier and the requester is a suitably authorised node (authorisation criteria tbd) then the path record is overwritten with the contents of this primitive.

FIGURE 37/T.132

RTLM-Roster-Register-Path - sequence of primitives

TABLE 17/T.132

RTLM-Roster-Register-Path Primitive

| Parameter | Request | Confirm |
| --- | --- | --- |
| Request Handle | M | M(=) |
| Path Identifier | O | O(=) |
| Pipeline List |  | C |
| Result |  | M |

Request Handle: Locally unique number to allow the requesting node to match confirms.

Path Identifier: Unique identifier for the pipe being registered.

Pipeline List: List of pipelines used to make up the path.

Result: Successful, insufficient privilege, invalid pipeline(s)

### 36 RTLM-Roster-Delete-Path

The RTLM-Roster-Delete-Path request primitive may be used to remove a single path entry from the routing roster.  This primitive may only be issued by the owner of the path or an authorised node (authorisation criteria tbd).

FIGURE 38/T.132

RTLM-Roster-Delete-Path - sequence of primitives

TABLE 18/T.132

RTLM-Roster-Delete-Path Primitive

| Parameter | Request | Confirm |
| --- | --- | --- |
| Request Handle | M | M(=) |
| Roster Key | O | O(=) |
| Result |  | M |

Request Handle: Locally unique number to allow the requesting node to match confirms.

Roster Key: Specifies the record(s) to remove.  A Roster Key consists of two components, one (Entity Type) which identifies the type of record to be monitored (path, pipeline, pipe or link) and the other which uniquely identifies the specific entity to be monitored.  Both components must be provided.

Result: Successful, insufficient privilege, entry not found.

## 37 Capability Roster

Associated with the routing roster is the capability roster, which contains the media specific capabilities of each node.  All nodes within the Virtual MCU maintain a routing roster for the entire conference and a capability roster for locally connected links.  Requests for a node’s capabilities may be directed to the node concerned or any node which has a direct connection to the node.

The following capability roster primitives are provided:

RTLM-Retrieve-Node-Capability

RTLM-Monitor-Node-Capability

### 38 RTLM-Retrieve-Node-Capability

The RTLM-Retrieve-Node-Capability request primitive may be used to determine the media specific capabilities of a single link.  The primitive may be issued at any time to the node whose capabilities are being requested, or any other node which has a direct real time link to that node.

FIGURE 39/T.132

RTLM-Retrieve-Node-Capability - sequence of primitives

TABLE 19/T.132

RTLM-Retrieve-Node-Capability Primitive

| Parameter | Request | Confirm |
| --- | --- | --- |
| Request Handle | M | M(=) |
| Node Identifier | M | M(=) |
| Node Capability Table |  | C |
| Result |  | M |

Request Handle: Locally unique number to allow the requesting node to match confirms.

Node Identifier: The GCC User ID of the node whose capabilities are being requested.

Node Capability Table:  Contains capability records for each media format supported by the node.  It may optionally include a list of valid operating configurations (i.e. combinations of media formats that can be supported simultaneously).

Result: Successful, invalid node, capabilities not held at this node.

### 39 RTLM-Node-Monitor

RTLM provides a mechanism for monitoring changes in node capabilities.  While enabled, the requesting entity is notified of all changes to the specified node capabilities via the RTLM-Node-Monitor indication primitive.

Permission to issue the RTLM-Node-Monitor request primitive may be restricted by the conference convenor and/or conductor.

FIGURE 40/T.132

RTLM-Node-Monitor - sequence of primitives

TABLE 20/T.132

RTLM-Node-Monitor Primitive

| Parameter | Request | Indication | Confirm |
| --- | --- | --- | --- |
| Request Handle | M |  | M(=RQ) |
| Node Identifier | M | M | M(=RQ) |
| Enable/Disable | M |  | M(=RQ) |
| Node Capability Table |  | M |  |
| Result |  |  | M |

Request Handle: Locally unique number to allow the requesting node to match confirms.

Node Identifier: The GCC User ID of the node to be monitored.

Enable/Disable: TRUE to deliver indications of changes, FALSE to suppress indications.

Node Capability Table:  Contains capability records for each media format supported by the node.  It may optionally include a list of valid operating configurations (i.e. combinations of media formats that can be supported simultaneously).

Result: Successful, invalid node, insufficient privilege, capabilities not held at this node.

# Annex I

Real Time Link Examples

(this annex  forms an integral part of this Recommendation)

The following figure illustrates the scope of RTLM influence on networks where real time streams are transported with T.120 data over a single multiplex.  In such cases the partitioning of bandwidth between real time streams and other signals is managed by the AVC QoS manager.  The AVC QoS Manager determines the total bandwidth requirement for T.120 data and real time streams.

END OF T.132