| Question: 10/8 INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- |
| TELECOMMUNICATION STANDARDISATION SECTOR STUDY PERIOD 1993 - 1996 | COM 8-__ February 1996 Original: English 31 July, 2026 |

|Geneva 6th - 15th February 1996

STUDY GROUP 8 – CONTRIBUTION

| SOURCE: | BT |
| --- | --- |

| CONTACT: | John Boucher / Trevor Peers, BT Labs, Martlesham Heath, IPSWICH, Suffolk, UK IP5 7RE. | Tel: +44 1473 644354 / +44 1473 647697 Fax: +44 1473 643791 E-mail:j_boucher@bt-web.bt.co.uk E-Mail: peers_t_w@bt-web.bt.co.uk |  |
| --- | --- | --- | --- |
| TITLE: | Draft recommendation T.130 Real-time Architecture for Multimedia Conferencing |  |  |

___________________

Real Time Architecture for Multimedia Conferencing

Draft Recommendation T.130

# Summary

This recommendation defines the Audio Video Control System architecture and  describes the system model showing the inter-relationships between it’s constituent parts.  It provides an overview of each of the recommendations, collectively they define the control of audio and video communication within a T.120 conferencing environment..

Draft Recommendation T.130

Real-time Architecture for Multimedia Conferencing

CONTENTS

Summary

1. Scope

2. Normative References

3. Definitions

4. Abbreviations

5. Overview of Multipoint Audio and Video Communications

6. Introduction to the  Real-Time Audio Video Control  System

6.1. Uniform Control of Real-Time Streams

6.2. Cross Network Stream Control Protocols

6.3. Processing Elements As Part of the Network Infrastructure

6.4. Quality Of Service in Multimedia Communications

6.5. Distributed Management

7.   Real Time Audio Visual Control Architecture

7.1. Provisions

7.2. The RAMC Protocol Set - System Model

7.3. Terminology

8. Introduction to the Infrastructure Recommendations

8.1. Real Time Link Management - T.132 (RTLM)

8.1.1. Real Time Link Quality of Service - Infra-structure QoS

8.1.2. The Virtual MCU.

8.2. Network Specific Mappings (NSM)

8.2.1. Network Specific Mapping for PSTN

8.2.2. Network Specific Mapping for ISDN

8.2.3. Network Specific Mapping for LAN

8.2.4. Network Specific Mapping for ATM

8.2.5. Network Specific Mapping For DSM-CC controlled Environment.

9. Introduction to the Audio Visual Control Services Recommendation

9.1.  Stream Routing Service (SRS)

9.2. Remote Device Control (RDC)

9.3. Audio Video Conference Services (AVCS)

9.3.1. Audio and video source selection.

9.3.2. Video Switching Service

9.3.3. Continuous Presence Video Service

9.3.4. Audio Mixing Service

9.3.5. Audio and video source identification

9.3.6. "On Air" Indication Service

9.3.7. Audio and Video Floor Control Service

9.3.8. Privacy

ANNEX A  EXAMPLES OF  AVC ROUTE ESTABLISHMENT

Real-time Architecture for Multimedia Conferencing

# 1 Scope

This Recommendation introduces a set of standards, collectively referred to as  Real-time Architecture Multimedia Conferencing (RAMC),  which together provide the management and control of real time streams within a multipoint communication environment.

This Recommendation describes the system architecture and its functional elements and illustrates the relationship with other parts of the T.120 series. It also provides an introduction to and functional description of the recommendations that go to make up the AVC system.

This Recommendation defines the criteria for compliance when AVC is used in a conferencing or group-working environment for multipoint communications.

# 2 Normative References

The following Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid.  All Recommendations and other references are subject to revision: All users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below.  A list of the currently valid ITU-T Recommendations is regularly published.

–	ITU-T Recommendation T.120 (1996), Transmission Protocols for Multipoint Data Conferencing.

–	ITU-T Recommendation T.122 (1993), Multipoint Communication Service for audiographics and audiovisual conferencing service definition.

–	ITU-T Recommendation T.123 (1994), Protocol Stack for Audiographics and Audiovisual Teleconference Applications.

–	ITU-T Recommendation T.124 (1995), Generic Conference Control.

–	ITU-T Recommendation T.125 (1994), Multipoint Communication Service protocol specification.

# 3 Definitions

For the purposes of this Recommendation, the following definitions apply:

Adjacent Node: An adjacent node is a node that forms one of a pair of nodes that are connected and thus associated by a real time link.

Application Protocol: Any standard or non-standard protocol specification that makes use of T.120 services.

Application Protocol Entity: The instantiation of an Application Protocol in a terminal or MCU.

Application Protocol Session: A group of peer Application Protocol Entities communicating with each other.

Controllable Device: A device that has the capability to be controlled remotely in a conference environment.

Communications Port: A physical interface at a node that possesses AVC media capabilities and is able to provided a connection to a real time link.

Device: An entity which provides a specific function, or set of functions, to the conference. Some devices such as cameras and microphones are capable of generating a source to the conference. Other devices provide control and processing functions for streams.

Inter-MCU-Link: A real time link that interconnects two adjacent MCU nodes.

MCU (Multipoint Control Unit): A device used to interconnect multiple terminals and/or further MCUs to allow multiple participants to be connected to single  conference.

Multipoint Service: One or more functions that are provided and managed by MCUs.

Node: Any entity in the network that runs the T.120/T.132 (RTLM) protocols. This includes both terminal equipment and network elements such as routers and bridges (otherwise known as MCUs).

Pipe: A communications path within a real time link that carries a single stream between adjacent nodes.

Pipeline: A collection of connected  pipes used to convey a real time stream from its source to one or more destinations.

Real Time Link: A connection between two adjacent nodes used for the transfer of real time information in a single direction.  A real time link may provide one or more communication paths (termed pipes), each of which conveys a single stream between the two nodes.

Source: The origin of a real time media stream.

Stream: A flow of real time information of a specific media type (e.g. audio) and format (e.g. G.722) from a single source to one or more destinations.

Video Processor: A device that modifies the content of video streams, e.g. a transcoder, continuous presence processor.

Virtual MCU: Term used to describe the collection of MCUs within a real time conference which present a single logical interface to terminals.

User Application: The instantiation of an Application Protocol in a terminal or MCU.

# 4 Abbreviations

APE			Application Protocol Entity

AVC		Audio-Visual Control

ATM		Asyncronous Transfer Mode

B-ISDN		Broadband Integrated Services Digital Network

GCC		Generic Conference Control

ISDN		Integrated Services Digital Network

LAN		Local Area Network

MCS		Multipoint Communication Service

MCU		Multipoint Control Unit

RTLM		Real Time Stream Control

PDU		Protocol Data Unit

PSDN		Packet Switched Digital Network

PSTN		Public Switched Telephone Network

TCP			Transmission Control Protocol

MCS	Multipoint Communication Service

MCU	Multipoint Control Unit

PDU	Protocol Data Unit

SAP	Service Access Point

# 5 Overview of Multipoint Audio and Video Communications

Traditionally, telephony services have been constrained to point-to-point operation. In order to support group activities such as meetings and conferences involving physically separated participants, there is a requirement to join together more than two locations; the term multipoint communication simply describes the interconnection of multiple terminals.  Normally, a special network element known as a Multipoint Control Unit (MCU), or more simply a bridge, is required in order to provide this function.  Much of the functionality specified by the RAMC set of  Recommendations relates to or depends upon MCUs for its provision.

A multipoint video conference involves  a number of suitably equipped conference terminals inter-connected using an MCU. Conferencing terminals may take a number of different forms. A common form of video conference terminal is room based.  It may consist of one or more video displays, with one or more video cameras to capture face images, possibly supplemented by document cameras and other video sources.  One or more microphones capture the speech.  More formal environments may dictate the use of individual microphones for participants.  Another form of conference terminal is the  'personal or 'desk-top' terminal which typically supports a single participant.

# 6 Introduction to the  Real-Time Audio Video Control  System

The Real-time Audio Video extensions to T.120 consists of a suite of recommendations that encompass the necessary mechanisms to establish a platform to support real time multimedia communications applications.  The RAMC series build on existing ITU-T conferencing recommendations that have gained wide acceptance.  These include the H.320 audio-visual conferencing series and the T.120 series of collaborative data conferencing recommendations.  The RAMC comprises a set of  recommendations that establish an architecture and protocols to support the use of real time streams in applications.

The value of RAMC is found in the unique problems that it solves. These can be categorised as follows:

- Uniform control of real-time streams in a network and platform indepenedent manner

- Facilities for cross network bridging of data stream protocols.

- Facilities for using processing elements as part of the network infrastructure.

- Delivering Quality of Service  to multimedia communications applications.

- Distributed management philosophy

Each of these features are described below:

## 7 Uniform Control of Real-Time Streams

Currently there are several standardised methods for managing communication of real time streams.  On ISDN connections, H.242 and H.243 provide limited multipoint control functionality for conferencing applications, whilst on PSTN, ATM and LAN,  H.245 offers a point-to-point control mechanism. The RAMC set provides a single uniform  platform independent control mechanism for managment of real time streams over multipoint connections.  It also incorporates mappings to all networks that are to be supported.

## 8 Cross Network Stream Control Protocols

Most network boundaries require protocol bridging elements that currently have no standard methods for control.  This means that it is difficult to embrace such connections seamlesslly in a communications session.  RAMC  defines a control protocol to enable streams to be routed across network boundaries, whilst providing consistent communication characteristics.

## 9 Processing Elements As Part of the Network Infrastructure

Processing elements are key components which may be used to bridge between  nodes which support a common media type (e.g. video), but different format.  For example, an MPEG based video service cannot communicate directly with an H.320 terminal that uses H.261 for its video coding.  However, by inserting a transcoding element in the communication path between these nodes, they can both operate within the range of their capabilities, and communicate with each other.  RAMC  provides management services to establish real time communications paths with routing via processing elements, as required. These paths allow terminals to communicate in environments that previously had no support for functionally different terminals.

## 10 Quality Of Service in Multimedia Communications

Most Quality of Service (QoS) mechanisms available prior to RAMC provide some means to request and receive communication paths with an associated QoS.  The use of these mechanisms requires the application to have knowledge of the network and of the specific details of the codecs that create the streams flowing through the network.  Applications desire the capability to request a certain audio or video usage requirement, and the exact communication QoS requested from the network to be hidden.  When streams cross network or functional boundaries, the QoS shall be arbitrated amongst all applications wishing to share the same resources.  Mechanisms to support this are provided in RAMC by QoS services that handle the translation of generic QoS requests from applications and manage other QoS interfaces and events that occur during the use of a dynamic real-time network.  All this is done in such a way that the application need only be concerned that its audio/video requirements will be fulfilled.

## 11 Distributed Management

All of the services above are defined in a way that will allow distributed management.  If desired, networks can be configured and managed such that a node only need be concerned with the communication paths that it will use.  The remainder of the network can be ignored.  If desired, the network can also be configured with a more centralised control to allow tighter control of valuable communication resources.

# 12 Real-time Architecture for Multimedia Conferencing.

The RAMC set of recommendations set out procedures for the management and control of real-time streams (for example audio and video streams) within a conferencing environment.  Whilst the real time stream control protocols are conveyed within T.120 data channels (i.e. MCS channels), the streams themselves are transported in independent pipelines due to the transmission requirements of real time information flows.

## 13 Provisions

The infrastructure to support the management of real time streams is designed to be independent of the underlying network, although there are components of the RAMC set that define the interaction between the generic real time stream control mechanism and existing network specific control protocols.

The RAMC set has provisions for:

- establishing a real time stream control infrastructure

- dynamically arbitrating link capabilities

- establishing and managing pipelines to transport real time streams

- quality of service management to preserve the integrity of real time streams

- stream access control and registration

- interaction with existing network specific real time stream control protocols

- remote device control

- multipoint network based services

The RAMC set supports multiple real time stream sources (transmitters) and sinks (receivers) at each node and allows each node to send and receive multiple streams, with no requirement for symmetry.

## 14 The Real-time Architecture - System Model

The RAMC model is comprised of a real time stream control infrastructure and the network services that make use of it.  Figure 1 serves to show both the scope of the RAMC set of recommendations (indicated by the dark shaded background) and the relationship between each of the recommendations and other components in the system.  It also illustrates the relationship with the T.120 suite of recommendations

FIGURE 2/T.130

Architecture Model  for the Real Time Audio Video Extensions

## 15 Terminology

The Multipoint Real Time Architecture is built up from a  number of elements to provide an unambiguous nomenclature for controlling a conference.  This section defines the elements of the model.

The logical connection between two adjacent nodes that is used to convey real time information between those nodes is termed a real time link.  Multiple real time links between adjacent nodes are permitted if there are multiple independent physical connections between those nodes.  A real time link may be subdivided into a number of pipes.  A pipe is used to convey a single uni-directional flow of information of a specific media type (e.g audio or video) between the two nodes.  Each independent information flow requires its own pipe.

The number of pipes within a link and the transmission characteristics of those pipes can be varied dynamically throughout the course of a conference.  There is no requirement for symmetry within a link, thus the number of pipes in each direction and the number per media type may differ.

FIGURE 3/T.130

Typical Link

A stream is a uni-directional flow of real time information from a specified origin to one or more destinations. The logical communication path used to transport a stream end to end is termed a pipeline.  The pipeline itself is comprised of one or more pipes, each of which is used to convey the stream over a single link between the source and its destination(s).

The origin of a stream is termed its source.  A source may either be a physical device such as a camera or microphone, or it may be some other entity that is introduced into the conference such as an 'off-air' signal.  The destination of a stream is termed a sink.

This suite of recommendations imposes no requirement for the flow of audio and video information between adjacent nodes to share the same physical link, although this would typically be the case.  Similarly, the T.120 data channel to control the real time stream infrastructure may be routed independently.

FIGURE 4/T.130

Stream flow to multiple destinations

# 16 Introduction to the Infrastructure Recommendations

Figure 1/T.130 shows the three component recommendations that comprise the audio video extensions T.132 (RTLM) and T.NSM are infra-structure recommendations and described in this section. T.133 is an Application protocol and is described in section 9.

## 17 Real Time Link Management - T.132 (RTLM)

Note- This recomendation was formerly known as T.RTSC but has been renamed to more correctly reflect its functionality.

The RAMC Audio-Visual Control Architecture is based upon the provision of a Real Time Link Management (RTLM) as defined in T.132 (RTLM).  This service supports the establishment of multiple logical communication paths, termed Pipelines which allow real time streams to be routed from a source to one or more destinations.  RTLM facilitates the creation of multipoint real time pipelines by providing a routing overlay on the conference network topology.  It uses a distributed management architecture to allow use in environments with large numbers of end points.

The RAMC routing infrastructure is derived from the conference topology and may have a different topology for each media type if the real time streams are routed independently. The RAMC routing infrastructure is not required to map directly onto the T.120 topology and allows each node to have a direct connection to any other node within the conference.

The following terms are used to describe the building blocks that form the basis of the RTLM infrastructure:

Unicast Real Time (URT) Link: a connection between two nodes, either terminals or MCUs.  The link will consist of one or more  pipes, each of which is used to convey a single flow of information of a specific media type and format (e.g.  G.711 audio, H.261 video) in a single direction.

Multicast Real Time (MRT) Link: a connection between one node and a group of commonly connected nodes.  It is defined to allow the mapping of a multicast topology (e.g Local Area Network) into the RTLM environment.

FIGURE 5/T.130

Example of T.132 (RTLM) Network Topology

T.132 (RTLM) provides the following features:

- A link capability negotiation mechanism, which determines the transmission capabilities of each link in the conference and media types (e.g. audio, video) which can be conveyed over each link.  The link negotiation is performed when the RTLM infrastructure is established, but may also be carried out at any subsequent time if there is a change of capabilities.  This enables RTLM to build up a map of the routing infrastructure describing the connectivity of bandwidth between nodes in the conference.  Overlaid on the routing infrastructure is the capability infrastructure which describes the media types that can be conveyed on each link.  This includes media specific encodings (e.g video compression algorithms) and will support dependent capabilities.  The routing infrastructure is held at all switching nodes (MCUs) within the conference, with each MCU maintaining only that portion of the capability infrastructure that applies to its immediately connected links.  A sample link capability record is illustrated in figure 6.

- Inheritance of existing connections and configurations from network specific real time control protocols.

- A real time pipeline creation and routing service, to a specified Quality of Service (QoS).  The pipeline management primitives are similar to those provided by MCS and support:

- Pipeline create (activating the pipeline)

- Pipeline closing (de-activating the pipeline)

- Pipeline disbanding (removal of the pipeline and all records associated with it)

- Pipeline joining

- Pipeline leaving

- Admission control for private pipelines

- A routing roster which allows nodes to register an interest in specific media paths, pipelines, pipes or links in order that they may receive notification of any changes (e.g. in routing, quality of service) applied to those entities.

FIGURE 7/T.130

Example of link capability record

### 18 Real Time Link Quality of Service - Infra-structure QoS

RTLM allows applications to request Quality of Service (QoS) parameters for real time streams and defines a  notification mechanism to inform applications of changes to QoS parameters.  The following QoS parameters are supported:

- Source Traffic Description - The manner in which the application's traffic will be injected into the network. This specification is made in terms of a token bucket traffic model and includes values for the token rate (i.e. mean bandwidth), the token bucket size, and the peak bandwidth.  Note that the bandwidth requirement being expressed in terms of a token rate does not mean that hosts must implement token buckets.  Any traffic management scheme that yields equivalent behaviour is permitted.

- Latency - Upper limits on the amount of delay and delay variation that are acceptable.

- Level of service guarantee -  Whether or not an absolute guarantee is required as opposed to best effort.  Note that providers which have no feasible way to provide the level of service requested are expected to reject the request.

- Provider-specific parameters - The structure itself can be extended in ways that are particular to specific providers.

QoS parameters for a pipeline are established when the pipeline is opened and may be modified at any point during its lifetime.  It is possible that a QoS request may be rejected due to limited resources; in such cases the requesting application may scale down its QoS requirements and try again or simply give up.

After every attempt to open a pipeline (successful or otherwise), RTLM providers update the associated QoS structures to indicate the current link conditions.  Clients expect to be able to use this information about current link conditions to guide their use of the connection, including any subsequent QoS requests.  Note however, that information provided by the link service provider in the updated QoS structure only applies to the local link as opposed to the complete end-to-end connection or set of links.

Even after a pipeline is established, conditions in the network may change or one of the communicating parties may invoke a QoS renegotiation which results in a reduction (or increase) in the available service level.  A notification mechanism is included to allow the RTLM provider to indicate to the client that QoS levels have changed.  If the updated level of service is not acceptable, the client may adjust itself to accommodate it, attempt to renegotiate QoS, or close the pipeline.

### 19 The Virtual MCU.

By treating the switching elements (MCUs) within a conference as a single logical entity (a Virtual MCU) as illustrated in Figure 4, it is possible to present a single logical interface between the network infrastructure and terminals, thus simplifying interactions between the two elements.

To achieve this, an MCS channel (the MCU Broadcast Channel) is reserved for communications between terminals and the Virtual MCU. In this way, terminals have a standard way to communicate with the network  and its MCU elements, independent of the conference network topology.  A second MCS channel (the MCU Control Channel) is reserved for communications between the MCUs that comprise the Virtual MCU.

Distributed management of the Multipoint Real Time Link Infrastructure is the preferred approach, to facilitate a scalable architecture, although there may be a requirement for centralised control of specific functions.  In the latter case, the MCU with ultimate authority is referred to as the Top Provider.

FIGURE 8/T.130

The Virtual MCU

## 20 Network Specific Mappings (NSM)

The RTLM service provides a generic network independent mechanism for managing real time streams.  In order to realise some of the RTLM functionality, it is necessary for RTLM to interact with existing network specific control protocols such as H.245.  Typically, these are point-to-point protocols which provide a capability exchange mechanism and limited real time stream control functionality.  RTLM inherits the capabilities negotiated by the network specific protocols on each link and maps these point-to-point  connections into a multipoint environment.  To achieve control of real time streams, RTLM may make use of the underlying network specific protocol where appropriate.  A mapping between RTLM and each network specific control protocol is thus required to specify how the control elements interact to achieve the required functionality.Each of the Network Specific Mapping (NSM) components provide a mapping of the RTLM infrastructure onto a specific network i.e. PSTN, ISDN, LAN and ATM.  They specify the interaction of the RTLM infrastructure with the appropriate network multiplex control element and define procedures for the following functions:

- Establishment of RTLM session

- Mapping of point-to-point connections into a multipoint environment

- Inheritance by T.132 (RTLM) of link capabilities arbitrated by network specific capability exchange mechanisms

- Interaction between T.132 (RTLM) and network multiplex to achieve QoS management

- Dynamic adjustment of link parameters

- Termination of T.132 (RTLM) session

On some networks, audio and video communications may be established before RTLM is activated.  In such cases, RTLM inherits the capabilities and configurations negotiated by the underlying network specific control protocol (e.g. H.242 on ISDN).  The Network Specific Mapping translates the negotiated capabilities and real time link configuration into a network independent format which can be understood by RTLM.  RTLM is thus made aware of any existing audio and video streams within the conference and the capabilities of each of the real time links it is to control.  Any existing real time pipeline routing will be maintained in the absence of RTLM intervention.

On some networks, RTLM may be established before real time audio and video communications.  In such cases, RTLM is responsible for performing the initial link negotiation process.

In order to establish new real time pipelines or to modify the characteristics of existing pipelines, RTLM needs to control the configuration of each real time link.  The NSM is thus required to map generic RTLM link configuration commands into specific configuration commands appropriate to the network in use.

### 21 Network Specific Mapping for PSTN

This  specifies the interaction between T.132 (RTLM), T.133 and H.245 in a PSTN environment.

### 22 Network Specific Mapping for ISDN

This part  specifies the interaction between T.132 (RTLM), T.133 and H.242/H.243 and identifies those parts of H.243 that are superseded by the provisions of the RAMC series.

### 23 Network Specific Mapping for LAN

This part specifies the interaction between T.132 (RTLM), T.133 and H.245 in a LAN environment.

### 24 Network Specific Mapping for ATM

This part specifies the interaction between T.132 (RTLM), T.133 and H.245 in an ATM environment.

### 8.2.5. Network Specific Mapping For DSM-CC controlled Environment.

This part specifies the interaction between T.132 (RTLM), T.133 and DSM-CC in an ATM environment.

# 25 Introduction to the Audio Visual Control Services Recommendation

Whilst the Multipoint Real Time Extensions infrastructure recommendations provide mechanisms for establishing and managing real time pipelines, the Audio Visual Control Services Recommendation(T.133)  provides mechanisms for manipulating the streams that flow through those pipelines and control of the devices that source and sink the real time information. The recommendation is comprised of three parts:

- Stream Routing  Service and Resource Management

- Remote Device Control Services (RDC)

- Audio Visual Conference Services (AVCS)

## 26  Stream Routing Service (SRS)

A key element of the core service is to manage the creation and manipulation of real time streams in a heterogeneous network environment.  This function is performed by the AVC Stream Routing Service (SRS).  SRS allows users to request a path between a source node and one or more destination nodes for the distribution of real time information.  The requesting application need only specify the endpoints to which a stream is to be delivered.  SRS identifies whether any intermediate processing (e.g transcoding) is required and determines the routing of the stream, utilising the pipeline management services provided by RTLM.

Applications may specify Quality of Service parameters for a path.  These are tailored to the specific media types (audio, video or real time data) and may be expressed in generic terms, independent of encoding format.  For example, the QoS criteria for a path conveying a video stream may include frame rate, resolution, transmission delay and jitter.  This method of QoS definition allows the applications to be network and platform independent.  SRS does not preclude applications which have knowledge of the network and node characteristics from specifying explicit QoS related parameters  (e.g. H.261, CIF, 15 frames/second).

The routing used to realise a path may require media processing elements such as transcoders, mixers and switches.  The inclusion of such devices may depend on the requested QoS and the capabilities of the conference nodes.

Demands on network resources to service media paths may exceed the transmission capability of the network.  Similarly, processing elements may not be available to service a particular request.  In such circumstances SRS arbitrates between competing requests for limited resources.  The algorithms used for arbitration are outside the scope of this recommendation but may take account of criteria such as conference requirements (e.g. the relative priorities of media paths) and network constraints.  QoS decisions may be made either as a result of user requests or a change in network characteristics.

In addition  to providing a path creation service, SRS allows existing paths to be modified or destroyed.  It also provides a monitoring service to allow interested nodes to be notified of changes in characteristics of a path.  All SRS services may be managed in a centralised or distributed manner to allow flexibility and  scalability.

These services differ from application protocols currently defined (such as T.126/7) since they are essentially client-server (with the virtual MCU as server and terminals as clients) rather than peer-peer. In order to use some of these services, it is necessary for a service to be supported by at least one MCU and one or more terminals.

## 27 Remote Device Control (RDC)

RDC defines  protocol to support remote control of devices within a group working or conferencing environment.  It defines a number of standard device types such as cameras and microphones, each of which has a number of associated control attributes.  It has provision for :-

- selection of audio and video sources

- configuration and control of devices

RDC is a part of the AVC application protocol which makes use of the services of both MCS and GCC. Remote device control may be used to configure and control a device at a remote location in a T.120 environment.

A number of standard device classes are available to construct a virtual device. These classes aim to address mandatory or commonly required functionality for use in support of multipoint multimedia communication and in particular conferencing. The device classes allow the construction of a device that may :-

- be capable of

-  only sourcing a real time audiovisual output stream.

-  only receiving a real time audiovisual input stream.

-  both sourcing or receiving a real time audiovisual stream.

- not have any direct interaction with any real time stream but still perform a useful function.

- be capable of performing a mixing function where more than one input stream is combined to produce one output stream.

- be capable of performing a switching function that is used to control the routing of input streams to output streams.

- be capable of performing processing on a real time audiovisual stream.

A device may be classified as :-

- a user device which is exposed to conference participants for use by those participants.

- a management device which is hidden from participants but may be used by a T.133 service to service a user request.

Remote device control provides the following facilities to the conference :-

- declaration of devices and their associated attributes to enable a node to determine a profile of device attached to a remote node.

- a mechanism to allow a node to request exclusive access to a remote device in order to perform device control or configuration.

- mechanisms to allow a node to control, configure, receive event notification and obtain the status of a remote device.

## 28 Audio Video Conference Services (AVCS)

This part of T.133 specifies a number of commonly required network based (i.e. MCU) services which manage real time streams using the underlying RTLM infrastructure. Each service is assigned a standard  identifier which is used to represent it in the application roster and  advertises the capability for that service to the conference. Each service may consist of a mandatory part and optional enhancements. The use of enhancements is negotiated using the GCC capability exchange mechanism. The standard services provided by AVC are listed below:-

·   Audio and video source selection.

·   Video switching

·   Video processing (including support for continuous presence)

·   Audio mixing

·   Audio and video source identification

·   On Air Indication

·   Floor control

·   Privacy

The capability to support a service is determined from GCC's application Roster. The Roster entry for each service indicates if a node wishes to activate that service. If the service is available at two or more nodes and one or more nodes has requested activation on start-up then the roster exchange will cause the service to be launched. Once launched the service will continue until the end of the session or until terminated. Each individual service defines  any restrictions that apply to the launch or termination of a  service.

An authorised node may  activate a service by issuing  a request to the MCU Communication Channel. All MCUs receive the request and respond by launching or terminating the specified service.

Any of the services  may utilise standardised  Devices and their associated functionality; within a session there may be a number of physical devices in different nodes that actually provide the like functionality, these devices communicating with each other using the MCU control channel. Terminals needing to communicate with the AVC switching services do so by using the MCU communications channel which provides a common interface independent of the conference configuration and  network components.

Note- devices being used by services are termed normally locked and not available for user control except through the service utilizing them.

### 29 Audio and video source selection.

.......................................

### 30 Video Switching Service

Switching services  utilise standardised  switch devices; within a session there may be a number of physical devices in different nodes that actually provide the switching functionality, these devices communicating with each other using the MCU control channel. Terminals needing to communicate with the AVC switching services do so by using the MCU communications channel which provides a common interface independent of the conference configuration and  network components.

In a multipoint conference  the connection topology shall always be arranged such that the Top Provider for the conference shall be an MCU. In conference configurations where more than one MCU is present, it shall be the top provider MCU that decides if any given external request is to acted upon. Through  inter-MCU communication the top provider can assign privileges to it client MCUs to act independently, carrying out designated functions.

In a conducted conference  switching mode selection  may be restricted (for example, only available to the conductor or a conference organiser).

The switching service provides a high level interface based on participant roles. A number of 'participant' roles are defined for the services, to provide a focal point for the switching algorithms. Roles may be pre-assigned or dynamically allocated;  they may also be re-allocated at will during a session. The selected switching mode  determines whether and how the roles are used to determine stream routing.

- Current-speaker

- Previous-Speaker

- Overider

- Designated Presenter(s)

- Conference Conductor

- Designated Stream

- Conference Convenor

- Viewer(s)

### 31 Continuous Presence Video Service

The primary objective of Continuous Presence systems is to allow many or all of the participants to be seen all of the time. Practical considerations may however restrict the ability to provide a true Continuous Presence system, particularly in large conference situations. The following factors will all impact on what is achievable:-

· 	The throughput capacity of the video pipes to and from terminals and on any inter-MCU links

·	The  topology of the conference

·	 Number of participants on Video Equipped Terminals.

·	The video protocol and encoding in use

The T.120 Continuous Presence (CP) service has the role of providing control and indications, in  support of  multipoint video conferences where some form of CP system is operating.

AVC services identify three methods of providing CP systems:

	Time-division Multiplex

	Transcoder-based picture composition

	Array-processor picture composition

### 32 Audio Mixing Service

The audio mixing service allow the way audio is mixed in the session to be determined. The audio mixing service supports a number of operational modes:

- Voice Switched Mode

- Conductor Controlled Mixing

### 33 Audio and Video Source Identification

This service exploits the T.120 capabilities to offer an enhanced functionality to the   conference. It provides the capability for the origin and content of a video stream to be communicated.

Each node in the conference is uniquely identified by its GCC's User ID. If a node is only capable of originating a single video stream then its User ID is all that will be needed to uniquely identify it as the originating node for that stream. Where a node is capable of originating more than one stream then this recommendation supports numeric identifiers. These identifiers should either correspond to a tag embedded within the stream or be used to map to some logical ordering of transmission or positioning relating of the pipes relative to each other.

In a multipoint conference; when an AVC aware node provides or changes the source of its stream, it shall send an identification message to the MCU Communication channel. If no identifier is received the MCU(s) will assume the default stream and source (face camera, #1 microphone) and extract a name if available from the GCC Conference Roster entry for that site. The identification message will allow the pipe number, the stream source device  and a string identifier to be defined to replace the default determination.

An identification message can be associated with each  received video image and audio stream by an MCU. It is the MCU’s responsibility to relay the identification onward to correspond to the routeing applied to the stream. If the MCU carries out any manipulation of the stream it shall generate a new Identification message to correlate with the changes.

The information will be used to identify; to the receiver, who or what is being seen and also from what location the stream originates. It may be used to generate a caption for displaying with the image (...). MCUs shall send Identification messages individually link by link to the  AVC User ID for any given node.

### 34 "On Air" Indication Service

In a multipoint conference MCU(s) provide control and routing of audio and video streams. When a stream is routed onwards by an MCU  to a terminal  capable of  interpreting the stream it indicates to any peer MCUs that source is 'On Air' . The MCU directly connected to the source originator has the responsible for passing the 'On Air' indication to that terminal.

### 35 Audio and Video Floor Control Service

This service provides the mechanism that allow conference participants to make requests to speak in a conducted conference and for the conductor to enable and disable participants from speaking. It is only operational in a multipoint conference in conducted mode.  This service makes use of the audio mixer device(s)  in order to provide the service.

### 36 Privacy

.........

END OF T.130

# ANNEX A  EXAMPLES OF  AVC ROUTE ESTABLISHMENT

The following examples illustrate the processes involved when real-time Route requests are made to AVC. They show how the routing and need for processing on the media stream is taken into account and how RTLM’s services are utilised.

A.1 Transcoding example

Figures 1-3 show accompany the following text for a simple transcoding example.

- A real time user application requests a Route from a media endpoint on node A to a media endpoint on node F. The ROUTE request may include optionally the acceptability of switching, mixing, processing, exclusive use or a preference. Figure 1 shows the logical interconnection.

- AVC requests the possible routing from RTLM. Figure 2 shows the physical interconnections between nodes. RTLM returns routes R1 & R2 (and associated nodes on routes). R1 involves nodes A,B,C,D,E,F whilst R2 involves A,B,C,E,F.

- AVC examines the capabilities of the endpoint nodes and also the requested QoS for the media route. In this example,  in order to provide this Route it is determined that transcoding needs to be performed on the media stream.

- AVC requests a list of nodes capable of providing the requisite level of transcoding. In this example nodes C and D are capable.

- AVC chooses one of the nodes for transcoding. This decision is matter local to AVC but it may consider items such as cost of routing, knowledge of bandwidth available on each route or a user preference (eg. user prefers the characteristics of one transcoder and has specified this in the Route request).

- AVC then determines the Pipelines required for a particular transcoding node. In this example, as shown in figure 3, the Pipelines would be P1 and P3 if node C performs the transcode, or P4 and P2 if node D performs the transcode.

- AVC requests RTLM to establish the Pipelines (P1 and P3 or P4 and P2).

- Assuming the Pipelines are successfully established AVC will configure the endpoint nodes and the transcoding node enabling the Route for the media stream.

- AVC will return a successful indication to the real time user application, also indicating that the transcoding was involved.

Figure 1 - Route request from node A to node F

Figure 2 - Possible physical routings to support requested Route

Figure 3 - Possible Pipelines to support requested Route

A.2 Switch example

With reference to Figures 4-6,

- A number of Route requests are made to AVC for Routes A->E, B->E and D->E. Each Route has an associated QoS.

- AVC determines the possible routings for these Routes. Route A->E has two alternatives R1 or R2 whilst the others only have one choice. This is shown in figure 5.

- AVC also examines the capabilities of all endpoint nodes and determines that node E is only capable of receiving one stream of this media type at a time.

- This means that in order for E to receive streams from all 3 a switch (or mixer) is required.

- AVC determines which nodes have a suitable switching (or mixing) capability and together with the routing information determines the Pipelines necessary to achieve that Route. The Pipelines are shown in figure 6.

- AVC requests RTLM to establish Pipelines P1, P2, P3, P4.

- Assuming the Pipelines are successfully established AVC will configure the endpoint nodes (A,B,D,E)  and the switching node (C)  enabling the Routes for the media streams.

Figure 4 - Multiple Route requests to node E

Figure 5 - Possible physical routings for requested Routes

Figure 6 - Pipelines and Switching necessary to achieve Routes

END OF DOCUMENT