| INTERNATIONAL TELECOMMUNICATION UNION Temporary Document - |  |  |
| --- | --- | --- |
|  | TELECOMMUNICATION STANDARDISATION SECTOR STUDY PERIOD 1993 - 1996 | COM 8-__ November 1995 Original: English 31 July, 2026 |

Question:  10/8

Geneva 6th - 15th February 1996

STUDY GROUP 8 – CONTRIBUTION

| SOURCE: | Rapporteur |  |  |  |
| --- | --- | --- | --- | --- |
| CONTACT: | See Page 2 |  |  |  |
| TITLE: | Draft recommendation T.133 - Audio Video Control Application Protocol |  |  |  |

Draft recommendation T.133

Audio Video Control Application Protocol

Summary

This recommendation consists of three parts:

The Core Services define a real-time stream distribution service for user applications. They provides the overall management of the infrastructure to deliver real-time streams from a source to one or more destinations at a negotiated Quality of Service. Furthermore the Core Services has overall responsibility for resource management (including QoS) within a conference.

Remote Device Control supports the remote configuration and control of  devices commonly used within a conferencing environment.

Audio Video Conference Services comprise a suite of  dedicated services that provide enhanced support for multipoint multimedia conferencing. These services include video switching, audio mixing, continuous presence and stream identification.

This application protocol makes use of the infrastructure recommendation T.132 in order to deliver its services

| EDITORS: | John Boucher, BT Labs, Martlesham Heath, IPSWICH, Suffolk, UK IP5 7RE. |  | Tel: +44 1473 644354 / +44 1473 647697 Fax: +44 1473 643791 E-mail:j_boucher@bt-web.bt.co.uk (Co-Editor Section 1-7, 9,10 and Team Leader) |
| --- | --- | --- | --- |
|  | Loc-Kin Yuen GPT Video Systems 6 Roxborough Way, Foundation Park, Maidenhead, Berkshire, UK SL6 3UD | Tel: +44 1628 772132 Fax: +44 1628 772299 E-mail: loc-kin.yuen@vid.bsg,mf.gpt.co.uk (Co-editor Section 8) |  |
|  | Andrew Woollett, ImageCom Ltd., New Century Park, Cordwallis Street, Maidenhead, Berkshire, UK SL7 6BE | Tel: +44 1628 788810 Fax: +44 1628 39131 E-mail: a_woollett@imagecom.demon.co.uk (Co-editor Section 9) |  |
|  | Greg Kisor, Intel Corporation, 2111 NE 25th Avenue, JF3 - 212, Hillsboro, OR 97006, USA | Tel: +1 503 264 9324 Fax: +1 503 264 3375 E-mail: gkisor@ibeam.intel.com (Co-editor section 8) |  |

CONTENTS

1. Scope

2. Normative References

3. Definitions

4. Abbreviations

5. Conventions

6. Overview

7. Use Of T.120 Infrastructure.

7.1 AVC Communication Channels

8. AVC Part1 - Core Services for Routing and Quality of Service Management

8.1 AVC Routing and quality of service management - Introduction

8.1.1 Terminology

8.1.2 ARQM  Operation

8.1.3 ARQM control of arbitration mechanisms

8.1.4 Utilisation of RTLM Pipeline service

8.1.5 Utilisation of RTLM ROSTER Primitives

8.1.6 Usage of MCS channels

8.2 Route QOS

8.2.1 QoS Ranges

8.2.2 Media QoS Parameters

8.3 Route Priority

8.4 AVC Routing Service Definition

8.4.1 AVC-Route-Create

8.4.2 AVC-Route-Delete

8.4.3 AVC-Route-Modify

8.4.4 AVC-Route-List

8.4.5 AVC-Route-Query

8.4.6 AVC-Route-Monitor

8.4.7 AVC-T120BW-modify

9. AVC Part 2 - Remote Device Control

9.1 Remote Device Control - Introduction

9.2 Device Control

9.3 The Remote Device Control Application

9.3.1 Application Enrolment

9.4 Determining Attributes for a Remote Device

9.4.1 Requesting Device Attributes

9.5 Remote Device Control

9.5.1 Exclusive Remote Device Access

9.5.2 Requesting Remote Device Control

9.6 Remote Device Status

9.6.1 Requesting Remote Device Status

9.7 Remote Device Events

9.7.1 Requesting Remote Device Notification

9.7.2 Device Attribute Changes

9.8 Source Selection

9.9 Stream Routing

9.10 Conducted Mode Behaviour

9.10.1 Conductor Privilege Request

9.10.2 Conductor Privilege Assign

9.11 Standard RDC Devices

9.11.1 Device Control Specifications

9.11.2 Standard Device Classes

9.11.3 Control Action Ranges

9.11.4 Standard Device Control Specifications

10. AVC PART 3 Conference Services

10.1 Summary of Conference Services

10.2 Conference Services

10.2.1 Video Switching Service (VSS).

10.2.2 Video Service - Identification.

10.2.3 "On Air" Indication Service

10.2.4 AVC-Video-Service-On-Air  - Sequence of primitives

10.2.5 Continuous Presence Video Service

10.2.6 CIF Image Sizes

10.2.7 Audio and Visual Floor Control Service

10.2.8 Audio Mixing Service

10.2.9 Conductor Controlled Mixing

10.3 Service Functions

10.3.1 Privacy Function

10.3.2 Conducted Mode Behaviour

10.4 Network and Management Device Defintions

10.4.1 Switching Device Class

10.4.2 Audio Mixer Device Class

10.5 The Virtual MCU.

10.6 AVCPDU Definitions

# Scope

The AVC recommendation is the application protocol that forms part of the T.120 Real-time Audio Visual Control extensions.  It comprises three parts:

AVC Part 1, the Core Service defines a real time stream distribution service for user applications. It provides the overall management of the infrastructure to deliver realtime streams from a source to one or more destinations at a negotiated Quality of Service. Furthermore the Core Service Resource Manager has overall responsibility for resource management (including QoS) within a conference.

AVC Part 2, Remote Device Control supports the remote configuration and control of  devices commonly used within a conferencing environment.

AVC Part 3 is the  Audio Video Conference Service is a suite of  dedicated services that provide enhanced support for multipoint multimedia conferencing. These services include video switching, audio mixing, continuous presence and stream identification.

AVC makes use of the infrastructure recommendation T.132  in order to deliver its services.

below shows the relationship between T.133(AVC) and the other component recommendations that provide the Real-time Audio Video Control for conferencing in a multimedia communications environment.

Figure 1/T.133.  Scope of AVC within T.120

# Normative References

The following Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid.  All Recommendations and other references are subject to revision: All users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below.  A list of the currently valid ITU-T Recommendations is regularly published.

–	ITU-T Recommendation T.120 (1996), Transmission Protocols for Multipoint Data Conferencing - under development.

–	ITU-T Recommendation T.122 (1993), Multipoint Communication Service for audiographics and audiovisual conferencing service definition.

–	ITU-T Recommendation T.123 (1994), Protocol Stack for Audiographics and Audiovisual Teleconference Applications.

–	ITU-T Recommendation T.124 (1995), Generic Conference Control.

–	ITU-T Recommendation T.125 (1994), Multipoint Communication Service protocol specification.

# Definitions

For the purposes of this Recommendation, the following definitions apply:

Adjacent Node: An adjacent node is a node that forms one of a pair of nodes that are connected and thus associated by a real time link.

Application Protocol: Any standard or non-standard protocol specification that makes use of T.120 services.

Application Protocol Entity: The instantiation of an Application Protocol in a terminal or MCU.

Application Protocol Session: A group of peer Application Protocol Entities communicating with each other.

Controllable Device: A device that has the capability to be controlled remotely in a conference environment.

Communications Port: A physical interface at a node that possesses AVC media capabilities and is able to provided a connection to a real time link.

Conference:  A number of nodes that are joined together and that are capable of exchanging audiographic and audiovisual information across various communication networks.

Device: An entity which provides a specific function, or set of functions, to the conference. Some devices such as cameras and microphones are capable of generating a source to the conference. Other devices provide control and processing functions for streams.

GCC Provider:  Agent providing GCC services to local Node Controller and Application Protocol Entities at a terminal or MCU.

Inter-MCU-Link: A real time link that interconnects two adjacent MCU nodes.

MCS Domain:  A hierarchy of MCS connections between nodes.  Nodes may use MCS services to communicate within a single domain, but not between separate domains. A GCC Conference corresponds one-to-one with a single MCS Domain.

MCS User ID:  Unique identification number assigned by MCS to an MCS User.  GCC Providers as well as Application Protocol Entities are MCS Users.  An MCS User ID assigned to a GCC Provider is referred to as a Node ID.  An MCS User ID assigned to an Application Protocol Entity is referred to as an Application User ID.  An MCS User ID is valid only within a single MCS Domain.

Media Endpoint:  The ‘lowest’ interface in an endpoint conference node where the media stream is unprocessed. This interface may source or sink media Streams. One media endpoint is required for each media type and each media stream.

Multipoint:  The ability to exchange data among multiple nodes simultaneously as compared with point-to-point, where data is exchanged between two directly connected nodes.

MCU (Multipoint Control Unit): A device used to interconnect multiple terminals and/or further MCUs to allow multiple participants to be connected to single  conference. Commonly referred to as an MCU or bridge, a multi-port device that serves to connect terminals and other MCUs in a multipoint fashion.  A GCC-capable MCU runs GCC and MCS.  An MCU is not primarily intended as an end-point for user communication.

Multiport Terminal:  End-point audiographic or audiovisual equipment that also includes the ability to bridge T.120 information.  Like a terminal, the behaviour of a multiport terminal is typified by automatic establishment of a single conference.  But, like an MCU, for a given conference in a multiport terminal, there may be more than one MCS connection.

Multipoint Service: One or more functions that are provided and managed by MCUs.

Node: Any entity in the network that runs the T.120/AVC protocols. This includes both terminal equipment and network elements such as routers and bridges (otherwise known as MCUs).

Node Controller:  A functional entity for which there is one for each terminal or MCU, which serves as the controller of that node.

Node ID:  MCS User ID assigned by MCS to the GCC Provider at a node.

Path:  The unidirectional logical connection between one or more Media Endpoints. A Path may have one source and one or more destination endpoints.

Pipe: A communications path within a real time link that carries a single stream between adjacent nodes.

Pipeline: A collection of connected  pipes used to convey a real time stream from its source to one or more destinations.

Quality of Service: A 'form factor' for the quality of the audio, video and data Paths.

Resource:  Something that can be used and shared by nodes in a conference.  A resource comprises both channels and tokens.

Real Time Link: A connection between two adjacent nodes used for the transfer of real time information in a single direction.  A real time link may provide one or more communication paths (termed pipes), each of which conveys a single stream between the two nodes.

Source: The origin of a real time media stream.

Stream: A flow of real time information of a specific media type (e.g. audio) from a single source to one or more destinations.

Video Processor: A device that modifies the content of video streams, e.g. a transcoder, continuous presence processor.

Virtual MCU: Term used to describe the collection of MCUs within a real time conference which present a single logical interface to terminals.

User Application: The instantiation of an Application Protocol in a terminal or MCU.

# Abbreviations

APE		Application Protocol Entity

AVC		Audio-Visual Control

ATM		Asyncronous Transfer Mode

B-ISDN		Broadband Integrated Services Digital Network

GCC		Generic Conference Control

ISDN		Integrated Services Digital Network

LAN		Local Area Network

MCS		Multipoint Communication Service

MCU		Multipoint Control Unit

RTLM		Real Time Stream Control

PDU		Protocol Data Unit

PSDN		Packet Switched Digital Network

PSTN		Public Switched Telephone Network

TCP		Transmission Control Protocol

MCS		Multipoint Communication Service

MCU		Multipoint Control Unit

PDU		Protocol Data Unit

SAP		Service Access Point

# Conventions

The primitive parameters of the abstract services defined in this Recommendation use the following key:

M:		parameter is mandatory

C:		parameter is conditional

O:		parameter is optional

Blank:	parameter is absent

(=):		value of the parameter is identical to the value of the corresponding parameter of the preceding primitive, where preceding is defined relative to the order:  request, indication, response, confirm.

(=RQ):	value of the parameter is identical to the value of the corresponding parameter in a preceding primitive, where RQ = request, IN = indication, RS = response, and CF = confirm.

# Overview

Within the context of the ITU-T Audio-visual Conferencing Service, a conference refers to a group of nodes that are joined together and that are capable of exchanging information across various communication networks. Participants taking part in a conference may have access to various types of media handling capabilities such as audio only (telephony), audio and data (audiographic), audio and video (audio-visual) and audio, video and data (multimedia).

This recommendation provides a generic and flexible framework for the control and management of real-time streams devices and services. These streams are communicated external to the T.120 protocol, and provide interactive audio and visual components to a conference. AVC depends upon the T.120 infrastructure recommendations T.122 and T.125 (MCS), T.124 (GCC) and T.123 (MPS) for conference establishment and control. Additionally, AVC utilises the real time infrastructure defined in T.132(RTLM).

This recommendation provides mechanisms to support and manage multiple audio and/or video streams in a conference, on any given link or combination of links in either direction.

A remote device control (RDC) service is also supported, where nodes can advertise their capability to provide controllable devices to the conference. Sources may be remotely selected and devices remotely controlled. This service provides modelling and classification of devices for the purpose of allowing remote activation and control within a conference environment. This is achieved by defining a template of control and configuration attributes for a set of commonly used devices. RDC also provides support to allow nodes to utilise the RDC model to define and advertise proprietary devices using standard attributes from the toolkit.

The recommendation also defines a number of MCU based services in support of conferences. These are shown in the list below. It defines a number of supplementary conference services, each encapsulating all the functionality required to provide a specific type of service in a complete and comprehensive manner. These services can be regarded as a toolkit from which functionality may be selected as appropriate for any given conference requirement.

All AVC resources, the QoS management of real-time streams, are managed by the AVC and these services must negotiate and ultimately compete with each  other for the use  of available resources.

Services defined in this recommendation include:

·   Remote device control.

·   Audio and video source selection.

·   Video switching

·   Video processing (including support for continuous presence)

·   Audio mixing

·   Audio and video source identification

·   On Air Indication

·   Floor control

# Use Of T.120 Infrastructure.

## AVC Communication Channels

For a multipoint session the AVC application utilises 3 MCS static channels to provide management and control services for  external interactive audio and video channels:-

·	AVC Broadcast Channel

·	MCU Control Channel

·	MCU Communication Channel

For a point to point session only the AVC Broadcast channel will be in use.

AVC Broadcast Channel

The AVC broadcast channel  is joined by all terminals supporting AVC capabilities and all MCU's. It is used for broadcasting information to all nodes that support the AVC application.

MCU Control Channel

The  MCU control channel is joined by all MCU's participating in a T.120 conference. It shall not be joined or used by terminals. Its use is reserved for MCU specific communications.  MCU's providing time critical management functions are permitted to use this channel at the highest priority that is normally reserved for GCC management  functions.

MCU Communication Channel

The  MCU Communication channel is also joined by all MCU's participating in a T.120 conference. This channel may not be joined by terminals but in a multipoint session they may send to it, in order to communicate with the conference MCU's. It provides a standard channel for communication to MCU's that is independent of the number of MCU's and the conference topology. It effectively allows terminals to see the MCU network as a single virtual  MCU.

FIGURE 2/T.133.

Example Conference configuration showing AVC channels in a point to point environment.

FIGURE 3/T.133.

Example Conference configuration showing AVC channels in a multipoint environment.

-

Annex A Relationship and Inter working with services Provided by the ITU H series recommendations

(This Annex Forms An integral Part Of This Recommendation)

A.1.	Interaction with ITU H. Series Recommendations.

·	The general rule will be that T.120 will take precedence over ITU H.243 where the same functional area's are addressed by both.

·	The optional extensions in ITU recommendation H.243 will in general be replaced by T.120 functionality.

·	In cases where a terminal only supporting H.243 is in a conference running the T.120 AVC application, then it is an implementation matter as to what level of support MCU's provide to that terminal.

The ITU recommendation H.243 sections 2 and 3 deal with the establishment of connections to MCU's. As part of this establishment procedure the MLP data channel capability will be determined. Where a MLP conference is established, a T.120 session will be initiated within the MLP channel.

This recommendation defines a flexible Audio-visual control architecture  which provides a comprehensive  range of services and options. T.133 shall inherit the default start up mode established by H243 including voice switched video. T.133 functionality  replaces all of the remaining H.243 functionality  offering a greatly enhance and more flexible approach than that possible with the BAS Code signalling method.

H.243 (1994) Sections 4.2.1 onward  shall be superseded by T.133 functionality.

In cases where terminal nodes do not support AVC but do support H.243, the protocol makes no provision to support  inter working between the two protocols.  In such cases MCU may be capable of providing a proxy function on behalf of the 'H.243 only' terminal, but this is a local matter beyond the scope of this recommendation.

{Note :- The H series recommendations currently makes no provision for multiple streams on  links, when the provision is made the existing Freeze Frame and Fast Update mechanisms must also be extended.}

A.2	T.120 Conference Start-up Issues.

T.124 "Generic Conference Control" states that a participant is not deemed to be part of the conference until their terminal has 'announced  presence' into the conference using the assigned GCC primitive.

In an H.221 environment  the video and audio may be introduced into the conference prior to the establishment a  T.120 connection.

If  the H.242 capability exchange declares T.120 available then it may be required that each terminal/user be vetted by the convenor or other designated person before fully connected to the conference. An alternative to this also applies to the conference access controlled by password. In such cases a recorded screen with a standard icon or recorded audio announcements and/or text data messages should be used.

END OF ANNEX A

Appendix A   EXAMPLES OF  AVC ROUTING ESTABLISHMENT

The following examples illustrate the processes involved when real-time Route requests are made to AVC. They show how the physical routing and need for processing on the media stream is taken into account and how RTLM’s services are utilised.

A.1 Transcoding example

Figures A1-3 show accompany the following text for a simple transcoding example.

- A real time user application requests a Route from a media endpoint on node A to a media endpoint on node F. The Route request may include the acceptability of switching, mixing, processing or a preference. Additionally, specific network processing nodes may be specified. Figure A1 shows the logical interconnection.

- AVC requests the possible physical routing from RTLM. Figure A2 shows the physical interconnections between nodes. RTLM returns physical routes R1 & R2 (and associated nodes on routes). R1 involves nodes A,B,C,D,E,F whilst R2 involves A,B,C,E,F.

- AVC examines the capabilities of the endpoint nodes and also the requested QoS for the media Route. In this example,  in order to provide this Route it is determined that transcoding needs to be performed on the media stream.

- AVC requests a list of nodes capable of providing the requisite level of transcoding. In this example nodes C and D are capable.

- AVC chooses one of the nodes for transcoding. This decision is matter local to AVC but it may consider items such as cost of routing, knowledge of bandwidth available on each route or a user preference (eg. user prefers the characteristics of one transcoder and has specified this in the Route request).

- AVC then determines the Pipelines required for a particular transcoding node. In this example, as shown in figure A3, the Pipelines would be P1 and P3 if node C performs the transcode, or P4 and P2 if node D performs the transcode.

- AVC requests RTLM to establish the Pipelines (P1 and P3 or P4 and P2).

- Assuming the Pipelines are successfully established AVC will configure the endpoint nodes and the transcoding node enabling the Route for the media stream.

- AVC will return a successful indication to the real time user application, also indicating that the transcoding was involved.

Figure A1 - Route request from node A to node F

Figure A2 - Possible physical routings to support requested Route

Figure A3 - Possible Pipelines to support requested Route

A.2 Switch example

With reference to Figures A4-6,

- A number of Route requests are made to AVC for Routes A->E, B->E and D->E. Each Route has an associated QoS.

- AVC determines the possible physical routings for these Routes. Route A->E has two alternatives R1 or R2 whilst the others only have one choice. This is shown in figure A5.

- AVC also examines the capabilities of all endpoint nodes and determines that node E is only capable of receiving one stream of this media type at a time.

- This means that in order for E to receive streams from all 3 a switch (or mixer) is required.

- AVC determines which nodes have a suitable switching (or mixing) capability and together with the physical routing information determines the Pipelines necessary to achieve that Route. The Pipelines are shown in figure A6.

- AVC requests RTLM to establish Pipelines P1, P2, P3, P4.

- Assuming the Pipelines are successfully established AVC will configure the endpoint nodes (A,B,D,E)  and the switching node (C)  enabling the Routes for the media streams.

Figure 4 - Multiple Route requests to node E

Figure 5 - Possible physical routings for requested Routes

Figure 6 - Pipelines and Switching necessary to achieve Routes

Appendix B   Examples of effect of MEDIA  PRIORITY and QoS definition

The following tables provide examples of how the media priority and QoS requested for each Route may affect the conference operational mode.

Table B1

|  |  | QoS |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Route ID | Maximum QoS | Preferred QoS | Minimum QoS | Priority | Comment |
| AVC-Video-0 | a | a | b | 0 |  |
| AVC-Video-1 |  |  |  |  | Only 3 media Routes are in use. Audio |
| AVC-Video-2 |  |  |  |  | QoS ‘e’ is to be used and this must not |
| AVC-Video-3 |  |  |  |  | be reduced, but may be increased. |
|  |  |  |  |  | Video QoS ‘a’ is desired however, this |
|  |  |  |  |  | may be reduced to QoS ‘b’ should the |
| AVC-Audio-0 | f | e | e | 0 | need arise. There is no Real Time data |
| AVC-Audio-1 |  |  |  |  | Route, but the T.120 Partition has a |
| AVC-Audio-2 |  |  |  |  | QoS (bandwidth) ‘n.’ The T.120 |
| AVC-Audio-3 |  |  |  |  | Partition may be reduced to ‘p’ to |
|  |  |  |  |  | accommodate other media. |
|  |  |  |  |  | Note the minimum T.120 bandwidth |
|  |  |  |  |  | should never be 0. |
| AVC-RTData-0 |  |  |  |  |  |
| AVC-RTData-1 |  |  |  |  |  |
| AVC-RTData-2 |  |  |  |  |  |
| AVC-T120 | n | n | p | 0 |  |

Table B2

|  |  | QoS |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Route ID | Maximum QoS | Preferred QoS | Minimum QoS | Priority | Comment |
| AVC-Video-0 | z | a | a | 0 | This example has 3 video Routes the |
| AVC-Video-1 | b | b | c | 1 | first of whose QoS cannot be reduced |
| AVC-Video-2 | b | b | c | 2 | from ‘a’ though it can be increased to |
| AVC-Video-3 |  |  |  |  | ‘z’. With the next 2, they have the same |
|  |  |  |  |  | QoS, but should one need to be |
|  |  |  |  |  | reduced, then ARQM-Video-2 would be |
| AVC-Audio-0 | f | e | e | 0 | the first. Similarly, should bandwidth be |
| AVC-Audio-1 |  |  |  |  | available to increase the video QoS, |
| AVC-Audio-2 |  |  |  |  | then ARQM-Video-1 would be the first |
| AVC-Audio-3 |  |  |  |  | candidate. Audio QoS must not be |
|  |  |  |  |  | reduced from ‘e’. One Real Time data |
|  |  |  |  |  | Route exists. Its QoS may be varied |
|  |  |  |  |  | between ‘p’ and ‘q’. Finally the T.120 |
| AVC-RTData-0 | p | p | q | 0 | Partition may have its bandwidth |
| AVC-RTData-1 |  |  |  |  | varied between ‘n’ and ‘y’ with a |
| AVC-RTData-2 |  |  |  |  | preferred value of ‘x’. |
|  |  |  |  |  | Note the minimum T.120 bandwidth |
|  |  |  |  |  | should never be 0. |
| AVC-T120 | n | x | y | 10 |  |

Table B3

|  |  | QoS |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Route ID | Maximum QoS | Preferred QoS | Minimum QoS | Priority | Comment |
| AVC-Video-0 | z | a | 0 | 0 |  |
| AVC-Video-1 | b | b | 0 | 1 | This example has 4 video Routes, each |
| AVC-Video-2 | b | b | 0 | 1 | of which may be reduced and removed |
| AVC-Video-3 | d | d | 0 | 2 | if necessary. Route 3 will be the first to |
|  |  |  |  |  | go, whilst it is arbitrary which of Route |
|  |  |  |  |  | 1 or 2 is removed first. Two audio |
| AVC-Audio-0 | f | e | e | 0 | Routes exist, one of which must not be |
| AVC-Audio-1 | g | g | f | 0 | reduced but can be increased. The |
| AVC-Audio-2 |  |  |  |  | other can be reduced to ‘f’. Two Real |
| AVC-Audio-3 |  |  |  |  | Time data Route exists, each of which |
|  |  |  |  |  | must not be varied or removed. Finally |
|  |  |  |  |  | the T.120 Partition may have its bandwidth varied between ‘n’ and ‘y’ |
|  |  |  |  |  | with a preferred value of ‘x’. |
| AVC-RTData-0 | q | p | p | 0 | Note the minimum T.120 bandwidth |
| AVC-RTData-1 | r | r | r | 0 | should never be 0. |
| AVC-RTData-2 |  |  |  |  |  |
| AVC-T120 | n | x | y | 10 |  |