|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	T.124

TELECOMMUNICATION	(08/95)
STANDARDIZATION  
OF  ITU

TERMINALS  FOR  TELEMATIC  SERVICES

GENERIC  CONFERENCE  CONTROL

ITU-T  Recommendation  T.124

(Previously “CCITT  Recommendation”)

# FOREWORD

The ITU-T (Telecommunication Standardization Sector) is a permanent organ of the International Telecommunication Union (ITU). The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU-T is covered by the procedure laid down in WTSC Resolution No. 1 (Helsinki, March 1-12, 1993).

ITU-T Recommendation T.124 was prepared by ITU-T Study Group 8 (1993-1996) and was approved under the WTSC Resolution No. 1 procedure on the 11th of August 1995.

___________________

NOTE

In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

  ITU  1996

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Recommendation T.124     (08/95)

Page

1	Scope		1

2	Normative References		1

3	Definitions		2

4	Abbreviations		5

5	Conventions		6

6	Overview		6

6.1	System model for a conference node		7

6.2	Conference establishment and termination		8

6.3	The conference roster		9

6.4	The application roster		9

6.5	The application registry		9

6.6	Conference conductorship		9

6.7	Miscellaneous functions		10

6.8	Summary of GCC abstract services		10

7	GCC service definition		13

7.1	Conference establishment and termination		13

7.1.1	The conference profile		13

7.1.2	Description of abstract services		14

7.1.2.1	GCC-Conference-Create		14

7.1.2.2	GCC-Conference-Query		18

7.1.2.3	GCC-Conference-Join		20

7.1.2.4	GCC-Conference-Invite		23

7.1.2.5	GCC-Conference-Add		26

7.1.2.6	GCC-Conference-Lock		29

7.1.2.7	GCC-Conference-Unlock		30

7.1.2.8	GCC-Conference-Lock-Report		31

7.1.2.9	GCC-Conference-Disconnect		31

7.1.2.10	GCC-Conference-Terminate		32

7.1.2.11	GCC-Conference-Eject-User		34

7.1.2.12	GCC-Conference-Transfer		35

7.1.3	Conference establishment requirements		37

7.1.4	Examples of conference establishment procedures		37

7.1.4.1	Meet-me conference establishment		37

7.1.4.2	Call-out conference establishment		38

7.1.4.3	Call-through conference establishment		39

7.1.4.4	Point-to-point conference establishment		39

7.1.4.5	Conference establishment among multiport terminals		39

7.2	The conference roster		40

7.2.1	Description of abstract services		40

7.2.1.1	GCC-Conference-Announce-Presence		41

7.2.1.2	GCC-Conference-Roster-Report		42

7.2.1.3	GCC-Conference-Roster-Inquire		44

7.3	The application roster		45

7.3.1	Contents of the application roster		45

7.3.2	Description of the application roster exchange process		48

7.3.3	Description of abstract services		49

Page

7.3.3.1	GCC-Application-Permission-To-Enroll		49

7.3.3.2	GCC-Application-Enroll		50

7.3.3.3	GCC-Application-Roster-Report		52

7.3.3.4	GCC-Application-Roster-Inquire		55

7.3.3.5	GCC-Application-Invoke		56

7.4	The Application Registry		58

7.4.1	Registry keys		58

7.4.2	Ownership and persistence		58

7.4.3	Dynamic allocation		58

7.4.4	Description of abstract services		59

7.4.4.1	GCC-Registry-Register-Channel		59

7.4.4.2	GCC-Registry-Assign-Token		61

7.4.4.3	GCC-Registry-Set-Parameter		61

7.4.4.4	GCC-Registry-Retrieve-Entry		63

7.4.4.5	GCC-Registry-Delete-Entry		64

7.4.4.6	GCC-Registry-Monitor		65

7.4.4.7	GCC-Registry-Allocate-Handle		67

7.5	Conference conductorship		68

7.5.1	Description of abstract services		68

7.5.1.1	GCC-Conductor-Assign		69

7.5.1.2	GCC-Conductor-Release		70

7.5.1.3	GCC-Conductor-Please		71

7.5.1.4	GCC-Conductor-Give		71

7.5.1.5	GCC-Conductor-Inquire		72

7.5.1.6	GCC-Conductor-Permission-Ask		74

7.5.1.7	GCC-Conductor-Permission-Grant		75

7.6	Miscellaneous functions		76

7.6.1	Description of abstract services		76

7.6.1.1	GCC-Conference-Time-Remaining		77

7.6.1.2	GCC-Conference-Time-Inquire		78

7.6.1.3	GCC-Conference-Extend		79

7.6.1.4	GCC-Conference-Assistance		80

7.6.1.5	GCC-Text-Message		80

8	GCC Protocol Specification		81

8.1	General operation		81

8.2	Conference establishment and termination		82

8.2.1	Conference creation		82

8.2.2	Querying conferences		87

8.2.3	Joining a conference		89

8.2.4	Inviting a node to a conference		93

8.2.5	Requesting to add a node to a conference		98

8.2.6	Locking a Conference		100

8.2.7	Unlocking a conference		102

8.2.8	Disconnecting from a conference		103

8.2.9	Terminating a conference		104

8.2.10	Ejecting a node from a conference		105

8.2.11	Transferring nodes between conferences		107

8.3	The conference and application rosters		109

8.3.1	Overview		109

8.3.2	Nodes entering a conference		109

8.3.3	Enrolling Application Protocol Entities		111

8.3.4	Updating a conference roster entry		112

8.3.5	Propagation of roster updates to the Top GCC Provider		112

8.3.6	Distribution of the conference and application rosters		113

8.3.7	Nodes leaving a conference		114

8.3.8	Collapsing application capabilities lists		114

8.3.9	Application and conference roster inquiry		115

Page

8.3.10	An Example of a roster update		115

8.3.11	Remotely invoking an Application Protocol Entity		116

8.4	The Application Registry		116

8.4.1	Registering a channel		117

8.4.2	Assigning a token		118

8.4.3	Setting a parameter		119

8.4.4	Retrieving an entry		120

8.4.5	Deleting an entry		121

8.4.6	Monitoring an entry		121

8.4.7	Allocation of unique handles		123

8.4.8	Changes in ownership and registry cleanup		124

8.5	Conference Conductorship		124

8.5.1	Grabbing conductorship		124

8.5.2	Releasing conductorship		125

8.5.3	Conductor assignment and release indications		125

8.5.4	Asking to be given conductorship		126

8.5.5	Giving conductorship		126

8.5.6	Getting conductorship status		127

8.5.7	Conductorship announcement when new nodes enter a conference		127

8.5.8	Unexpected disconnection of the conductor		127

8.5.9	Asking to be given conducted-mode permission		128

8.5.10	Granting conducted-mode permission		128

8.6	Miscellaneous functions		129

8.6.1	Timed conferences		129

8.6.2	Requesting conference assistance		130

8.6.3	Broadcasting a text message		131

8.7	GCCPDU definitions		131

9	Use of the Multipoint Communication Service		148

9.1	MCS services		148

9.2	Channel allocation		149

9.3	Token allocation		149

9.4	Use of MCS data transmission services		150

9.5	Encoding of PDUs in MCS primitives		152

9.6	Format of User Data parameter of MCS-Connect-Provider		152

9.7	Interpretation of the MCS Domain Selector		152

Annex A – Static channel and token ID assignments		153

A.1.	Static channel ID assignments		153

A.2.	Static token ID assignments		153

Annex B – Object Identifier assignments		154

SUMMARY

This Recommendation provides a high-level framework for conference management and control of multimedia terminals and Multipoint Control Units (MCUs). It encompasses Generic Conference Control (GCC) functions such as conference establishment and termination, managing the roster of terminals participating in a conference, managing the roster of applications and application capabilities within a conference, registry services for use by applications, coordination of conference conductorship, as well as other miscellaneous functions. It depends on companion Recommendations T.122 and T.125 (MCS) and T.123 as part of the T.120 infrastructure.

Recommendation T.124

Recommendation T.124     (08/95)

GENERIC  CONFERENCE  CONTROL

(Geneva, 1995)

# 1	Scope

This Recommendation provides a high-level framework for conference management and control of audiographic and audiovisual terminals and Multipoint Control Units (MCUs). It encompasses Generic Conference Control (GCC) functions such as conference establishment and termination, managing the roster of nodes participating in a conference, managing the roster of Application Protocol Entities and Application Capabilities within a conference, registry services for use by Application Protocol Entities, coordination of conference conductorship, as well as other miscellaneous functions.

This Recommendation is defined within the framework of Recommendation T.120. Included within this framework are companion Recommendations T.122 and T.125, which define the multipoint delivery mechanism used in this Recommendation, and Recommendation T.123, which specifies the Audiovisual Protocol Stacks for each of the communication networks supported.

Figure 1-1 presents an overview of the scope of this Recommendation and its relationship to the other elements of the T.120 framework within a single node.

# 2	Normative References

The following Recommendations and International Standards contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and Standards are subject to revision, and parties to agreements based on this Recommendation are encouraged to investigate the possibility of applying the most recent edition of the Recommendations and Standards listed below. Members of IEC and ISO maintain registers of currently valid International Standards. The ITU-T Secretariat maintains a list of the currently valid ITU-T Recommendations.

–	CCITT Recommendation F.710 (1991), General principles for audiographic conference service.

–	ITU-T Recommendation H.221 (1993), Frame structure for a 64 to 1920 kbits/s channel in audiovisual teleservices.

–	CCITT Recommendation T.35 (1991), Procedure for the allocation of CCITT defined codes for non-standard facilities.

–	ITU-T Recommendation T.1201), Overview of the T.120-Series Recommendations.

–	ITU-T Recommendation T.122 (1993), Multipoint Communication Service for Audiographics and Audiovisual Conferencing Service Definition.

–	ITU-T Recommendation T.123 (1994), Protocol stacks for audiographic and audiovisual teleconference applications.

–	ITU-T Recommendation T.125 (1994), Multipoint Communication Service Protocol Specification.

–	ITU-T Recommendation T.126 (1995), Multipoint Still Image and Annotation Protocol.

–	ITU-T Recommendation T.127 (1995), Multipoint binary file transfer protocol.

–	ITU-T Recommendation X.680 (1994), Information technology – Abstract Syntax Notation One (ASN.1): Specification of basic notation.

–	ITU-T Recommendation X.691 (1995), Information technology – ASN.1 encoding rules: Specification of Packed Encoding Rules (PER).

–	ISO/IEC 10646-1:1993, Information technology – Universal Multiple-Octet Coded Character Set (UCS) – Architecture and Basic Multilingual Plane.

FIGURE 1-1/T.124...[D01]  16.5 CM

# 3	Definitions

For the purposes of this Recommendation, the following definitions apply.

3.1	application protocol: Any standard or non-standard protocol specification which is designed to make use of T.120 services.

3.2	application protocol entity: The instantiation of an Application Protocol in a terminal or MCU. Application Protocol Entities are employed by User Applications, but are not themselves User Applications. An Application Protocol Entity communicates with GCC through the GCC Provider present at its local terminal or MCU via a GCCSAP. Only Application Protocol Entities communicate with GCC Providers; User Applications do not. Multiple Application Protocol Entities based on the same Application Protocol may enroll at a single node. These may be either in the same or separate Application Protocol Sessions. A single Application Protocol Entity is assumed to communicate with the local GCC Provider via a single GCCSAP, and is also assumed to have a single MCS User ID if it has enrolled in the active state. An Application Protocol Entity which has enrolled in the active state is part of a single Application Protocol Session as indicated by its Session ID (or lack of a Session ID which indicates that it is part of the Default Session). An

Application Protocol Entity which has enrolled in the inactive state is not considered part of any Application Protocol Session, however an inactive Application Protocol Entity may make use of GCC services and information associated with such an Application Protocol Entity is included in the Application Roster.

3.3	application protocol key: The Application Protocol Key identifies the type of Application Protocol for an Application Protocol Session. Multiple Application Protocol Sessions of the same type would be identified using the same Application Protocol Key (but different Session IDs). An Application Protocol Key is either an ASN.1 OBJECT IDENTIFIER belonging to a Recommendation, Standard, or non-standard protocol, or, alternatively, it is a non-standard identifier using the encoding conventions of Recommendation H.221.

3.4	application protocol session: A set of Peer Application Protocol Entities.

3.5	application record: A set of information for a specific Application Protocol Entity at a specific node. This set includes the Application User ID, the Active/Inactive flag, as well as other parameters.

3.6	application registry: A central repository located at the Top GCC Provider where an Application Protocol Entity can register its use of tokens, channels, and other parameters. Peer Application Protocol Entities can then access the registry to discover this registered information.

3.7	application roster: The set of all Application Records from all enrolled Application Protocol Entities at all nodes in a conference, including the Application Capabilities List for each Application Protocol Session.

3.8	application user ID: MCS User ID assigned by MCS to an Application Protocol Entity.

3.9	conducted mode: Conducted Mode allows a Conductor node the ability to control Application Protocol Entities at all nodes in a conference, and to restrict operation of Application Protocol Entities by other nodes. Conducted mode is established when a Conductor has been assigned to a conference. This is achieved when a node has successfully grabbed the conference Conductor Token.

3.10	conductor: The Conductor, if present, is a node in a conference which controls certain aspects of the conference (e.g. control of communication between Application Protocol Entities, control over conference participants, and conference termination). There shall be either zero or one Conductors in a conference. A node becomes conductor by grabbing the conductor token, or by requesting or accepting conductorship from the current conductor.

3.11	conference: A number of nodes that are joined together and that are capable of exchanging audiographic and audiovisual information across various communication networks.

3.12	conference application roster: A data-base maintained by each GCC Provider consisting of a set of Application Records, one for every Application Protocol Entity at every node in the conference, as well as other information such as the Application Capabilities List for each Application Protocol Session.

3.13	conference profile: A data-base maintained by each GCC Provider consisting of information pertinent to a conference as a whole such as Conference Name, Password (if any), etc.

3.14	conference roster: A data-base maintained by each GCC Provider consisting of a list of nodes in a conference. For each node, this list includes the Node ID of the node, type of the node, the name of the node, and may include a list of participants at the node, as well as other optional information.

3.15	control GCC service access point: The communication interface between a GCC Provider and the Node Controller within a single node.

3.16	convener: Node that created a conference by issuing the GCC-Conference-Create request primitive.

3.17	convener password: An identifying numeric string, as well as optional text string, which may be used when a conference is created to allow the convener of the conference to leave the conference and re-enter at a later time using the password to regain convener privileges. A Convener Password must be included during conference creation (in one of its two possible forms) for this to be possible. Use of the correct Convener Password allows joining of a locked conference, but it does not avoid the need to specify the correct Password when joining a Password protected conference.

3.18	default session: Active Application Protocol Entities with no Session ID included in their Session Key are considered part of a separate unique session referred to as the Default Session.

3.19	entity ID: A 16-bit numeric identifier used to identify each Application Protocol Entity enrolled at a node. The value of the Entity ID is unique among all Application Protocol Entities of any type within a single node. It need not be unique between nodes. A particular Application Protocol Entity in a conference may be uniquely identified by the combination of the Entity ID and the Node ID corresponding to the node at which the Application Protocol Entity is enrolled.

3.20	GCC provider: Agent providing GCC services to local Node Controller and Application Protocol Entities at a terminal or MCU.

3.21	GCC service access point: The communication interface between a GCC Provider and an Application Protocol Entity within a single node.

3.22	handle: A 32-bit integer number allocated by the Top GCC Provider using the GCC-Registry-Allocate-Handle request primitive. This number is guaranteed to be unique within a conference.

3.23	local application roster: A data-base maintained by each GCC Provider consisting of one Application Record as well as other information, such as the Application Capabilities List, for each Application Protocol Entity which has locally enrolled with the GCC Provider. This is used to form the information exchanged with other nodes to determine the Conference Application Roster.

3.24	MCS domain: A hierarchy of MCS connections between nodes. Nodes may use MCS services to communicate within a single domain, but not between separate domains. A GCC Conference corresponds one-to-one with a single MCS Domain.

3.25	MCS domain selector: A locally unique identifier of an MCS Domain.

3.26	MCS user ID: Unique identification number assigned by MCS to an MCS User. GCC Providers as well as Application Protocol Entities are MCS Users. An MCS User ID assigned to a GCC Provider is referred to as a Node ID. An MCS User ID assigned to an Application Protocol Entity is referred to as an Application User ID. An MCS User ID is valid only within a single MCS Domain.

3.27	multipoint: The ability to exchange data among multiple nodes simultaneously as compared with point-to-point, where data is exchanged between two directly connected nodes.

3.28	multipoint control unit: Commonly referred to as an MCU or bridge, a multiport device that serves to connect terminals and other MCUs in a multipoint fashion. A GCC-capable MCU runs GCC and MCS. An MCU is not primarily intended as an end-point for user communication.

3.29	multiport terminal: End-point audiographic or audiovisual equipment that also includes the ability to bridge T.120 information. Like a terminal, the behavior of a multiport terminal is typified by automatic establishment of a single conference. But, like an MCU, for a given conference in a multiport terminal, there may be more than one MCS connection.

3.30	node: A terminal, multiport terminal, or MCU. A single node comprises a single GCC provider. A single node may consist of one or more physical devices. Similarly, one physical device may host several logical nodes.

3.31	node controller: A functional entity for which there is one for each terminal or MCU, which serves as the controller of that node.

3.32	node ID: MCS User ID assigned by MCS to the GCC Provider at a node.

3.33	non-conducted mode: The mode in which a conference has no Conductor.

3.34	participant: A person participating in a conference at a node.

3.35	password: A numeric string, as well as an optional text string, which may be specified when a conference is created. If so, when attempting to join a conference, a node must include this Password (in one of the two possible forms) in the GCC-Conference-Join primitive in order for that node to be accepted into the conference.

3.36	peer application protocol entity: Application Protocol Entities which have enrolled in the active state using identical Session Keys, including the Session ID portion of the Session Key. Peer Application Protocols are those which may communicate with each other during a conference.

3.37	resource: Something that can be used and shared by nodes in a conference. A resource comprises both channels and tokens.

3.38	session ID: An optional parameter included in a Session Key used to distinguish between multiple sets of Peer Application Protocol Entities which are based on the same base Application Protocol. Each set of Peer Application Protocol Entities, defined by the use of a common Application Protocol and identical Session IDs, separately communicate among themselves. Active Application Protocol Entities with no Session ID included in their Session Key are considered part of a separate unique session referred to as the Default Session. A set of Peer Application Protocol Entities is referred to as an Application Protocol Session. Session IDs are in the form of MCS Channel IDs.

3.39	session Key: An identifier which is common to Peer Application Protocol Entities. A Session Key consists of two components. One component, the Application Protocol Key identifies the type of Application Protocol. The second component, which is an optional part of a Session Key, is the Session ID which identifies the specific session for this Application Protocol (the lack of a Session ID indicates the Default Session). Application Protocol Entities whose entire Session Keys are in common, including the Application Protocol Key as well as the Session ID, if any, are considered Peer Application Protocol Entities.

3.40	terminal: End-point audiographic or audiovisual equipment. A GCC-capable terminal runs GCC and MCS. A terminal is limited, within a conference, to a single MCS connection.

3.41	top GCC Provider: The GCC Provider which is co-resident with the Top MCS Provider in a conference. The Top GCC Provider has responsibilities not required of other GCC Providers in a conference. The location of the Top GCC Provider remains unchanged for the duration of a conference.

3.42	unicode: Multilingual text string format as defined by the Basic Multilingual Plane of ISO/IEC Specification 10646-1.

3.43	unicode Row 00: A subset of Unicode consisting of 256 code positions containing the Basic Latin and Latin-1 Supplement character sets plus control characters and reserved codes.

3.44	user application: An entity which makes use of one or more Application Protocol Entities. A User Application is limited in its scope to those tasks which have no effect on the interpretation of information between Peer Application Protocol Entities such as presentation to the end-user. User Applications therefore do not require specification, either by standard Recommendation or otherwise, to allow interoperability between the Application Protocol Entities which they make use of and their Peer Application Protocol Entities.

# 4	Abbreviations

For the purposes of this Recommendation, the following abbreviations are used:

CSDN	Circuit Switched Data Network

GCC	Generic Conference Control

GCCSAP	Generic Conference Control Service Access Point

ISDN	Integrated Services Digital Network

MCS	Multipoint Communication Service

MCU	Multipoint Control Unit

PDU	Protocol Data Unit

PSDN	Packet Switched Data Network

PSTN	Public Switched Telephone Network

The following abbreviations are used in this Recommendation, and defined in Recommendation T.122:

MCS	Multipoint Communication Service

MCSAP	Multipoint Communication Service Access Point

# 5	Conventions

The primitive parameters of the abstract services defined in this Recommendation use the following key:

M:	parameter is mandatory

C:	parameter is conditional

O:	parameter is optional

Blank	parameter is absent

(=):	value of the parameter is identical to the value of the corresponding parameter of the preceding primitive, where preceding is defined relative to the order: request, indication, response, confirm.

(=RQ):	value of the parameter is identical to the value of the corresponding parameter in a preceding primitive, where RQ = request, IN = indication, RS = response, and CF = confirm.

Primitives are categorized in up to four types: Request, Indication, Response, and Confirm. Some primitives support all of these types, while others do not. These four types are defined as follows:

Request primitive:	Those that are sourced from a Node Controller or Application Protocol Entity to initiate a certain action.

Indication primitive:	Those that are sourced from a GCC Provider either as a result of a Request primitive, or as a result of a GCC initiated action.

Response primitive:	Those that are sourced from a Node Controller or Application Protocol Entity in response to an Indication primitive which is defined to require a response.

Confirm primitive:	Those that are sourced from a GCC Provider as a result of a Response primitive.

PDUs are categorized into three types. PDU names all include the words Request, Indication, or Response to indicate the intended use of the PDU. These are defined as follows:

Request PDUs:	Those that require a Response PDU in return. If the request is for a function which is not supported by the receiving node (e.g. an optional or non-standard PDU), a generic response PDU, FunctionNotSupportedResponse, shall be used to provide a response to the requesting node (sent to the Node ID Channel of the requesting node).

Indication PDUs:	Those that do not require a response (e.g. those that are for informational purposes).

Response PDUs:	Those which are in response to a particular Request PDU to be sent on the Node ID Channel of the requesting node.

# 6	Overview

Within the context of the ITU-T Audiovisual Conferencing Service, a conference refers to a group of geographically dispersed nodes that are joined together and that are capable of exchanging audiographic and audiovisual information across various communication networks. Participants taking part in a conference may have access to various types of media handling capabilities such as audio only (telephony), audio and data (audiographics), audio and video (audiovisual), and audio, video, and data (multimedia).

The F-, G-, H-, and T-Series Recommendations provide a framework for the interworking of audio, video, and graphics terminals on a point-to-point basis through existing telecommunication networks. They also provide the capability for three or more terminals in the same conference to be interconnected by means of an MCU.

This Recommendation provides a high-level framework for conference management and control of audiographics and audiovisual terminals, and MCUs. It coexists with companion Recommendations T.122 and T.125 (MCS) and T.123 (AVPS) to provide a mechanism for conference establishment and control. Recommendations T.122, T.123, T.124, and T.125 form the minimum set of Recommendations to develop a fully functional terminal or MCU.

This Recommendation includes the following Generic Conference Control (GCC) functional components: conference establishment and termination, managing the Conference Roster, managing the Application Roster, Application Registry services, and conference conductorship. The service definitions for the primitives associated with these functional components are contained in clause 7. The corresponding protocol definitions are contained in clause 8.

Figure 6-1 shows an example of how GCC components are distributed throughout an MCS domain. The GCC components are shown in white. Each terminal or MCU contains a GCC Provider that provides GCC services to the local Node Controller and Application Protocol Entities.

FIGURE 6-1/T.124...[D02] = 11 CM

## 6.1	System model for a conference node

Each Node participating in a GCC conference consists of an MCS layer, a GCC layer, a Node Controller and may also include one or more Application Protocol Entities. The relationship between these components within a single node is illustrated in Figure 6-2. The Node Controller is the controlling entity at a node, dealing with the aspects of a conference that apply to the entire node. The Node Controller interacts with GCC, but does not interact directly with MCS. Application Protocol Entities also interact with GCC, as well as directly with MCS. The services provided by GCC to Application Protocol Entities are primarily to enable Peer Application Protocol Entities to communicate directly, via MCS. Local communication between individual Application Protocol Entities or between Application Protocol Entities and the Node Controller may take place, but is a local implementation matter not covered by this Recommendation. Within a node, more than one Application Protocol Entity may be based on the same Application Protocol. In this case, they may either be part of the same Application Protocol Session, allowing them to communicate within the node as well as to other Peer Application Protocol Entities at other nodes, or they may be part of separate Application Protocol Sessions, allowing them to communicate separately among their peers, but using the same protocol.

The service primitives as described in clause 7 apply to the GCC Service Access Point and the Control GCC Service Access Point as indicated in 6-2. The PDUs described in clause 8 are communicated using MCS service primitives available at the Control MCS Access Point (MCSAP).

NOTES

1	The normative intent of this Recommendation is to specify the procedures and contents of external communication — sequences of primitive operations and data exchanges acting through the Control MCSAP for purposes of conference control. The internal decomposition of a node suggested in Figure 6-2 serves to motivate features of the GCC protocol but is not normative. GCC service primitives whose effect is purely local need not exist at all nodes in the form that they are described here. Statements about what a node controller or an application protocol entity shall do in certain circumstances should be interpreted loosely if the same results in external communication can be achieved through different internal mechanisms.

2	The system model assumes that service requests may be issued at any time. A Node Controller or Application Protocol Entity need not wait until it has received a confirm from the GCC Provider corresponding to a previous request before issuing another request. A specific implementation may, however, impose stricter requirements.

FIGURE 6-2/T.124...[D03] = 12 CM

## 6.2	Conference establishment and termination

GCC provides a set of services for establishment and termination of conferences. A conference can be viewed as a meeting room in which any number of participants may meet in order to exchange audiographic or audiovisual information. As with physical meeting rooms, services such as finding out what conferences are in progress, joining a conference, leaving a conference, restricting access to a conference, etc., are meaningful in audiographic and audiovisual conferences as well.

Prior to joining a conference, participants at a node may not know all of the information needed to join. GCC provides a means for participants to view a list of Conference Names and select the one they wish to join. This service is analogous to the conference schedule typically posted in a lobby, allowing someone to find the meeting room in which a particular meeting is taking place.

GCC provides a means to create new conferences. This may be done either by a conference participant, or by a conference administrator. When a new conference is created, its characteristics, referred to as its Conference Profile, are specified by its creator. The Conference Profile includes such things as the Conference Name, whether it has restricted access by means of a Password, whether it is open to be freely joined (unlocked) or restricted to be joined by invitation only (locked).

Expanding an existing conference may be initiated by the joining node, or by a conference Convener (or convener designated node). If a conference is Password protected, a joining node is required to supply the correct Password to be allowed into the conference. If a conference is locked, joining is only allowed at the request of the Convener, or convener designated node. Nodes may join or be added at any time during a conference. It is possible to be joined with more than one conference simultaneously. GCC also provides a means of transferring participants from one conference to another. This function may be used to achieve the effect of merging two conferences, or splitting a conference into more than one conference.

At any time a node may wish to disconnect from the conference, leaving the other nodes to continue the conference. Depending on the choice of termination method, a conference is either automatically terminated if all nodes disconnect from it, or manually terminated by an explicit termination request. The convener, or a convener-designated node may also forcibly terminate the entire conference at any time, or eject a particular node from it.

## 6.3	The conference roster

Once a node has joined a conference, it announces its presence to all other nodes in the conference. A GCC primitive is provided to allow each node in a conference to announce its presence in a conference, and another to provide each node with the full Conference Roster. The Conference Roster is a list of all nodes in the conference. For each node it includes information such as the name of the node, a list of participants at that node, as well as other information needed for proper communication between nodes. A node is not considered part of a conference until it has been included in the Conference Roster.

## 6.4	The application roster

GCC provides a means of identifying which Application Protocol Entities are available at each node and to provide necessary information for Peer Application Protocol Entities to communicate with each other. Upon joining a conference, each node sends to all other nodes its local list of Application Protocol Entities – its Local Application Roster – which it may update at any time thereafter. From this information, the Conference Application Roster is formed and broadcast to all nodes. Relevant portions of this roster are then communicated locally to each Application Protocol Entity as well as to the Node Controller. In addition to a simple roster, GCC also provides a service for Application Protocol Entities to include a list of Application Protocol–specific capabilities in the information exchanged. GCC applies a fixed set of rules to this information from all nodes in the conference to determine a common set of Application Capabilities. This information is also communicated locally to each Application Protocol Entity.

## 6.5	The application registry

The Application Registry is an active data-base residing at the Top GCC Provider that may be used to manage channels, tokens, and other shared resources used in a conference. The Application Registry can aid in establishing communication among peer Application Protocol Entities.

## 6.6	Conference conductorship

GCC provides a method for allowing a node to become a conductor for a conference. A token is used by GCC to determine whether a conference is conducted or non-conducted. The node which grabs the conductor token becomes the conductor of the conference. A node may also request conductorship or accept conductorship from the current conductor. Upon request, GCC provides the identity of the current conference conductor. On creation of a conference, it may be specified that conducted mode is not permitted for the duration of the conference.

Conducted mode is available as a means to provide order to the course of a conference. The actual means by which this order is provided is determined by the Application Protocols. Specifically, Recommendations specifying Application Protocols may define alternative procedures depending on whether the conference is conducted or non-conducted. GCC does provide a mechanism for basic conducted operation which may be made use of by Application Protocols. A

mechanism is provided by which a node may request permission from the conducting node, and if permission is granted, all Application Protocol Entities at that node may act accordingly as specified by the Application Protocol specification. Application Protocols may, for example, specify strict limitations in the allowed operations for nodes that do not have permission from the conductor, while removing some or all of these limitations for nodes which have this permission. Application protocols may also, for example, specify that once overall permission has been granted by the conductor, further permission must be granted by the Peer Application Protocol Entity at the conducted node, if one exists, before allowing certain operations to be performed.

## 6.7	Miscellaneous functions

A method is provided for coordinating timed conferences. A mechanism is provided for a node to find out how much time is remaining in a timed conference, as well as a mechanism for announcing to all nodes how much time is remaining (which would typically be used to announce that the time is almost up), and a mechanism for nodes to request more time to be added, if available.

A method is also provided to request assistance from an unspecified operator. Another function is provided to allow transmission of simple text messages.

## 6.8	Summary of GCC abstract services

Table 6-1 is a list of all GCC primitives and their associated PDUs. The table also shows whether or not each primitive is mandatory (M), conditionally required (C), or optional (O) for a terminal or an MCU. For a multiport terminal, for each primitive, the requirement shall be taken to be the most restrictive of either a terminal or MCU for that primitive. A conditionally required primitive is one which is required if the Application Protocol specification for one or more Application Protocol Entities located at that node mandates its use. The table also shows, for the corresponding PDUs, whether these are mandatory (M) or conditionally required (C) for both the transmit (T) and receive (R) directions. Again, for a multiport terminal, the requirement shall be taken to be the most restrictive of either a terminal or MCU for that PDU. A conditionally required PDU is one which is required only if the corresponding primitive is to be supported at that node. In the case that a primitive is not mandatory, but its corresponding PDU is mandatory, this implies that there is some portion of the protocol, not related to the primitive, that relies on the use of that PDU which is required to be supported.

TABLE  6-1/T.124

GCC Primitives and PDUs

| Functional unit | Primitives | Term | MCU | Associated PDUs | Dir. | Term | MCU |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Conference | GCC-Conference-Create request | M | O | ConferenceCreateRequest | T | M | C |
| establishment | GCC-Conference-Create indication | M | M | ConferenceCreateRequest | R | M | M |
| and | GCC-Conference-Create response | M | M | ConferenceCreateResponse | T | M | M |
| termination | GCC-Conference-Create confirm | M | O | ConferenceCreateResponse | R | M | C |
|  |  |  |  | UserIDIndication | T, R | M | M |
|  | GCC-Conference-Query request | M | M | ConferenceQueryRequest | T | M | M |
|  | GCC-Conference-Query indication | M | M | ConferenceQueryRequest | R | M | M |
|  | GCC-Conference-Query response | M | M | ConferenceQueryResponse | T | M | M |
|  | GCC-Conference-Query confirm | M | M | ConferenceQueryResponse | R | M | M |
|  | GCC-Conference-Join request | M | O | ConferenceJoinRequest | T | M | C |
|  | GCC-Conference-Join indication | O | M | ConferenceJoinRequest | R | C | M |
|  | GCC-Conference-Join response | O | M | ConferenceJoinResponse | T | C | M |
|  | GCC-Conference-Join confirm | M | O | ConferenceJoinResponse | R | M | C |
|  |  |  |  | UserIDIndication | T, R | M | M |

TABLE  6-1/T.124  (continued)

GCC Primitives and PDUs

| Functional unit | Primitives | Term | MCU | Associated PDUs | Dir. | Term | MCU |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | GCC-Conference-Invite request | O | M | ConferenceInviteRequest | T | C | M |
|  | GCC-Conference-Invite indication | M | M | ConferenceInviteRequest | R | M | M |
|  | GCC-Conference-Invite response | M | M | ConferenceInviteResponse | T | M | M |
|  | GCC-Conference-Invite confirm | O | M | ConferenceInviteResponse | R | C | M |
|  |  |  |  | UserIDIndication | T, R | M | M |
|  | GCC-Conference-Add request | O | O | ConferenceAddRequest | T | C | C |
|  | GCC-Conference-Add indication | O | O | ConferenceAddRequest | R | C | C |
|  | GCC-Conference-Add response | O | O | ConferenceAddResponse | T | C | C |
|  | GCC-Conference-Add confirm | O | O | ConferenceAddResponse | R | C | C |
|  | GCC-Conference-Lock request | O | O | ConferenceLockRequest | T | C | C |
|  | GCC-Conference-Lock indication | O | O | ConferenceLockRequest | R | C | C |
|  | GCC-Conference-Lock response | O | O | ConferenceLockResponse | T | C | C |
|  | GCC-Conference-Lock confirm | O | O | ConferenceLockResponse | R | C | C |
|  | GCC-Conference-Unlock request | O | O | ConferenceUnlockRequest | T | C | C |
|  | GCC-Conference-Unlock indication | O | O | ConferenceUnlockRequest | R | C | C |
|  | GCC-Conference-Unlock response | O | O | ConferenceUnlockResponse | T | C | C |
|  | GCC-Conference-Unlock confirm | O | O | ConferenceUnlockResponse | R | C | C |
|  | GCC-Conference-Lock-Report indication | O | O | ConferenceLockIndication | T, R | C | C |
|  |  |  |  | ConferenceUnlockIndication | T, R | C | C |
|  | GCC-Conference-Disconnect request | M | M | – | – | – | – |
|  | GCC-Conference-Disconnect indication | M | M | – | – | – | – |
|  | GCC-Conference-Disconnect confirm | M | M | – | – | – | – |
|  | GCC-Conference-Terminate request | O | O | ConferenceTerminateRequest | T | C | C |
|  |  |  |  | ConferenceTerminateRequest | R | M | M |
|  | GCC-Conference-Terminate indication | M | M | ConferenceTerminateIndication | T | M | M |
|  |  |  |  | ConferenceTerminateIndication | R | M | M |
|  | GCC-Conference-Terminate confirm | O | O | ConferenceTerminateResponse | T | M | M |
|  |  |  |  | ConferenceTerminateResponse | R | C | C |
|  | GCC-Conference-Eject-User request | O | O | ConferenceEjectUserRequest | T | C | C |
|  |  |  |  | ConferenceEjectUserRequest | R | M | M |
|  | GCC-Conference-Eject-User indication | M | M | ConferenceEjectUserIndication | T | M | M |
|  |  |  |  | ConferenceEjectUserIndication | R | M | M |
|  | GCC-Conference-Eject-User confirm | O | O | ConferenceEjectUserResponse | T | M | M |
|  |  |  |  | ConferenceEjectUserResponse | R | C | C |
|  | GCC-Conference-Transfer request | O | M | ConferenceTransferRequest | T | C | M |
|  |  |  |  | ConferenceTransferRequest | R | M | M |
|  | GCC-Conference-Transfer indication | M | M | ConferenceTransferIndication | T | M | M |
|  |  |  |  | ConferenceTransferIndication | R | M | M |
|  | GCC-Conference-Transfer confirm | O | M | ConferenceTransferResponse | T | M | M |
|  |  |  |  | ConferenceTransferResponse | R | C | M |
| Conference | GCC-Conference-Announce-Presence request | M | M | RosterUpdateIndication | T, R | M | M |
| roster | GCC-Conference-Announce-Presence confirm | M | M | – | – |  |  |
|  | GCC-Conference-Roster-Report indication | M | M | RosterUpdateIndication | T, R | M | M |
|  | GCC-Conference-Roster-Inquire request | O | O | – | – | – | – |
|  | GCC-Conference-Roster-Inquire confirm | O | O | – | – | – | – |
| Application roster | GCC-Application-Permission-To-Enroll indication | M | C | – | – | – | – |
|  | GCC-Application-Enroll request | M | C | RosterUpdateIndication | T, R | M | M |
|  | GCC-Application-Enroll confirm | M | C | – | – |  |  |
|  | GCC-Application-Roster-Report indication | M | C | RosterUpdateIndication | T, R | M | M |
|  | GCC-Application-Roster-Inquire request | O | O | – | – | – | – |
|  | GCC-Application-Roster-Inquire confirm | O | O | – | – | – | – |

TABLE  6-1/T.124  (continued)

GCC Primitives and PDUs

| Functional unit | Primitives | Term | MCU | Associated PDUs | Dir. | Term | MCU |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | GCC-Application-Invoke request | O | O | ApplicationInvokeIndication | T | C | C |
|  | GCC-Application-Invoke indication | O | O | ApplicationInvokeIndication | R | C | C |
|  | GCC-Application-Invoke confirm | O | O | – | – | – | – |
| Application | GCC-Registry-Register-Channel request | C | C | RegistryRegisterChannelRequest | T | C | C |
| registry |  |  |  | RegistryRegisterChannelRequest | R | M | M |
|  | GCC-Registry-Register-Channel confirm | C | C | RegistryResponse | T | M | M |
|  |  |  |  | RegistryResponse | R | C | C |
|  | GCC-Registry-Assign-Token request | C | C | RegistryAssignTokenRequest | T | C | C |
|  |  |  |  | RegistryAssignTokenRequest | R | M | M |
|  | GCC-Registry-Assign-Token confirm | C | C | RegistryResponse | T | M | M |
|  |  |  |  | RegistryResponse | R | C | C |
|  | GCC-Registry-Set-Parameter request | C | C | RegistrySetParameterRequest | T | C | C |
|  |  |  |  | RegistrySetParameterRequest | R | M | M |
|  | GCC-Registry-Set-Parameter confirm | C | C | RegistryResponse | T | M | M |
|  |  |  |  | RegistryResponse | R | C | C |
|  | GCC-Registry-Retrieve-Entry request | C | C | RegistryRetrieveEntryRequest | T | C | C |
|  |  |  |  | RegistryRetrieveEntryRequest | R | M | M |
|  | GCC-Registry-Retrieve-Entry confirm | C | C | RegistryResponse | T | M | M |
|  |  |  |  | RegistryResponse | R | C | C |
|  | GCC-Registry-Delete-Entry request | C | C | RegistryDeleteEntryRequest | T | C | C |
|  |  |  |  | RegistryDeleteEntryRequest | R | M | M |
|  | GCC-Registry-Delete-Entry confirm | C | C | RegistryResponse | T | M | M |
|  |  |  |  | RegistryResponse | R | C | C |
|  | GCC-Registry-Monitor request | C | C | RegistryMonitorEntryRequest | T | C | C |
|  |  |  |  | RegistryMonitorEntryRequest | R | M | M |
|  | GCC-Registry-Monitor indication | C | C | RegistryMonitorEntryIndication | T | M | M |
|  |  |  |  | RegistryMonitorEntryIndication | R | C | C |
|  | GCC-Registry-Monitor confirm | C | C | RegistryResponse | T | M | M |
|  |  |  |  | RegistryResponse | R | C | C |
|  | GCC-Registry-Allocate-Handle request | C | C | RegistryAllocateHandleRequest | T | C | C |
|  |  |  |  | RegistryAllocateHandleRequest | R | M | M |
|  | GCC-Registry-Allocate-Handle confirm | C | C | RegistryAllocateHandleResponse | T | M | M |
|  |  |  |  | RegistryAllocateHandleResponse | R | C | C |
| Conference | GCC-Conductor-Assign request | O | O | – | – | – | – |
| conductorship | GCC-Conductor-Assign indication | C | C | ConductorAssignIndication | T | M1) | M1) |
|  |  |  |  | ConductorAssignIndication | R | C | C |
|  | GCC-Conductor-Assign confirm | O | O | – | – | – | – |
|  | GCC-Conductor-Release request | O | O | ConductorReleaseIndication | T | C | C |
|  |  |  |  | ConductorReleaseIndication | R | M1) | M1) |
|  | GCC-Conductor-Release indication | C | C | ConductorReleaseIndication | T | M1) | M1) |
|  |  |  |  | ConductorReleaseIndication | R | C | C |
|  | GCC-Conductor-Release confirm | O | O | – | – | – | – |
|  | GCC-Conductor-Please request | O | O | – | – | – | – |
|  | GCC-Conductor-Please indication | O | O | – | – | – | – |
|  | GCC-Conductor-Please response | O | O | – | – | – | – |
|  | GCC-Conductor-Please confirm | O | O | – | – | – | – |
|  | GCC-Conductor-Give request | O | O | – | – | – | – |
|  | GCC-Conductor-Give indication | O | O | – | – | – | – |
|  | GCC-Conductor-Give response | O | O | ConductorAssignIndication | T | C | C |
|  | GCC-Conductor-Give confirm | O | O | – | – | – | – |
|  | GCC-Conductor-Inquire request | C | C | – | – | – | – |
|  | GCC-Conductor-Inquire confirm | C | C | – | – | – | – |

TABLE  6-1/T.124  (end)

GCC Primitives and PDUs

| Functional unit | Primitives | Term | MCU | Associated PDUs | Dir. | Term | MCU |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | GCC-Conductor-Permission-Ask request | O | O | ConductorPermissionAskIndication | T | C | C |
|  | GCC-Conductor-Permission-Ask indication | O | O | ConductorPermissionAskIndication | R | C | C |
|  | GCC-Conductor-Permission-Ask confirm | O | O | – | – | – | – |
|  | GCC-Conductor-Permission-Grant request | O | O | ConductorPermissionGrantIndication | T | C | C |
|  | GCC-Conductor-Permission-Grant indication | O | O | ConductorPermissionGrantIndication | R | C | C |
|  | GCC-Conductor-Permission-Grant confirm | O | O | – | – | – | – |
| Miscellaneous | GCC-Conference-Time-Remaining request | O | O | ConferenceTimeRemainingIndication | T | C | C |
| functions | GCC-Conference-Time-Remaining indication | O | O | ConferenceTimeRemainingIndication | R | C | C |
|  | GCC-Conference-Time-Remaining confirm | O | O | – | – |  |  |
|  | GCC-Conference-Time-Inquire request | O | O | ConferenceTimeInquireIndication | T | C | C |
|  | GCC-Conference-Time-Inquire indication | O | O | ConferenceTimeInquireIndication | R | C | C |
|  | GCC-Conference-Time-Inquire confirm | O | O | – | – |  |  |
|  | GCC-Conference-Extend request | O | O | ConferenceTimeExtendIndication | T | C | C |
|  | GCC-Conference-Extend indication | O | O | ConferenceTimeExtendIndication | R | C | C |
|  | GCC-Conference-Extend confirm | O | O | – | – | – | – |
|  | GCC-Conference-Assistance request | O | O | ConferenceAssistanceIndication | T | C | C |
|  | GCC-Conference-Assistance indication | O | O | ConferenceAssistanceIndication | R | C | C |
|  | GCC-Conference-Assistance confirm | O | O | – | – | – | – |
|  | GCC-Text-Message request | O | O | TextMessageIndication | T | C | C |
|  | GCC-Text-Message indication | O | O | TextMessageIndication | R | C | C |
|  | GCC-Text-Message confirm | O | O | – | – | – | – |
|  | – | – | – | FunctionNotSupported | T, R | M | M |
| 1) Reception of ConductorReleaseIndication PDUs is mandatory to allow the Top GCC Provider to properly handle requests which require privileges. Privileges depend on whether the conference is in conducted or non-conducted mode. Transmission of ConductorAssignIndication and ConductorReleaseIndication PDUs by the Top GCC Provider is mandatory for handling new nodes joining a conference. |  |  |  |  |  |  |  |

# 7	GCC service definition

## 7.1	Conference establishment and termination

In this clause, primitives needed for conference establishment and conference termination are described. All of the primitives in this clause are intended for use only by the Node Controller at a terminal or MCU.

### 7.1.1	The conference profile

All conferences have the following characteristics which are defined when the conference is created and communicated to each node as it enters the conference. These characteristics remain unchanged for the duration of the conference. This information is collectively referred to as the Conference Profile:

•	Conference name – A numerical string and an optional Unicode Row 00 text string identifying the conference. If both forms of Conference Name are used when a conference is created, when that conference is joined, either form may be specified to indicate the conference to be joined.

•	Conference description – An optional text string to describe the conference. For a listed conference, this string is reproduced in the Conference Descriptor List in response to a GCC-Conference-Query request.

•	Password protected vs. not password protected – Choice of whether the conference is Password protected or not.

•	Listed vs. unlisted – Choice of whether the conference is listed or not listed on the conference list provided when querying the list of available conferences.

•	Conductible vs. non-conductible – Choice of whether the conference is able to be placed in conducted mode or whether the conference is always non-conducted.

•	Termination method – Choice of whether conference should last until explicitly terminated (manually terminating), or if it should last until all participants disconnect (automatically terminating).

•	Privilege lists – A set of lists indicating which privileges, normally only available to the Convener, are also allowed to the Conductor, to any node in a conducted conference, or to any node in a non-conducted conference.

### 7.1.2	Description of abstract services

The following is a list of the primitives defined in this subclause and a brief summary of the function of each:

•	GCC-Conference-Create – Used by the Node Controller to create a new conference, specifying the characteristics of that conference.

•	GCC-Conference-Query – Used by the Node Controller to query what conferences are currently in progress as well as the information needed to attempt to join them.

•	GCC-Conference-Join – Used by the Node Controller to join an existing conference.

•	GCC-Conference-Invite – Used by the Node Controller to invite a node into an existing conference.

•	GCC-Conference-Add – Allowed only by the conference convener or convener-designated node, this is used by the Node Controller to request that a node be added to the conference by dialing out from an MCU.

•	GCC-Conference-Lock – Allowed only by the conference convener or convener-designated node, this is used by the Node Controller to prevent new participants from joining a conference without being explicitly added.

•	GCC-Conference-Unlock – Allowed only by the conference convener or convener-designated node, this is used by the Node Controller to allow new participants to join a conference.

•	GCC-Conference-Lock-Report – Provides an indication to the Node Controller that a conference has changed from being locked to being unlocked or vice versa.

•	GCC-Conference-Disconnect – Used by a Node Controller to disconnect the local node from an ongoing conference.

•	GCC-Conference-Terminate – Allowed only by the conference convener or convener-designated node, this is used by the Node Controller to terminate an entire conference, disconnecting all nodes.

•	GCC-Conference-Eject-User – Allowed only by the conference convener or convener-designated node (or by the node directly above the ejected node in the connection hierarchy), this is used by the Node Controller to disconnect a specific node from an ongoing conference.

•	GCC-Conference-Transfer – Allowed only by the conference convener or convener-designated node, this is used by the Node Controller to transfer nodes joined with one conference to another conference. This may be used as part of the process of merging or splitting conferences.

#### 7.1.2.1	GCC-Conference-Create

The GCC-Conference-Create request primitive is used by a Node Controller to create a new conference at a remote node to which the local node is automatically joined. This primitive may be issued at any time. When a conference is created, the node to which the creation request is directed (the node which receives the GCC-Conference-Create indication) is also automatically joined to the conference and becomes the Top GCC Provider for that conference. This node remains the Top GCC Provider for the conference as long as the conference continues to exist. In some implementations, it may

be possible to create a conference locally without the use of GCC primitives. In this case, the node at which the conference is created becomes the Top GCC Provider. Table 7-1 shows the parameters and types of this primitive. Figure 7-1 shows the sequence of events when using this primitive.

TABLE  7-1/T.124

GCC-Conference-Create – Types of primitives and their parameters

| Parameter | Request | Indication | Response | Confirm |
| --- | --- | --- | --- | --- |
| Conference Name | M | M() | M() | M() |
| Conference Name Modifier | C |  | C |  |
| Conference ID |  | M | M() | M |
| Convener Password | O | O() |  |  |
| Password | O | O() |  |  |
| Conference Locked | M | M() |  |  |
| Conference Listed | M | M() |  |  |
| Conference Conductible | M | M() |  |  |
| Termination Method | M | M() |  |  |
| Conductor Privilege List | O | O() |  |  |
| Conducted-mode Conference Privilege List | O | O() |  |  |
| Non-conducted-mode Conference Privilege List | O | O() |  |  |
| Conference Description | O | O() |  |  |
| Caller Identifier | O | O() |  |  |
| Calling Address | O | O() |  |  |
| Called Address | O | O |  |  |
| Domain Parameters | M | M | M | M() |
| Quality of Service | M | M | M | M() |
| Local Network Address | O |  | O |  |
| User Data | O | O() | O | O() |
| Result |  |  | M | M() |

Conference Name: Name by which the conference to be created is identified. This consists of a numerical string along with an optional Unicode Row 00 text string, from zero to 255 characters each. If both forms of a Conference Name are used, if a node wishes to join this conference, it may specify either form of the name in the join request.

Conference Name Modifier: If the requesting or responding node is already joined to a conference with the same Conference Name (either numerical or text portion) as that included in the request, this parameter shall also be included in the corresponding request or response primitive. The value of this parameter shall be unique among all conferences at the corresponding node which have this Conference Name. This modifier, if included, shall be used as the Called Node Conference Name Modifier parameter in a GCC-Conference-Join request by another node attempting to join the conference through a direct connection with the corresponding node. This modifier is also included in the response to a GCC-Conference-Query directed at this node. This parameter is a numerical string up to 255 digits in length.

Conference ID: Locally allocated identifier of the newly created conference. All subsequent references to the conference are made using the Conference ID as a unique identifier. The Conference ID shall be identical with the MCS Domain Selector used locally to identify the MCS Domain associated with the conference.

Convener Password: This optional parameter contains a numeric string, as well as an optional Unicode Row 00 text string, used for the convener to identify itself in later operations, allowing the convener to disconnect and later rejoin the conference, maintaining convener privileges (only when rejoined with a direct connection to the Top GCC Provider). This is the private password which will allow the convener to perform convener-only operations (maximum 255 digits and 255 characters). If this parameter is NULL, then it is not possible for the convener to disconnect and later rejoin maintaining convener privileges.

Password: This is a numeric string, as well as an optional Unicode Row 00 text string, to serve as a Password to enter the conference (maximum 255 digits and 255 characters). If no Password is specified, the conference is not Password protected.

NOTE – If the conference is Password protected, the Node controller must specify a numeric Password, and may also specify a text Password. The numeric Password is required to allow for nodes which have no suitable text entry mechanism. In the case that a text password is used, there is no assumption that the numeric Password that must also be included is generated by the user. It may be more convenient and secure to use a machine generated numeric Password.

Conference Locked: Setting this flag immediately locks a conference, preventing anyone from joining this conference unless they are explicitly added using the GCC-Conference-Add primitive. To lock a conference at any time after issuing this primitive, the primitive GCC-Conference-Lock may be used. To unlock a conference, GCC-Conference-Unlock may be used. Since support of the GCC-Conference-Add and GCC-Conference-Unlock primitives are optional, a locked conference created at an MCU (which is not also a terminal) which did not support either of these primitives would be of little use. Such an MCU may reject the conference creation stating locked-conference-not-supported as the reason. If either GCC-Conference-Add indication/response (along with GCC-Conference-Invite request/confirm) or GCC-Conference-Unlock indication/response primitives are supported, an MCU shall allow creation of a locked conference and shall not make use of this reason for rejection.

Conference Listed: The TRUE setting of this flag indicates that this conference may be listed when using the conference-query facility. The FALSE setting of this flag indicates that this conference shall not be listed.

Conference Conductible: The TRUE setting of this flag indicates that this conference may be placed in conducted mode using the GCC-Conductor-Assign primitive. The FALSE setting of this flag indicates that this conference shall be non-conducted only, and attempts to assign a conductor shall be rejected.

Termination Method: This parameter indicates whether the conference shall remain in existence until explicitly terminated by the Convener or convener-designated node using the GCC-Conference-Terminate primitive (manually terminating), or if the conference will terminate when there are no nodes joined to it or if explicitly terminated (automatically terminating).

Conductor Privilege List: This is a list of flags indicating which functions the convener is designating as allowable to be used by the conference conductor, if any. The flags in this list correspond to the operations: GCC-Conference-Terminate, GCC-Conference-Eject-User, GCC-Conference-Add, GCC-Conference-Lock, GCC-Conference-Unlock and GCC-Conference-Transfer.

Conducted-mode Conference Privilege List: This is a list of flags indicating which functions the convener is designating as allowable to be used by any node in a conducted-mode conference. The flags in this list correspond to the same operations as for the above parameter.

Non-conducted-mode Conference Privilege List: This is a list of flags indicating which functions the convener is designating as allowable to be used by any node in a non-conducted-mode conference. The flags in this list correspond to the same operations as for the above parameter.

Conference Description: An optional Unicode text string, up to 255 characters in length, which may be used to describe the conference. This string is maintained by the GCC Providers in the conference to use as part of the response to GCC-Conference-Query requests.

Caller Identifier: Optional Unicode text string (maximum 255 characters) which may be used to identify the calling node to the node at which the conference is to be created. The use of this information at this node is beyond the scope of this Recommendation. It may be used, for example, to allow a user at that node to select among a limited set of

participants which are allowed to create conferences. Because this string can be set to any value, it does not necessarily add to the security of a conference to do this, however.

Calling Address: Optional address to be included in the MCS-Connect-Provider primitive on establishing an MCS connection. See Recommendation T.122 for the interpretation of this parameter.

Called Address: Optional address to be included in the MCS-Connect-Provider primitive on establishing an MCS connection. See Recommendation T.122 for the interpretation of this parameter.

Domain Parameters: Domain parameters to be included in the MCS-Connect-Provider primitive on establishing an MCS connection. See Recommendation T.122 for the interpretation of this parameter.

Quality of Service: Quality of Service parameters to be included in the MCS-Connect-Provider primitive on establishing an MCS connection. See Recommendation T.122 for the interpretation of this parameter.

Local Network Address: If included in either the request or response, the local GCC Provider at the corresponding node shall use this information to include as the Network Address parameter in the Conference Descriptor List sent as part of the response to a GCC-Conference-Query request from another node.

User Data: Optional user data which may be used for functions outside the scope of this Recommendation such as authentication, billing, etc.

Result: An indication of whether the request was accepted or rejected, and if rejected, the reason why. It contains one of a list of possible results: successful, user rejected, resources not available, rejected for symmetry-breaking, locked conference not supported, Conference Name and Conference Name Modifier already exist, domain parameters unacceptable, domain not hierarchical, lower-layer initiated disconnect, unspecified failure to connect. A negative result in the GCC-Conference-Create confirm does not imply that the physical connection to the node to which the connection was being attempted is disconnected.

FIGURE 7-1/T.124...[D04] = 8.5 CM

#### 7.1.2.2	GCC-Conference-Query

The GCC-Conference-Query request primitive may be used by a Node Controller to determine what conferences are currently in existence at a particular MCU. Table 7-2 shows the parameters and types of this primitive. Figure 7-2 shows the sequence of events when using this primitive.

TABLE  7-2/T.124

GCC-Conference-Query – Types of primitives and their parameters

| Parameter | Request | Indication | Response | Confirm |
| --- | --- | --- | --- | --- |
| Node Type | M | M() | M | M() |
| Asymmetry Indicator | C | C() | C | C() |
| Conference Descriptor List |  |  |  | C |
| Calling Address | O | O() |  |  |
| Called Address | O | O |  |  |
| User Data | O | O() | O | O() |
| Result |  |  | M | M() |

Node Type: The node type is either terminal, MCU, or multiport terminal.

Asymmetry Indicator: This is a field which is needed for certain conference establishment procedures which require knowledge of which node across a single connection was the calling node (the initiator of the physical connection), and which is the called node. This parameter is required in the case of a physical connection between the two nodes (that is, an underlying connection between the two nodes using the PSTN, ISDN, or CSDN cases of Recommendation T.123). Otherwise, this parameter is optional. If a node is aware of its status as the calling or called node, it shall set this indicator to the proper value. In some cases a node may not be certain whether it is the calling or called node. In this case, this parameter shall include a 32-bit random number. If both nodes indicate that they are not certain of their status, the random numbers are used to determine which node should be considered the calling node and which the called node for the purposes of the conference establishment procedure (note that this may not correctly reflect the true calling or called node). The node which transmitted the largest of the two random numbers shall be considered the calling node for the purpose of the conference establishment procedure. If both random numbers are identical, this decision shall be considered unsatisfied and the requester shall re-issue the GCC-Conference-Query request using a different random number (a different random number shall also be used in the resulting response). If both nodes respond with actual values for this parameter (indicating knowledge of whether they are the called or calling node), but the resulting exchange indicates a disagreement (i.e. both think they are the calling node or both think they are the called node), this decision shall also be considered unsatisfied and the requester shall re-issue the GCC-Conference-Query request using a random number (the response shall also use a random number). During the period where a query request remains unconfirmed on one side of a given connection, the value for this parameter in all query requests and query responses issued by this node over the same physical connection shall remain unchanged. The random numbers should be generated to be uniformly distributed over the entire numerical range.

Conference Descriptor List: A variable length list of conference descriptors each indicating an active conference available to be joined. This list does not include conferences which had been designated unlisted at their time of creation. If there are no available conferences, the list contains zero entries. Each conference descriptor includes the parameters shown in Table 7-3.

Calling Address: Optional address to be included in the MCS-Connect-Provider primitive on establishing an MCS connection.

Called Address: Optional address to be included in the MCS-Connect-Provider primitive on establishing an MCS connection.

User Data: Optional user data which may be used for functions outside the scope of this Recommendation such as authentication, billing, etc.

Result: An indication of whether the request was accepted or rejected, and if rejected, the reason why. It contains one of a list of possible results: successful, user-rejected, domain parameters unacceptable, domain not hierarchical, lower-layer initiated disconnect, unspecified failure to connect.

TABLE  7-3/T.124

Contents of a Conference Descriptor

| Parameter | Description |
| --- | --- |
| Conference Name | Conference Name of the conference. If the requesting node wishes to join this conference, this parameter is the value that shall be used in the Conference Name parameter of the GCC-Conference-Join request. This parameter is a numerical string along with an optional Unicode Row 00 text string, maximum 255 characters each. If both forms of the name are given, either form may be specified in the join request. |
| Conference Name Modifier (conditional) | If at the node returning the response, the conference is known by a name which includes a Conference Name Modifier, this parameter is included. If the requesting node wishes to join this conference, this is the Conference Name Modifier that shall be used in the Called Node Conference Name Modifier parameter of the GCC-Conference-Join request. This parameter is a numerical string up to 255 digits in length. |
| Conference Description (conditional) | An optional Unicode text string, up to 255 characters in length, used to describe the conference. This parameter may be particularly useful in cases where more than one conference in the Conference Descriptor List has the same Conference Name as a means of distinguishing between these conferences. |
| Locked/Unlocked | Flag indicating whether the conference is currently locked or unlocked. |
| Password In The Clear Required | Indicates that the conference is password protected with a password that may be used without encryption in a GCC-Conference-Join request without first being challenged for the password. |
| Network Address (conditional) | Address information provided to the requesting node. This is provided only if the optional Network Address parameter had been included in the connection establishment primitive at that node (either GCC-Conference-Create, GCC-Conference-Join, or GCC-Conference-Invite). |

FIGURE 7-2/T.124...[D05] = 9 CM

#### 7.1.2.3	GCC-Conference-Join

The GCC-Conference-Join request primitive may be used by the Node Controller to cause the local node to join an existing conference. This primitive may be issued at any time. A node may be joined to more than one conference simultaneously. If the conference is Password protected, the Password parameter must contain the correct information for the conference join to be successful. Table 7-4 shows the parameters and types of this primitive. Figure 7-3 shows the sequence of events when using this primitive.

NOTE – It is up to the Node Controller, not the Top GCC Provider to determine if the Password is correct. It is possible that its definition of correct may be less stringent than a strict character-by-character match. For example, in the case of the text form of a Password, the Node Controller may choose to use a case-insensitive matching criterion.

TABLE 7-4/T.124

GCC-Conference-Join – Types of primitives and their parameters

| Parameter | Request | Indication | Response | Confirm |
| --- | --- | --- | --- | --- |
| Conference Name | M |  |  | M |
| Called Node Conference Name Modifier | C |  |  | C(RQ) |
| Calling Node Conference Name Modifier | C |  |  | C(RQ) |
| Conference ID |  | M | M() | M |
| Convener Password | O | O() |  |  |
| Password | C | C() | C | C() |
| Caller Identifier | O | O() |  |  |
| Calling Address | O | O() |  |  |
| Called Address | O | O |  |  |
| Domain Parameters | M |  |  | M |
| Quality of Service | M |  |  | M |
| Password In The Clear Required |  |  |  | M |
| Conference Locked |  |  |  | M |
| Conference Listed |  |  |  | M |
| Conference Conductible |  |  |  | M |
| Termination Method |  |  |  | M |
| Conductor Privilege List |  |  |  | C |
| Conducted-mode Conference Privilege List |  |  |  | C |
| Non-conducted-mode Conference Privilege List |  |  |  | C |
| Conference Description |  |  |  | C |
| Local Network Address | O |  |  |  |
| User Data | O | O() | O | O() |
| Result |  |  | M | M() |

Conference Name: Name of the conference being joined. In the request, this parameter is either a numeric string or a Unicode Row 00 text string, maximum 255 characters. If both the numeric and text parts of the Conference Name were used at the time the conference was created, either part may be included in this request. In the confirm, this parameter includes the full Conference Name including both numerical and text forms, if both were used at the time the conference was created.

Called Node Conference Name Modifier: If the node directly connected to the joining node (the node to which a connection is attempting to be established) has included a Conference Name Modifier as part of the name by which this conference is known, this parameter shall be included in the request primitive and shall indicate the Conference Name Modifier as it is known to the node directly connected to the joining node. This parameter is a numeric string up to 255 digits in length.

Calling Node Conference Name Modifier: If a conference already exists at the node issuing the join request with a name identical with the Conference Name of the conference to be joined, this parameter shall be included in the GCC-Conference-Join request and shall indicate the Conference Name Modifier by which the conference shall be known at the local node. This parameter, if included, shall be different from any Conference Name Modifier already in use for any other conference the local node is currently joined to with the same Conference Name. If used, this parameter becomes the Called Node Conference Name Modifier by which another node attempting to join this conference through a connection to the local node refers to this conference. This modifier is also included as the Conference Name Modifier parameter in any GCC-Conference-Query response from this node (if the conference is listed). This parameter is a numeric string up to 255 digits in length.

Conference ID: At the Top GCC Provider (the indication/response primitives) this parameter is the Conference ID of the conference to which the requesting node wishes to join. In the confirm primitive, this parameter is returned by GCC indicating the locally allocated ID by which all subsequent references to the conference are indicated. The Conference ID shall be identical with the MCS Domain Selector used locally to identify the MCS Domain associated with the conference.

Convener Password: This is an optional parameter which is either a numeric string or a Unicode Row 00 text string which may be used by a conference convener rejoining a conference after disconnecting (maximum 255 digits or characters). If this identifier matches the corresponding identifier used when the conference was created, the joining node is given the privileges of the convener, but only if joining with a direct connection to the Top GCC Provider (rather than via an intermediate MCU). The convener, with the correct Convener Password, is allowed to join even conferences which are locked. If the conference is Password protected, the correct Password must be given in addition to the Convener Password to successfully join the conference. If the Convener Password is present but does not match, the request to join shall be rejected. The criterion used to determine if the Convener Password matches the originally specified value is determined by the Node Controller.

Password: The password parameter is used to gain access to a password protected conference. In the request form of this primitive, this parameter shall only contain a password if a result of Challenge Response Required has been received in a previous GCC-Conference-Join confirm for this conference or if the Password In The Clear Required parameter is set in the Conference Descriptor for this conference in a previous GCC-Conference-Query confirm. In the case of a password in the clear this is either a numeric string or a Unicode Row 00 text string (maximum 255 digits or characters). A text string may only be used if a text password was defined at the time of conference creation in addition to the numeric password. In the case of an encrypted password this parameter contains the password encoded using one of the algorithms specified in the previously received challenge. In the case of a password sent in response to a challenge (either in the clear or encrypted), this parameter shall also include a tag which shall be identical to the tag received in the challenge. In the case of a password initiated in response to the Password In The Clear Required flag in the GCC-Conference-Query indication, no tag is required. In the request form of this primitive, this parameter may also include a challenge to the receiving node. There are no restrictions on when a challenge may be included in this parameter.

In the response form of this primitive, this parameter may contain a challenge to the requester indicating that a password is required for joining this conference. For this case, this parameter includes information specifying which forms of the password will be accepted (either in the clear, and/or encrypted via a list of non-standard encryption algorithms), an integer tag used to identify this challenge, and any additional information required for encryption. Should this parameter contain challenge, the result parameter of this primitive shall be set to Challenge Response Required. In this case, no connection is established by this exchange. This parameter in the response form of this primitive may also include a Password (either in the clear or encrypted) in response to a challenge by the requesting node.

If this parameter in the indication is not in a format satisfactory to the receiving node, that node should issue a response with Invalid Challenge Response as the result. If this parameter in the indication is of the correct format, but does not contain the correct password, the response should include Invalid Password as the result.

Caller Identifier: Optional Unicode text string (maximum 255 characters) which may be used to identify the calling node to the node at which the Top GCC Provider resides. The use of this information at this node is beyond the scope of this Recommendation. It may be used, for example, to allow a user at that node to select among a limited set of participants to allow into the conference. Because this string can be set to any value, it does not necessarily add to the security of a conference to do this, however.

Calling Address: Optional address to be included in the MCS-Connect-Provider primitive on establishing an MCS connection. See Recommendation T.122 for the interpretation of this parameter.

Called Address: Optional address to be included in the MCS-Connect-Provider primitive on establishing an MCS connection. See Recommendation T.122 for the interpretation of this parameter.

Domain Parameters: Domain parameters to be included in the MCS-Connect-Provider primitive on establishing an MCS connection. See Recommendation T.122 for the interpretation of this parameter.

Quality of Service: Quality of Service parameters to be included in the MCS-Connect-Provider primitive on establishing an MCS connection. See Recommendation T.122 for the interpretation of this parameter.

Password In The Clear Required: This is a Boolean parameter that indicates that the conference is password protected with a password that may be used without encryption in a GCC-Conference-Join request without first being challenged for the password. If the joining node is an MCU, this information is used by the GCC Provider in generating the response to a GCC-Conference-Query indication. This information shall be used by the Node Controller in determining whether or not the Password parameter may be included in a GCC-Conference-Transfer request.

Conference Locked: This flag indicates whether or not the joined conference is locked or unlocked. If the joining node is an MCU, this information shall be used in generating the response to a GCC-Conference-Query indication.

Conference Listed: This flag indicates whether or not the joined conference is listed or unlisted. If the joining node is an MCU, this information shall be used in generating the response to a GCC-Conference-Query indication. In particular, conferences which are indicated as being unlisted shall not be listed in the conference list provided with the GCC-Conference-Query response.

Conference Conductible: The TRUE setting of this flag indicates that this conference may be placed in conducted mode using the GCC-Conductor-Assign primitive. The FALSE setting of this flag indicates that this conference shall be non-conducted only, and attempts to assign a conductor shall be rejected.

Termination Method: This flag indicates the termination rule for the joined conference. The conference may be either manually or automatically terminating.

Conductor Privilege List: This is a list of flags indicating which functions the convener has designated as allowable to be used by the conference conductor, if any. The flags in this list correspond to the operations: GCC-Conference-Terminate, GCC-Conference-Eject-User, GCC-Conference-Add, GCC-Conference-Lock, GCC-Conference-Unlock and GCC-Conference-Transfer.

Conducted-mode Conference Privilege List: This is a list of flags indicating which functions the convener has designated as allowable to be used by any node in a conducted-mode conference. The flags in this list correspond to the same operations as for the above parameter.

Non-conducted-mode Conference Privilege List: This is a list of flags indicating which functions the convener has designated as allowable to be used by any node in a non-conducted-mode conference. The flags in this list correspond to the same operations as for the above parameter.

Conference Description: This parameter is a Unicode text string, up to 255 characters in length, describing the conference being joined. It is present in the confirm primitive only if included at the time of conference creation.

Local Network Address: If included in the request, the local GCC Provider at the corresponding node shall use this information to include as the Network Address parameter in the Conference Descriptor List sent as part of the response to a GCC-Conference-Query request from another node.

User Data: Optional user data which may be used for functions outside the scope of this Recommendation such as authentication, billing, etc.

Result: An indication of whether the request was accepted or rejected, and if rejected, the reason why. It contains one of a list of possible results: successful, user-rejected, invalid conference, invalid Password, Challenge Response required, invalid Challenge Response, invalid Convener Password, domain parameters unacceptable, domain not hierarchical, lower-layer initiated disconnect, unspecified failure to connect. A negative result in the GCC-Conference-Join confirm does not imply that the physical connection to the node to which the connection was being attempted is disconnected.

FIGURE 7-3/T.124...[D06] = 9 CM

#### 7.1.2.4	GCC-Conference-Invite

The GCC-Conference-Invite request primitive may be used by a Node Controller to invite a node to join a conference. This primitive may be used as a result of GCC-Conference-Add indication, or may be issued directly by the inviting node. If the add is successful, the adding MCU invites the added node to join the conference by issuing the GCC-Conference-Invite request primitive. Note that even if the conference is Password protected, no Password is needed by the invited node in order to accept the invitation to join the conference. Table 7-5 shows the parameters and types of this primitive. Figure 7-4 shows the sequence of events when using this primitive.

Conference ID: Identifier of the conference to which the primitive refers.

Conference Name: The name of the conference as specified in the Conference Profile. If the Conference Name includes both numerical and text forms, both forms shall be included in this parameter.

Conference Name Modifier: If a conference already exists at the node issuing the invite response with a name identical with the Conference Name of the conference to be joined, this parameter shall be included in the GCC-Conference-Invite response and shall indicate the Conference Name Modifier by which the conference shall be known at the local node. This parameter if included shall be different from any Conference Name Modifier already in use for any other conference the local node is currently joined to with the same Conference Name. If used, this parameter becomes the Calling Node Conference Name Modifier by which another node attempting to join this conference through a connection to the local node refers to this conference. This name is also included as the Conference Name Modifier parameter in any GCC-Conference-Query response from this node (if the conference is listed). This is a numerical string up to 255 digits in length.

Caller Identifier: Optional Unicode text string (maximum 255 characters) which may be used to identify the calling node to the node at which the conference is to be created. The use of this information at this node is beyond the scope of this Recommendation. It may be used, for example, to allow a user at that node to select among a limited set of participants which are allowed to invite this node into a conference. Because this string can be set to any value, it does not necessarily add to the security of a conference to do this, however.

TABLE  7-5/T.124

CC-Conference-Invite – Types of primitives and their parameters

| Parameter | Request | Indication | Response | Confirm |
| --- | --- | --- | --- | --- |
| Conference ID | M | M | M(IN) | M(RQ) |
| Conference Name |  | M |  |  |
| Conference Name Modifier |  |  | C |  |
| Caller Identifier | O | O() |  |  |
| Calling Address | O | O() |  |  |
| Called Address | O | O |  |  |
| Domain Parameters |  | M | M |  |
| Quality of Service |  | M | M |  |
| Password In The Clear Required |  | M |  |  |
| Conference Locked |  | M |  |  |
| Conference Listed |  | M |  |  |
| Conference Conductible |  | M |  |  |
| Termination Method |  | M |  |  |
| Conductor Privilege List |  | C |  |  |
| Conducted-mode Conference Privilege List |  | C |  |  |
| Non-conducted-mode Conference Privilege List |  | C |  |  |
| Conference Description |  | C |  |  |
| Local Network Address |  |  | O |  |
| User Data | O | O() | O | O() |
| Result |  |  | M | M() |

Calling Address: Optional address to be included in the MCS-Connect-Provider primitive on establishing an MCS connection. See Recommendation T.122 for the interpretation of this parameter.

Called Address: Optional address to be included in the MCS-Connect-Provider primitive on establishing an MCS connection. See Recommendation T.122 for the interpretation of this parameter.

Domain Parameters: Domain parameters to be included in the MCS-Connect-Provider primitive on establishing an MCS connection. See Recommendation T.122 for the interpretation of this parameter.

Quality of Service: Quality of Service parameters to be included in the MCS-Connect-Provider primitive on establishing an MCS connection. See Recommendation T.122 for the interpretation of this parameter.

Password In The Clear Required: This is a Boolean parameter that indicates that the conference is password protected with a password that may be used without encryption in a GCC-Conference-Join request without first being challenged for the password. If the joining node is an MCU, this information is used by the GCC Provider in generating the response to a GCC-Conference-Query indication. This information shall be used by the Node Controller in determining whether or not the Password parameter may be included in a GCC-Conference-Transfer request.

Conference Locked: This flag indicates whether or not the joined conference is locked or unlocked. If the joining node is an MCU, this information shall be used in generating the response to a GCC-Conference-Query indication.

Conference Listed: This flag indicates whether or not the joined conference is listed or unlisted. If the joining node is an MCU, this information shall be used in generating the response to a GCC-Conference-Query indication. In particular, conferences which are indicated as being unlisted shall not be listed in the conference list provided with the GCC-Conference-Query response.

Conference Conductible: The TRUE setting of this flag indicates that this conference may be placed in conducted mode using the GCC-Conductor-Assign primitive. The FALSE setting of this flag indicates that this conference shall be non-conducted only, and attempts to assign a conductor shall be rejected.

Termination Method: This flag indicates the termination rule for the joined conference. The conference may be either manually or automatically terminating.

Conductor Privilege List: This is a list of flags indicating which functions the convener has designated as allowable to be used by the conference conductor, if any. The flags in this list correspond to the operations: GCC-Conference-Terminate, GCC-Conference-Eject-User, GCC-Conference-Add, GCC-Conference-Lock, GCC-Conference-Unlock and GCC-Conference-Transfer.

Conducted-mode Conference Privilege List: This is a list of flags indicating which functions the convener has designated as allowable to be used by any node in a conducted-mode conference. The flags in this list correspond to the same operations as for the above parameter.

Non-conducted-mode Conference Privilege List: This is a list of flags indicating which functions the convener has designated as allowable to be used by any node in a non-conducted-mode conference. The flags in this list correspond to the same operations as for the above parameter.

Conference Description: This parameter is a Unicode text string, up to 255 characters in length, describing the conference. It is present in the confirm primitive only if included at the time of conference creation.

Local Network Address: If included in the response, the local GCC Provider at the corresponding node shall use this information to include as the Network Address parameter in the Conference Descriptor List sent as part of the response to a GCC-Conference-Query request from another node.

User Data: Optional user data which may be used for functions outside the scope of this Recommendation such as authentication, billing, etc.

Result: An indication of whether the request was accepted or rejected, and if rejected, the reason why. It contains one of a list of possible results: successful, user-rejected, invalid conference, domain parameters unacceptable, domain not hierarchical, lower-layer initiated disconnect, unspecified failure to connect. A negative result in the GCC-Conference-Invite confirm does not imply that the physical connection to the node to which the connection was being attempted is disconnected.

FIGURE 7-4/T.124...[D07] = 9 CM

#### 7.1.2.5	GCC-Conference-Add

The GCC-Conference-Add request primitive may be used by a Node Controller to add a single additional node to an existing conference by requesting that an MCU dial-out the specified node. This primitive is only valid if issued by the Convener or a convener-designated node. If the Adding MCU is specified, the sequence of primitives is direct between the requester and the adding MCU. If the Adding MCU is not specified, the indication is issued from the Top GCC Provider. In this case, if a port capable of performing the add is available at this node, the exchange is also direct. If the Node Controller at the Top GCC Provider maintains or has access to a central data-base of port information for all MCUs in the conference, it may perform the add indirectly by issuing another GCC-Conference-Add request, specifying an Adding MCU (even if the top MCU is not a convener-designated privileged node for this operation).

Once provision has been made to set-up the physical connection to the Added Node (if needed), the GCC-Conference-Invite request primitive shall be issued by the Adding MCU to invite the Added Node into the conference. If the Adding MCU already has a physical connection to the Added Node, the Adding MCU may issue the GCC-Conference-Invite request directly to the Added Node without establishing a new physical connection. If this connection had been established without knowledge of the Network Address of the Added Node (e.g. the Added Node had dialed into the Adding MCU to connect to a separate conference), the Network Address parameters in the Conference Roster may be used to provide the information needed to match the Network Address of an already connected node with that of a node to be added. Table 7-6 shows the parameters and types of this primitive. Figures 7-5 and 7-6 show the sequence of events when using this primitive in the direct and indirect cases, respectively.

TABLE  7-6/T.124

GCC-Conference-Add – Types of primitives and their parameters

| Parameter | Request | Indication | Response | Confirm |
| --- | --- | --- | --- | --- |
| Conference ID | M | M | M(IN) | M(RQ) |
| Network Address | M | M() | M() | M() |
| Adding MCU Node ID | O |  |  |  |
| Requesting Node ID |  | M | M() |  |
| User Data | O | O() | O | O() |
| Result |  |  | M | M() |

Conference ID: Identifier of the conference to which the primitive refers.

Network Address: This parameter specifies the address of the node to be added for use by the Adding MCU in making the network connection to the node to be added. This parameter consists of a list of one or more independent addresses, each specifying a portion of the total logical connection over which data as well as other information (such as audio or video) may be carried. A single address in this list may be specified either as a dialing string for a switched connection (aggregated channel), a transport connection, or an address with a non-standard format assumed to be understood by the Adding MCU. In the case of a switched connection (aggregated channel), the address is specified to include the parameters listed in Table 7-7. In the case of a transport connection, the address is specified to include the parameters listed in Table 7-8.

The NSAP part of a transport address may encapsulate within itself certain patterns that by convention suggest the choice of a switched connection or a connectionless network protocol. In such cases, the listed element stands alone as an independent alternative to any aggregated channel or non-standard addresses that may also be present. On the other hand, a transport address may instead be incomplete and may depend implicitly on the shared use of a data channel opened within a multimedia multiplex of aggregated channels, as specified in Recommendation T.123 or in some non-standard way. In these cases, a locally specified NSAP address and/or transport selector may be needed to steer new connections in the data channel to the correct termination point. NSAP addresses are conveyed, according to Recommendation T.123, as Calling or Called Party Sub-address information elements in SETUP, and transport selectors as TSAP identifiers in an X.224 connection request.

TABLE  7-7/T.124

Network address parameters for a switched connection (aggregated channel)

| Parameter | Description |
| --- | --- |
| Transfer Mode | This is a choice of either a PSTN speech connection, a PSTN voice-band data connection, a CSDN digital connection at 56 kbit/s, an ISDN digital connection at 64, 128, 192, 256, 320, 384, 512, 768, 1152, 1472, 1536, or 1920 kbit/s, an ISDN frame-mode connection, a PSDN packet mode connection, or an ATM connection. One or more of these choices must be selected. If more than one choice is indicated, this is to be interpreted as a choice left to the discretion of the Adding MCU. |
|  | NOTE – ISDN rates exceeding 64 kbit/s relate to the corresponding code points in Recommendation H.221. They represent single multirate connections, e.g. H0, H11, H12, rather than multiple B or H0 calls. The latter require multiple address elements (at 64 and/or 384 kbit/s) only if different numbers should be called for initial and additional channels. Otherwise, the number of channels can be resolved through the exchange of capability and command BAS codes. If an MCU operates by video switching, new nodes will generally be assigned the same number of channels as existing nodes in the conference or they will be given secondary status. |
| International Number | This is a string of digits, up to 16 digits in length, which represents the full international number of the node to be added. |
| Sub-Address (optional) | This is an optional parameter, valid only in the case of ISDN transfer modes, which represents the ISDN sub-address of the node to be added. This is a string of digits, up to 40 digits in length. |
| Extra Dialing String (optional) | This is an optional parameter which indicates that additional information is needed to reach the node to be added once the physical connection has been established. In the case of a speech or voiceband data connection, for example, this may represent DTMF tones to be transmitted over the voice channel once it has been established. Alternatively, the extra dialing may represent a virtual private network number. This is a string up to 255 characters which may be either the digits 1 through 9, the “#” character, the “*” character, or the “,” (comma) character. The comma character is meant to represent a one second delay the Adding MCU is to insert prior to the characters which follow. |
| High Layer Compatibility Information (optional) | This is an optional parameter, valid only in the case of ISDN transfer modes, which indicates the mode of operation this portion of the connection is to use. This information is required for connections made in some countries. The modes of operation are one or more of telephony at 3 kHz bandwidth, telephony at 7 kHz bandwidth, videotelephony, videoconferencing, audiographics, audiovisual, or multimedia. If more than one of these is selected, this indicates that the Adding MCU may use one of the indicated modes at its discretion. The definitions of these choices may depend on the country of operation of the Adding MCU. |

TABLE  7-8/T.124

Network address parameters for a transport connection

| Parameter | Description |
| --- | --- |
| NSAP Address | This is an Octet String of up to 20 octets in length which is the preferred binary encoding (per A.8.3.1/X.213) of the Network Service Access Point address of the node to be added. |
| Transport Selector (optional) | This is an optional parameter which may be used to select the Transport Service Access Point at the node to be added. |

Adding MCU Node ID: This is an optional parameter which may be used to specify the particular MCU node from which the called node is to be added. If this parameter is not specified, the called node may be added from any available MCU in the conference. In this case, the indication occurs from the Top GCC Provider. The node controller at that MCU may either service it locally, or issue another request to a specific MCU.

Requesting Node ID: Node ID of the requesting node.

User Data: Optional user data which may be used for functions outside the scope of this Recommendation such as authentication, billing, etc.

Result: An indication of whether the request was accepted or rejected, and if rejected, the reason why. It contains one of a list of possible results: successful, invalid conference, invalid adding MCU, not convener or convener-designated node, invalid network type, invalid network address, added node busy, network busy, connection unsuccessful, no ports available.

FIGURE 7-5/T.124...[D08] = 7cm

Erreur! Signet non dŽfini.

FIGURE 7-6/T.124....[D09] = 8cm

#### 7.1.2.6	GCC-Conference-Lock

The GCC-Conference-Lock request primitive may be used by a Node Controller to lock a conference, preventing other nodes from dialing into the conference at all. This primitive is valid only if issued by the Convener or a convener-designated node. While locked, participants may be added to a conference only by using the GCC-Conference-Add primitive. The order of GCC-Conference-Lock and GCC-Conference-Unlock primitives exchanged between a node and the Top GCC Provider is preserved. Table 7-9 shows the parameters and types of this primitive. Figure 7-7 shows the sequence of events when using this primitive.

TABLE  7-9/T124

GCC-Conference-Lock – Types of primitives and their parameters

| Parameter | Request | Indication | Response | Confirm |
| --- | --- | --- | --- | --- |
| Conference ID | M | M | M(IN) | M(RQ) |
| Source Node ID |  | M | M() |  |
| Result |  |  | M | M() |

Conference ID: Identifier of the conference to which the primitive refers.

Source Node ID: Node ID of the requesting node.

Result: An indication of whether the request was accepted or rejected, and if rejected, the reason why. It contains one of a list of possible results: successful, invalid conference, function not available, already locked, not convener or convener-designated node.

FIGURE 7-7/T.124...[D10] = 12cm

#### 7.1.2.7	GCC-Conference-Unlock

The GCC-Conference-Unlock request primitive may be used by a Node Controller to unlock a previously locked conference. Unlocking a conference allows other nodes to join the conference by dialing into it in a meet-me style. If the conference was created specifying the use of Password protection, the Password is still required for any participant attempting to join the unlocked conference. This primitive is valid only if issued by the Convener or a convener-designated node. The order of GCC-Conference-Lock and GCC-Conference-Unlock primitives exchanged between a node and the Top GCC Provider is preserved. Table 7-10 shows the parameters and types of this primitive. Figure 7-8 shows the sequence of events when using this primitive.

TABLE  7-10/T.124

GCC-Conference-Unlock – Types of primitives and their parameters

| Parameter | Request | Indication | Response | Confirm |
| --- | --- | --- | --- | --- |
| Conference ID | M | M | M(IN) | M(RQ) |
| Source Node ID |  | M | M() |  |
| Result |  |  | M | M() |

Conference ID: Identifier of the conference to which the primitive refers.

Source Node ID: Node ID of the requesting node.

Result: An indication of whether the request was accepted or rejected, and if rejected, the reason why. It contains one of a list of possible results: successful, invalid conference, function not available, already unlocked, not convener or convener-designated node.

FIGURE 7-8/T.124...[D11] = 12 cm

#### 7.1.2.8	GCC-Conference-Lock-Report

The GCC-Conference-Lock-Report indication primitive is issued to the Node Controller at all nodes in a conference as a result of a successful GCC-Conference-Lock request or a successful GCC-Conference-Unlock request. Figures 7-7 and 7-8 show the sequence of events leading to the use of this primitive. See also Table 7-11.

TABLE  7-11/T.124

GCC-Conference-Lock-Report – Types of primitives and their parameters

| Parameter | Indication |
| --- | --- |
| Conference ID | M |
| Locked/Unlocked | M |

Conference ID: Identifier of the MCS Domain corresponding to the locked conference.

Locked/Unlocked: Flag indicating if the conference has switched into locked or unlocked mode.

#### 7.1.2.9	GCC-Conference-Disconnect

The GCC-Conference-Disconnect request primitive is used by a Node Controller to disconnect itself from a conference. Disconnecting from a conference does not imply disconnecting the corresponding physical connection. Once disconnected from the conference, a terminal may then join another conference. If GCC detects that a node has abnormally disconnected from a conference (e.g. the physical call has been disconnected), it shall send a GCC-Conference-Disconnect indication to all remaining nodes in the conference. Table 7-12 shows the parameters and types of this primitive. Figures 7-9 and 7-13 show the sequence of events when using this primitive for client initiated disconnects. Figure 7-11 shows the case of a GCC initiated abnormal disconnect.

TABLE  7-12/T.124

GCC-Conference-Disconnect – Types of primitives and their parameters

| Parameter | Request | Indication | Confirm |
| --- | --- | --- | --- |
| Conference ID | M | M | M(RQ) |
| Reason for disconnect |  | M |  |
| Disconnecting Node ID |  | M |  |
| Result |  |  | M |

Conference ID: Identifier of the conference to which the primitive refers.

Reason for disconnect: Indication of the reason for disconnecting from the conference. Either user-initiated, ejected node, or unknown.

Disconnecting Node ID: Node ID corresponding to the disconnected node.

Result: An indication of whether the request was accepted or rejected, and if rejected, the reason why. It contains one of a list of possible results: successful, invalid conference.

NOTE – if it is necessary to allow a Node Controller to disconnect from a conference prior to completion of the connection establishment (e.g. prior to reception of a GCC-Conference-Join confirm), it would be necessary to provide a local means of association of the conference to be disconnected with the conference being established. This is because the Conference ID is not known by the Node Controller until after creating or joining a conference. The mechanism for doing so is a local matter, beyond the scope of this Recommendation.

FIGURE 7-9/T.124...[D12] = 8 CM

#### 7.1.2.10	GCC-Conference-Terminate

The GCC-Conference-Terminate request primitive is used by a Node Controller to terminate an entire conference. This primitive shall only be issued by the conference Convener or a convener-designated node. Terminating a conference does not imply termination of the corresponding physical connection. Table 7-13 shows the parameters and types of this primitive. Figures 7-10, 7-11 and 7-12 show the sequence of events when using this primitive for client initiated and GCC initiated cases. The GCC initiated case can result either from abnormal termination at the lower layers of the protocol, or at the Top GCC Provider, if the conference had been created as an automatically terminating conference and all other nodes have disconnected from the conference.

NOTE – In most cases, reception of the GCC-Conference-Terminate indication implies that the entire conference has been terminated – i.e. all members have been disconnected. In the case that error termination is given as the reason code, this primitive implies that the local node has been unexpectedly removed from the conference, but does not necessarily imply that the entire conference has been terminated.

TABLE  7-13/T.124

GCC-Conference-Terminate – Types of primitives and their parameters

| Parameter | Request | Indication | Confirm |
| --- | --- | --- | --- |
| Conference ID | M | M | M(RQ) |
| Requesting Node ID |  | C |  |
| Reason for termination | O | O() |  |
| Result |  |  | M |

Conference ID: Identifier of the conference to which the primitive refers.

Requesting Node ID: Node ID of the node which initiated the termination. This parameter is present except in the case of GCC initiated termination.

Reason for termination: Indication of the reason for termination of the conference. This contains one of a list of possible reasons: requested normal termination, requested timed conference termination, no more participants in automatically terminating conference, error termination.

Result: An indication of whether the request was accepted or rejected, and if rejected, the reason why. It contains one of a list of possible results: successful, invalid conference, not convener or convener-designated node.

FIGURE 7-10/T.124...[D13] = 10 CM

FIGURE 7-11/T.124...[D14] = 8 CM

FIGURE 7-12/T.124...[D15] = 9 CM

#### 7.1.2.11	GCC-Conference-Eject-User

The GCC-Conference-Eject-User request primitive is used by a Node Controller to force a particular node to be disconnected from a conference. This primitive shall only be issued by the conference Convener or a convener-designated node or by a node directly above the ejected node in the connection hierarchy. Being ejected from a conference does not imply termination of the corresponding physical connection. When a node is ejected, a GCC-Conference-Disconnect indication is issued to the Node Controller at all nodes remaining in the conference indicating that the ejected node has disconnected due to being ejected. Table 7-14 shows the parameters and types of this primitive. Figure 7-13 shows the sequence of events when using this primitive.

TABLE  7-14/T.124

GCC-Conference-Eject-User – Types of primitives and their parameters

| Parameter | Request | Indication | Confirm |
| --- | --- | --- | --- |
| Conference ID | M | M | M(RQ) |
| Ejected Node ID | M | M() | M() |
| Reason for ejection | O | O() |  |
| Result |  |  | M |

Conference ID: Identifier of the conference to which the primitive refers.

Ejected Node ID: The node to be ejected.

Reason for ejection: Indication of the reason for ejection: user-initiated, higher node disconnected, or higher node ejected.

Result: An indication of whether the request was accepted or rejected, and if rejected, the reason why. It contains one of a list of possible results: successful, invalid conference, invalid user to eject, not convener or convener-designated node.

FIGURE 7-13/T.124...[D16] = 7.5 CM

#### 7.1.2.12	GCC-Conference-Transfer

The GCC-Conference-Transfer request primitive is used by a Node Controller to cause selected nodes in a conference to disconnect from that conference and join another conference. This primitive shall only be issued by the conference Convener or a convener-designated node. Some MCUs in a conference may be already joined to both the originating and destination conferences prior to the transfer taking place. If so, these MCUs shall not be included in the list of Destination Nodes in this request. If an MCU is not joined with both conferences but will be connected to nodes which, after the transfer, will be joined to both conferences (if not all nodes are transferred), then that MCU shall be joined to the destination conference prior to issuing the GCC-Conference-Transfer request. Any MCU which is included in the list of Destination Nodes (those which are intended to transfer) shall complete the transfer operation (disconnecting from the originating conference and joining the destination conference) prior to processing any new GCC-Conference-Join indications. This allows the join request from the nodes below that MCU in the connection hierarchy to be successfully completed. Nodes which are in the process of transferring and receiving a GCC-Conference-Terminate indication for the originating conference may proceed directly to joining the destination conference without disconnecting if they have not already done so. This situation could arise if the MCU to which the node is connected was also instructed to transfer. Table 7-15 shows the parameters and types of this primitive. Figure 7-14 shows the sequence of events when using this primitive.

TABLE  7-15/T.124

GCC-Conference-Transfer – Types of primitives and their parameters

| Parameter | Request | Indication | Confirm |
| --- | --- | --- | --- |
| Conference ID | M | M | M(RQ) |
| Destination Conference Name | M | M() | M() |
| Destination Conference Name Modifier | O | O() | O() |
| Destination Network Address | O | O() |  |
| Transferring Nodes (List of Node IDs or null) | O |  | O() |
| Password | C | C() |  |
| Result |  |  | M |

Conference ID: Identifier of the MCS Domain corresponding to the conference to which the designated nodes are joined prior to the transfer operation.

Destination Conference Name: Name of the conference to which the designated nodes are instructed to join. This is the name by which the conference is known at the MCU or MCUs to which the destination nodes are connected. If the conference had been created using both the numerical and text forms of the Conference Name, either form may be used in this primitive.

Destination Conference Name Modifier: If an MCU in the destination conference had chosen to use the optional Conference Name Modifier (as a result of local naming uniqueness problem), nodes to be transferred which are connected to this MCU must be transferred separately, with a separate exchange of the GCC-Conference-Transfer primitive from those nodes which are connected to MCUs using the unmodified (or differently modified) base Conference Name. The manner by which the requesting node becomes aware of Conference Name Modifiers at various MCUs is considered a matter outside the scope of this Recommendation.

Destination Network Address: Optional Called Address parameter to be included in the GCC-Conference-Join request to be issued by the transferring nodes.

Transferring nodes (List of Node IDs or null): List of Node IDs identifying GCC Providers to which the indication should go, or omitted, to indicate it should go to all nodes in the conference designated by the Conference ID.

Password: This parameter indicates the password that the transferring nodes shall use in the GCC-Conference-Join request to join the new conference. This is a numeric string or a Unicode Row 00 text string (maximum 255 digits or characters). This parameter shall only be used if the Password In The Clear Required flag is set for this conference.

Result: An indication of whether the request was accepted or rejected, and if rejected, the reason why. It contains one of a list of possible results: successful, invalid conference, not convener or convener-designated node.

FIGURE 7-14/T.124...[D17] = 11.5 CM

### 7.1.3	Conference establishment requirements

Recommendation T.124     (08/95)

In order to avoid the possibility of a deadlock situation in which both sides of a physical connection fail to initiate the GCC conference establishment procedure, waiting for the other to do so, the following requirements are defined for the conference establishment procedure when a physical connection is established (that is, an underlying connection between the two nodes using the PSTN, ISDN, or CSDN cases of Recommendation T.123). These requirements may be superseded by bilateral agreement between the nodes involved, either via a priori arrangement, or via an exchange of information defined outside of the scope of this Recommendation.

•	It must first be known by both ends of the physical connection which end is the calling node and which end is the called node. It must also be known what type of node each of the two nodes are (terminal, multiport terminal, or MCU). This information may be obtained either using the GCC-Conference-Query primitive, or it may be known a priori. For determining the calling vs. called node, if neither node is aware of which it is, the symmetry breaking procedure defined for the GCC-Conference-Query primitive is used to arbitrarily choose one node to be the caller for purposes of the conference establishment procedure.

•	The calling node shall be responsible for initiating the initial conference establishment procedure (either requesting creation of a new conference, joining a conference at the called node, or inviting the called node into a conference). Note that this, in general, does not preclude the called node from taking actions to establish a conference over the same physical connection as well, although care must be taken in this case to ensure that this action does not interfere with the action of the calling node.

•	When establishing calls among the various node types (terminals, multiport terminals, and MCUs), Table 7-16 constraints are placed on the actions of the calling node in initiating the initial connection. The definition of the actions shown in the table, numbered 1, 2, 3, and 4 are as follows:

1)	Calling node requests creation of a new conference at the Called node.

2)	Calling node attempts to join existing conference at the Called node.

3)	Calling node creates a conference locally and invites the Called node.

4)	Calling node invites the Called node into an existing conference.

TABLE  7-16/T.124

Actions of the calling node for conference establishment

| Calling Node | Called Node |  |  |
| --- | --- | --- | --- |
|  | Terminal | Multiport terminal | MCU |
| Terminal | Either 1 or 3 | Either 1 or 2 | Either 1 or 2 |
| Multiport terminal | Either 3 or 4 | Either 1, 2, 3, or 4 | Either 1 or 2 |
| MCU | Either 3 or 4 | Either 3 or 4 | Either 1, 2, 3, or 4 |

### 7.1.4	Examples of conference establishment procedures

Conference establishment may be done in a variety of ways and under a variety of conditions. The simplest conference is of the point-to-point variety where there is no MCU involved in the call. In the cases where the conference is established through one or more MCUs, a call would typically be done in either the meet-me style (all participants call into an MCU), in the call-out style (the MCUs set up the call by calling out to all participants), or in the call-through style (one participant calls into an MCU, then adds other participants which are called by the MCU).

#### 7.1.4.1	Meet-me conference establishment

In a meet-me conference, a conference is established at an MCU and terminal nodes (as well as other MCUs if necessary) call into the MCU and join the conference. If other MCUs have joined the conference, terminal nodes may call into any of these MCUs to join the conference.

Initial creation of a meet-me conference may be done out-of-band (e.g. locally initiated at the MCU), or the conference may be created by the first node to call into the MCU. In the former case, GCC is not involved. In the latter, the conference is created at the MCU by issuing a GCC-Conference-Create request over the connection from the convening node to the MCU. In either case, the node at which the conference was created becomes the Top-GCC-Provider. The convening node (the MCU itself in the former case, and the requesting node in the latter) is granted special status as the Conference Convener.

When a conference is created remotely (using a GCC-Conference-Create request), a Conference Name is specified in the request primitive. If that name is already in use at the MCU, a Conference Name Modifier is assigned by the Node Controller at the MCU to make the name locally unique at that MCU. When other nodes attempt to join the conference at that MCU, this name modifier must be specified as part of the join request.

When a conference is created remotely it may also include an optional Convener Password. This password is needed only if the Convener intends to disconnect from the conference and rejoin at a later time expecting Convener privileges to continue.

A meet-me conference would typically be specified as non-locked so that other nodes may join. A meet-me conference may optionally be created to require Password protection to prevent unwanted nodes from joining the conference.

A node joining a meet-me conference would issue a GCC-Conference-Join request over the connection from the node to any MCU joined to the conference. Typically, a node does not know a priori the name of the conference to be joined. In this case, prior to joining the conference, a node may query the MCU for a list of conferences that are available to be joined. This is done by issuing a GCC-Conference-Query request. The response is a GCC-Conference-Query confirm which indicates the type of node to which the terminal is connected (i.e. an MCU in this case), and a list of all listed conferences to which that MCU is currently joined. This list includes the Conference Name of each conference, a Conference Name Modifier if one is needed, as well as other characteristics of the conference such as whether or not the conference is Password protected, which may be used to request a Password from the user prior to attempting to join the conference. Once the name of the conference is selected, the conference may be joined by issuing a GCC-Conference-Join request specifying the Conference Name of the desired conference, and if needed, the Conference Name Modifier. If the conference requires a Password, it is included in the join request.

Note that it is typical that the case of a meet-me conference with no a priori knowledge of the conference to join is identical to the case of a point-to-point conference. That is, the sequence of events used in starting up the conference cannot be different from that of a point-to-point conference since the a priori knowledge in each case is the same. It is only the identification in the GCC-Conference-Query confirm that the directly connected node is an MCU and that there are ongoing conferences to be joined that allow a distinction to be made.

In the case that the Conference Name is distributed to joining nodes out-of-band, a node may directly join the conference without first querying the available conferences by specifying the Conference Name in the GCC-Conference-Join request. If there are multiple independent MCUs to which nodes may connect to join a conference, it is possible that, due to naming conflicts, a Conference Name Modifier is needed on some MCUs. In this case, a node joining a conference when connected to one of these MCUs must specify the Conference Name Modifier for that MCU. As it may be difficult to determine this modifier without the use of the GCC-Conference-Query primitive, it is recommended that if there is any possibility of a naming conflict, GCC-Conference-Query should be used prior to attempting to join a meet-me conference. It is also recommended that a Conference Description be used when a meet-me conference is first created. In this case, it is more likely that multiple conferences with the same Conference Name may be distinguished by having different Conference Descriptions. In general, however, when creating a meet-me conference, it is better to choose a Conference Name which will be unique at all MCUs without the need of a Conference Name Modifier.

#### 7.1.4.2	Call-out conference establishment

In call-out conference establishment, a conference is created locally at the MCU and the conference participants are called and invited to the conference by that MCU. The conference would typically be created specifying that it is a locked conference and may also be specified as unlisted.

The convening MCU would then make physical connections to each of the terminals to participate in the conference followed by inviting each node in turn to the conference. This is done by issuing a GCC-Conference-Invite request to the node to be invited. Since the conference was created at the MCU, that MCU is the Top-GCC-Provider of a call-out conference.

#### 7.1.4.3	Call-through conference establishment

A call-through conference is very similar to call-out case with the exception that the conference is initially created remotely by an initiating terminal. In this case, the terminal connects to the MCU and creates a conference using a GCC-Conference-Create request. As in the call-out case, it would typically be locked and unlisted. A call-through conference would typically be created as an automatically terminating conference indicating that the conference will be terminated when all nodes disconnect. Typically, a NULL Conference Name would be specified for a call-through conference since there is no need for nodes to explicitly join the conference. If there is already a conference at the convening node with a NULL conference name, the Node Controller would simply choose an arbitrary unique name to use as the Conference Name. In either case, the name need not be human readable since it will never be used for joining.

#### 7.1.4.4	Point-to-point conference establishment

A point-to-point conference is distinct from the other varieties in that it involves only two terminal nodes with no MCU present. In the case where it is known which terminal is the calling terminal (the initiator of the physical connection) and which is the called terminal, a point-to-point conference may be established by the calling terminal first querying the called terminal by issuing a GCC-Conference-Query request. This allows the terminal to determine if the other node is a terminal, MCU, or multiport terminal without requiring a priori knowledge to that effect. The GCC-Conference-Query confirm generated in response to this request indicates, in the case of a point-to-point call, that the directly connected node is a user-terminal. Once it is known that the directly connected node is a terminal, the conference is established by the calling terminal by issuing a GCC-Conference-Create request to create a new conference or by creating a conference locally, and inviting the other terminal by issuing a GCC-Conference-Invite request. Typically, the conference would be specified with an arbitrary Conference Name such as “0”, and would be unlocked, listed, and automatically terminating.

In the case that it is not known by a node which terminal is the calling or called terminal, that terminal should issue GCC-Conference-Query request to determine whether the other node is the called or calling node (unless it has already received a GCC-Conference-Query indication which has this information). In the request, the Asymmetry Indicator parameter indicates that it is unknown whether the local node is the called or calling node. If the other node does have this knowledge about itself, it will so indicate in the resulting GCC-Conference-Query confirm. If so, it is now known which is the calling node and the appropriate actions are taken as described above. If neither node knows its called/calling status, the confirm (or the contents of a received GCC-Conference-Query indication) will specify the unknown setting. The unknown settings of this parameter include a 32-bit random number. In this case, the random number is used to break the resulting symmetry. The node which had generated the smaller of the two random numbers should be considered the called node and should not attempt to establish the conference. The node which generated the larger number should be considered the calling node and should attempt to establish the conference.

If a terminal does have a priori knowledge that the call is point-to-point between two terminals (and in the case that it is known whether that terminal is the called or calling terminal), that terminal need not issue a GCC-Conference-Query request. Instead, if the terminal is the calling terminal, it may issue the GCC-Conference-Create request or the GCC-Conference-Invite request immediately. If it is the called terminal with this a priori knowledge, it may also skip the GCC-Conference-Query request and simply wait to receive a GCC-Conference-Create indication or GCC-Conference-Invite indication. If the far-end terminal does not have a priori knowledge of the connection type, it is possible that the local terminal will receive a GCC-Conference-Query indication from the far-end terminal to which it is required to respond.

#### 7.1.4.5	Conference establishment among multiport terminals

A multiport terminal is a device which is, in general, to be treated as a terminal, but has the ability to establish connections to multiple nodes simultaneously as an MCU does. When a terminal or multiport terminal calls a node which it finds to be a multiport terminal (either through a priori knowledge or via the GCC-Conference-Query exchange), the action taken depends on whether or not there are conferences available on that multiport terminal and on the characteristics of those conferences.

In general, it is typically desirable that a connection of this kind be made automatically, like a point-to-point call rather than like a meet-me conference. Specifically, if neither the called nor calling nodes are already part of conferences connecting them to other nodes, the call should be treated exactly as a point-to-point call. If the calling node already has an ongoing conference, it is typical that this node would simply invite the new node into the existing conference.

If instead the called node has an ongoing conference, the action may depend on whether that conference is locked or unlocked, listed or unlisted, or password protected. If the conference is unlisted, without a priori information, the calling node would not be aware of its presence and would treat the call as if there were no conference present at that node (if it

did have a priori information that an unlisted conference was present, it could join that conference). If the conference is locked, the calling node has no way to join that conference and again would treat the call as if that conference were not present. If the conference is password protected, again, the calling node would likely treat the call as if there were no conference present at that node, although it does have the option of prompting the user for a password and attempting to join the conference. In either of these cases, once a new conference is established, the called node may choose to transfer the nodes connected to the previous conference into the new conference. If there is a single conference at the called node which is “open” (not locked, unlisted, or password protected) the calling node should simply attempt to join this conference. If there is more than one “open” conference at the calling node, this would typically require a user to decide explicitly which one to join – in this case, the procedure would not be entirely automatic.

If there are conferences present at both nodes (assuming an “open” conference at the called node), the calling node would normally either invite the called node into its conference, or would attempt to join the called node’s conference. In either case, the multiport terminal with the existing conference that was not enlarged to include the other multiport terminal should transfer all subordinate nodes to the conference that it has newly become joined (or invited) to.

Transferring nodes to the new conference in these cases may be done in one of two ways. The GCC-Conference-Transfer primitive may be used to command each of the nodes to disconnect from the current conference and join the new conference. This, of course, can only be done if the new conference is not locked, and if the requesting node had Transfer privilege for the previous conference (or was the convener of the previous conference). Alternatively, the GCC-Conference-Invite primitive could be used to invite the directly connected nodes to the new conference. Typically this would be done after disconnecting these nodes from the previous conference. If there were more nodes originally present than the directly connected nodes, it is the responsibility of the directly connected nodes, recursively, to invite the nodes directly connected to them into the new conference. That is, the invitations would propagate through the hierarchy of physically connected nodes. Nodes would typically do this; however, there is no assurance that this action would be taken. For this reason, the use of GCC-Conference-Transfer is a safer mechanism in the cases where it can be performed.

## 7.2	The conference roster

The Conference Roster allows a node participating in a conference to learn what other nodes are participating in the same conference, and provides information about each node. Immediately after joining a conference (by means of either creating, joining, or being invited to the conference), the Node Controller at that node shall announce its presence to the conference by issuing a GCC-Conference-Announce-Presence request. As a result of this request, the GCC Providers of the nodes in the conference exchange information needed to update the Conference Roster to include the newly joined node. The updated Conference Roster is distributed to all nodes in the conference, generating a GCC-Conference-Roster-Report indication. A node joining a conference is not considered an active member of the conference until it has received a Conference Roster in which it is included.

The information in the Conference Roster may be changed at any time during a conference by the Node Controller. This is done by re-issuing a GCC-Conference-Announce-Presence request. This results in an updated roster being distributed to all nodes, generating another GCC-Conference-Roster-Report indication.

As nodes leave a conference for any reason, a new Conference Roster is distributed by GCC, again generating a GCC-Conference-Roster-Report indication.

### 7.2.1	Description of abstract services

The following is a list of the primitives defined in this subclause and a brief summary of the function of each:

•	GCC-Conference-Announce-Presence – Used by the Node Controller to announce the presence of a node into a conference. Use of this primitive is required immediately after joining or being joined to any conference.

•	GCC-Conference-Roster-Report – Generated by GCC in response to any change to the Conference Roster due to nodes either entering or leaving a conference.

•	GCC-Conference-Roster-Inquire – Used by either the Node Controller or by Application Protocol Entities to retrieve the current Conference Roster at any time during a conference.

#### 7.2.1.1	GCC-Conference-Announce-Presence

Immediately after a node has joined any conference, it shall announce its presence to all other nodes in the conference using the primitive GCC-Conference-Announce-Presence request. The Node Controller is responsible for issuing this primitive. This primitive may be re-issued at any time during a conference if the included information has changed. Table 7-17 shows the parameters and types of this primitive. Figure 7-15 shows the sequence of events when using this primitive.

NOTE – The Conference Roster includes the list of all terminals, and MCUs currently joined to a conference. A user display of the roster, as well as user indications of nodes entering and leaving the conference are likely to only include terminals and multiport terminals. The Node Type parameter of this primitive may be used to distinguish among these node types, excluding uninteresting node types from user displays. In addition, the Node Properties parameter also adds information as to the use of the device, specifying whether a node is a management device, and whether a node is a peripheral, subordinate to another node. Both of these characteristics may also be used by a system to determine whether or not to include a particular node in its display of conference participants. Typically, neither management devices, nor peripherals would be included in such a list.

TABLE  7-17/T.124

GCC-Conference-Announce-Presence – Types of primitives and their parameters

| Parameter | Request | Confirm |
| --- | --- | --- |
| Conference ID | M | M(=RQ) |
| Node Type | M |  |
| Node Properties | M |  |
| Node Name | O |  |
| Participant Name(s) | O |  |
| Site Information | O |  |
| Network Address | O |  |
| Alternative Node ID | O |  |
| User Data | O |  |
| Result |  | M |

Conference ID: Identifier of the conference to which the primitive refers.

Node Type: The node type is either a terminal, MCU, or multiport terminal.

Node Properties: Made up of two independent flags. One indicating whether or not the node is a management device (e.g. a reservation system), and the other indicating whether or not the node is a peripheral, subordinate to another node.

Node Name: Unicode text string containing the name of this node (e.g. “London”). Maximum 255 characters.

Participant Name(s): A list of Unicode text strings each containing the name of a meeting participant. Maximum 255 characters for each string.

Site Information: A Unicode text string containing other information about the node. It may be used, for example, to indicate such things as the voice-phone or FAX numbers at the site.

Network Address: Optional parameter to indicate the network address of this node. This parameter includes sub-fields which specify network type information followed by the actual network address or addresses. This parameter should be used if it is possible that the announcing node may later be attempted to be added to another conference via the GCC-Conference-Add primitive. This gives an MCU knowledge of the Network Address which may be compared to the Network Address of a node to be added to determine if a physical connection already exists. If this parameter is not included, it may not be possible to later add this node to another conference over the same physical connection. This parameter should also be used if it is possible that an initial point-to-point conference may be automatically re-routed through an MCU to add additional nodes. This parameter allows one of the original two nodes to re-connect with the other by use of a GCC-Conference-Add primitive through the MCU.

Alternative Node ID: This field may be used to associate the announcing node (and its corresponding Node ID) with an alternative node ID which has been defined for some other purpose. This alternative node ID is not intended to represent IDs in the same numbering space as Node IDs, but rather a separate numbering scheme not specified by this Recommendation. For example, in the case of ISDN, for nodes supporting Recommendation H.243, the alternative node ID could be the Recommendation H.243 site ID which is two octets in length.

User Data: Optional user data which may be used for functions outside the scope of this Recommendation. Note that this data is stored within the Top GCC Provider as part of Conference Roster. Therefore, this parameter is not intended to contain large amounts of information. Doing so could risk being involuntarily ejected from the conference.

Result: An indication of whether the request was accepted or rejected, and if rejected, the reason why. It contains one of a list of possible results: successful, invalid conference.

FIGURE 7-15/T.124...[D18] = 9 CM

#### 7.2.1.2	GCC-Conference-Roster-Report

Whenever the Conference Roster has changed for any reason (a new node entering the conference, a node leaving the conference, or updated information in a roster entry), the roster is distributed by GCC to all nodes by issuing a GCC-Conference-Roster-Report primitive to the Node Controller at each node. Table 7-18 shows the parameters and types of this primitive. Figure 7-16 shows the sequence of events when using this primitive.

TABLE  7-18/T.124

GCC-Conference-Roster-Report – Types of primitives and their parameters

| Parameter | Indication |
| --- | --- |
| Conference ID | M |
| Conference Roster | M |

Conference ID: Identifier of the conference to which the primitive refers.

Conference Roster: A list of each node joined to the specified conference. The format of the Conference Roster is shown in Table 7-19.

TABLE  7-19/T.124

Contents of the Conference Roster parameter

| Parameter | Description |
| --- | --- |
| List of Conference Nodes | A list of the Nodes joined to the conference along with information about each node. The contents of each entry in this list is shown in Table 7-20. |
| Instance Number | The instance number for the Conference Roster. This is a 16-bit number which is incremented modulo 216 each time the contents of the Conference Roster changes. |
| Nodes Added Flag | A flag indicating whether one or more Nodes have been added to the Conference Roster since the last instance. This flag is not mutually exclusive of the Nodes Removed Flag. |
| Nodes Removed Flag | A flag indicating whether one ore more Nodes have been removed from the Conference Roster since the last instance. This flag is not mutually exclusive of the Nodes Added Flag. |

TABLE  7-20/T.124

Contents of each entry in the List of Conference Nodes

| Parameter | Description |
| --- | --- |
| Node ID | MCS User ID of the GCC Provider at this node. |
| Node ID of Superior Node (conditional) | MCS User ID of the GCC Provider at the node directly above this node in the connection hierarchy, if any (not present for the Top GCC Provider). |
| Node Type | The node type is either a terminal, MCU, or multiport terminal. |
| Node Properties | Made up of two independent flags. One indicating whether or not the node is a management device (e.g. a reservation system), and the other indicating whether or not the node is a peripheral, subordinate to another node. |
| Node Name (conditional) | Unicode text string containing the name of this node (e.g. “London”). Maximum 255 characters. |
| Participants Names (conditional) | A list of Unicode text strings each containing the name of a meeting participant. Maximum 255 characters for each string. |
| Site Information (conditional) | A Unicode text string containing other information about the node. It may be used, for example, to indicate such things as the voice-phone or FAX numbers at the site. |
| Network Address (conditional) | The network address of this node. This parameter includes sub-fields which specify network type information followed by the actual network address or addresses. |
| Alternative Node ID (conditional) | This field is used to associate the announcing node (and its corresponding Node ID) with an alternative node ID which has been defined for some other purpose. This alternative node ID is not intended to represent IDs in the same numbering space as Node IDs, but rather a separate numbering scheme not specified by this Recommendation. For example, in the case of ISDN, for nodes supporting Recommendation H.243, the alternative node ID could be the H.243 site ID which is two octets in length. |
| User Data (conditional) | Optional user data which may be used for functions outside the scope of this Recommendation. |

FIGURE 7-16/T.124...[D19] = 8 CM

#### 7.2.1.3	GCC-Conference-Roster-Inquire

The GCC-Conference-Roster-Inquire request primitive returns the complete Conference Roster for the specified conference. This primitive is available to Application Protocol Entities as well as the Node Controller, allowing them to independently obtain a Conference Roster from their local GCC Provider. Table 7-21 shows the parameters and types of this primitive. Figure 7-17 shows the sequence of events when using this primitive.

TABLE  7-21/T.124

GCC-Conference-Roster-Inquire – Types of primitives and their parameters

| Parameter | Request | Confirm |
| --- | --- | --- |
| Conference ID | M | M(=) |
| Conference Name |  | M |
| Conference Description |  | C |
| Conference Roster |  | M |
| Result |  | M |

Conference Name: The Conference Name as contained within the Conference Profile.

Conference Description: The Conference Description as contained within the Conference Profile. This parameter is present, only if a Conference Description had been defined at the time of conference creation.

Conference ID: Identifier of the conference to which the primitive refers.

Conference Roster: A list of each node joined to the specified conference. The format of the Conference Roster is shown in Table 7-22.

Result: An indication of whether the request was accepted or rejected, and if rejected, the reason why. It contains one of a list of possible results: successful, invalid conference.

TABLE  7-22/T.124

Contents of the Conference Roster parameter

| Parameter | Description |
| --- | --- |
| List of Conference Nodes | A list of the Nodes joined to the conference along with information about each node. The contents of each entry in this list is shown in Table 7-20. |
| Instance Number | The instance number for the Conference Roster. This is a 16-bit number which is incremented modulo 216 each time the contents of the Conference Roster changes. |

FIGURE 7-17/T.124...[D20] = 8 CM

## 7.3	The application roster

The Application Roster allows a node participating in a conference to learn what Application Protocol Entities are available at nodes in the conference, and provides sufficient information about these Application Protocol Entities to allow direct communication between Peer Application Protocol Entities to begin. The roster may include Application Protocol Entities that are based on standard Application Protocols as well as those based on non-standard Application Protocols.

### 7.3.1	Contents of the application roster

For each conference, a GCC Provider compiles a list of information associated with each Application Protocol Entity. These lists form the Local Application Roster which will be exchanged with other nodes as part of the Application Roster exchange procedure. This exchange will result in a list of information associated with all Application Protocol Entities in the entire conference collectively known as the Conference Application Roster.

The Local Application Roster is held by each GCC Provider and stored in its local data-base. At an appropriate point in the set-up of a conference, a node will exchange its Local Application Roster with all other conference nodes and the GCC Provider will receive the Conference Application Roster which it will report to all Application Protocol Entities as well as the Node Controller. This exchange occurs within GCC and is apparent to Application Protocol Entities only through primitives that may be generated.

The Local and Conference Application Rosters consist of the following components:

•	Local Application Roster – For each Application Protocol Entity which has enrolled with the local GCC Provider, the Local Application Roster includes a Session Key, an Application Record, and an Application Capabilities List.

•	Conference Application Roster – For each Application Protocol Session, the Conference Application Roster includes the Session Key for that Session, an Application Capabilities List containing the negotiated capability set for the session, and the list of Application Protocol Entities enrolled in the session which includes, for each, the Node ID of the node at which that Application Protocol Entity had enrolled, an identifier which identifies the particular Application Protocol Entity within its local node, and an Application Record.

Each set of Peer Application Protocol Entities – those within a single Application Protocol Session – are characterized by having enrolled using the same value of the Session Key. A Session Key is defined as follows:

•	The Session Key – An identifier used to uniquely identify an Application Protocol Session. A Session Key consists of two components. One component identifies the Application Protocol. The second component, which is optional, identifies a particular session of that Application Protocol.

The first component of a Session Key, the Application Protocol Key, identifies either a standard or non-standard Application Protocol specification. Keys are structured so that any Application Protocol, whether standard or non-standard, may be defined to have a unique key with no possibility of conflict. Keys may be specified in one of two forms. A key may be specified as an ASN.1 type Object Identifier. This form of Key may be used to specify standard Application Protocols. The assignment of Object Identifier components for Standards and Recommendations is described in Annex B/X.680 and Annex C/X.680, respectively. Object Identifiers may also be used to specify non-standard Application Protocols in the case of national or private administrative authorities which have been directly or indirectly authorized by ISO or ITU.

A key may also be specified as an ASN.1 type OCTET STRING interpreted in a manner similar to the pattern adopted in Recommendation H.221 to designate non-standard commands and capabilities. In this case, the first two octets of the OCTET STRING define a country code, the second two octets define a manufacturer code. The first octet of the country code is assigned according to Recommendation T.35; the second octet and the manufacturer code are assigned nationally. Octets beyond this shall be freely chosen by the responsible manufacturer or national body.

The second component of a Session Key is an optional Session ID. The Session ID is an MCS Channel ID which is used as the unique identifier of an Application Protocol Session and may be used as a communication channel by the Application Protocol Entities taking part in that session (as determined by the Application Protocol specification). To ensure uniqueness, this Channel ID shall remain allocated for the duration of the session. The lack of a Session ID in a Session Key of an active Application Protocol Entity identifies a distinct session – the Default Session.

In the case of an inactive Application Protocol Session, the Session ID is not required. Its absence in this case may be interpreted as an indication of support for the indicated Application Protocol and the ability to invoke one or more Application Protocol Entities as part of any session, default or otherwise.

Each Application Record contains the following parameters, some of which are optional:

•	Active/Inactive Flag – An Application Protocol Entity may enroll but not yet be ready to receive data (e.g. it has not yet joined the appropriate channels). To indicate this to other nodes, an Application Protocol Entity may enroll with this flag set to Inactive. This may be done to allow the Application Protocol Entity to make use of GCC services (such as the registry) in preparation for becoming active, or it may simply be to allow other nodes to become aware of the presence of this Application Protocol Entity without becoming active until it is known that there are like Application Protocol Entities at other nodes. When an inactive Application Protocol Entity becomes active, it may re-enroll setting the flag to Active. This flag may be changed by the Application Protocol Entity at any time by re-enrolling.

•	Application User ID – The MCS User ID associated with the Session Key. The Application User ID is the only means by which a node can include another node as a participant when convening a private channel for that Application Protocol Session to use. This parameter is optional for Application Protocol Entities

enrolled as inactive, but mandatory for active Application Protocol Entities. If it is not included because the Application Protocol Entity has not yet attached to the MCS Domain, it may later be added, once the Application Protocol Entity has attached, by re-enrolling.

•	Conducting Operation Capable – This is a flag which indicates whether the Application Protocol Entity is capable of operating as a conducting Application Protocol Entity if the corresponding Application Protocol specification defines the procedures for such an Application Protocol Entity to follow. The GCC Provider within each node chooses no more than one Peer Application Protocol Entity per Application Protocol Session to include with this flag set in the exchanged Application Roster. If a node becomes the conference conductor, the designated Application Protocol Entity, if any, for each Application Protocol Session becomes the designated conducting Application Protocol Entity. This flag only applies to Active Application Protocol Entities. If the Active/Inactive flag is set to Inactive when enrolling, the Conducting Operation Capable flag is ignored and assumed by the GCC Provider to be FALSE.

•	Non-Collapsing Capabilities List – This is an optional parameter which allows Application Protocol Entities to list capabilities (either standard or non-standard) which are to be maintained in the roster as part of the Application Record of each Application Protocol Entity rather than being collapsed by a set of rules as the capabilities listed in the Application Capabilities List would be.

The Application Capabilities List is defined as follows:

•	Application Capabilities List – This is an optional parameter list which may be used to specifically list the capabilities of the Application Protocol Entity. While the capabilities themselves are Application Protocol-specific, they are listed along with information which allows GCC to determine the common capability set for the Application Protocol Session and inform the Peer Application Protocol Entities of this set. This avoids the need for a complete exchange of a full capability list between all nodes. Each capability in the capability list is tagged with a class specifier. The class specifier determines the rule that is applied to determine the common capability set. Table 7-23 lists all capability classes.

TABLE  7-23/T.124

Capability combination rules

R

| Class | Description |
| --- | --- |
| Logical | If any Peer Application Protocol Entity in a conference indicates the use of this capability, the final capability list indicates the number of Peer Application Protocol Entities that have indicated this capability. |
| Unsigned-Minimum | The parameter octets are treated as a single unsigned integer. The final capability list indicates the minimum value among all Peer Application Protocol Entities which indicated this capability as well as the number of Peer Application Protocol Entities which have indicated this capability. |
| Unsigned-Maximum | The parameter octets are treated as a single unsigned integer. The final capability list indicates the maximum value among all Peer Application Protocol Entities which indicated this capability as well as the number of Peer Application Protocol Entities which have indicated this capability. |

In addition to the basic classifications, it is also possible to nest capabilities. Nesting is not done explicitly, but rather is done by appropriate interpretation of the three capability classes. For example, an Application Protocol may define a particular capability, Y, to be conditional on a Logical capability, X. If the rule is applied within the Application Protocol specification that capability Y may only be issued if capability X had been issued, then the value of Y in the final capability set may be interpreted as, in the case of a Logical capability Y, the number of Peer Application Protocol Entities, among those which have capability X, which also have capability Y, and in the case of a numerical capability Y, the minimum or maximum value of Y among those Peer Application Protocol Entities which have capability X. Note that in the case of a numerical capability Y, if the number of Peer Application Protocol Entities which indicate the capability Y is less than the number of Peer Application Protocol Entities indicating the capability X, this

indicates that some Peer Application Protocol Entities do not support a value of Y beyond its default value. In this case, the negotiated value of Y should be ignored. The negotiated value of Y is valid in this case only if the number of Peer Application Protocol Entities with both the X and Y capabilities are equal.

### 7.3.2	Description of the application roster exchange process

Each Application Protocol Entity makes its local GCC Provider aware of its presence through a GCCSAP over which it communicates with the GCC Provider. Creation and management of the GCCSAP is a local matter not covered by this Recommendation.

Application Protocol Entities are made aware of the existence of a conference to which the node is joined by the GCC-Application-Permission-To-Enroll indication. Once an Application Protocol Entity has been made aware of an existing conference, it shall issue a GCC-Application-Enroll request to the GCC Provider. This enroll request may indicate that the Application Protocol Entity is enrolling itself in the conference (the enroll/un-enroll flag set to enroll), or it may indicate that it is not enrolling itself into the conference (the enroll/un-enroll flag set to un-enroll). In the latter case, the Application Record parameters need not be included in the primitive. Any time thereafter, until permission to enroll is revoked (upon receiving a GCC-Application-Permission-To-Enroll primitive with the grant/revoke flag set to revoke), an Application Protocol Entity which did not choose to initially enroll itself into the conference, may enroll by issuing a GCC-Application-Enroll request.

There are several parameters in the GCC-Application-Enroll request which, if used, require steps to be taken prior to issuing the GCC-Application-Enroll request. First is the Application User ID which is mandatory in the request if enrolling with the Active/Inactive flag is set to Active. To get an Application User ID, the Application Protocol Entity must first attach to the MCS Domain indicated in the GCC-Application-Permission-To-Enroll indication. This is done by issuing the MCS primitive MCS-Attach-User request. On reception of the MCS-Attach-User confirm, the Application Protocol Entity will have been allocated an Application User ID. This ID may be included in the GCC-Application-Enroll request. If the attachment to the domain should fail due to the connection to the domain being lost during the intervening time, the Application Protocol Entity shall assume that the node is no longer part of the conference and shall wait until receiving another GCC-Application-Permission-To-Enroll indication before proceeding further. When the node disconnects from the conference, the Application Protocol Entity will be notified directly by MCS that the attachment is no longer valid. The Application Protocol Entity shall assume that the node is no longer part of the conference and shall wait until receiving another GCC-Application-Permission-To-Enroll indication before proceeding further. If permission to enroll is revoked by a GCC-Application-Permission-To-Enroll primitive, the Application Protocol Entity shall not attempt to attach to the corresponding domain.

The second such parameter is the optional Session ID which is part of the Session Key. The Session ID shall be formed from an MCS Channel ID (which is not a User ID). When enrolling into a session being invoked, the Session ID is obtained from the GCC-Application-Invoke indication. If joining a session already in progress, the Session ID may be obtained from the most recently received Application Roster. If creating a new session, then a new Channel ID may be allocated using an MCS-Channel-Join primitive or MCS-Channel-Convene primitive.

As a consequence of a GCC-Application-Enroll request primitive, the GCC Provider creates a new entry in its Local Application Roster. If an entry already exists for an Application Protocol Entity, the contents of the Local Application Roster entry are modified to contain the new information, or in the case of the un-enroll flag being set, the entry is removed from the roster.

At the start of a conference, the GCC Provider, after issuing the GCC-Application-Permission-To-Enroll primitive, waits for all Application Protocol Entities which are connected to the GCC Provider through a GCCSAP to issue a GCC-Application-Enroll primitive either enrolling, or indicating their explicit desire not to enroll. Once all Application Protocol Entities have responded, the GCC Provider exchanges the Local Application Roster with the other nodes in the conference. During a conference in progress, any changes to the Local Application Roster are immediately exchanged with other nodes.

In either case, whenever a new exchange takes place, initiated by any node, a new Conference Application Roster is generated and distributed to all nodes. The GCC Provider issues the entire roster to the Node Controller, and issues portions of this roster to enrolled Application Protocol Entities using the GCC-Application-Roster-Report indication primitive. To an Application Protocol Entity, the roster report includes the portion of the Conference Application Roster which specifically applies to that Application Protocol Session (and may include other portions as well). For each session, this is in the form of a list. Each entry in the list includes the Node ID and Application Protocol Entity ID

which, together, identify the Application Protocol Entity, and the Application Record for that Application Protocol Entity. It may also include the Application Capabilities List for the Application Protocol Session. This is a list of the common capabilities for this Application Protocol Session based on application of the capability class rules to the capabilities announced by all Peer Application Protocol Entities. In the case of the node controller, the GCC-Application-Roster-Report indication includes the entire Conference Application Roster. That is, for each entry in the list, it includes the node identifier, and the Application Records for all Application Protocols enrolled at that node. It also may include, for each Application Protocol, an Application Capabilities List.

An Application Protocol Entity is not considered part of a conference until it has received a GCC-Application-Roster-Report indication in which it is included in the Application Roster. If, at any time, an Application Protocol Entity which had previously been enrolled receives a GCC-Application-Roster-Report indication in which it is no longer included, that Application Protocol Entity shall be considered no longer enrolled. The Application Protocol Entity may attempt to re-enroll by issuing a GCC-Application-Enroll request.

At any time an Application Protocol Entity may remove itself from the Application Roster by issuing a GCC-Application-Enroll request primitive with the enroll/un-enroll flag set to un-enroll. This removes the Application Record for that Application Protocol Entity from the Local Application Roster as well as from the Conference Application Roster at all nodes in the conference.

At any time during a conference, an Application Protocol Entity or the Node Controller may request a portion of the Conference Application Roster. For each node in the conference, all Application Records which match the Session Key given in the GCC-Application-Roster-Inquire request are returned. Using a null key will result in the complete Conference Application Roster.

An Application Protocol Entity or Node Controller may attempt to remotely invoke an Application Protocol Entity at another node. This is done by issuing a GCC-Application-Invoke request. This request may optionally include a specified list of destination nodes. The corresponding GCC-Application-Invoke indication is received by the Node Controller at the specified destination nodes (or all nodes if no destination is specified). The node controller may optionally respond to this indication by invoking a Peer Application Protocol Entity which may then enroll itself in the conference.

### 7.3.3	Description of abstract services

The following is a list of the primitives defined in this subclause and a brief summary of the function of each:

•	GCC-Application-Permission-To-Enroll – Generated by GCC and issued to all Application Protocol Entities which have made the GCC Provider aware of their presence whenever the local node has been joined to a conference. This indicates that the Application Protocol Entity may enroll. With the Revoke flag set, this revokes the ability to enroll when the node is no longer joined to the conference.

•	GCC-Application-Enroll – Used by Application Protocol Entities to establish (or terminate) communications with other Peer Application Protocol Entities in a conference. Use of this primitive generates (or modifies, or removes) an entry in the Application Roster exchanged with other nodes.

•	GCC-Application-Roster-Report – Generated by GCC in response to any change in the Application Roster due to Application Protocol Entities enrolling or un-enrolling or nodes entering or leaving a conference.

•	GCC-Application-Roster-Inquire – Used by either the Node Controller or by Application Protocol Entities to retrieve all or a portion of the current Application Roster at any time during a conference.

•	GCC-Application-Invoke – Used to signal a set of other nodes in a conference to invoke an Application Protocol Entity for a particular session of an Application Protocol.

#### 7.3.3.1	GCC-Application-Permission-To-Enroll

The primitive GCC-Application-Permission-To-Enroll indication tells a local Application Protocol Entity that the local node is now joined to the specified conference and that the Application Protocol Entity may enroll itself with that conference. This primitive is also used to revoke such permission if the local node is no longer joined to a conference. While an MCS-attached Application Protocol Entity will receive notification from MCS when the conference terminates at that node, an Application Protocol Entity which has chosen not to attach (or to enroll) will not necessarily receive such notification. This primitive indicates to such an Application Protocol Entity that it may no longer enroll with that conference or attach to the corresponding MCS Domain. The GCC-Application-Permission-To-Enroll indication is issued by a GCC Provider to all Application Protocol Entities which have made the GCC Provider aware of their presence only after the node has been successfully joined to a conference. Table 7-24 shows the parameters and types of this primitive. Figure 7-18 shows the sequence of events when using this primitive.

TABLE  7-24/T.124

GCC-Application-Permission-To-Enroll – Types of primitives and their parameters

| Parameter | Indication |
| --- | --- |
| Conference ID | M |
| Grant/Revoke Flag | M |

Conference ID: Identifier of the Conference to which the Application Protocol Entity may enroll. This parameter is equal to the MCS Domain to which the Application Protocol Entity may attach.

Grant/Revoke Flag: This flag indicates whether the Application Protocol Entity is being given permission to enroll, or if that permission is being revoked.

FIGURE 7-18/T.124...[D21] = 8 CM

#### 7.3.3.2	GCC-Application-Enroll

The GCC-Application-Enroll request primitive is issued by an Application Protocol Entity to establish itself as part of the specified conference. This primitive shall be issued in response to a GCC-Permission-To-Enroll indication, although the Application Protocol Entity may set the Enroll/Un-enroll flag to Un-enroll, indicating that it does not wish to enroll at this time. At any time until permission to enroll is revoked, the Application Protocol Entity may enroll, re-enroll (to change entries in its Application Record), or un-enroll. If an Application Protocol Entity which is already un-enrolled attempts to un-enroll, the request is accepted with a successful result, but no change is made to the status of that Application Protocol Entity. When an Application Protocol Entity is enrolled, the associated parameters form the Application Record which is broadcast to the conference during the Application Roster exchange. Application Protocol Entities that are not enrolled will not be available to any local or remote participants and will not receive GCC-Application-Roster-Report indications. Table 7-25 shows the parameters and types of this primitive. Figure 7-19 shows the sequence of events when using this primitive.

TABLE  7-25/T.124

GCC-Application-Enroll – Types of primitives and their parameters

| Parameter | Request | Confirm |
| --- | --- | --- |
| Conference ID | M | M(=) |
| Session Key | M | M(=) |
| Active or Inactive | M |  |
| Application User ID | C |  |
| Conducting Operation Capable | O |  |
| Start-up Channel | O |  |
| Non-Collapsing Capabilities List | O |  |
| Application Capabilities List | O |  |
| Enroll or Un-enroll | M |  |
| Application Protocol Entity ID |  | C |
| Node ID |  | C |
| Result |  | M |

Conference ID: Identifier of the conference to which the primitive refers.

Session Key: Unique identifier of a particular Application Protocol Session. This corresponds to the identifier of the Application Protocol, optionally concatenated with a Session ID.

Active or Inactive: This flag is used to indicate whether the enrolling Application Protocol Entity is fully active (indicating that it can perform tasks required by the specification of the corresponding Application Protocol), or inactive. An Application Protocol Entity might indicate itself to be inactive in the case that it must enroll to make use of certain GCC services, but is not yet fully operational (e.g. it has not yet joined dynamic channels, but must first check the registry to determine which channels to join). Alternatively, it might enroll inactively to indicate support for the corresponding Application Protocol without the need to become active until it is known that there are like Application Protocol Entities at other nodes. The specific meaning of this flag shall be defined in the specification of each Application Protocol.

Application User ID: MCS User ID given to this Application Protocol Entity. This parameter is optional in the case of inactive Application Protocol Entities, but is mandatory for active Application Protocol Entities. It is an error for an Application Protocol Entity that is enrolled with an MCS User ID to re-enroll with a different MCS User ID. This parameter is not needed in the case of un-enrolling.

Conducting Operation Capable: This is a flag which indicates whether the Application Protocol Entity is capable of operating as a conducting Application Protocol Entity if the corresponding Application Protocol specification defines the procedures for such an Application Protocol Entity to follow. The GCC Provider within each node chooses no more than one Peer Application Protocol Entity per Application Protocol Session to include with this flag set in the exchanged Application Roster. If a node becomes the conference conductor, the designated Application Protocol Entity, if any, for each Application Protocol Session becomes the designated conducting Application Protocol Entity. The enrolling Application Protocol Entity shall not assume itself to be in the designated conducting role unless it determines that its corresponding entry in the received Application Roster has this flag set. This flag only applies to Active Application Protocol Entities. If the Active/Inactive flag is set to Inactive when enrolling, the Conducting Operation Capable flag is ignored and assumed by the GCC Provider to be FALSE.

Start-up Channel: This is an optional parameter which may take on the values Static, Dynamic Multicast, Dynamic Private, or Dynamic UserId. This parameter specifies the type of MCS Channel the Application Protocol Entity will use for start-up sequencing. The exact interpretation of this parameter as well as any requirements for the use of this parameter are Application Protocol-specific. In some cases, certain of these channel types may not be valid for particular Application Protocols.

Non-Collapsing Capabilities List: This is an optional parameter which allows Application Protocol Entities to list capabilities (either standard or non-standard) which are to be maintained in the roster as part of the Application Record of each Application Protocol Entity rather than being collapsed by a set of rules as the capabilities listed in the Application Capabilities List would be. Each entry in this list includes a capability ID which may be either standard or non-standard, as well as a data field for Application Protocol-specific data.

Application Capabilities List: This is an optional list of Application Protocol–specific capabilities. Each capability is tagged with a class identifier indicating the rule to be applied by GCC in determining the common capability set for this Application Protocol Session. The class identifiers are one of the choices listed in Table 7-23.

Enroll or Un-enroll: This is a flag used to indicate whether the Application Protocol Entity wishes to add, or change its Application Record (enroll), or to remove its Application Record (un-enroll). When an Application Protocol Entity has received a GCC-Permission-To-Enroll indication, it shall respond with a GCC-Enroll-Request. If that Application Protocol Entity wishes not to enroll, it shall set this flag to Un-enroll.

Application Protocol Entity ID: Present only in the case of a successful new enroll (not in the case when already enrolled, in the case of an unsuccessful enroll, or in the case of an un-enroll). An identifier which uniquely identifies the Application Protocol Entity within the local node. This corresponds to the same parameter in each entry of the Application Roster. This allows an Application Protocol Entity to determine if it is included in subsequent updates of the Application Roster.

Node ID: Present only in the case of a successful enroll (not in an unsuccessful enroll or an un-enroll). This provides the enrolling Application Protocol Entity the Node ID of its local node. This helps to allow the Application Protocol Entity to find its corresponding entry in received Application Rosters to determine when it actually becomes enrolled into the conference. The combination of the Node ID and the Application Protocol Entity ID uniquely determine the Application Protocol Entity.

Result: An indication of whether the request was accepted or rejected, and if rejected, the reason why. It contains one of a list of possible results: successful, invalid conference, not permitted to change MCS User ID.

FIGURE 7-19/T.124...[D22] = 8 CM

#### 7.3.3.3	GCC-Application-Roster-Report

This primitive is used by a GCC Provider to send to each enrolled Application Protocol Entity as well as to the Node Controller some or all of the Conference Application Roster. A GCC-Application-Roster-Report indication may be generated automatically by a GCC Provider whenever the GCC Provider finds out that a change in any portion of the roster has occurred. This may happen, for example, as a result of a GCC-Application-Enroll request, or on detection of an Application Protocol Entity detaching from the conference, or a node leaving the conference altogether. When any portion of the roster has changed for any Application Protocol Entity, the GCC-Application-Roster-Report indication shall be issued to the Node Controller. The GCC Provider shall issue a GCC-Application-Roster-Report indication to an active Application Protocol Entity when any portion of the roster has changed for that any Peer Application Protocol Entity. In this case it is required to include only the portion of the roster which applies to the corresponding Application Protocol Session. The GCC Provider may also issue a GCC-Application-Roster-Report indication to an Application Protocol Entity at other times (such as when entries other than for Peer Application Protocol Entities have changed) and may include in the indication primitive portions of the roster beyond those corresponding to Peer Application Protocol Entities. For inactive Application Protocol Entities, if the Application Protocol Entity has enrolled with a specific Session ID, the same rules apply as for an active Application Protocol Entity. For an inactive Application Protocol Entity which has enrolled with no Session ID, the GCC Provider shall issue a GCC-Application-Roster-Report when the contents of the roster have changed for all Application Protocol Sessions which are based on the same Application Protocol as the enrolled Application Protocol Entity. Table 7-26 shows the parameters and types of this primitive. When issued to the Node Controller, each of the Application Protocol–specific parameters (all except the Conference ID) is repeated separately for each Application Protocol Session. Figure 7-20 shows the sequence of events when using this primitive.

TABLE  7-26/T.124

GCC-Application-Roster-Report – Types of primitives and their parameters

R

| Parameter | Indication |
| --- | --- |
| Conference ID | M |
| Updated Application Roster | M |

Conference ID: Identifier of the MCS Domain corresponding to the indicated conference.

Updated Application Roster: The updated Application Roster includes the information shown in Table 7-27 for each Application Protocol Session being indicated by the indication.

TABLE  7-27/T.124

Contents of the Updated Application Roster for each Application Protocol Session

| Parameter | Description |
| --- | --- |
| Session Key | The Session Key (including the Session ID, if any) designating the particular Application Protocol Session. |
| Application Protocol Entity List Updated Flag | A flag indicating whether the Application Protocol Entity List for this Session Key has been updated in this report. If so, the Application Protocol Entity List is included as the following parameter. |
| Application Protocol Entity List (conditional) | A list of the Application Protocol Entities enrolled in the conference as part of this Application Protocol Session including an Application Record for each. The contents of each entry in this list is shown in Table 7-28. |
| Instance Number | The instance number for the Application Roster for this Session Key. This is a 16-bit number which is incremented modulo 216 each time the contents of the Application Roster changes. This allows Application Protocol Entities to perform operations with respect to a particular capability set which may be in the process of changing avoiding any race conditions. |
| Peer Entities Added Flag | A flag indicating whether one or more Peer Application Protocol Entities have been added to the Application Roster at any node since the last instance. This flag is not mutually exclusive of the Peer Entities Removed Flag. |
| Peer Entities Removed Flag | A flag indicating whether one ore more Peer Application Protocol Entities have been removed from the Application Roster at any node since the last instance. This flag is not mutually exclusive of the Peer Entities Added Flag. |
| Application Capabilities List Updated Flag | A flag indicating whether the Application Capabilities List for this Session Key has been updated in this report. If so, the Application Capabilities List is included as the following parameter. |
| Application Capabilities List (conditional) | The fully collapsed Application Capabilities List for this Session Key. |

TABLE  7-28/T.124

Contents of each entry of the Application Protocol Entity List

| Parameter | Description |
| --- | --- |
| Node ID | Node ID identifying the node at which the Application Protocol Entity has enrolled. |
| Application Protocol Entity ID | An identifier which uniquely identifies the Application Protocol Entity within the node specified by the Node ID. |
| Active/Inactive Flag | A flag indicating whether the enrolled Application Protocol Entity is inactive or active. |
| Application User ID (conditional) | The MCS User ID associated with the enrolled Application Protocol Entity. |
| Conducting Operation Capable | This is a flag which indicates whether the Application Protocol Entity is capable of operating as a conducting Application Protocol Entity if the corresponding Application Protocol specification defines the procedures for such an Application Protocol Entity to follow. There is no more than one Peer Application Protocol Entity per node with this flag set. If a node becomes the conference conductor, the designated Application Protocol Entity at that node, if any, for this Application Protocol Session becomes the designated conducting Application Protocol Entity. |
| Start-up Channel (conditional) | This parameter, if present, may take on the values Static, Dynamic Multicast, Dynamic Private, or Dynamic UserId. This parameter indicates the type of MCS Channel the Application Protocol Entity will use for start-up sequencing. The exact interpretation of this parameter is Application Protocol-specific. In some cases, certain of these channel types may not be valid for particular Application Protocols. |
| Non-Collapsing Capabilities List (conditional) | This is a list of Application Protocol-specific capabilities (either standard or non-standard) which are maintained in the roster as part of the Application Record of each Application Protocol Entity. |

FIGURE 7-20/T.124...[D23] = 8 CM

#### 7.3.3.4	GCC-Application-Roster-Inquire

An Application Protocol Entity may request a portion of the Conference Application Roster corresponding to a single Application Protocol Session, a group of these, or all Application Protocol Entities, using the GCC-Application-Roster-Inquire request primitive. This information is returned by GCC in the GCC-Application-Roster-Inquire confirm primitive. Table 7-29 shows the parameters and types of this primitive. Figure 7-21 shows the sequence of events when using this primitive.

TABLE  7-29/T.124

GCC-Application-Roster-Inquire – Types of primitives and their parameters

| Parameter | Request | Confirm |
| --- | --- | --- |
| Conference ID | M | M(=) |
| Session Key | M | M(=) |
| Application Roster |  | M |
| Result |  | M |

R

Conference ID: Identifier of the conference to which the primitive refers.

Session Key: Unique identifier of an Application Protocol Session. This may be a partial Session Key or a null key, indicating that the Application Information for all Application Protocol Entities is desired.

Application Roster: The Application Roster includes a list of roster entries for each Application Protocol Session specified by a Session Key whose octets match the Session Key in the request up to the length of the Session Key in the request, and if a Session ID is specified as part of the Session Key, those with the identical Session ID. For each matched entry, the information returned is shown in Table 7-30.

TABLE  7-30/T.124

Contents of the Application Roster for each Application Protocol Session

| Parameter | Description |
| --- | --- |
| Session Key | The Session Key (including the Session ID, if any) designating the particular Application Protocol Session. |
| Application Protocol Entity List | A list of the Application Protocol Entities enrolled in the conference as part of this Application Protocol Session including an Application Record for each. The contents of each entry in this list is shown in Table 7-28. |
| Instance Number | The instance number for the Application Roster for this Session Key. This is a 16-bit number which is incremented modulo 216 each time the contents of the Application Roster changes. This allows Application Protocol Entities to perform operations with respect to a particular capability set which may be in the process of changing avoiding any race conditions. |
| Application Capabilities List | The fully collapsed Application Capabilities List for this Session Key. |

R

Result: An indication of whether the request was accepted or rejected, and if rejected, the reason why. It contains one of a list of possible results: successful, invalid conference.

FIGURE 7-21/T.124...[D24] = 8 CM

#### 7.3.3.5	GCC-Application-Invoke

This primitive may be issued by an Application Protocol Entity or by the Node Controller to request that an Application Protocol Entity or list of Application Protocol Entities be invoked at a specified set of nodes. At the specified nodes, the indication form of this primitive is issued to the Node Controller which may invoke the specified Application Protocol Entities. For each listed Application Protocol Entity this indication indicates the desire to create a new Application Protocol Entity with the Session Key as specified in the request which would then enroll actively into the conference, becoming part of the corresponding Application Protocol Session. Alternatively, if an inactive Application Protocol Entity already exists at a destination node with the identical Session Key, receipt of this indication indicates the desire to make that Application Protocol Entity re-enroll in the active state. If an Application Protocol Entity exists at a destination node with the identical Session Key which is already in the active state, this indication may be ignored. Table 7-31 shows the parameters and types of this primitive. Figure 7-22 shows the sequence of events when using this primitive.

TABLE  7-31/T.124

GCC-Application-Invoke – Types of primitives and their parameters

| Parameter | Request | Indication | Confirm |
| --- | --- | --- | --- |
| Conference ID | M | M | M(=) |
| Application Protocol Entity Invoke List | M | M(=) | M(=) |
| Destination Nodes (List of Node IDs or NULL) | O |  |  |
| Invoking Node ID |  | M |  |
| Result |  |  | M |

Conference ID: Identifier of the conference to which the primitive refers.

Application Protocol Entity Invoke List: This is a list of one or more Application Protocol Entities to be invoked.  The contents of each entry in this list are shown in Table T-32.

TABLE  7-32/T.124

Contents of each entry in the Application Protocol Entity Invoke List

| Parameter | Description |
| --- | --- |
| Session Key | The Session Key (including the Session ID, if any) designating the particular Application Protocol Session to invoke. |
| Expected Capability Set | This is an optional list of application capabilities (in the same form as the Application Capabilities List) which indicates the expected set of capabilities that the Application Protocol Entity to be invoked must have. If the Application Protocol Entity at a particular node cannot satisfy this, it shall not be invoked at that node. If this parameter is not included, no constraints are placed on the invoked Application Protocol Entity. Note that the capabilities class definitions are used to determine the interpretation of this expected capability set, but are not interpreted by GCC as they are in the case of the Application Capabilities List. For a capability of the Unsigned-MAX class, the invoked Application Protocol Entity must have a capability less-than-or-equal-to the specified capability, while for an Unsigned-MIN class, the invoked Application Protocol Entity must have a capability greater-than-or-equal-to the specified capability. For a capability in the Logical class, the invoked Application Protocol Entity simply must have the identical capability. |
| Start-up Channel (conditional) | Either Static, Dynamic Multicast, Dynamic Private, or Dynamic UserId. This parameter specifies the type of MCS Channel the Application Protocol Entity should assume for use when invoking. In some cases, certain of these channel types may not be valid for particular Application Protocols. If an invalid setting is used, the Application Protocol Entity shall not be invoked. |
| Mandatory/Optional Flag | This flag indicates whether this Application Protocol Entity must be invoked in order to invoke the other Application Protocol Entities in this list. This flag is used by the invoking node to indicate that the destination nodes should only invoke the Application Protocol Entities in this list if all of the Application Protocol Entities marked as Mandatory can successfully be invoked. |

Destination Nodes (List of Node IDs or NULL): List of Node IDs identifying GCC Providers to which the request should go, or null, to indicate it should go to all nodes in the conference designated by the Conference ID.

Invoking Node ID: The Node ID of the node which initiated the corresponding GCC-Application-Invoke request.

Result: An indication of whether the request was accepted or rejected, and if rejected, the reason why. It contains one of a list of possible results: successful, invalid conference.

FIGURE 7-22/T.124...[D25] = 8 CM

## 7.4	The Application Registry

The Application Registry is a functional component of GCC. The registry offers a set of functions to Application Protocol Entities which operate on a central data-base located at the Top GCC Provider. The contents of the registry are unique to a single conference. The significance of information stored in the registry data-base is defined by Application Protocols. The central repository can assist in establishing communication among Application Protocol Entities. It can help peers to discover a common Channel ID on which to communicate, a common Token ID to regulate exclusive access, or a common parameter setting (a parameter is a registry entry with an Application Protocol-specific use). It offers an alternative to the static reservation of Channel IDs, Token IDs, and other parameter values. By supporting dynamic discovery, the Application Registry can facilitate the introduction of new features, standardized or non-standardized, that enhance audiographic and audiovisual conferencing. The registry also includes a general purpose service for allocation of numeric handles which are globally unique within a conference.

### 7.4.1	Registry keys

The indexes used to store and retrieve entries in the registry data-base are Application Protocol-specified keys. Both standard and non-standard keys are allowed. Standard keys are allocated by Recommendations like this. Non-standard keys may be proprietary. Both are structured to avoid conflicts in the choice of key values.

Registry keys consist of a Session Key as defined in 7.3.1 in combination with a Resource ID. The Session Key used for any registry operation requiring a Registry Key shall be identical to the Session Key of an Application Protocol Session present in the Application Roster (with either active or inactive members). The Resource ID is of ASN.1 type OCTET STRING. The Resource ID allows a single Application Protocol Entity to make use of multiple Registry Keys. The value of the Resource IDs are defined by the specifications of the Application Protocols which make use of them.

### 7.4.2	Ownership and persistence

The registry yields ownership of a Registry Key to the first Application Protocol Entity that requests to store an item there via either the GCC-Registery-Register-Channel, GCC-Registry-Assign-Token, or GCC-Registry-Set-Parameter request primitives.

In the case of a Parameter registry item, when the parameter is created, its owner may specify the scope of Application Protocol Entities allowed to modify the value of that entry. The owner may specify either owner, session, or public modification rights – the parameter may be modifiable by the owner only (as in the case of Channels and Tokens), by all Peer Application Protocol Entities (those within the same Application Protocol Session), or by any Application Protocol Entity in the conference, respectively. Modification of the contents of a parameter by an Application Protocol Entity other than the owner does not alter the identity of the owner of that entry. The owner of a parameter may at any time re-define the scope of modification rights. If another node attempts to modify the modification rights as part of issuing a GCC-Registry-Set-Parameter request, the rights will not be modified, although the parameter will be set to the requested value if the requester had modification rights.

For all registry item types (Channel, Token, or Parameter), ownership is required to delete the entry. An entry, once deleted, has no owner and may be taken over by a different Application Protocol Entity.

Registry entries are not deleted automatically when the owner detaches from the conference. Their content persists unchanged indefinitely. However, ownership of an entry is removed when the owner detaches. This allows a surviving Application Protocol Entity to modify (for a Parameter entry only) or delete an orphaned entry if its usefulness has expired. The first Application Protocol Entity that requests to store an item in an orphaned Parameter entry becomes its new owner.

When all Application Protocol Entities in the session that corresponds to the Session Key used to form the Registry Key for a registry entry become un-enrolled, that registry entry is automatically deleted.

### 7.4.3	Dynamic allocation

MCS distinguishes between static and dynamic Channel IDs. The latter comprise User IDs, private Channel IDs, and assigned Channel IDs. Dynamic Channel IDs are created and deleted directly by Application Protocol Entities, separately from their use of registry services. The Application Registry stores dynamic Channel IDs in a central repository for retrieval by other Application Protocol Entities. It does not test or operate on these Channel IDs with MCS primitives.

MCS makes no distinction between static and dynamic Token IDs. To maintain similar semantics, an artificial division is imposed by GCC. Token IDs 1 to 16 383 are designated static and are reserved for assignment by other specifications. Token IDs 16 384 to 65 535 are designated dynamic and are allocated by the Top GCC Provider upon request, as part of creating an entry in the registry data-base. When the associated entry is deleted, the Token ID it held is made available for reassignment. The registry does not invoke MCS primitives, like grab and release, on the Token IDs it assigns. It merely chooses specific ID values and disseminates them. Application Protocol Entities are free to operate on assigned Token IDs according to their own logic.

### 7.4.4	Description of abstract services

The following is a list of the primitives defined in this subclause and a brief summary of the function of each:

•	GCC-Registry-Register-Channel – Used by an Application Protocol Entity to register the Channel ID of an MCS dynamic channel. Application Protocol Entities may examine the entry using GCC-Registry-Retrieve-Entry to determine if a node has already registered the Channel ID, and if so, retrieve the value of the Channel ID.

•	GCC-Registry-Assign-Token – Used by an Application Protocol Entity to allocate a dynamic token and register the assigned Token ID. Application Protocol Entities may examine the entry using GCC-Registry-Retrieve-Entry to determined if a node has already registered the Token ID, and if so, retrieve the value of the Token ID.

•	GCC-Registry-Set-Parameter – Used by an Application Protocol Entity to set a value in the registry data-base which may be examined or modified from any node in a conference.

•	GCC-Registry-Retrieve-Entry – Used by an Application Protocol Entity to extract the current contents of any registry entry.

•	GCC-Registry-Delete-Entry – Used by an Application Protocol Entity to remove an entry in the registry data-base.

•	GCC-Registry-Monitor – Used by an Application Protocol Entity to enable (or disable) monitoring of a registry entry. Once enabled, the indication form of this primitive notifies the requesting Application Protocol Entities of any changes to the content of the entry (including deletion).

•	GCC-Registry-Allocate-Handle – Used to generate a 32-bit Handle which is unique within the scope of a single conference.

For a particular requester, the order of registry request primitives, the resulting action at the Top GCC Provider (if successful), and their associated confirm primitives is preserved.

#### 7.4.4.1	GCC-Registry-Register-Channel

The GCC-Registry-Register-Channel request primitive may be issued by an Application Protocol Entity to inform Application Protocol Entities at other nodes that a particular MCS channel has been designated for use by the Application Protocol Entity in the manner indicated by the Registry Key. Once any Application Protocol Entity registers using a particular Registry Key, Application Protocol Entities at other nodes may find out if a channel has been registered for this key (and if so, the value of the channel ID) by issuing the GCC-Registry-Retrieve-Entry request primitive specifying the Registry Key in question. Once a channel is registered, if any Application Protocol Entity attempts to register using the same Registry Key (including the owner), their attempt will be rejected with an indication that this Registry Key has already been used. Table 7-33 shows the parameters and types of this primitive. Figure 7-23 shows the sequence of events when using this primitive.

TABLE  7-33/T.124

GCC-Registry-Register-Channel – Types of primitives and their parameters

| Parameter | Request | Confirm |
| --- | --- | --- |
| Conference ID | M | M(=) |
| Registry Key | M | M(=) |
| Channel ID | M |  |
| Registry Item |  | C |
| Owner |  | C |
| Result |  | M |

Conference ID: Identifier of the conference to which the primitive refers.

Registry Key: The data-base index at which the item is to be stored.

Channel ID: A dynamic channel ID (user ID, private, or assigned) specified by the Application Protocol Entity.

Registry Item: The value of the entry after the request has taken effect. If the request is successful, this is the value of the parameter, if not successful due to inconsistent type or index already owned, this is the value prior to the request. It is either a Channel ID, Token ID, Parameter Value, or None if the entry is vacant (both the type and the value, if any, are indicated by this parameter). This parameter is not present if the confirm is a locally generated error condition.

Owner: This parameter indicates the current owner of this registry entry. If owned, this parameter includes the Node ID of the node at which the owner resides and the Entity ID of the owning Application Protocol Entity. This parameter also indicates if the entry is not owned. This parameter is not present if the confirm is a locally generated error condition.

Result: An indication of whether the request was accepted or rejected, and if rejected, the reason why. It contains one of a list of possible results: successful, invalid conference, registry full, index already exists, inconsistent type, invalid requester.

FIGURE 7-23/T.124...[D26] = 8.5 CM

#### 7.4.4.2	GCC-Registry-Assign-Token

The GCC-Registry-Assign-Token request primitive may be issued by an Application Protocol Entity to assign a token to be associated with a particular Registry Key. If the request is successful, the token ID is returned as a parameter in the confirm primitive. Once any Application Protocol Entity gets a token assigned using a particular Registry Key, Application Protocol Entities at other nodes may find out if a token has been assigned for this key (and if so, the value of the Token ID) by issuing the GCC-Registry-Retrieve-Entry request primitive specifying the Registry Key in question. Once the token is assigned, if any Application Protocol Entity attempt to get a token assigned using the same Registry Key (including the owner), their attempt will be rejected with an indication that this Registry Key has already been used. Table 7-34 shows the parameters and types of this primitive. Figure 7-24 shows the sequence of events when using this primitive.

TABLE  7-34/T.124

GCC-Registry-Assign-Token – Types of primitives and their parameters

| Parameter | Request | Confirm |
| --- | --- | --- |
| Conference ID | M | M(=) |
| Registry Key | M | M(=) |
| Token ID |  | C |
| Registry Item |  | C |
| Owner |  | C |
| Result |  | M |

Conference ID: Identifier of the conference to which the primitive refers.

Registry Key: The data-base index at which the item is to be stored.

Token ID: A dynamic token ID (16 384 or greater) assigned by the Top GCC Provider. This parameter is not present if the confirm is a locally generated error condition.

Registry Item: The value of the entry after the request has taken effect. If the request is successful, this is the value of the parameter, if not successful due to inconsistent type or index already owned, this is the value prior to the request. It is either a Channel ID, Token ID, Parameter Value, or None if the entry is vacant (both the type and the value, if any, are indicated by this parameter). This parameter is not present if the confirm is a locally generated error condition.

Owner: This parameter indicates the current owner of this registry entry. If owned, this parameter includes the Node ID of the node at which the owner resides and the Entity ID of the owning Application Protocol Entity. This parameter also indicates if the entry is not owned. This parameter is not present if the confirm is a locally generated error condition.

Result: An indication of whether the request was accepted or rejected, and if rejected, the reason why. It contains one of a list of possible results: successful, invalid conference, registry full, index already exists, inconsistent type, invalid requester.

#### 7.4.4.3	GCC-Registry-Set-Parameter

The GCC-Registry-Set-Parameter request primitive may be issued by an Application Protocol Entity to set or modify the value of a registry parameter. If the registry entry had been designated to be monitored using the GCC-Registry-Monitor request primitive, each successful GCC-Registry-Set-Parameter request results in a GCC-Monitor-Indication to Application Protocol Entities at all nodes in the conference which have enabled monitoring for this entry. If a registry entry exists for a particular key, a request to set a parameter shall only be accepted if the entry is already a parameter. Table 7-35 shows the parameters and types of this primitive. Figure 7-25 shows the sequence of events when using this primitive.

FIGURE 7-24/T.124...[D27] = 9 CM

TABLE  7-35/T.124

GCC-Registry-Set-Parameter – Types of primitives and their parameters

| Parameter | Request | Confirm |
| --- | --- | --- |
| Conference ID | M | M(=) |
| Registry Key | M | M(=) |
| Parameter Value | M |  |
| Registry Item |  | C |
| Owner |  | C |
| Modification Rights | O | C |
| Result |  | M |

Conference ID: Identifier of the conference to which the primitive refers.

Registry Key: The data-base index at which the item is to be stored.

Parameter Value: An octet string specified by the Application Protocol Entity.

Registry Item: The value of the entry after the request has taken effect. If the request is successful, this is the value of the parameter, if not successful due to inconsistent type, this is the value prior to the request. It is either a Channel ID, Token ID, Parameter Value, or None if the entry is vacant (both the type and the value, if any, are indicated by this parameter). This parameter is not present if the confirm is a locally generated error condition.

Owner: This parameter indicates the current owner of this registry entry. If owned, this parameter includes the Node ID of the node at which the owner resides and the Entity ID of the owning Application Protocol Entity. This parameter also indicates if the entry is not owned. If successful completion of this primitive results in the requester becoming the owner of this entry, the requester is indicated as the new owner in the confirm primitive. This parameter is not present if the confirm is a locally generated error condition.

Modification Rights: This optional parameter specifies the scope of Application Protocol Entities allowed to make modifications to the value of this registry entry. The value of this parameter may be either Owner, Session, or Public. The Owner setting specifies that only the owner (as long as the entry is owned) may modify this entry. The Session setting specifies that any Application Protocol Entity which is part of the same Application Protocol Session as the owner may modify this entry. The Public setting specifies that any Application Protocol Entity enrolled in the conference may modify this entry. If this parameter is not present when the entry is first created, a value of Public is assumed. If not present at other times, its value is left unchanged. Only the owner of this registry entry (as long as the entry is owned) can change the modification rights. If a non-owner attempts to change the modification rights, this change will not take place, however any modification to the parameter entry itself will occur as long as the requester has modification rights. In the confirm primitive, this parameter is present independent of whether it was in the request primitive and indicates the actual modification rights. This parameter is not present if the confirm is a locally generated error condition.

Result: An indication of whether the request was accepted or rejected, and if rejected, the reason why. It contains one of a list of possible results: successful, invalid conference, registry full, inconsistent type, invalid requester.

FIGURE 7-25/T.124...[D28] = 8.5 CM

#### 7.4.4.4	GCC-Registry-Retrieve-Entry

The GCC-Registry-Retrieve-Entry request primitive may be issued by an Application Protocol Entity to determine the contents of a single registry entry. This primitive may be issued at any time and will indicate the contents of the entry as well as whether the entry is a channel ID, a token ID, a parameter, or if the entry is empty. Table 7-36 shows the parameters and types of this primitive. Figure 7-26 shows the sequence of events when using this primitive.

TABLE  7-36/T.124

GCC-Registry-Retrieve-Entry – Types of primitives and their parameters

| Parameter | Request | Confirm |
| --- | --- | --- |
| Conference ID | M | M(=) |
| Registry Key | M | M(=) |
| Registry Item |  | C |
| Owner |  | C |
| Modification Rights |  | C |
| Result |  | M |

Conference ID: Identifier of the conference to which the primitive refers.

Registry Key: The data-base index of the entry to be retrieved.

Registry Item: A Channel ID, Token ID, Parameter Value, or None if the entry is vacant (both the type and the value, if any, are indicated by this parameter). This parameter is not present if the confirm is a locally generated error condition.

Owner: This parameter indicates the current owner of this registry entry. If owned, this parameter includes the Node ID of the node at which the owner resides and the Entity ID of the owning Application Protocol Entity. This parameter also indicates if the entry is not owned. This parameter is not present if the confirm is a locally generated error condition.

Modification Rights: This parameter, included only in the case of Parameter entries, indicates the scope of Application Protocol Entities allowed to make modifications to the value of this registry entry. The value of this parameter may be either Owner, Session, or Public. The Owner setting specifies that only the owner (as long as the entry is owned) may modify this entry. The Session setting specifies that any Application Protocol Entity which is part of the same Application Protocol Session as the owner may modify this entry. The Public setting specifies that any Application Protocol Entity enrolled in the conference may modify this entry. If this parameter is not present when the entry is first created, a value of Public is assumed. This parameter is not present if the confirm is a locally generated error condition.

Result: An indication of whether the request was accepted or rejected, and if rejected, the reason why. It contains one of a list of possible results: successful, entry not found, invalid conference.

FIGURE 7-26/T.124...[D29] = 8 CM

#### 7.4.4.5	GCC-Registry-Delete-Entry

The GCC-Registry-Delete-Entry request primitive may be issued by an Application Protocol Entity to remove a registry entry. Removal of a registry entry is only allowed by the Application Protocol Entity which owns that entry (unless the last owner disconnected from the conference and no new owner has been assigned). If the registry entry had been designated to be monitored using the GCC-Registry-Monitor request primitive, a successful GCC-Registry-Delete-Entry request results in a GCC-Monitor-Indication to Application Protocol Entity at all nodes in the conference which have enabled monitoring for this entry with a NULL Registry Item parameter to indicate that the entry has been deleted. Table 7-37 shows the parameters and types of this primitive. Figure 7-27 shows the sequence of events when using this primitive.

TABLE  7-37/T.124

GCC-Registry-Delete-Entry – Types of primitives and their parameters

| Parameter | Request | Confirm |
| --- | --- | --- |
| Conference ID | M | M(=) |
| Registry Key | M | M(=) |
| Result |  | M |

Conference ID: Identifier of the conference to which the primitive refers.

Registry Key: The data-base index of the entry to be vacated, removing any item previously stored.

Result: An indication of whether the request was accepted or rejected, and if rejected, the reason why. It contains one of a list of possible results: successful, invalid conference, index already owned by another GCC Provider.

FIGURE 7-27/T.124...[D30] = 8 CM

#### 7.4.4.6	GCC-Registry-Monitor

GCC provides a mechanism for continuously monitoring particular registry entries of any type to determine if they have been changed (either altered, deleted, changed owners, or in the case of a parameter entry, changed modification rights) without the need for continuous polling. The GCC-Registry-Monitor request primitive may be used by an Application Protocol Entity to enable (or disable) monitoring of a particular registry entry. While enabled, the requesting Application Protocol Entity is notified of all changes to this entry via the GCC-Registry-Monitor indication primitive. An indication will be generated as a result of any modification of the contents of the registry entry or deletion of the entry. Only registry entries which exist may be monitored. Once an entry has been deleted, if it is recreated, the monitor request must be re-issued to begin monitoring again. Table 7-38 shows the parameters and types of this primitive. Figure 7-28 shows the sequence of events when using the request form of this primitive.

NOTE – Particular GCC Provider implementations may choose not to keep track of which Application Protocol Entities have enabled or disabled monitoring for each entry. In that case, Application Protocol Entities may receive GCC-Registry-Monitor indications for entries that they have not requested to be monitored, or for entries for which they specifically disabled monitoring.

TABLE  7-38/T.124

GCC-Registry-Monitor – Types of primitives and their parameters

| Parameter | Request | Indication | Confirm |
| --- | --- | --- | --- |
| Conference ID | M | M(=) | M(=) |
| Enable/Disable | M |  | M(=) |
| Registry Key | M | M(=) | M(=) |
| Registry Item |  | M |  |
| Owner |  | M |  |
| Modification Rights |  | C |  |
| Result |  |  | M |

Conference ID: Identifier of the conference to which the primitive refers.

Enable/Disable: TRUE to deliver indications of registry contents; FALSE to suppress indications.

Registry Key: The data-base index of an entry to be monitored (in the request/confirm), or that has changed (in the indication).

Registry Item: A Channel ID, Token ID, Parameter Value, or None indicating that the entry has been deleted (both the type and the value, if any, are indicated by this parameter).

Owner: This parameter indicates the current owner of this registry entry. If owned, this parameter includes the Node ID of the node at which the owner resides and the Entity ID of the owning Application Protocol Entity. This parameter also indicates if the entry is not owned. This parameter is not present if the confirm is a locally generated error condition.

Modification Rights: This parameter, included only in the case of Parameter entries, indicates the scope of Application Protocol Entities allowed to make modifications to the value of this registry entry. The value of this parameter may be either Owner, Session, or Public. The Owner setting specifies that only the owner (as long as the entry is owned) may modify this entry. The Session setting specifies that any Application Protocol Entity which is part of the same Application Protocol Session as the owner may modify this entry. The Public setting specifies that any Application Protocol Entity enrolled in the conference may modify this entry. If this parameter is not present when the entry is first created, a value of Public is assumed. This parameter is not present if the confirm is a locally generated error condition.

Result: An indication of whether the request was accepted or rejected, and if rejected, the reason why. It contains one of a list of possible results: successful, invalid conference, registry entry does not exist.

FIGURE 7-28/T.124...[D31] = 8 CM

#### 7.4.4.7	GCC-Registry-Allocate-Handle

The GCC-Registry-Allocate-Handle request primitive may be issued by an Application Protocol Entity to request that a numerical value (or list of values) be allocated to that Application Protocol Entity which is globally unique within the scope of a single conference. Handles are allocated by the Top GCC Provider in increasing numerical order in the order that requests are received. Blocks of handles are also allocated in increasing numerical order. As a result, only the first handle in a block is returned if the number of handles is more than one. Table 7-39 shows the parameters and types of this primitive. Figure 7-29 shows the sequence of events when using this primitive.

TABLE  7-39/T.124

GCC-Registry-Allocate-Handle – Types of primitives and their parameters

| Parameter | Request | Confirm |
| --- | --- | --- |
| Conference ID | M | M(=) |
| Number of Handles | M | M(=) |
| First Handle |  | M |
| Result |  | M |

Conference ID: Identifier of the conference to which the primitive refers.

Number of Handles: The number of handles requested to be allocated and returned in the confirm primitive. This may range from 1 to 1024.

First Handle: 32-bit unsigned integer value. If the number of requested handles is equal to one, this is the value of the allocated handle. If the number of handles is greater than one, the set of allocated handles are those contiguous values (modulo 232) that range from the value of First Handle, through the value (First Handle  Number of Handles) mod 232.

Result: An indication of whether the request was accepted or rejected, and if rejected, the reason why. It contains one of a list of possible results: successful, invalid conference, no handles available, too many handles requested.

FIGURE 7-29/T.124...[D32] = 8 CM

## 7.5	Conference conductorship

GCC provides a method for allowing a node to become a conductor for a conference. A token is used by GCC to determine whether a conference is conducted or non-conducted. The Node Controller at a node may acquire the Conductor token by issuing a GCC-Conductor-Assign request primitive. When any node successfully acquires the Conductor token, the conference is placed in Conducted Mode. The Node Controller as well as all Application Protocol Entities at all nodes in the conference are made aware when a conference switches into Conducted Mode by means of a GCC-Conductor-Assign indication. This indication also indicates which node has become the conductor.

Whether a particular conference may operate in conducted mode or not is determined when the conference is created. If the Conference Conductible flag had been set, the conference may be placed in conducted mode. If not, any attempt to place the conference in conducted mode will be rejected.

A node may release conductorship and return the conference into Non-conducted Mode by issuing a GCC-Conductor-Release request primitive. The Node Controller as well as all Application Protocol Entities at all nodes in the conference are made aware when a conference switches into Non-conducted Mode by means of a GCC-Conductor-Release indication.

A node which is currently the conductor of a conference may pass conductorship directly to another node without placing the conference in Non-Conducted mode during the transition by issuing a GCC-Conductor-Give request specifying the desired recipient node. If the recipient accepts conductorship, the Node Controller as well as all Application Protocol Entities at all nodes in the conference are made aware of this transition by means of a GCC-Conductor-Assign indication which indicates which node is the new conductor. A node may explicitly request from the conductor that conductorship be given to it by issuing a GCC-Conductor-Please request primitive. The current conductor may choose to give the requesting node conductorship, or may ignore the request.

The Node Controller as well as Application Protocol Entities at any node may also inquire as to which node, if any, currently holds the Conductor token by using the GCC-Conductor-Inquire request primitive.

When an Application Protocol Entity is made aware that the conference to which it is joined is in Conducted Mode, it shall immediately begin operating according to its Conducted Mode behaviour as prescribed by the specification of the corresponding Application Protocol. When an Application Protocol Entity is made aware that the conference to which it is joined is in Non-conducted mode, it shall immediately begin operating according to its Non-conducted Mode behaviour as prescribed by the specification of the corresponding Application Protocol. A typical Application Protocol specification may, for example, state that the Application Protocol Entity at any node when in Conducted Mode must request permission from the Peer Application Protocol Entity at the conductor before taking any action, while in Non-conducted Mode, no such permission is required. However, the behaviour defined by Application Protocol specifications while in Conducted or Non-conducted Mode is a matter outside of the scope of this Recommendation.

### 7.5.1	Description of abstract services

The following is a list of the primitives defined in this subclause and a brief summary of the function of each:

•	GCC-Conductor-Assign – Used by a Node Controller to request conductorship of a conference. When a new node becomes conductor, the indication form of this primitive is used to announce this to all Node Controllers in the conference as well as to all Application Protocol Entities at all nodes in the conference.

•	GCC-Conductor-Release – Used by a Node Controller to release conductorship of a conference. When any node releases conductorship, the indication form of this primitive is used to announce this to all Node Controllers in the conference as well as to all Application Protocol Entities at all nodes in the conference.

•	GCC-Conductor-Please – Used by a Node Controller to request that conductorship be given to it from the current conductor.

•	GCC-Conductor-Give – Used by a Node Controller to pass conductorship to a specified node.

•	GCC-Conductor-Inquire – Used by a Node Controller or an Application Protocol Entity to determine whether the conference is currently conducted nor non-conducted, and if conducted, the Node ID of the current conductor.

•	GCC-Conductor-Permission-Ask – Used by a Node Controller to request permission for Application Protocol Entities at that node to take actions which, in conducted-mode, require permission from the conductor.

•	GCC-Conductor-Permission-Grant – Used by the Node Controller at the conducting node to indicate which subset of nodes in a conference have been granted conducted-mode permission.

#### 7.5.1.1	GCC-Conductor-Assign

During non-conducted mode the Node Controller at any node has the possibility to issue a request to become conductor, by issuing a GCC-Conductor-Assign request primitive. The receipt of a GCC-Conductor-Assign confirm primitive indicates whether the requester has become the conductor or not, depending on the result parameter within the primitive. A successful GCC-Conductor-Assign request shall be accompanied by GCC-Conductor-Assign indications to all Node Controllers in the conference as well as to all enrolled Application Protocol Entities at all nodes in the conference signalling that the conference has become conducted, and giving information about the identity of the conductor. The indication form of this primitive may also be issued to an Application Protocol Entity upon enrolling into a conference to inform it that the conference is in conducted mode. The order of GCC-Conductor-Assign and GCC-Conductor-Release indications represents the actual order of conductorship transitions. Table 7-40 shows the parameters and types of this primitive. Figure 7-30 shows the sequence of events when using this primitive.

NOTE – If the conductor disconnects from the conference for any reason, the conference reverts to non-conducted mode until another node issues a request to become conductor.

TABLE  7-40/T.124

GCC-Conductor-Assign – Types of primitives and their parameters

| Parameter | Request | Indication | Confirm |
| --- | --- | --- | --- |
| Conference ID | M | M | M(=RQ) |
| Requesting Node ID |  | M |  |
| Result |  |  | M |

Conference ID: Identifier of the conference to which the primitive refers.

Requesting Node ID: The Node ID of the requesting node.

Result: An indication of whether the request was accepted or rejected, and if rejected, the reason why. It contains one of a list of possible results: successful, invalid conference, non-conductible conference, token already owned.

FIGURE 7-30/T.124...[D33] = 8.5 CM

#### 7.5.1.2	GCC-Conductor-Release

In order to change its status from conductor to normal participant, the Node Controller at the conductor node issues the GCC-Conductor-Release request primitive. As soon as the conductor has requested the release of conductorship, all Node Controllers in the conference as well as all enrolled Application Protocol Entities at all nodes in the conference are informed of the change in operational mode by means of the GCC-Conductor-Release indication. The order of GCC-Conductor-Assign and GCC-Conductor-Release indications represents the actual order of conductorship transitions. Any participant issuing a GCC-Conductor-Release, but not being the current conductor, shall be answered by GCC with a GCC-Conductor-Release confirmation containing a negative result and reason description. In this situation, no indications shall be sent to the other participants. Apart from this user initiated transition to non-conducted mode, the release of conductorship can also be initiated by GCC itself, for instance because the conductor disconnected from the conference. The indication form of this primitive may also be issued to an Application Protocol Entity upon enrolling into a conference to inform it that the conference is in non-conducted mode. Table 7-41 shows the parameters and types of this primitive. The sequences of primitives belonging to both situations are depicted in Figures 7-31 and 7-32, respectively.

TABLE  7-41/T.124

GCC-Conductor-Release – Types of primitives and their parameters

| Parameter | Request | Indication | Confirm |
| --- | --- | --- | --- |
| Conference ID | M | M | M(=RQ) |
| Result |  |  | M |

Conference ID: Identifier of the conference to which the primitive refers.

Accept/Reject: This flag indicates whether the GCC-Conductor-Release request was accepted or rejected.

Result: An indication of whether the request was accepted or rejected, and if rejected, the reason why. It contains one of a list of possible results: successful, invalid conference, did not own token.

FIGURE 7-31/T.124...[D34] = 8.5 CM

FIGURE 7-32/T.124...[D35] = 8.5 CM

#### 7.5.1.3	GCC-Conductor-Please

The primitive GCC-Conductor-Please request may be issued by a Node Controller to ask the current conductor to give conductorship to the requesting node. The GCC-Conductor-Please indication is forwarded to the current conducting node. The current conductor may then choose to give conductorship to the requester by using the GCC-Conductor-Give primitive. The confirm portion of this primitive is only local confirmation that the request was accepted by the local GCC Provider. No confirmation from the conductor is given directly. Table 7-42 shows the parameters and types of this primitive. Figure 7-33 shows the sequence of events when using this primitive.

TABLE  7-42/T.124

GCC-Conductor-Please – Types of primitives and their parameters

| Parameter | Request | Indication | Confirm |
| --- | --- | --- | --- |
| Conference ID | M | M | M(=RQ) |
| Requesting Node ID |  | M |  |
| Result |  |  | M |

Conference ID: Identifier of the conference to which the primitive refers.

Requesting Node ID: The Node ID of the requesting node.

Result: An indication of whether the request was accepted or rejected, and if rejected, the reason why. It contains one of a list of possible results: successful, invalid conference, not in conducted mode.

#### 7.5.1.4	GCC-Conductor-Give

The primitive GCC-Conductor-Give request may be issued by a Node Controller to transfer conductorship to a specific node. If the conductorship is not accepted by the intended recipient, conference conductorship continues to be held by the original conductor. Table 7-43 shows the parameters and types of this primitive. Figure 7-34 shows the sequence of events when using this primitive.

FIGURE 7-33/T.124...[D36] = 8.5 CM

NOTE – It is possible to use this primitive (in conjunction with others) to ensure that a particular node becomes the conference conductor with no opportunity for another node to acquire conductorship. The procedure would be for the conference convener first to create a conference which is locked and to then acquire conductorship using the GCC-Conductor-Assign primitive. Once the convener has become the conference conductor, it may then allow other nodes into the conference either by unlocking it using GCC-Conference-Unlock and/or by adding the other nodes directly. Once the intended conducting node has joined the conference, conductorship may be passed directly to that node by use of the GCC-Conductor-Give primitive.

TABLE  7-43/T.124

GCC-Conductor-Give – Types of primitives and their parameters

| Parameter | Request | Indication | Response | Confirm |
| --- | --- | --- | --- | --- |
| Conference ID | M | M | M(=IN) | M(=RQ) |
| Recipient Node ID | M |  |  | M(=) |
| Result |  |  | M | M(=) |

Conference ID: Identifier of the conference to which the primitive refers.

Recipient Node ID: The Node ID of the node to which conductorship is being transferred.

Result: An indication of whether the request was accepted or rejected, and if rejected, the reason why. It contains one of a list of possible results: successful, invalid conference, not conductor, give not accepted.

#### 7.5.1.5	GCC-Conductor-Inquire

The primitive GCC-Conductor-Inquire request may be issued at any time by either a Node Controller or Application Protocol Entity in order to find out whether the conference is conducted or not, and if so, which node is the conductor, and if the requesting node has been granted conducted-mode permission. Table 7-44 shows the parameters and types of this primitive. Figure 7-35 shows the sequence of events when using this primitive.

FIGURE 7-34/T.124...[D37] = 12 CM

TABLE  7-44/T.124

GCC-Conductor-Inquire – Types of primitives and their parameters

| Parameter | Request | Confirm |
| --- | --- | --- |
| Conference ID | M | M(=) |
| Conducted vs. Non-Conducted Flag |  | M |
| Conductor Node ID |  | C |
| Permission flag |  | C |
| Result |  | M |

Conference ID: Identifier of the conference to which the primitive refers.

Conducted vs. Non-Conducted Flag: A flag indicating whether the indicated conference is currently in conducted mode or non-conducted mode.

Conductor Node ID: The Node ID of the node that is currently conductor. Not present if currently in non-conducted mode.

Permission Flag: If in conducted mode, this flag indicates whether or not the local node has been granted conducted-mode permission.

Result: An indication of whether the request was accepted or rejected, and if rejected, the reason why. It contains one of a list of possible results: successful, invalid conference.

FIGURE 7-35/T.124...[D38] = 8 CM

#### 7.5.1.6	GCC-Conductor-Permission-Ask

The primitive GCC-Conductor-Permission-Ask request may be issued by a Node Controller to ask the current conductor to grant (or release) permission to allow Application Protocol Entities at the requesting node to perform any actions for which permission from the conductor is required. The definition of which specific actions require this permission is a matter for the individual Application Protocol specifications to define. The GCC-Conductor-Permission-Ask indication is forwarded to the current conducting node. The order of GCC-Conductor-Permission-Ask indications from a single node represents the actual order of the requests from that node. The current conductor may then choose to grant (or release) conducted-mode permission to the requester by using the GCC-Conductor-Permission-Grant primitive, or to ignore the request. The confirm portion of this primitive is only local confirmation that the request was accepted by the local GCC Provider. No confirmation from the conductor is given directly. Table 7-45 shows the parameters and types of this primitive. Figure 7-36 shows the sequence of events when using this primitive.

TABLE  7-45/T.124

GCC-Conductor-Permission-Ask – Types of primitives and their parameters

| Parameter | Request | Indication | Confirm |
| --- | --- | --- | --- |
| Conference ID | M | M | M(=RQ) |
| Grant/Release Flag | M | M(=) | M(=) |
| Requesting Node ID |  | M |  |
| Result |  |  | M |

Conference ID: Identifier of the conference to which the primitive refers.

Give/Release Flag: This flag indicates whether the requester desires to be granted conducted-mode permission, or if the requester desires to release conducted-mode permission.

Requesting Node ID: The Node ID of the requesting node.

Result: An indication of whether the request was accepted or rejected, and if rejected, the reason why. It contains one of a list of possible results: successful, invalid conference, not in conducted mode.

FIGURE 7-36/T.124...[D39] = 8 CM

#### 7.5.1.7	GCC-Conductor-Permission-Grant

The primitive GCC-Conductor-Permission-Grant request may be issued by a Node Controller of the conducting node to grant or revoke conducted-mode permission from one or more nodes in a conference. The corresponding GCC-Conductor-Permission-Grant indication is broadcast to every node in the conference and indicates which nodes currently have conducted-mode permission, and also which have requested, but are still waiting for permission. The latter list may be given in the order that the conductor believes permission will ultimately be granted. The indication is given to the Node Controller as well as all enrolled Application Protocol Entities at each node. The order of multiple GCC-Conductor-Permission-Grant indications represents the actual order of the requests from the conductor – that is, the most recently received indication applies. Conducted-mode permission is typically given in response to a GCC-Conductor-Permission-Ask, but may also be given unsollicited by the conductor. The conducted node, itself, is assumed to have conducted-mode permission whether or not it is explicitly listed in the list of nodes granted permission. When a conference first becomes conducted it is to be assumed that no nodes have permission – this is true even if the conference had previously been in conducted mode. If the conductorship changes hands via a successful GCC-Conductor-Give operation, the state of permission are left as they were last broadcast by the original conductor. When a new node joins a conference, the node shall assume that it has no permission. In this case, the Node Controller at the conducting node may re-broadcast the permission list by re-issuing a CC-Conductor-Permission-Grant request so that the new node is made aware of the permission status of other nodes in the conference. The confirm portion of this primitive is only local confirmation that the request was accepted by the local GCC Provider. Table 7-46 shows the parameters and types of this primitive. Figure 7-37 shows the sequence of events when using this primitive.

TABLE  7-46/T.124

GCC-Conductor-Permission-Grant – Types of primitives and their parameters

| Parameter | Request | Indication | Confirm |
| --- | --- | --- | --- |
| Conference ID | M | M | M(=RQ) |
| List of Nodes Granted Permission | M | M(=) |  |
| List of Nodes Waiting for Permission | O | O(=) |  |
| Permission Flag |  | M |  |
| Result |  |  | M |

Conference ID: Identifier of the conference to which the primitive refers.

List of Nodes Granted Permission: A list of Node IDs, one for each node which the conductor has granted conducted-mode permission. If this list is empty, then no nodes have conducted-mode permission.

List of Nodes Waiting for Permission: An ordered list of Node IDs, one for each node which is being considered by the conductor to receive conducted-mode permission, but has not yet received it. The conductor may order the list in the order in which it is expected that permission will be granted. If so, the first item in the list is to be considered the next node likely to receive permission.

Permission Flag: This flag indicates whether or not the local node is present on the list of nodes granted permission. This is primarily to allow Application Protocol Entities to easily determine their mode of operation without requiring them to search through the returned lists.

Result: An indication of whether the request was accepted or rejected, and if rejected, the reason why. It contains one of a list of possible results: successful, invalid conference, not conductor.

FIGURE 7-37/T.124...[D40] = 8.5 CM

## 7.6	Miscellaneous functions

### 7.6.1	Description of abstract services

The following is a list of the primitives defined in this subclause and a brief summary of the function of each:

•	GCC-Conference-Time-Remaining – Allows the Node Controller at the conference convener to notify all nodes that a timed conference is scheduled to end at a particular time.

•	GCC-Conference-Time-Inquire – Allows the Node Controller at any node to find out how much time is remaining in a timed conference.

•	GCC-Conference-Extend – Allows the Node Controller at any node to request that the convener extend a timed conference beyond its allocated time duration.

•	GCC-Conference-Assistance – Used to request some unspecified form of assistance from a conference operator.

•	GCC-Text-Message – Used to send an arbitrary text message to a set of other nodes for display to the user or users at those nodes.

1)	Presently at the stage of draft.